3GPP TSG-RAN WG4 Meeting#117                                R4-2521751

Dallas, USA, 17th – 21st, Nov. 2025

Agenda item:	8.10

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on 6G Sensing study

Document for:	Approval

Introduction

In RAN#108 meeting, a new SID on Study on 6G Radio [1] was approved. In this contribution, we would like to share some initial understandings on 6GR ISAC study.

- Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914] - PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4] - Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1] - Aspects of integration with communication services [RAN1] - higher layer procedures and protocol aspects [RAN2] - RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4] Note: RAN1 identify detailed requirements, if triggered by TSG RAN RAN4#116bis: Agreement: Before April 2026, the agenda for sensing will be kept. RAN4 discussion will primarily focus on the following aspect - Sensing related regulatory status and requirement survey - Potential architecture consideration based on the RAN plenary use case study - views sharing on the potential RAN4 scope based on the identified use cases out of RAN plenary study - Identify less RAN1 design dependent RAN4 aspects if any

Discussion

## 2.1. Use case of sensing

In the last RAN-P meeting, there were some discussions on 6G sensing service and RAN reached the following consensus which is captured in TR 38.914. From RAN4 perspective, we propose to follow the RAN-P agreement and prioritize one or two use case in RAN4 to trigger the initial technical discussions.

The 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs.

Proposal 1: Propose RAN4 to start the discussion on the following use case (UAVs, human, vehicles and AGVs) and prioritize one or two use cases (e.g. UAV) to trigger the initial technical discussions.

## 2.2. Sensing mode

In the last RAN4 meeting, there were some suggestions proposing the following sensing mode, however from the workload perspective, it would be better to prioritize one or two sensing mode similar as the workload management for use case.

TRP monostatic, UE monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-TRP bistatic, and UE-UE bistatic and multistatic

For TRP monostatic sensing mode, this has already been included in 5GA and RAN1 has already started the relevant evaluation for this sensing mode for UAV case, therefore we also think that this should be supported in 6G day 1.

In addition, UE-TRP bistatic method might be also one appropriate scenario with UE transmitting sensing signal towards the sensing target and TRP/BS could fully utilize its large antenna array to conduct the relevant angle estimation. If this kind of sensing measurement is conducted from UE side, due to the omini antenna assumption and limited number of Rx chains, this might be not feasible to achieve the targeted measurement accuracy e.g. from horizontal and vertical relevant positioning accuracy at least.

Proposal 2: Propose RAN4 to prioritize the following two sensing mode TRP monostatic and UE-TRP bistatic in RAN4.

## 2.3.Basic operation mechanism for ISAC deployment

### 2.3.1. General

In the following section, we would like to share some initial understanding on the design principle for UAV sensing detection based on the monostatic sensing mode. For UE-TRP bistatic method could be further discussed later on.

### 2.3.2. Basic algorithm: distance estimation

Here we use UAV sensing detection as an example in Figure 2.3.2-1 to clarify the distance estimation theory from our understanding. According to the channel estimation based on reference signals of ISAC operation designed in RAN1, the timing delay profile for distance estimation can be obtained by transforming the estimation results in frequency domain to timing domain by IFFT operation to get channel impulse response CIR (similar to channel truncation in channel estimation function.) .

Figure 2.3.2-1. Illustration of basic principle of distance estimation for ISAC operation

If the transmission bandwidth is assumed as, the finest granularityin time domain can be expressed as follows:

As shown in the figure 2.3.2-1, the distance estimation for targeted UAV can be calculated as follows:

If we want to make sure that two targeted devices could be distinguished in the distance domain, then the the timing difference from the receiver side should be at least equal to or larger than the finest timing granularity as follows.

According to the above derivation, then the ideal finest distance estimation granularity can be calculated as follows:

For example, the ideal finest distance estimation granularity according to different sensing BW is calculated as shown in the following Table 2.3.2-1 :

Table 2.3.2-1: the ideal finest distance estimation granularity according to different sensing BW

| BW [MHz] | Ideal finest distance estimation granularity [m] | Remark |
| --- | --- | --- |
| 50MHz | 3.0 | Might be worse than Horizontal localization accuracy: 2m for RAN SI for IMT-2030 |
| 100MHz | 1.5 | FR1 high bands |
| 200MHz | 0.75 | FR1 high bands |
| 400MHz | 0.375 | FR1 high bands and FR2-1 |
| 800MHz | 0.1875 | FR1 high bands and FR2-1 |

Observation 1: In order to meet the RAN Sensing KPI for IMT-2030, the measurement bandwidth for sensing signal should be at least larger than 100MHz;

Observation 2: In order to meet more than 100MHz sensing signal bandwidth, FR1 high TDD bands and FR2-1 bands are more suitable for ISAC deployment;

In addition, in RAN1#122 meeting, RAN1 has already reached the following consensus for 5GA sensing evaluation, from our understanding, RAN4 could also follow these assumption to conduct the relevant coexistence study in RAN4.

| Parameters | Assumptions |
| --- | --- |
| Scenario | UMa-AV, Optional RMa-AV |
| Carrier frequency | Mandatory: one value either 4 GHz or 4.9GHz. optional for FR1: 6 GHz [optional for FR2: 28 GHz] |
| System bandwidth | 100 MHz |
| Numerology | SCS = 30kHz |
| BS Layout | Hexagonal grid, 7 macro sites, 3 sectors per site. See Note1 3 sectors with 30, 150, 270 degrees |
| Inter-BS (2D) distance | Uma-AV: 500m, optional 1000m, RMa-AV: 1732m |
| BS antenna height | 25m for UMa-AV, 35m for RMa-AV |
| Wrap-round | No wrap-round |

Proposal 3: For 6G ISAC study, consider FR1 TDD with available spectrum wider than 100MHz at least, FR2-1 bands and FFS for 7-24GHz;

In signal processing theory, then the achievable timing estimation performance according to Cramér-Rao Lower Bound, CRLB theory can be calculated as follows:

Then, the corresponding achievable distance estimation performance can be calculated as follows:

### 2.3.3. Basic algorithm: Angle estimation

Here we use UAV sensing detection as an example in Figure 2.3.3-1 to clarify the azimuth/zenith angle estimation theory from our understanding. The angle arrival direction could be estimated by the phase difference among different sub-arrays or antenna elements of each digital chain from frequency domain similar to other legacy timing estimation method.

Figure 2.3.3-1.Illustration of basic principle of azimuth and Zenith angle estimation for ISAC operation

According to the antenna theory, the finest angle estimation granularity could be calculated by basic 3dB beamdwidth as follows:

In signal processing theory, then the achievable angle estimation performance according to Cramér-Rao Lower Bound can be calculated as follows:

### 2.3.4. Basic algorithm: velocity estimation

Here we use UAV sensing detection as an example in Figure 2.3.4-1 to clarify the velocity estimation theory from our understanding. According to the channel estimation results and its conjugate of reference signal of ISAC operation designed in RAN1, then we could get the following conjugate estimation results:

Figure 2.3.4-1. Illustration of basic principle of velocity estimation for ISAC operation

In addition, considering estimated doppler shift reflected in the phase domain and also the accurate phase detection threshold without phase estimation ambiguity, then we could get the velocity estimation results as follows:

In signal processing theory, then the achievable velocity estimation performance according to Cramér-Rao Lower Bound, CRLB theory can be calculated as follows:

### 2.3.5. Basic sensing mode for ISAC operation

For the UAV use case as shown in the following figures, there are two potential modes including the mono-static and bi-static mode.

As shown in Figure 2.3.5-1, for mono-static deployment, ISAC reference signal is transmitted and received by the same BS which is also the basic assumption for 5G-A assumption (e.g. without special optimization for network signalling for bi-static use case).

Figure 2.3.5-1. Monostatic mode for ISAC operation

As shown in Figure 2.3.5-2, for bi-static deployment, ISAC reference signal is transmitted and received by different BSs. For this use case, the timing requirement across different BSs will have the direct impacts on the measurement performance. The existing 3us timing error between different BSs will make the targeted measurement accuracy not possible to be met.

Figure 2.3.5-2. Bi-static mode for ISAC operation [existing timing accuracy requirement as 3us]

### 2.3.6. Basic mechanism for ISAC operation

For the mono-static deployment, as shown in Figure 2.3.6-1/2, there are two potential basic mechanism for ISAC operation from our understanding.

Option 1: TDM&Full array&High Tx power.

Option 2: Full duplex&half array&Low Tx power

In last RAN1#122bis meeting, for 5GA sensing discussion, there are the following agreement as highlighted in yellow to indicate that both option 1 and option 2 can be studied for sensing evaluation.

Agreement When Tx/Rx operates simultaneously (baseline case), the following parameters are used for the evaluation on NR ISAC: Antenna isolation; - Option 1: 65dB - Option 2: 80dB. Maximum BS Tx power; - Option 1: 37dBm - Option 2: 52dBm The above options are calculated with BS_maxpower = BS Rx saturation power + antenna isolation by assuming the BS Rx saturation power = -28dBm and the antenna isolation = 65dB and 80dB, respectively. - Optionally: companies should report the maximum BS Tx power when it is assumed that Tx and Rx don’t operate simultaneously. Companies should report how the maximum BS Tx power is derived. - Agreement When Tx/Rx operates simultaneously, the following evaluation parameters are agreed for the evaluation on NR ISAC. Self-interference; The residual leakage interference/noise is modelled e.g. by additional additive white Gaussian noise, -94+X dBm in 100 MHz, X is up to company report. Companies to provide details on their modelling. Note: X = -Infinity corresponds to not modelling self-interference.

For the option 1, due to non-simultaneous transmission and reception in TDM, the corresponding coverage is expected to be larger. The drawback for the option 1 is the blind detection area due to the transmission period and switching period.

Figure 2.3.6-1. Illustration of Pulse signal: TDM&Full array&High Tx power.

For the option 2, due to simultaneous transmission and reception, the areas close to the ISAC BS can be also detected without any blind detection regions compared with option 1. However self interference and BS2BS co-channel interference will also result in lower power transmission and smaller coverage compared with option 1.

Figure 2.3.6-2. Illustration of Pulse signal:Full duplex&half array&Low Tx power [similar to Rel-19 SBFD digital IC calibration process]

Figure 2.3.6-3. Comparison between pulse signal and continuous-wave signal [3]

Proposal 4: For the 6G ISAC study, consider TDM based sensing operation and full duplex based sensing operation.

### 2.3.7. Basic physical layer processing for UAC use case of ISAC operation

As shown in Figure 2.3.7-1, the sensing algorithm used in the evaluation includes the following steps:

- Frequency-domain Channel Estimation

Use the received sensing signal to estimate frequency-domain channel.

- Range-doppler (RD) Map Construction

After obtaining multiple channel estimations in different time, apply an IFFT to the channel coefficients of the sub-carriers and an FFT to different channel estimations, and the range-Doppler (RD) map is obtained.

- CFAR Detection

CFAR is used to detect the target in the RD map, and the range index and Doppler index of each target can be obtained. Note that a coarse clustering is used to combine the peaks whose the range index and Doppler index are both close.

- Angle Estimation

For each detected target, the Azimuth-of-Arrival (AoA) and Zenith-of-Arrival (ZoA) are further estimated.

- Position and velocity determination

Determine the coordinate (x, y, z) and the radical velocity according to CFAR detection results and angle estimations.

- Multi-node Collaboration

For each target, combine the points (x, y, z) provided by different TX-RX pairs into a single point, and combine the radical velocities into a velocity vector only when the number of TX-RX pairs is not less than three.

Figure 2.3.7-1. Algorithm procedure to process the received sensing signal

- Measurement report

In the whole pipeline to generate sensing results with different levels, we summarize all procedures as four main steps, which is related to four kinds of measurement data, from specific to abstract. The whole pipeline is shown as follows:

Figure 2.3.7-2. Pipeline to derive measurement data with different levels

Once receiving the echo signal from the Rx side, the processor at Rx could record the whole echo signal from multiple channels. The raw echo signal would be too dense to be transferred by any network entity. The general echo signal recorded at Rx could be visualized as follows:

Figure 2.3.7-3 Echo signal from multiple channels

Step 1: Channel estimation (Channel Impulse Response (CIR) I/Q sample)

Then the Rx would conduct the channel estimation to derive the channel impulse response (CIR) I/Q sample, based on the recorded echo signal and the known reference signal. The peak in CIR I/Q sample profile would be used to indicate the delay/distance information of each multi-path, which is potentially related to sensing results.

Figure 2.3.7-4 CIR I/Q profiles from multiple channels

The raw data size of CIR I/Q samples would also be very large. As an example, we have following configurations for sensing RS:

- Number of symbols used for sensing:

- Sub-carrier space:

- Bandwidth:

- Number of TRPs:

- Number of channels:

- Number of trajectory points:

If the single precision floating point number is used to describe the CIR I/Q samples, the data size would be about 480 Gbits, based on the following calculation:

If the double precision floating point number is used to describe the CIR I/Q samples, the data size would be about 960 Gbits, based on the following calculation:

It could be seen that the data size of CIR I/Q samples leads to the situation that the delivery of such amount of data would cost too much resources for communication and calculation for sensing operation.

Step 2: Accumulating and FFT transforming (RD profile)

Given the CIR I/Q samples with multiple channels, the processor could accumulate the CIR profile on Doppler dimension and implement FFT operation on Doppler dimension to derive the range-Doppler (RD) profile. The accumulation and FFT operation could also be implemented on both Doppler dimension and spatial dimension, to derive the range-Doppler-angle (RDA) profile. The step1 only digs out the sparsity on time domain, and the step2 could derive the sparsity in Doppler domain and spatial domain. Then any peak in such RD profile (or RDA profile) would indicate the delay and velocity (and angle) of a potential sensing target.

The general RD profile could be visualized as following figure:

Figure 2.3.7-5. RD profile of one channel

The RD profiles would have comprehensive information about sensing target, including the position, velocity, RSRP, micro-Doppler features, etc., which is the most useful and important tool to analyse the status of sensing target. However, if the data size of RD profile would be same as the CIR I/Q profile, with the same configuration as step1, the data size of RD profile stored as single precision floating point number would be 480 Gbits, and the data size of RD profile stored as double precision floating point number would be 960 Gbits.

Step 3: Target detection (Point cloud)

It could be seen that most of points in the RD profile carries with very limited power, and only small number of points in the point cloud would indicate the sensing target. Thus the detection algorithm could be implemented on the RD profile, such as spectrum peak searching, or constant false alarm rate searching (CFAR), to obtain the point cloud which could consist of the TOA, AOA, ZOA, Doppler and other information of potential sensing target. One set of detected points cloud could be visualized in the following figure:

Figure 2.3.7-6. A cluster of points cloud

The data size of points cloud could be simplified a lot. For example, if each point in points cloud would consist of TOA, AOA, ZOA, and Doppler frequency, which have data dimension of , then the data size would be decided by the total number of detected points in points cloud, as shown in following calculation.

When 1000 points are detected:

- If single precision floating point number is used to described the point cloud, the data size of point cloud would be 14.6 Mbits

- If double precision floating point number is used to described the point cloud, the data size of point cloud would be 29.3 Mbits

When 10000 points are detected:

- If single precision floating point number is used to described the point cloud, the data size of point cloud would be 146.5 Mbits

- If double precision floating point number is used to described the point cloud, the data size of point cloud would be 293.0 Mbits

Step 4: Tracking and Deduplication (X-Y-Z coordinates)

Each point in points cloud could be seen as single measurement at one time, which could have limited performance and duplicating information. For example, multiple points could be used to describe one same sensing target because of the high resolution of sensing system, and then one set of points cloud could be clustered as one single trajectory. What’s more, sensing results from different Rxs could be merged together. All the deduplication operations could decrease the data size of measurement report a lot.

Figure 2.4.6-6 A list of trajectory point

We suppose that each trajectory point would have four dimensions of statement: X-Y-Z coordinates, and velocity. If the single precision floating point number is used, the data size of the trajectory point would be . If the double precision floating point number is used, the data size of the trajectory point would be .

Actually both the sensing results from step3 and step4 could be acceptable considering the data size. The sensing results from step3 or step4 could also be merged together with the following format:

- 3D coordinates (X-Y-Z location)

- 3D velocity (X-Y-Z speed)

- Orientation angles

- RSRP/RSRPP

- TimeStamp

- Uncertainty

- ......

Both points derived from step 3 and step 4 could include part or all of the above information elements. The difference between the measurement report from step 3 and step 4 would only be the number of points to be reported and also ID linking multiple points to form the trajectory of an object.

## 2.4. Architecture and RF feasibility for ISAC study

In the last RAN4#116bis meeting, we reached the following consensus on potential architecture based on RAN-P use case study. Indeed, the architecture could be interpreted in different way in different working group. From RAN study perspective, the architecture might refer to the following issues e.g. whether to place sensing function SF in RAN or core network, data privacy with localized data storage and processing or not etc. However all of these aspects are more relevant with RAN3 architecture. From RAN4 perspective, we propose to focus on RF architecture of sensing for different use case.

- Potential architecture consideration based on the RAN plenary use case study

Proposal 5: For the architecture study for 6G sensing in RAN4, propose to focus on the RF architecture of sensing node for different use case.

In the following section, we would like to share our initial views on the potential RF architecture for option 1 and option 2 as mentioned above.

For the RF hardware assumption of option 1 as shown in Figure 2.4-1, although it is expected to work as TDD BS, however it will go to receive the backscattering signal from the sensing target or potential co-channel interference from other sensing BS transmitting signals, therefore in RAN4, we also need to further consider the RF feasibility evaluation from the receiver’s in-channel linearity and also out of carrier blocking performance.

Figure 2.4-1. Illustration of RF architecture for pulse wave signal [similar to the legacy TDD BS]

Proposal 6: For the 6G ISAC BS with option 1 TDM operation, consider the RF feasibility evaluation from the receiver’s in-channel linearity and also out of carrier blocking performance.

For the generic baseband signal generation of option 1 as shown in Figure 2.4-2, it is somehow similar to LP-WUS signal generation to create the short pulse signal to minimize the blind detection regions.

Figure 2.4-2 Generation of pulse signal by traditional DFT-s-OFDM waveform [3]

For the RF hardware assumption of option 2 as shown in Figure 2.4-3, this is similar to SBFD RF architecture as specified in Rel-19. To enable this sensing feature, the Tx/Rx panels are separated to ensure the large isolation to minimize the receiver impacts. In addition, the sensing operation is full carrier transmission instead of DL sub-band transmission which is one significant difference compared with Rel-19 SBFD operation.

Figure 2.4-3. Illustration of RF architecture for contiguous wave signal [similar to Rel-19 SBFD digital IC calibration process]

Proposal 7: For the 6G ISAC BS with option 2 FDM operation, consider the RF feasibility including fully overlapping transmission/reception between DL and UL.

## 2.5. Coexistence study for ISAC study

### 2.5.1.Coexistence case for ISAC study

Since the ISAC system is designed as new RAT with different measurement metric compared with the legacy wireless communication system, in general, the appropriate coexistence study is needed to guarantee the coexistence with the legacy system and new ISAC system.

As shown in the following Figure 2.5-1/2 for ISAC BS with option 1/2 implementation, the coexistence studies between ISAC system and legacy system should be considered in 6G SI phase.

In addition, ISAC system coexisting with ISAC system in the adjacent channel should be also considered similar to Rel-20 SBFD coexisting with SBFD.

Figure 2.5-1. co-existence case for ISAC BS with option 1 assumption [similar to SBFD co-existence study]

Figure 2.5-2. co-existence case for ISAC BS with option 2 assumption [similar to SBFD co-existence study]

Proposal 8: for the 6G ISAC BS, the coexistence studies between ISAC system(s) and the legacy system need to be conducted to figure out the appropriate RF requirements.

In addition, for the different use case of ISAC as mentioned in section 2.1, the deployment is expected to be different. In the following table 2.5-1, we want to highlight the potential ISAC deployment to make the evaluation case more realistic and closer to the deployment.

Table 2.5. 1-1. the potential deployment for different ISAC use case

| Use case | Deployment | Remarks |
| --- | --- | --- |
| UAV detection and tracking | WA deployment | 1st priority |
| Pedestrian detection and tracking | WA deployment |  |
| Vehicle detection and tracking | WA deployment |  |
| AGV detection and tracking | Indoor factory scenario |  |
| Ship detection and tracking | WA deployment |  |
| Infrastructure collapse monitoring | WA deployment |  |
| Micro-deformation detection | WA deployment |  |

Proposal 9: for the different use case of ISAC deployment, propose to consider differentiating the deployment to make the evaluation more realistic and closer to the deployment.

### 2.5.2.Coexistence case for ISAC study

In the following table 2.5.2-1, we provide some initial understanding on the potential coexistence cases for ISAC study in RAN4.

Table 2.5. 2-1. the potential deployment for different ISAC use case

| Scenario | FR | Aggressor | Victim |
| --- | --- | --- | --- |
| 1 | FR1 4GHz | Urban Macro | Urban Macro |
| 2 | FR1 4GHz | Indoor factory | Indoor factory |
| 3 | FR2-1 28GHz | Urban Macro | Urban Macro |
| 4 | FR2-1 28GHz | Indoor factory | Indoor factory |
| 5 | Around 7GHz | Urban Macro | Urban Macro |
| 6 | Around 7GHz | Indoor factory | Indoor factory |
| Note 1: More scenarios could be added based on the consensus in RAN1/4. |  |  |  |

In the following table 2.5.2-2/3, we provide some initial analysis on adjacent channel interference for ISAC study from both mono-static and bi-static implementation.

Table 2.5.2-2: Adjacent channel co-existence cases for mono-static deployment

| Case | Aggressor | Aggressor baseline | Victim | Remarks |
| --- | --- | --- | --- | --- |
| 1 | ISAC | TDD DL | TDD DL | Option 1: Option 2: |
| 2 | TDD DL | No aggressor network | ISAC |  |
| 3 | ISAC | No aggressor network | ISAC |  |
| NOTE: assume the ISAC to be deployed in the DL symbols to ensure the coverage as 1st priority |  |  |  |  |

Table 2.5.2-3: Adjacent channel co-existence cases for bi-static deployment

| Case | Aggressor | Aggressor baseline | Victim | Remarks |
| --- | --- | --- | --- | --- |
| 4 | ISAC | TDD DL | TDD DL | Option 1: Option 2: |
| 5 | TDD DL | No aggressor network | ISAC |  |
| 6 | ISAC | No aggressor network | ISAC |  |
| NOTE: assume the ISAC to be deployed in the DL symbols to ensure the coverage as 1st priority |  |  |  |  |

### 2.5.3.Simulation assumption for ISAC coexistence study

For coexistence study between ISAC and NR system, we

In the previous RAN1#122 and RAN1#122bis meeting, RAN1 has already reached the following agreement on 5G-A sensing evaluation assumption. From RAN4 sensing coexistence evaluation, we propose to follow the agreement reached in 5G-A sensing to conduct the initial coexistence study.

Agreement The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. Scenario; UMa-AV, Optional RMa-AV. Carrier frequency; Mandatory: one value either 4 GHz or 4.9GHz. optional for FR1: 6 GHz [optional for FR2: 28 GHz]. System bandwidth; 100 MHz. Numerology; SCS = 30kHz. BS Layout; Hexagonal grid, 7 macro sites, 3 sectors per site. See Note1 3 sectors with 30, 150, 270 degrees. Inter-BS (2D) distance; Uma-AV: 500m, optional 1000m, RMa-AV: 1732m. BS antenna height; 25m for UMa-AV, 35m for RMa-AV. Wrap-round; No wrap-round. Note1: target(s) are dropped only in the center site, and inter-BS interference is not modelled. Agreement The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. Target type; UAV with small size (0.3m x 0.4m x 0.2m). Target distribution when target(s) are dropped; N = 5 targets per sector in the center site. Optional: N is uniformly distributed from 1 to 10. Horizontal plane: uniformly distributed in a sector Vertical plane: Uniformly distributed between 25m and 300m, optionally distributed between 1.5m and 300m. Mobility; horizontal speed: uniformly distributed between 0 and 180km/h vertical speed: 0km/h. Note: N=0 will be discussed in a later proposal in relation to false alarm. Agreement The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. Minimum BS-target 3D distance; 10 m. Minimum target-target (3D) distance; 10 m. Target outdoor/indoor proportion; 100% outdoor. LOS/NLOS; LOS and NLOS. Orientation; Random in horizontal domain. Agreement The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. BS antenna configuration; For 4GHz, 4.9GHz: - Option 1 as optional - Tx: (12,16, 2,1,1;2,16) - Rx: (12,16,2,1,1;2,16) - Option 2 - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8); For 6GHz: - Option 1 as optional - Tx: (16,16, 2,1,1;4,16) - Rx: (16,16,2,1,1;4,16) - Option 2: - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8). Mandatory: dH,dV = (0.5, 0.8)λ, +45°/-45° polarization Optional: dH,dV = (0.5, 0.5)λ, +45°/-45° polarization. - Note: Tx antenna elements and Rx antenna elements are operating simultaneously Agreement The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. BS antenna radiation pattern; Table 9 in Report ITU-R M.2412. BS antenna mechanic tilt (downtilt angle without electrical tilt); 90° in GCS (pointing to horizontal direction). BS antenna electrical tilt (downtilt angle after mechanic and electrical tilt); Option 1: no electrical tilt Option 2: 102° in GCS. Polarized antenna model; Model-2 in clause 7.3.2 in TR 38.901. Agreement The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. gNB-target link; gNB-aerial UE of UMa/RMa parameters in 36.777 with Alternative 3. Concatenation of TX-target and target-RX links; Up to company choice between two options for concatenation defined in Step 9 in section 7.9.4.1, TR 38.901. The power threshold for path dropping after concatenation for target channel; -25dB and -40dB are respectively used for the two options for concatenation. Agreement Up to company to model low power cluster in the evaluation of NR ISAC Agreement When Tx/Rx operates simultaneously (baseline case), the following parameters are used for the evaluation on NR ISAC: Antenna isolation; - Option 1: 65dB - Option 2: 80dB. Maximum BS Tx power; - Option 1: 37dBm - Option 2: 52dBm The above options are calculated with BS_maxpower = BS Rx saturation power + antenna isolation by assuming the BS Rx saturation power = -28dBm and the antenna isolation = 65dB and 80dB, respectively. - Optionally: companies should report the maximum BS Tx power when it is assumed that Tx and Rx don’t operate simultaneously. Companies should report how the maximum BS Tx power is derived. - Agreement When Tx/Rx operates simultaneously, the following evaluation parameters are agreed for the evaluation on NR ISAC. Self-interference; The residual leakage interference/noise is modelled e.g. by additional additive white Gaussian noise, -94+X dBm in 100 MHz, X is up to company report. Companies to provide details on their modelling. Note: X = -Infinity corresponds to not modelling self-interference. Agreement The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. Co-site inter-sector interference; Not modelled. Adjacent channel interference; Not modelled. BS receiver noise figure; 5dB.

Proposal 10: for the simulation assumption for UAV sensing evaluation in RAN4, propose to follow the agreement of simulation assumption reached in RAN1.

Proposal 11: for the simulation assumption for TN BS, propose to follow the simulation assumption as captured in TR 38.858 for TDD network.

In addition, in the last RAN4 meeting, there were some proposal mentioning the RCS for sensing target. The modelling for RCS modelling is captured in clause 7.9.2.1 of TR 38.901. From RAN4 coexistence perspective, we could only consider the RCS in the large scale pathloss model as shown in the following table with two target path included.

| Parameters | Values |
| --- | --- |
| Coupling loss for target channel | power scaling factor (pathloss, shadow fading, and RCS component σM included): LTX−SPST−RX=PLdBd1+PLdBd2+10lgc24πf2−10lgσRCS,A+SFdB,1+SFdB,2 |
| STX/SRX selection | Best N = 4 STX-SRX pairs to be selected for the target. see note 1 |
| Metrics | Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the coupling loss between Tx and one reference point) see note 2 |
| NOTE 1: Based on the STX-SRX pairs with the smallest power scaling factor of the target channel. NOTE 2: CDFs can be separately generated for target channel, background channel. |  |

Proposal 12: for the RCS of sensing target, propose to consider the RCS model as captured in clause 7.9.2.1 of TR 38.901 and simplify the RCS in the large scale pathloss model if necessary.

### 2.5.4. Metrics for ISAC coexistence study

Similar to other legacy coexistence study in the past, in order to quantify the performance impacts, we need to discuss the acceptable performance degradation for both ISAC and legacy system or 6GR communication system due to the adjacent channel leakage.

Proposal 13: For the UAV use case of 6G ISAC study at least, propose to consider the distance, angle estimation and velocity estimation as the basic metric for the study in RAN4.

Proposal 14: For the UAV use case of 6G ISAC study at least, propose to consider the distance accuracy impacts, angle estimation accuracy impacts and velocity estimation accuracy impacts to quantify the performance impacts from both co-channel and adjacent channel.

Proposal 15: for the 6G ISAC coexistence study in RAN4, for 6GR performance metric, propose to use the legacy throughput loss as basic metric.

## 2.6. System parameters and RF requirement impacts for ISAC study

In the following table 2.6-1, we provide some initial view on the 6G ISAC BS requirement comparing to 6GR BS RF requirements.

Table 2.6-1: general view on the relationship between 6GR BS and 6G IASC requirement

|  | 6GR BS | 6G ISAC |
| --- | --- | --- |
| carrier bandwidth | Yes | The same as 6GR BS |
| Numerology | Yes | The same as 6GR BS |
| Channel raster | Yes | ## The same as 6GR BS |
| Sync raster | Yes | ## N/A |
| Base station output power | Yes | ## The same as 6GR BS |
| Output power dynamics | Yes | N/A |
| Transmit ON/OFF power | Yes | ## The same as 6GR BS |
| Transmitted signal quality | Yes | ## N/A |
| OBW | Yes | ## The same as 6GR BS |
| Tx intermodulation | Yes | ## The same as 6GR BS |
| ACLR | Yes | ## Depends on coexistence study |
| Operating band unwanted emissions | Yes | Depends on coexistence study |
| Transmitter spurious emissions | Yes | The same as 6GR BS |
| Rx part |  |  |
| REFSENS | Yes | E.g.distance, angle estimation and velocity estimation for UAV |
| dynamic range | Yes | FFS |
| ACS | Yes | Depends on coexistence study |
| blocking | Yes | Depends on coexistence study |
| NBB | Yes | FFS |
| RX IMD | Yes | Depends on coexistence study |
| RX NB IMD | Yes | FFS |
| RX spurious emission | Yes | The same as 6GR BS |
| RX ICS | Yes | FFS |

## 2.7. RRM requirement impacts for ISAC study

Similar to Rel-17 positioning WID, for 6G ISAC, it might be also necessary to define the certain RRM requirement at BS side to verify the sensing accuracy requirement, however these details is highly dependent on RAN1’s physical layer and framework design. We propose to postpone the discussion until there are sufficient progress made in RAN1/RAN2/RAN3.

Figure 2.7-1. Illustration of Rel-17 positioning framework

Proposal 16: for the 6G ISAC RRM requirement, propose to postpone the discussion until there are sufficient progress made in RAN1/RAN2/RAN3.

## 2.8. Testing impacts for ISAC study

For ISAC BS, since its conformance testing is expected to be different from the legacy 5G conformance testing due to the moving testing target, therefore we propose to discuss the applicable conformance testing for 6G ISAC in the OTA chamber. In the following figure 2.8.1, we provide some initial understanding how this test setup should be built upon.

Figure 2.8-1.Illustration of OTA conformance testing for ISAC BS

Proposal 17: for the 6G ISAC conformance testing, propose to discuss the OTA test setup for conformance testing of moving sensing target.

Conclusions

In this contribution, we shared some initial views on the 6GR ISAC study and proposals are made as follows:

Proposal 1: Propose RAN4 to start the discussion on the following use case (UAVs, human, vehicles and AGVs) and prioritize one or two use cases (e.g. UAV) to trigger the initial technical discussions.

Proposal 2: Propose RAN4 to prioritize the following two sensing mode TRP monostatic and UE-TRP bistatic in RAN4.

Observation 1: In order to meet the RAN Sensing KPI for IMT-2030, the measurement bandwidth for sensing signal should be at least larger than 100MHz;

Observation 2: In order to meet more than 100MHz sensing signal bandwidth, FR1 high TDD bands and FR2-1 bands are more suitable for ISAC deployment;

Proposal 3: For 6G ISAC study, consider FR1 TDD with available spectrum wider than 100MHz at least, FR2-1 bands and FFS for 7-24GHz;

Proposal 4: For the 6G ISAC study, consider TDM based sensing operation and full duplex based sensing operation.

Proposal 5: For the architecture study for 6G sensing in RAN4, propose to focus on the RF architecture of sensing node for different use case.

Proposal 6: For the 6G ISAC BS with option 1 TDM operation, consider the RF feasibility evaluation from the receiver’s in-channel linearity and also out of carrier blocking performance.

Proposal 7: For the 6G ISAC BS with option 2 FDM operation, consider the RF feasibility including fully overlapping transmission/reception between DL and UL.

Proposal 8: for the 6G ISAC BS, the coexistence studies between ISAC system(s) and the legacy system need to be conducted to figure out the appropriate RF requirements.

Proposal 9: for the different use case of ISAC deployment, propose to consider differentiating the deployment to make the evaluation more realistic and closer to the deployment.

Proposal 10: for the simulation assumption for UAV sensing evaluation in RAN4, propose to follow the agreement of simulation assumption reached in RAN1.

Proposal 11: for the simulation assumption for TN BS, propose to follow the simulation assumption as captured in TR 38.858 for TDD network.

Proposal 12: for the RCS of sensing target, propose to consider the RCS model as captured in clause 7.9.2.1 of TR 38.901 and simplify the RCS in the large scale pathloss model if necessary.

Proposal 13: For the UAV use case of 6G ISAC study at least, propose to consider the distance, angle estimation and velocity estimation as the basic metric for the study in RAN4.

Proposal 14: For the UAV use case of 6G ISAC study at least, propose to consider the distance accuracy impacts, angle estimation accuracy impacts and velocity estimation accuracy impacts to quantify the performance impacts from both co-channel and adjacent channel.

Proposal 15: for the 6G ISAC coexistence study in RAN4, for 6GR performance metric, propose to use the legacy throughput loss as basic metric.

Proposal 16: for the 6G ISAC RRM requirement, propose to postpone the discussion until there are sufficient progress made in RAN1/RAN2/RAN3.

Proposal 17: for the 6G ISAC conformance testing, propose to discuss the OTA test setup for conformance testing of moving sensing target.

References

- RP-251881,New SID: Study on 6G Radio, Approved.

- RP-251742, Study on 6G scenarios and requirements - ISAC, ZTE,

- R1-2506131,Discussion on 5G-A ISAC evaluation,ZTE, Noted.

- RP-252024 RAN KPIs for IMT-2030