3GPP TSG-RAN WG4 Meeting#117                                R4-2521754

Dallas, USA, 17-21 Nov, 2025

Agenda item:	7.11.3

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on Rel-20 A-IoT coexistence study

Document for:	Approval

Introduction

In the RAN#108 meeting, a New SID on Rel-20 enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices [1] was approved. In the last RAN4 meeting, there were some discussions on coexistence assumption for A-IoT outdoor scenario. In this contribution, we would like to share our further understanding on the open issues.

- Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. This objective includes: RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. - While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range - Maximum transmit power of -20 dBm and -10 dBm for device 2b Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources. - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations. - Define link budget calculation for coverage. RAN4-led - Study coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios. - Use band n8 as an example band. NOTE: Strive to minimize evaluation cases in RAN1 and RAN4.

Discussion

- 2.1. NR UE emission assumption

- In the last RAN4 meeting, it was agreed to use the NR UE in-band emission requirement to evaluate the in-band interference from NR UE towards A-IoT device D2R link, however there are some mismatch between IBE and D2R bandwidth from our understanding.

- NR UE IBE requirement is defined in RB level, which is 180kHz for the 900MHz case;

- A-IoT UE device bandwidth is agreed as 15kHz for coexistence study.

| NR UE Parameter | Values for calibration and formal simulation |
| --- | --- |
| UE TX power in dBm | -40 to 23 dBm |
| NR UE Antenna gain (dBi) | 0dBi |
| Height of UE antenna (m) | 1.5m |
| NR UE Tx emission（dB） (standalone) | Same with NR UE ACLR |
| NR UE Tx emission (dB) (inband) | Same with NR UE IBE |
| NR UE ACS（dB） | 33dB |
| NR UE Noise Figure（dB） | 9dB |
| Antenna configuration | Omni direction antenna |

There are basically two approaches to emulate the leakage from NR UE towards the D2R 15KHz for in-band deployment case.

- Option 1: NR UE IBE -10*log10(12)

- Option 2: using the following approach as NB-IoT in-band case

Figure 2.1-1. Leakage from LTE to NB-IOT for guard-band scenario

Figure 2.1-2. Leakage from LTE to NB-IOT for in-band scenario

Table 2.1-1. average LTE leakage to NB-IOT subcarrier

| Frequency domain | Leakage (dB) |
| --- | --- |
| No guard band, NB-IoT adjacent to LTE PRB. | -14.1 |

Table 2.1-2. LTE leakage to NB-IOT subcarrier

| Frequency domain | Leakage (dB) |
| --- | --- |
| No guard band, NB-IoT close to last LTE PRB. | -17.2 |

Proposal 1: Propose RAN4 to further discuss the following two options how to emulate the leakage from NR UE into D2R bandwidth as 15KHz.

- Option 1: NR UE IBE -10*log10(12)

- Option 2: using the following approach as NB-IoT in-band case (17.2dB for in-band case)

- 2.2. ACIR modelling

In the following Figures, we shared our understanding on the ACIR modelling between NR and A-IoT case.

If the aggressor bandwidth is larger than the victim bandwidth, then scaling factor is usually considered for the transmitter side. For receiver side, usually we only consider the pessimistic assumption without any scaling factor for ACS requirement. However considering the sampling bandwidth within DFE in the baseband might be two times of wanted signal for digital filtering, then it seems that such kind of scaling factor is also reasonable which is not used in the past.

Figure 2.2-1. NR DL interfering A-IoT DL

For the R2D interfering NR DL case, it is also fine to use to consider the two step ACLR modeling. It would be efficiently equal to 3.5 1st adjacent channel interference. For ACS side, since the sampling bandwidth might be 20MHz within DFE, the it should be okay to use 33dBc.

Figure 2.2-2. A-IoT DL interfering NR DL

For the NR UL interfering A-IoT D2R case, it is also fine to use to consider the two step ACLR modeling as proposed in the following figure 2.2-3, however for ACS requirement, whether baseband filter is 180kHz or 15kHz which is the same as D2R signal bandwidth, this might need a bit more discussion for it. So far, we still propose to use 15KHz as assumption to derive the scaling factor for 1st adjacent user.

Figure 2.2-3. NR UL interfering A-IoT UL

For A-IoT UL interfering NR UL case, if we consider the scaling factor as 10*log10(10/0.18/12) which is equal to 28dBc which is not reasonable at all (20/25-28.2dB ), we propose to start using 20/25dBc as staring point and further check the effective ACLR by single tone emission mask.

Figure 2.2-4. NR UL interfering A-IoT UL

Conclusions

In this contribution, we shared some initial views on Rel-20 A-IoT coexistence study and proposals are made as following:

Proposal 1: Propose RAN4 to further discuss the following two options how to emulate the leakage from NR UE into D2R bandwidth as 15KHz.

- Option 1: NR UE IBE -10*log10(12)

- Option 2: using the following approach as NB-IoT in-band case (17.2dB for in-band case)

- Proposal 2: Propose to consider the ACLR/ACS modelling in the following way:

References

- RP-251884, Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices, approved.

- R4-2515152, Way Forward for [116bis][321] Study of A-IoT for outdoor, LG