3GPP TSG-RAN WG4 Meeting #117	R4-2521767

# Dallas, USA, November 17th – 21st 2025

Source:	Ericsson

Title:	Discussion on Rel-19 NR SBFD demodulation requirements

Agenda item:	6.13.4

Document for:	Discussion

# Introduction

In RAN4#116-bis, companies can’t get agreement on whether introduce demodulation requirement for PUSCH with UL muting [1].

In this contribution, further analysis on FR2 interference modelling are delivered.

# Discussion

In previous meetings, only interference analysis for FR1 is focused. As for FR2, similar analysis based on link budget can also be applied.

If we take dense urban Macro 1 layer deployment configuration in TR 38.858 Annex E for FR2-1, we can take following values:

- ISD =200m, center frequency fc= 40GHz

- BS Tx power = 30dBm and EIRP_bs = 53dBm per panel (antenna array per panel (M,N,P,Mg,Ng)  = (4,16,2,2,2)).

- UE Tx power = 23dBm and EIRP_ue = 31dBm (antenna array is (M,N,P,Mg,Ng;Mp,Np) = (2,4,2,1,2;1,1)).

According to RF discussion on FR2-1 WA BS, we can have following assumptions:

- In-channel adjacent subband blocking requirement is -52dBm

- The coupling loss is 87 dBc.

If we assume victim UE is at the cell edge of the cell, we could have following results:

- Distance between BS dbs2bs=0.2 km, and distance between victim UE to BS due2bs=0.133 km

- Pathloss between BS is LFS_bs2bs=32.45+20lgdbs2bs+20lgfc = 110.5dB.

- Pathloss between victim UE to BS is LFS_ue2bs=32.45+20lgdue2bs+20lgfc = 107dB.

- The received wanted signal power is  SUE=EIPR_ue- LFSue2bs+(EIRPbs-bsPower)= -52.5 dBm

In that case, the maximum interference power level -52dBm is almost same as received signal power.

If we follow similar interfering condition as in FR1 analysis including two conditions: 1 strong interference + 1 weaker interference (10dB less than stronger interference) and two medium interferences (6dB weaker than strong interference). We can have following INR values:

Table 2-1 Revised INR values for FR2-1

| Conditions | FR2-1 |
| --- | --- |
| Interf1 > Interf2 | INR1 <= 0dB, INR2 <= -10dB |
| Interf1 = Interf2 | INR1 = INR2 <= -6dB |
| Note: The interference wave form could use PUSCH with 1+1 DM-RS and CP-OFDM to simplify simulation setup. |  |

The INR values of FR2-1 scenario are much smaller than FR1 scenario based on link budget analysis.

According to our simulation results for FR1 scenario [2], the gap between “Not use muted REs” and “Use muted REs” is less than 0.25dB in all cases which is too small to differentiate whether the product estimate the interference covariance matrix with muted RE or not. Furthermore, the lower INR values lead to smaller gap. In that case, the gap in FR2-1 condition would be smaller than in FR1.

The gap between “Not use muted REs” and “Use muted REs” is less than 0.25dB.

The lower INR inference cases have smaller gaps.

Based on observations above, we propose not to define PUSCH demodulation requirements for UL muting in both FR1 and FR2-1.

Proposal 1	Do not define PUSCH demodulation requirement for UL muting in both FR1 and FR2-1.

# Conclusions

In the previous sections we made the following observations:

Observation 1	The INR values of FR2-1 scenario are much smaller than FR1 scenario based on link budget analysis.

Observation 2	The gap between “Not use muted REs” and “Use muted REs” is less than 0.25dB.

Observation 3	The lower INR inference case have smaller gap.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Do not define PUSCH demodulation requirement for UL muting in both FR1 and FR2-1.

# References

[1] 	R4-2515038, Way Forward for [116bis][311] Rel-19 Demodulation_Part, Samsung

[2]	R4-2521768, Simulation results for NR SBFD demodulation, Ericsson