3GPP TSG RAN WG4 #117	R4-2521777

# Dallas, USA, Nov. 17 – 21, 2025

Agenda Item:	6.15.2.1

Source: 	Ericsson

Title:	Discussion on remaining issues for OD-SSB performance requirements

Document for:	Discussion

# Introduction

In last meeting, RAN4 made the agreement about the OD-SSB test case list as follow.

No.; Test category; Test scenario; Test configuration; Test purpose. TC1; SCell Activation and deactivation of unknown SCell in DRX based on OD-SSB; SCell activation Interruption at OD-SSB activation/deactivation; OD-SSB periodicity 10ms; OD-SSB Case 1 OD-SSB scenario 2a Unknown SCell activation FR1 and FR2. TC2; SCell Activation and deactivation of unknown SCell in DRX based on OD-SSB; SCell activation Interruption at OD-SSB activation/deactivation; OD-SSB periodicity 10ms; OD-SSB Case 1 OD-SSB scenario 2a Unknown SCell activation FR2. TC3; SCell Activation and deactivation of unknown SCell in DRX based on OD-SSB; SCell activation Interruption at OD-SSB activation/deactivation; OD-SSB periodicity 10ms AO-SSB periodicity 20ms; OD-SSB Case 2 Alt Time-C1 - OD-SSB(10ms) is overlapping with AO-SSB(20ms) OD-SSB scenario 2a Unknown SCell activation FR1. TC4; SCell Activation and deactivation of known SCell in non-DRX based on OD-SSB; SCell activation Interruption at OD-SSB activation/deactivation; OD-SSB periodicity 20ms AO-SSB periodicity 20ms; OD-SSB Case 2 Alt Time-C1 - The union of OD-SSB and AO-SSB meets the legacy SSB periodicity OD-SSB scenario 2a known SCell activation FR1. TC5; SCell Activation and deactivation of known SCell in non-DRX based on OD-SSB; SCell activation Interruption at OD-SSB activation/deactivation; OD-SSB periodicity 20ms AO-SSB periodicity 20ms; OD-SSB Case 2 Alt Time-C1 - The union of OD-SSB and AO-SSB meets the legacy SSB periodicity OD-SSB scenario 2a known SCell activation FR2. TC6; Direct SCell activation at SCell addition based on OD-SSB; Direct SCell activation; OD-SSB periodicity 20ms AO-SSB periodicity 80ms; OD-SSB Case 2 Alt Time-C1 OD-SSB scenario 2a FR1. TC7; Intra-frequency measurement(deactivated SCell) based on OD-SSB; Deactivated SCell measurement; Interruption during deactivated SCell measurement; OD-SSB periodicity 10ms; OD-SSB Case 1 FMW; UE behaviour relevant to T2 FR1. TC8; Intra-frequency measurement(deactivated SCell) based on OD-SSB; Deactivated SCell measurement; Interruption during deactivated SCell measurement; OD-SSB periodicity 20ms AO-SSB periodicity 20ms; OD-SSB Case 2 FMW FR2. TC9; Intra-frequency measurement- SA event triggered reporting tests without gap for SCell under non DRX based on OD-SSB; L3-RSRP measurement; OD-SSB periodicity 10ms; OD-SSB scenario 3B; OD-SMTC and SMTC FR1. TC10; Intra-frequency measurement- SA event triggered reporting tests without gap under non DRX based on OD-SSB; L3-RSRP measurement; OD-SSB periodicity 20ms AO-SSB periodicity 20ms; OD-SSB scenario 3B; OD-SMTC and SMTC FR2 FFS FR1. TC11; SSB based L1-RSRP measurement when DRX is not used based on OD-SSB; L1-RSRP measurement; OD-SSB periodicity 20ms -> 10ms; OD-SSB scenario 3B; Scheduling restriction to AO-SSB occasions; L1-RSRP based on EMP FR1.

There are still several uncertain test cases need to be discussed. In this contribution, we’ll discuss these maintenance test case [1].

# Test Case Maintenence

In last meeting, RAN4 listed the following test case as FFS.

Issue 3-1-1: Scenario - General - SCell activation No.; Test category; Test scenario; Test configuration; Test purpose. FFS; SCell Activation and deactivation of known SCell in non-DRX based on OD-SSB; SCell activation Interruption at OD-SSB activation/deactivation; OD-SSB periodicity 10ms; OD-SSB Case 1 OD-SSB scenario 2 and 2a Known SCell activation FR1. - Direct SCell Activation No.; Test category; Test scenario; Test configuration; Test purpose. FFS; Direct SCell activation at SCell addition based on OD-SSB; Direct SCell activation; OD-SSB periodicity 10ms; OD-SSB Case 1 OD-SSB scenario 2a FR1. - Deactivated SCell L3 measurement No.; Test category; Test scenario; Test configuration; Test purpose. FFS; Intra-frequency measurement(deactivated SCell) based on OD-SSB; Deactivated SCell measurement; Interruption during deactivated SCell measurement; OD-SSB periodicity 20ms AO-SSB periodicity 20ms; OD-SSB Case 2 FMW FFS FR1.

Known SCell activation in Case 1

The first case is for known SCell activation in OD-SSB Case 1. Currently, only unknown SCell activation test cases were defined. Thus, it looks reasonable to further define a known SCell activation test case. The test purpose should be further clarified that only test the scenario 2 without verify the FMW since the deactivated SCell measurement for OD-SSB has already been verified in TC7. The test should only verify the delay of known SCell activation for OD-SSB in Case 1.

Proposal 1: RAN4 to define the known SCell activation based on OD-SSB Case 1 as follows:

- Only verify the known SCell activation delay not verify the FMW.

- Only activate the OD-SSB transmission once other than verify both scenario 2 and scenario 2a.

Direct SCell activation in Case 1

Currently, RAN4 only defined one direct SCell activation test case for Case 2. Considering direct SCell activation is an important procedure commonly used in real field, we suggest to also verify the Case 1 with only OD-SSB.

Proposal 2: RAN4 to define the direct SCell activation for OD-SSB Case 1.

Diactivated SCell L3 measurement

In our understanding, the deactivated SCell measurement function has already been verified in TC7 and TC8. We don’t think it’s important to further define a test case to verify the FR1 behaviour for OD-SSB Case 2. The behaviour should be similar to FR2.

Proposal 3: RAN4 not to further define the deactivated SCell measurement test case in FR1.

- Summary

In this contribution, we have discussed the the performance part of Rel-19 NES. Based on the discussions, we made the following proposals and observations:

Proposal 1: RAN4 to define the known SCell activation based on OD-SSB Case 1 as follows:

- Only verify the known SCell activation delay not verify the FMW.

- Only activate the OD-SSB transmission once other than verify both scenario 2 and scenario 2a.

Proposal 2: RAN4 to define the direct SCell activation for OD-SSB Case 1.

Proposal 3: RAN4 not to further define the deactivated SCell measurement test case in FR1.

- References

- R4-2514808, “WF on RRM requirements for Netw_Energy_NR_enh_Part1”, Ericsson