3GPP TSG-RAN WG4 Meeting #117	R4-2521791

Dallas, USA, Nov. 17-21, 2025

Agenda item:	8.2.5.1

Source:            	Google

Title:                	Discussion on 6G system parameters

Document for:	Approval

# Introduction

In RAN#109, the latest revised SID for Study on 6G Radio was approved as one of Rel-20 study items [1]. The followings are the objectives quoted from the revised SID. In this contribution, we will focus on the 6GR system parameters and device types related discussion.

- 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - UE RF requirement aspects including band and band combination [RAN4] - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination - Study UE RF capabilities considering different device types and implementations - RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles - Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS - Aspects related to the testability [RAN4] - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity - Other aspects - Handling irregular channel bandwidths including the definition [RAN4, RAN1] - Definition of ‘frequency range(s)’[RAN4]

In RAN4#116bis, the 6GR system parameters has been discussed and the open issues are captured in the approved WF [4]. In this paper, we would like to share our view in the following.

# Topic #1: Waveform ## Framework study for waveform - Agreement - The primary purpose of RAN4 study on waveform is to evaluate candidate waveforms and potential PAPR reduction techniques based on agreements and inputs from RAN1 - To establish the foundational evaluation framework in RAN4 firstly, and RAN4 waveform study should focus on the following aspects - Tx assumption including PA model - Related RF requirements which should be taken into consideration - Both UL and DL are considered - Take the candidate waveforms identified/discussed in RAN1 as the baseline - Identify the evaluation metric, e.g. net gain - Implementation constraints - Model and evaluate the RF performance of different waveform candidates and PAPR reduction techniques pending on RAN1 inputs - Evaluation cases could be selected step-by-step, based on RAN1 progress - Whether to investigate RF impacts of DL DFT-s-OFDM pending on RAN1 agreement ## PA model - Agreement - Consider PA modelling at least for the following aspects for RAN4 discussion - Memory effects for UE supporting larger CBW - Different PA models for different sub-frequency ranges, e.g. around 7GHz (high priority, PC3/PC2) and lower frequency bands (PC3) - Calibration conditions - RF impairments used for MPR evaluation, e.g. carrier leakage, I/Q imbalance, etc. - PA models for different device types, handheld UE is prioritized - Applicable requirements - 5G-A requirements as starting point - PA models for RAN1 waveform evaluation and RAN4 requirements discussion can be decoupled # Topic #2: Modulation - Agreement - RAN4 evaluation work focus on the feasibility study and RF requirements impact - To establish the foundational evaluation framework in RAN4 firstly when no solid progress and inputs from RAN1 - Identify the main affected requirements for modulation evaluations - The existing 5G NR requirements will serve as the baseline, which are subject to future updates based on RAN4’s 6G UE RF discussions. - Align on the evaluation assumptions. - Study on how to align on the transmitter chain model, including PA model, for consistent evaluations on the modulation. - RF evaluation could be done firstly for 5G supported modulations with new assumptions for 6G study, such as assumed new spectrum, CBW, new PA models, etc. Co-ordination with 6G UE RF study is needed. - Both link-level and system-level simulations should be performed as usual for high-order modulations study done by RAN4 in prior releases, pending on the progress of RAN1. - Model and evaluate the performance and the implementation complexity of higher-order modulations, e.g. 1024QAM on the UL and/or new constellations - For high order QAM with uniform constellation, e.g. 1024QAM on the UL, RAN4 can work concurrently with RAN1 studies. - For new non-uniform constellation, the evaluation in RAN4 should depend on RAN1 progress and request. # Topic #3: Channel bandwidth ### Sub-topic 3-1: Max Channel Bandwidth - Agreement In the absence of RAN1 agreements, RAN4 could conduct preliminary assessments of the RF feasibility for the candidate maximum channel bandwidths. - RF performance evaluation with proposed max CBW - Identify the prioritized max CBW scenarios (e.g., TDD/FDD, frequency ranges) for evaluation - From transmitter perspective, evaluate the feasibility of meeting out-of-band emission requirements - From receiver perspective, study the impact on reference sensitivity, blocking, and ACS when receiving these wide carriers - 5G NR requirements could be considered as baseline for the evaluation on existing frequency bands, FFS for the new spectrum - The large CBW study can take into consideration on the potential spectrum availability for re-farming spectrum as well as the regulatory and WRC discussion of new 6G spectrum. - Assess the implementation feasibility and complexity and power consumption - Compare implementation options, e.g., for cases like 200MHz/400 MHz, evaluate the RF performance and implementation trade-offs of the different proposed UE architectures (e.g., single 16K FFT vs. multi-FFT vs. CA). The evaluation cases also depend on the discussion in RAN1. - Study the feasibility to support different NW and UE max CBW. - Collaborate with RAN1 on feasibility findings - Provide early feedback to RAN1 with RAN4's initial findings on the RF feasibility and trade-offs of the most prominent max CBW/SCS/FFT combinations. ### Sub-topic 3-2: Min Channel Bandwidth - Agreement - Compare options of defining min CBW, considering pros and cons - Study the following aspects from RAN4 perspective, meanwhile tracking RAN1/RAN progress - Whether 5MHz could be considered as a general baseline while 3MHz is allowed for particular bands - SCS-dependent framework - Other aspects are not precluded - Provide early feedback to RAN1 with RAN4's initial findings on min CBW from implementation and spectrum perspective. ### Sub-topic 3-3: FFT size - Agreement - Consider FFT size, maximum Channel Bandwidth and numerology as a framework to have feasibility and complexity study from implementation perspective, especially for 8K or 16K FFT size considering the associated SCS and also the frequency ranges - Provide RAN1 with early RAN4 feedback on the feasibility and trade-offs of the proposed FFT/CBW/SCS combinations to help guide their decisions. ### Sub-topic 3-4: Numerology - Agreement - Evaluate the following proposals regarding numerology from RAN4 perspective - "Single numerology" proposal - Frequency sub-range/Band specific SCS values proposal - Compare perf gain and implementation complexity for different SCS with same frequency range or specific band - Study numerology for SSB of initial cell search from RAN4 perspective - Other proposals not presented in this meeting are not precluded - Provide RAN1 with early RAN4 feedback on the feasibility and trade-offs of the proposed FFT/CBW/SCS combinations to help guide their decisions. ### Sub-topic 3-5: Spectrum utilization - Agreement - Agree on a set of common simulation assumptions for SU evaluation, including PA models, RF impairments (e.g., carrier leakage, I/Q imbalance, phase noise, etc.), and baseline RF requirements (e.g., SEM, ACLR, EVM). - 5G NR channel bandwidth, requirements can be considered as starting point for the SU evaluation with new assumptions for 6G - Evaluate the RF performance impact (complying with the affected requirements) of advanced spectral confinement techniques (e.g., better filtering, windowing) to understand how many RBs can be enabled - Considering trade-offs between SU, RF performance, and UE/BS complexity - Channel bandwidth and SCS with smaller SU should be prioritized - SU for larger channel bandwidth shall be evaluated based on progress on CBW - Closely coordinate with RAN1 on different waveform candidates and SCS configurations. ### Sub-topic 3-6: Asymmetric channel bandwidths - Agreement - Study the need and feasibility to enable asymmetric CBW # Topic #4: Channel arrangement ### Sub-topic 4-1: Channel raster - Agreement - Study 5G-6GR co-existence impact on channel raster with legacy NR refarmed bands - Note that NR bands could have 100kHz channel raster, 10kHz enhanced channel raster or SCS based channel raster - Investigate the interaction between the channel raster and the synchronization raster - Investigate the necessity of channel raster or alternative ways for the channel configuration - If channel raster needs to be specified, further investigate granularity including SCS based raster, and enhanced channel raster - Investigate the possibility of migrating to SCS based raster if legacy rasters are still to be supported - Study the listed main proposals especially for the migration and co-existence approaches - Other options not presented in this meeting are not precluded - Provide early feedback to RAN1 with RAN4's analysis on the RF coexistence performance and potential implementation complexity associated with the various proposed channel raster options (5 kHz, 10 kHz, SCS-based). ### Sub-topic 4-2: Sync raster - Agreement - Evaluation on sync raster from RAN4 perspective: - Investigate the interaction between the channel raster and the synchronization raster and other aspects identified in RAN4 - Study enhancement and/or simplification considering, e.g. fast UE cell search and power saving - Collaboration and planning: - Proactively collaborate with RAN1 to ensure that the evolving SSB design considers the practical RF and implementation constraints related to the sync raster from the outset. - Develop a flexible evaluation framework in RAN4 that can quickly assess different sync raster proposals once key parameters (like final SSB bandwidth and min CBW) are stabilized by RAN1. ### Sub-topic 4-3: Channel spacing - Agreement - Further study in RAN4 regarding channel spacing # Topic #5: Irregular channel bandwidth - Agreement - 5G concepts from TR 38.844 can be considered as starting point to study a more generic solution for 6G - The listed main proposals and identified issues should be taken into account for the following study of irregular/flexible/scalable channel bandwidths - Other options not presented in this meeting are not precluded # Topic #6: Number of Tx and Rx - Agreement - Study the framework of spec impact with different number of Tx/Rx - Evaluate number of Tx/Rx from both a performance and implementation complexity perspective. - E.g. device size (e.g. foldable smartphone)/form factors constraints, antenna design - Co-ordinate with RAN and RAN1 for number of Tx/Rx # Topic #7: Device types - Agreement - The following aspects could be further studied - Investigate detailed RF/BB implementation feasibility and constraints related to different devices assumption, for example, size/form factors and use cases (e.g., IoT, Wearable, Smartphone, FWA devices). These should include concrete assumptions, e.g. number of antennas, CBW, power class, and supported modulation per frequency range. - Study on the NW impact should also be considered - Study how to better support variety devices from RAN4 perspective - E.g. differentiate devices, use cases, features - Other aspects are not precluded - Collaborate on the framework with other WGs: - Provide necessary inputs to RAN/other WGs with RAN4's study outcome for the above-mentioned issues

# Discussion

PA Model

In WF [4], several aspects are collected for 6G PA model and one of these aspects is memory effect for PA model. For traditional memoryless AM-AM/AM-PM PA model, several companies shared the views that it may be insufficient to accurately capture the non-linear behavior of handheld UEs when operating at the larger channel bandwidths (e.g., 200MHz or 400MHz) targeted for 6G. Neglecting PA memory effects in initial 6G waveform evaluations may lead to over-optimistic results that do not reflect realistic implementation constraints of compact handheld devices, particularly in new higher frequency ranges which will be introduced in 6G. In our view, it is essential to avoid over-estimating RF performance by accurately modelling the memory effects in initial feasibility studies. Hence, to ensure realistic RF performance assessments, it is proposed to consider the memory effects for the PA model, e.g., existed PA model or new PA model, to evaluate 6G waveforms with bandwidth >100MHz for handheld device types

Proposal 1: For 6G PA model, to ensure realistic RF performance assessments, it is proposed to consider the memory effects for the PA model, e.g., the existed PA model or new PA model, to evaluate 6G waveforms with bandwidth >100MHz for handheld device types.

Modulation

In WF [4], the high order modulation for UL 1024 QAM is proposed to evaluate the performance and implementation complexity. In our understanding, it is observed that UL 1024QAM requires extremely high SNR conditions, which are rarely achievable for power-constrained handheld UEs in typical mobile deployment scenarios. Implementing UL 1024QAM on handheld devices would need significant MPR (Maximum Power Reduction) to meet the PA linearity requirements, severely degrading power efficiency and thermal issues without guaranteeing practical throughput gains. Since UL 1024QAM is primarily feasible only in static, high-SNR scenarios (e.g., FWA), it should have a lower priority in the initial 6G study compared to features essential for mainstream handheld devices. Hence, considering the implementation constraints and limited practical gains for mobile scenarios, it is proposed to deprioritize the UL 1024QAM study for 6G handheld devices.

Proposal 2: For 6G modulation, considering the implementation constraints and limited practical gains for mobile scenarios, it is proposed to deprioritize the UL 1024QAM study for 6G handheld devices.

Max Channel Bandwidth

In WF [4], it is suggested that RAN4 could conduct preliminary assessments of the RF feasibility for the 6G candidate maximum channel bandwidths. In our understanding, it is observed that supporting a 400 MHz single-carrier bandwidth (requiring 16K FFT) presents significant implementation challenges for handheld UEs in terms of baseband complexity and power consumption. Some companies’ views shows that 200 MHz represents a feasible and appropriate baseline for the maximum single-carrier bandwidth in new frequency ranges (e.g., around 7GHz) for initial 6G specifications. As for the feasibility for 400 MHz as maximum channel bandwidth, intra-band contiguous CA of 2x200 MHz could be the baseline for further study rather than mandating single-carrier 400 MHz support. Hence, it is proposed to adopts 200 MHz as the single-carrier maximum channel bandwidth for 6G handheld device in FR1 (including new bands around 7GHz).

Proposal 3: For 6G max channel bandwidth, it is proposed to adopts 200 MHz as the single-carrier maximum channel bandwidth for 6G handheld device in FR1 (including new bands around 7GHz).

For FR2-1 max channel bandwidth, it is observed that channel bandwidths wider than 200 MHz are still treated as optional capabilities in the early 5G FR2 specifications. From industrial views, 400 MHz can represent a mature feasible bandwidth for high-end handheld devices in FR2-1 from 5G-Advanced. However, mandating FR2-1 800 MHz single-carrier for all 6G handheld devices may poses significant risks regarding thermal management and power efficiency. Hence, considering ensuring a mature and power-efficient for 6G mmWave devices, it is proposed to adopt 400 MHz as the maximum channel bandwidth for 6G handheld devices in FR2-1.

Proposal 4: For 6G max channel bandwidth, considering ensuring a mature and power-efficient for 6G mmWave devices, it is proposed to adopt 400 MHz as the maximum channel bandwidth for 6G handheld devices in FR2-1.

Min Channel Bandwidth

In WF [4], it is suggested to study defining 6G min CBW by considering its pros and cons. It is observed that most companies shared the opinion that 5 MHz is the appropriate general baseline for minimum channel bandwidth in FR1 to maintain robust system performance (e.g., SSB design, PDCCH coverage). While 3 MHz is necessary for specific fragmented legacy bands (e.g., sub-1GHz), it is realized that mandating 3MHz as a general baseline across all bands could impact the design of common channels and sync raster, leading to suboptimal efficiency for the majority of 6G deployments. Hence, considering ensuring a robust initial access design, it is proposed to adopt 5 MHz as the general baseline minimum channel bandwidth for 6G handheld devices. For 3 MHz, it can be treated as an exception which is applicable only to specific operating bands with explicitly justification by regional spectrum regulations or operator requests.

Proposal 5: For 6G min channel bandwidth, considering ensuring a robust initial access design, it is proposed to adopt 5 MHz as the general baseline minimum channel bandwidth for 6G handheld devices. For 3 MHz, it can be treated as an exception which is applicable only to specific operating bands with explicitly justification by regional spectrum regulations or operator requests.

FFT Size

In WF [4], it is suggested to consider 6G FFT size, maximum channel bandwidth and numerology as a framework to have feasibility and complexity study from implementation perspective. It is observed from most companies’ view that 8K (8192) FFT represents a feasible and mature baseline for 6G FR1 handheld devices, which can enable support for up to 200 MHz channel bandwidth with 30 kHz SCS. However, mandating 16K (16384) FFT for handheld devices in 6G Day-1 to support single-carrier 400 MHz may introduce significant challenges in baseband complexity and power consumption. Hence, considering the baseband complexity and power consumption, it is proposed to adopt 8K FFT as the maximum FFT size for 6G handheld devices.

Proposal 6: For 6G FFT size, considering the baseband complexity and power consumption, it is proposed to adopt 8K FFT as the maximum FFT size for 6G handheld devices.

Numerology

In WF [4], it is proposed to evaluate 6G numerology from RAN4 perspective. In our understanding, it is widely observed from 5G NR deployments that the flexibility of supporting multiple SCS options per band provided limited practical benefit but it may significantly increase UE implementation and test complexity. Hence, considering fundamentally simplifying 6G UE design and reduce the test burden, it is proposed to adopt a single numerology per operating band principle as the baseline for 6G handheld devices.

Proposal 7: For 6G numerology, considering fundamentally simplifying 6G UE design and reduce the test burden, it is proposed to adopt a single numerology per operating band framework as the baseline for 6G handheld devices.

Allowing different numerologies for the SSB and the data channel (as 5G did) forces the UE to handle mixed configurations during the critical initial cell search phase. This directly increases baseband processing load and power consumption for handheld devices. Hence, considering reducing UE power consumption during initial access, it is proposed that the SSB numerology shall be aligned with single default data numerology defined on that operating band for 6G handheld devices.

Proposal 8: For 6G numerology, considering reducing UE power consumption during initial access, it is proposed that the SSB numerology shall be aligned with single default data numerology defined on that operating band for 6G handheld devices.

Channel raster

In WF [4], it is proposed to study 5G-6GR co-existence impact on channel raster with legacy NR refarmed bands. It is observed that 5G FR1 bands currently utilize a mix of 100 kHz, 10 kHz enhanced, and SCS-based rasters, leading to increased UE implementation complexity and conformance testing without clear long-term benefits. The 10 kHz enhanced channel raster was introduced in 5G NR to resolve PRB misalignment issues for the legacy 100 kHz raster, particularly when configuring narrower UE bandwidths within wider BS carriers. Adopting 10 kHz as the unified baseline for legacy or re-farmed bands in 6G can leverage 5G UE design, which avoid the complexity of maintaining multiple legacy rasters (100kHz) while providing sufficient flexibility for coexistence without needing a new 5 kHz definition. Hence, consider simplifying UE implementation and efficient spectrum utilization, it is proposed to adopt the 10 kHz instead of 100KHz for FR1 bands below the frequency, e.g., 2.4GHz or 3 GHz, as single baseline channel raster for 6G handheld devices. For the FR1 bands above the frequency, e.g., 2.4GHz or 3 GHz, it is proposed to adopt SCS-based channel raster as the baseline to ensure perfect PRB alignment in 5G-6GR co-existence for 6G handheld devices.

Proposal 9: For 6G channel raster, considering simplifying UE implementation and efficient spectrum utilization, it is proposed to adopt the 10 kHz instead of 100KHz for FR1 bands below the frequency, e.g., 2.4GHz or 3 GHz, as single baseline channel raster for 6G handheld devices. For the FR1 bands above the frequency, e.g., 2.4GHz or 3 GHz, it is proposed to adopt SCS-based channel raster as the baseline to ensure PRB alignment in 5G-6GR co-existence for 6G handheld devices.

Sync raster

In WF [4], it is proposed to evaluate 6G sync raster from RAN4 perspective. It is widely recognized by most companies’ views that the 5G NR synchronization raster was designed with unnecessarily high density, leading to increased UE power consumption and longer cell search times. For battery-critical handheld devices, minimizing the time to camp on a cell during initial access and reducing background search activity in idle mode are important. Hence, considering reducing cell search times and power consumption, it is proposed to adopt sparser sync raster, i.e., significantly reducing the number of sync raster entries per band compared to 5G, for 6G handheld devices.

Proposal 10: For 6G sync raster, considering reducing cell search times and power consumption, it is proposed to adopt sparser sync raster, i.e., significantly reducing the number of sync raster entries per band compared to 5G, for 6G handheld devices.

Number of Tx/Rx and Device type

In WF [4], it is proposed to study the number of Tx/Rx and device types. In our understanding, it is observed that the feasibility of #Tx/#Rx configurations is intrinsically tied to the physical constraints of the target device. While relaxed form-factor devices (e.g., FWA CPEs) can readily support higher antenna counts (e.g., 8Rx), supporting such configurations for highly integrated, power-limited handheld UEs (especially foldable architectures) faces severe thermal and spatial implementation barriers. While the final definition of 6G device types heavily relies on decisions from RAN Plenary, RAN4 still can develop working assumptions for some common device categories (e.g., IoT, Wearable, Handheld vs. FWA) and prioritize defining the RF boundary conditions, e.g., number of Tx/Rx, MIMO layer, max RF bandwidth, power class, etc.) for some device types assumption, where the example is shown as follows.

- Type IoT Set#1:

  - Tx ANT: 1Tx

  - Rx ANT: 1Rx or 2Rx

  - MIMO layer: 1 or 2

  - Max RF bandwidth: 20MHz

  - Power Class: FDD PC3, TDD PC2

- Type Wearable Set#2:

  - Tx ANT: 1Tx

  - Rx ANT: 1Rx

  - MIMO layer: 1

  - Max RF bandwidth: 20MHz

  - Power Class: FDD PC3, TDD PC2

- Type Smartphone Set#3:

  - Tx ANT: 2Tx

  - Rx ANT: 4Rx

  - MIMO layer: 4

  - Max RF bandwidth: 200MHz

  - Power Class: FDD PC3, TDD PC2

- Type FWA Set#4:

  - Tx ANT: 1-4Tx

  - Rx ANT: 4-8Rx

  - MIMO layer: 4-8

  - Max RF bandwidth: 400MHz

  - Power Class: FDD PC1, TDD PC1.5

Hence, considering establishing working assumptions for device types allows RAN4 to proactively initiate studies on key parameters and identifying potential RF/baseband implementation issues early in the 6G study phase, it is proposed to adopt IoT/Wearable/Smartphone/FWA as the baseline set of 6G device types working assumption and other device types can be further discussed.

Proposal 11: Considering establishing working assumptions for device types allows RAN4 to proactively initiate studies on key parameters and identifying potential RF/baseband implementation issues early in the 6G study phase, it is proposed to adopt IoT/Wearable/Smartphone/FWA as the baseline set of 6G device types working assumption and other device types assumption can be further discussed.

# Conclusion

The proposals in this contribution are summarized in the following.

PA Model

Proposal 1: For 6G PA model, to ensure realistic RF performance assessments, it is proposed to consider the memory effects for the PA model, e.g., the existed PA model or new PA model, to evaluate 6G waveforms with bandwidth >100MHz for handheld device types.

Modulation

Proposal 2: For 6G modulation, considering the implementation constraints and limited practical gains for mobile scenarios, it is proposed to deprioritize the UL 1024QAM study for 6G handheld devices.

Max Channel Bandwidth

Proposal 3: For 6G max channel bandwidth, it is proposed to adopts 200 MHz as the single-carrier maximum channel bandwidth for 6G handheld device in FR1 (including new bands around 7GHz).

Proposal 4: For 6G max channel bandwidth, considering ensuring a mature and power-efficient for 6G mmWave devices, it is proposed to adopt 400 MHz as the maximum channel bandwidth for 6G handheld devices in FR2-1.

Min Channel Bandwidth

Proposal 5: For 6G min channel bandwidth, considering ensuring a robust initial access design, it is proposed to adopt 5 MHz as the general baseline minimum channel bandwidth for 6G handheld devices. For 3 MHz, it can be treated as an exception which is applicable only to specific operating bands with explicitly justification by regional spectrum regulations or operator requests.

FFT Size

Proposal 6: For 6G FFT size, considering the baseband complexity and power consumption, it is proposed to adopt 8K FFT as the maximum FFT size for 6G handheld devices.

Numerology

Proposal 7: For 6G numerology, considering fundamentally simplifying 6G UE design and reduce the test burden, it is proposed to adopt a single numerology per operating band framework as the baseline for 6G handheld devices.

Proposal 8: For 6G numerology, considering reducing UE power consumption during initial access, it is proposed that the SSB numerology shall be aligned with single default data numerology defined on that operating band for 6G handheld devices.

Channel raster

Proposal 9: For 6G channel raster, considering simplifying UE implementation and efficient spectrum utilization, it is proposed to adopt the 10 kHz instead of 100KHz for FR1 bands below the frequency, e.g., 2.4GHz or 3 GHz, as single baseline channel raster for 6G handheld devices. For the FR1 bands above the frequency, e.g., 2.4GHz or 3 GHz, it is proposed to adopt SCS-based channel raster as the baseline to ensure PRB alignment in 5G-6GR co-existence for 6G handheld devices.

Sync raster

Proposal 10: For 6G sync raster, considering reducing cell search times and power consumption, it is proposed to adopt sparser sync raster, i.e., significantly reducing the number of sync raster entries per band compared to 5G, for 6G handheld devices.

Number of Tx/Rx and Device type

Proposal 11: Considering establishing working assumptions for device types allows RAN4 to proactively initiate studies on key parameters and identifying potential RF/baseband implementation issues early in the 6G study phase, it is proposed to adopt IoT/Wearable/Smartphone/FWA as the baseline set of 6G device types working assumption and other device types assumption can be further discussed.

# Reference

- RP-251816, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone, RAN#109

- R4-2514508, “Topic Summary for [116bis][101] 6G system parameter”, Huawei, HiSilicon, RAN4#116bis

- R4-2514623 , “AH minutes on [116bis][101] 6G system parameter”, Huawei, RAN4#116bis

- R4-2514641, “WF on [116bis][101] 6G system parameter”, Huawei, RAN4#116bis

- R4-2514650, “WF on 6G system parameter”, Huawei, RAN4#116bis

- R4-2514643, “LS on [116bis][101] 6G system parameter”, Huawei, RAN4#116bis