3GPP TSG-RAN4 Meeting #117	R4-2521896

# Dallas, USA, 17th – 21st November 2025

Title:	Views on the coexistence study for High Power UE

Source:	Qualcomm Incorporated

Agenda item:	7.1.3

Document for:	Approval

# Introduction

Following the Rel20 UE RF enhancement WI [1] approved in RAN#109, a WF was agreed in [2] on the HPUE coexistence aspects. The WF in [2] captured parameters and candidate options regarding simulation scenarios, assumptions, performance metrics and others for adjacent channel co-existence study for 4Tx HPUE with up to 32dBm maximum output power. In this contribution we continue to provide our views on [2] and present our preliminary coexistence results for HPUE to be further discussed.

# General discussion

## Simulation scenarios

In [2], the following scenarios were agreed to be studied in the coexistence study:

Table 1: Co-existence study scenario Usage scenario; Aggressor; Victim; Direction; Simulation frequency; Deployment Scenario; Note. eMBB; NR, 100MHz; NR, 100MHz; UL to UL; 2.6GHz; [Urban macro, Rural macro]. 3.5GHz, 4.9GHz, 7GHz; Urban macro. Other scenarios, if agreed, can be further considered in the co-existence study.

To facilitate the work in RAN4, it is proposed that RAN4 categorize the coexistence study into three different frequency ranges, namely, 2 GHz, 4GHz, and 7 GHz, to encompass all the bands listed in the WID [1]. Additionally, based on [2], it is observed that the urban macro is studied for the three frequency ranges, only rural macro is investigated for the lowest frequency. Because of the different simulation frequencies, it is expected that each frequency  can have different simulation assumptions and parameters. Although several parameters could be kept the same (e.g., UE maximum output power, UE distribution, propagation model, etc.), there are fundamental differences between others depending on the frequency range (e.g., BS AAS parameters, ISD).

Proposal 1: RAN4 to categorize the coexistence work to three frequency ranges, namely, 2GHz, 4GHz, and 7 GHz.

## Network layout and simulation parameters

In [2], the following parameters were captured:

Table 2: Network layout model for urban macro Parameters; Value. Network layout; hexagonal grid, 19 macro sites, 3 sectors per site with wrap around. Inter-site distance; Depending on frequency (ref. TR 37.829 for 2.6GHz, 3.5GHz and [TR 38.921, 600m] for other frequencies). Multi operators layout; Uncoordinated. BS antenna height; Depending on frequency (ref. TR 37.829 for 2.6GHz, 3.5GHz and TR 38.922 for other frequencies). UE location; Outdoor/indoor; Outdoor and indoor. Indoor UE ratio; 20% (TR 38.921). Low/high penetration loss ratio; 50% low loss, 50% high loss. LOS/NLOS; LOS and NLOS. UE antenna height; Same as 3D-UMa in TR 36.873 (TR 38.921). UE distribution (horizontal); Uniform. Minimum BS - UE distance (2D); 35 m For Uncoordinated deployment, minimum distance applies also to UEs and BS across different deployments.. Channel model; UMa.

It is important to agree within RAN4 on the listed parameters above, since they have an impact on the simulations.

Focusing first on the Urban Macro scenarios, the ISD for the 2GHz is set to 750m following urban macro scenario in Table A.1.3-1, TR 37.829. For the 4GHz and 7GHz, an ISD of 450m is considered, following Table 4.2.1.1-1 in TR 38.921. Note that the same ISD assumption was considered for urban scenarios in TR 38.922. For the BS antenna height, it is reasonable to agree on 45m for 2GHz and 20m for both 4GHz and 7GHz, aligned with the mentioned TRs in the table above. For consistency, the propagation models for UMa scenarios too should be per TR 38.901 as captured in [2].

In Summary, Table 1 captures better the detailed simulation assumptions for the three frequency ranges for the Urban Macro scenario.

Table 1 Simulation parameters for UMa scenarios

| Parameters | 2GHz | 4GHz | 7GHz |
| --- | --- | --- | --- |
| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |  |
| Inter-site distance | 750m | 450m | 450m |
| BS antenna height | 45m | 20m | 20m |
| Channel model | UMa in TR 38.901 |  |  |

Considering the Rural Macro deployment, relevant to the 2GHz scenario, an ISD of 6 km could be agreed, aligned with TR TR 37.829, and only outdoor UEs should be considered. In general it is desirable to align simulation parameters and underlying models as much as possible with TR 37.829, since this TR studies HPUE coexistence with several NR and LTE bands.

Observation 1: It is desirable to align simulation parameters and underlying model in this work item with HPUE coexistence work conducted in TR 37.829.

Regarding the propagation model, the Hata rural model used in TR 36.837 and TR 37.829 should be adopted, since it is more tailored to HPUE rural scenarios that were already studied in RAN4. The Path Loss is defined in TR 36.837, Section 5.4.1.1 as follows:

L (R)=  69.55 +26.16log10(f)–13.82log10(Hb)+[44.9-6.55log10(Hb)]log(R)  – 4.78(log10 (f))2+18.33 log10 (f) -40.94

where:

- R is the base station-UE separation in kilometres

- f is the carrier frequency in MHz

- Hb is the base station antenna height above ground in metres

In Figure 1 we plot the pathloss statistics as well as the single network throughput. Note that we have assumed the same parameters between the two runs with only changing the underlying pathloss model. It can be observed that the RMa 38.901 model might be too pessimistic and will lead to around of 40% of victim devices being out of coverage. This motivates us to adopt the Hata model given in TR 36.837, which was also used in TR 37.829, that studied HPUE coexistence aspects in LTE and NR bands. Accordingly, table 2 summarizes the simulation assumptions for the 2GHz for the Rural Macro scenario.

Figure 1 Comparison between RMa models in 3GPP TRs

Table 2 Simulation parameters for 2GHz RMa scenario

| Parameters | 2GHz |
| --- | --- |
| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance | 6000m |
| FWA/UE Outdoor/ indoor ratio | 100% outdoor |
| BS antenna height | 45m |
| Channel model | RMa in TR 37.829 |

Proposal 2: RAN4 to adopt Tables 1 and 2 for the simulation assumptions for UMa and RMa scenarios.

Finally, one of the important parameters for the UL simulations is the power control parameters, and specifically, the CLxile values adopted in the simulations. In [2], the following agreement was reached regarding the CLxile:

CLx-ile = [88 + 10*log10(200/X) + 11 – Y + Pmax – 23, where X is UL transmission BW (MHz) and Y is the BS noise figure] Pmax = 32dBm γ = 1

It is important to mention that the CLxile affects the UL transmit power and consequently affect the UL SNR target that the UEs can achieve. For UMa scenarios, we have selected the CLxile to meet an UL SNR target = 15 dB.  Moving to RMa scenarios, it is important first to understand the assumptions and whether the underlying assumptions are not impacting the basic UL coverage of the victim network. It is unrealistic to reuse the same CLxile value for the RMa, given the much larger ISD assumptions for the RMa, compared to the ISD. The WF in [2] does not differentiate between UMa and RMa. In this regard, it is important for RAN4 to agree on RMa assumptions prior discussing ACLR values.

Observation 3: WF from RAN4#116bis does not differentiate between UMa and RMa scenarios when calculating the CLxile.

In order to ensure UL coverage is not an issue, especially for the cell edge users in RMa scenarios, several measures could be considered in the simulation framework, such that SNR target reduction and PSD boosting via lowering the number of allocated PRBs. From our view, in order to have a meaningful UL SNR target, it is a viable option to consider PSD boosting via lowering the number of allocated PRBs along with having an SNR target lower than what was considered for the UMa scenario (i.e., 15 dB). A reasonable SNR target could be between 5 and 10 dB for RMa scenarios.

Proposal 3: If there is UL coverage issues in RMa scenarios, RAN4 to consider SNR target between 5 and 10 dB, along with PSD boosting via lowering the number of allocated PRBs for UL transmission.

# Preliminary simulation results

In this section we present our UMa preliminary simulation results. Our simulation assumptions are summarized in Table 3 below.

Tabel 3 Adjacent channel coexistence simulation parameters

| Category | Parameter | Value |
| --- | --- | --- |
| Network configuration | ISD | Follows Table 1 and 2 in this contribution |
|  | CHBW | 100 MHz |
|  | Propagation model | UMa in 38.901 (Pathloss, LOS probability, and O-to-I penetration loss) RMa in TR 36.837 |
|  | Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
|  | Victim network | TDD UL (UE to BS) with handheld PC3 |
|  | Victim Network Outdoor/Indoor Ratio | 20/80% |
|  | Aggressor network | TDD UL (UE to BS) with FWA (same BW as victim) (we also simulated TDD UL with handheld PC3 to benchmark) |
|  | Multi-operator layout | Uncoordinated (0% Grid Shift) |
|  | Shadowing correlation | Between cells: 1.0, Between sites: 0.5 |
|  | Additional shadowing @ FWA | 0 dB |
|  | FWA/PC3 Ratio | 100/0% as a starting point |
|  | ACLR/ACS | Follows [2] |
|  | Allocated BW to UE | 100 % (i.e., full allocation) |
| gNB configuration | AAS parameters | Follows [2] |
|  | Antenna height | Follows Table 1 and 2 in this contribution |
|  | BS noise figure | 5 dB for 2GHz and 4GHz 6 for 7GHz |

In Table 3, the edge throughput degradation for UMa scenario for the three different frequency ranges with two different aggressor networks is shown. The baseline case, which represent both victim and aggressor network have PC3 legacy devices. The other case where victim network is PC3 devices, while aggressor is FWA devices. Note that we have assumed that 100% of devices in the aggressor network are FWA devices with maximum output power of 32 dBm. We can observe that for 2GHz, and 4GHz, the required ACLR to meet the 5% edge throughput loss is around 31 dB, while for 7GHz, the target ACLR is 28 dB. The relaxation for the 7GHz could be explained due to the higher Rx gain (i.e., BS AAS directivity), since the number of antenna elements for the 7GHz is higher than the C-band frequency ranges. Table 3 quantifies the required ACLR based on our preliminary simulations.

Table 3 Target ACLR for UMa scenarios

| Frequency | Preliminary ACLR [dB] @5% TP loss |  |
| --- | --- | --- |
|  | w/ PC3 aggressor | w/ FWA aggressor |
| 2GHz | 29.6 dB | 30.7 dB |
| 4GHz | 29.9 dB | 30.4 dB |
| 7GHz | 26 dB | 27-28 dB |

For the RMa, Figure 2 presents the throughput degradation (mean and edge performance) for the 2GHz frequency range, based on the parameters given in Table 2. As observed, a similar ballpark ACLR (31.5dB) compared to the UMa is observed. Accordingly, an ACLR of 31-32 dB could be agreed for 2GHz and 4GHz (n4 n78, and n79), while an ACLR of 28 dB could be agreed for n104.

Figure 2 Target ACLR for 2GHz RMa scenarios

Observation 3: Based on our preliminary simulations, an ACLR of 31-32 dB could be agreed for 2GHz and 4GHz (n41, n78, and n79), while an ACLR of 27-28 dB could be agreed for n104.

# Conclusion

In this contribution, we provided our views on the coexistence aspects for the single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations. Our observations and proposals can be summarized as follows:

Proposal 1: RAN4 to categorize the coexistence work to three frequency ranges, namely, 2GHz, 4GHz, and 7 GHz.

Observation 1: It is desirable to align simulation parameters and underlying model in this work item with HPUE coexistence work conducted in TR 37.829.

Proposal 2: RAN4 to adopt Tables 1 and 2 for the simulation assumptions for UMa and RMa scenarios.

Table 1 Simulation parameters for UMa scenarios

| Parameters | 2GHz | 4GHz | 7GHz |
| --- | --- | --- | --- |
| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |  |
| Inter-site distance | 750m | 450m | 450m |
| BS antenna height | 45m | 20m | 20m |
| Channel model | UMa in TR 38.901 |  |  |

Table 2 Simulation parameters for 2GHz RMa scenario

| Parameters | 2GHz |
| --- | --- |
| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance | 6000m |
| FWA/UE Outdoor/ indoor ratio | 100% outdoor |
| BS antenna height | 45m |
| Channel model | RMa in TR 37.829 |

Observation 3: WF from RAN4#116bis does not differentiate between UMa and RMa scenarios when calculating the CLxile.

Proposal 3: If there is UL coverage issues in RMa scenarios, RAN4 to consider SNR target between 5 and 10 dB, along with PSD boosting via lowering the number of allocated PRBs for UL transmission.

Observation 3: Based on our preliminary simulations, an ACLR of 31-32 dB could be agreed for 2GHz and 4GHz (n41, n78, and n79), while an ACLR of 27-28 dB could be agreed for n104.

# References

- RP-252942, New WID: UE RF enhancements for NR FR1, Phase 5, vivo (Moderator), RAN Meeting #109, Beijing, China.

- R4-2515085, “Way Forward for [116bis][329] NR_UE_RF_Ph5_Part2 - HPUE coexistence”, Huawei, HiSilicon, RAN4#116bis, Prague.

- R4-2410576, LS on Parameters for 4400 to 4800 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27, RAN4.