3GPP TSG RAN WG4 Meeting #117														R4-2521912

Dallas, USA, Nov. 17-21, 2025

Source:	Ericsson

Title:	Test configurations for low-band carrier aggregation via switching

Agenda item:	6.3.2

Document for:	Discussion

# 1	Introduction

There was good progress in the last meeting and agreed on the list of test cases for LB CA via switching. In this contribution we discuss the test configuration for low-band carrier aggregation via switching and related test cases.

# 2	Discussion

The list of test cases agreed in the last meeting is presented in the Annexes. In this section, we discuss the test configuration for the LB CA via switching test cases. Some of the agreed test configurations are mentioned below.

Test configurations:

    - General configuration:

      - For test configuration and parameters, existing FR1 with 15kHz SCS test configuration and parameters could be baseline

      - CA between FDD PCell and SDL SCell is configured

      - PCell and SCell are co-located deployed and keep sync between them (Max RTD is 3us)

      - The SCS is 15kHz and the frequencies of PCell and SCell are lower than 1GHz

    - As a baseline, test partially overlapped scenario for LB CA RRM, i.e., SSB/SMTC is partially overlapped with LB-CA switching pattern.

      - Working assumption for TC: SSB/SMTC periodicity is 20ms and switching pattern periodicity is 40ms.

        - The details of switching pattern can be FFS.

      - RAN4 will pick one TC in the list to cover fully overlapping and partially overlapping cases.

As per the agreement, we are expected to test one TC where the SSB/SMTC occasions are fully available on the FDD carrier and not impacted by the switching pattern, while the SDL SCell SSB/SMTC occasions are partially overlapped with the switching pattern, and some occasions are not available due to the switching pattern.

We have illustrated this in Figure 1, which shows an example switching pattern. In the figure, green slots represent the slots available for the respective cell, and grey slots indicate the slots that are not available due to the switching pattern. As per the RAN1 spec, switch pattern is indicated as following. A bit value of ‘0’ for a first slot in the bitmap indicates that the UE can transmit/receive on the PCell in the first slot, and a bit value of ‘1’ for a second slot in the bitmap indicates that the UE can receive on the SCell in the second slot

Figure 1: Example switching pattern for LB CA test case.

To verify, LB CA switching functionality we need to define the switching pattern and the switching gap N1 and N2.

- N1: Configurable between 1 to 3 symbols

- N2: Configurable between 1 to 30 symbols

Switching gap = Switching time + Transient time + MRTD + UL TA

|  | RF switching delay | MRTD | Transient period | NTA offset | TA (18km RTD) | Total blanked symbols |
| --- | --- | --- | --- | --- | --- | --- |
| FDD to SDL (35 µs) | 35 | 3µs | 10µs | 13µs | NA | 1 |
| FDD to SDL (140 µs) | 140µs | 3µs | 10µs | 13µs | NA | 3 |
| SDL to FDD | 35 | 3µs | 10µs | 13µs | 60µs | 2 |
| SDL to FDD | 140 | 3µs | 10µs | 13µs | 60µs | 4 |

Since different UE may support different RF switching delay, we think we should have different N1 and N2 values for the UE supporting 35µs and 140µs RF switching delay.

- RAN4 to agree following parameters for the switching pattern.

| Parameter | Value |
| --- | --- |
| Switching pattern | 0000001111111110000000011100000111111000 |
| N1 | 35 µs RF switching delay UE, 1 140 µs RF switching delay UE, 3 |
| N2 | 35 µs RF switching delay UE, 2 140 µs RF switching delay UE, 4 |

To ensure that SSB/SMTC occasions are fully available on the FDD carrier and not impacted by the switching pattern, while the SDL SCell SSB/SMTC occasions are partially overlapped and some occasions are not available due to the switching pattern, we believe that the SDL SSB transmission should not be overlapped with the FDD PCell and shall be placed in the second half of the radio frame.

However, upon reviewing the SSB patterns defined in Clause A.3, we observe that no existing SSB pattern starts in the second half of the radio frame. Therefore, we propose to define a new SSB pattern that enables SDL SSB occasions to occur in the second half of the frame, allowing for partial overlap with the switching pattern while keeping FDD SSB occasions unaffected. Similarly, we did not find the SMTC pattern with 1ms duration and offset of 5 (that means the second half of the radio frame) is not found and hence we propose to define new SMTC pattern.

- RAN4 to agree on following SSB pattern 14 fort he LB CA via switching test cases.

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 10 MHz |
| SSB SCS | 15 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 2 | 2-5 |
| Slot numbers containing SSB Note 2 | 5 |
| SFN containing SSB | SFN mod (max(TSSB,10 ms)/10 ms) = 0 |
| PRB numbers containing SSB within channel BW | (RBJ, PRBJ+1,.…, PRBJ+19)Note 1 |
| NOTE 1: PRBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. NOTE 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

- RAN4 to agree on following SMTC pattern 13 fort he LB CA via switching test cases

| SMTC Parameters | Values |
| --- | --- |
| SMTC periodicity | 20 ms |
| SMTC offset | 5 ms |
| SMTC duration | 1 ms |

How do we test the switching gap:

We think the switching mechanism is tested by counting the percentage of HARQ NACK for both the PCell and SCell. The HARQ NACK percentage observed during the test shall not be lower than the HARQ NACK percentage measured for normal LB CA operation without the switching mechanism. To make sure we get enough HARQ Ack/NACKs fort he is counting, we suggest UEs are scheduled in every slot throughout the test time. The percentage of HARQ NACKs allowed can be set by RAN5.

- Switching gap duration is verified by counting the HARQ NACK percentage. The HARQ NACK percentage observed during the test shall not be lower than the HARQ NACK percentage measured for normal LB CA operation without the switching mechanism. Exact HARQ NACK percentage can be set by RAN5.

Events to use for Intra-frequency L3 measurement for activated SDL SCell or PCell test:

Since this is a collocation deployment and close by frequency bands, we suggest using event A3 for PCell and event A6 for the SCell.

Event A3: neighbour cell becoming an offset better than serving cell

Event A6: neighbour cell becoming offset better than SCell

This is particularly useful as the same neighbour cell can be used to trigger the measurement report on both PCell and SCell.

- For Intra-frequency L3 measurement for activated SDL SCell or PCell test, use event A3 and A6 in the test configuration.

# 3 	Summary

In this contribution we analyzed the performance requirements of low-band CA via switching and made the following observations and proposals.

- RAN4 to agree following parameters for the switching pattern.

| Parameter | Value |
| --- | --- |
| Switching pattern | 0000001111111110000000011100000111111000 |
| N1 | For a UE supporting RF switching delay of 35 µs, 1 For a UE supporting RF switching delay of 35 µs, 140 µs, 3 |
| N2 | For a UE supporting RF switching delay of 35 µs, 2 For a UE supporting RF switching delay of 35 µs, 140 µs, 4 |

- RAN4 to agree on following SSB pattern 14 fort he LB CA via switching test cases.

| SSB Parameters | Values |
| --- | --- |
| Channel bandwidth | 10 MHz |
| SSB SCS | 15 kHz |
| SSB periodicity (TSSB) | 20 ms |
| Number of SSBs per SS-burst | 1 |
| SS/PBCH block index | 0 |
| Symbol numbers containing SSB Note 2 | 2-5 |
| Slot numbers containing SSB Note 2 | 5 |
| SFN containing SSB | SFN mod (max(TSSB,10 ms)/10 ms) = 0 |
| PRB numbers containing SSB within channel BW | (RBJ, PRBJ+1,.…, PRBJ+19)Note 1 |
| NOTE 1: PRBs containing SSB can be configured in any frequency location within the cell bandwidth according to the allowed synchronization raster defined in TS 38.104 [13]. NOTE 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |

- RAN4 to agree on following SMTC pattern 13 fort he LB CA via switching test cases

| SMTC Parameters | Values |
| --- | --- |
| SMTC periodicity | 20 ms |
| SMTC offset | 5 ms |
| SMTC duration | 1 ms |

- Switching gap duration is verified by counting the HARQ NACK percentage. The HARQ NACK percentage observed during the test shall not be lower than the HARQ NACK percentage measured for normal LB CA operation without the switching mechanism. Exact HARQ NACK percentage can be set by RAN5.

- For Intra-frequency L3 measurement for activated SDL SCell or PCell test, use event A3 and A6 in the test configuration.

# References

[1] 		RP-251315, Revised WID on Low NR band carrier aggregation via switching, TELUS

# Annexes

| TC Index | Requirement | RS used for test | Target Cell for testing | Support this TC | Leading Company |
| --- | --- | --- | --- | --- | --- |
| 1 | Intra-frequency L3 measurement for activated SDL SCell or PCell | SSB | Both PCell and SCell in the same TC PCell and SCell can be configured with different events, details can be FFS. PCell is in partially overlapped case. SCell is in fully overlapping case. |  | Ericsson |
| 2 | Deactivated SDL SCell measurement and interruption requirement | SSB | Verify measurement requirement on SDL SCell. And in the same TC verify interruption requirement to the PCell. |  | MTK |
| 3 | SDL SCell activation/deactivation | SSB (for sync and L1-RSRP) and CSI-RS for CQI | SDL SCell. Only MAC CE based SCell activation is verified. |  | Nokia |
| 4 | BFD/CBD for SDL SCell | Option 1: SSB (Apple) Option 2: CSI-RS Option 3: both | Option 1: SCell only (Apple) Option 2: Both PCell and SCell (OPPO, HW, ZTE, Ericsson) [Ericsson]: If option 2 is adopted, test PCell/SCell in the different test. | FFS whether this TC shall be introduced. |  |
| 6 | PL-RS switch delay for PCell | Option 1: SSB (Apple) Option 2: CSI-RS Option 3: both | FDD PCell (Apple, OPPO, ZTE, Ericsson) | FFS whether this TC shall be introduced. |  |
| 7 | L1-RSRP for SDL SCell | SSB | SCell only |  | Huawei |
| 8 | RLM requirement for PCell | SSB | FDD PCell |  | OPPO |