3GPP TSG-RAN WG4 Meeting #116bis	R4-2521926

# Dallas, USA, Nov. 17-21, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on RRM aspects of R20 Low band carrier aggregation

via switching

Agenda Item:	7.8.4

Document for:	Discussion

- Introduction

In latest RAN#109 meeting, a new R20 WID of Low band carrier aggregation via switching was approved in [1] and led by RAN4. The objectives of this new WID are captured as below:

4.1 Objective of SI or Core part WI or Testing part WI The core part of WI: - Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible. 4.2 Objective of Performance part WI The performance part of WI: - Specify RRM test case(s) corresponding to the RRM core requirements.

This WID intends to support some low bands CA through switching between FDD band and SDL band such as n12+n29 and n28+n67. Although such band combinations are already specified in TS38.101-1, this practical implementation challenge may pose a barrier for the combination’s adoption in a wide range of devices. As a result the solution of switching between the two bands is necessary to facilitate CA with such band combinations.

The successful experience in the development of R19 low band CA could provide good reference for this new WID. It could be seen that better downlink performance is foreseen via enabling dual DL in this new WID compared with R19 WID. In last meeting, some initial progress were achieved and summarized in [2]. We provide our further analysis on this new WID in this paper.

- Discussion

2.1 Band combinations for low band CA

As it is noted in the WID, to support the low band CA through switching, specify requirements for the feature with the example band combination CA_n12A-n29A and CA_n28A-n67A, and with additional band combinations to be handled via the basket work item approach.

So before diving into the RRM aspect analysis, it is necessary to overview these example band combinations and compare with the typical band combination(CA_n5A-n29A) of R19 low band CA, so that to determine whether and how to leverage experience from R19 low band CA.

The following band combinations are proposed as potential scenarios to facilitate the deployment of low band CA through switching solution in [3]:

It can been seen that different from CA_n5A-n29A, why need to support CA through switching for CA_n12A-n29A and CA_n28A-n67A, mainly lies in the bottleneck of duplexer. Since the frequency of UL carrier in FDD cell is quite close to or even overlapping with the frequency of SDL cell, it is hard for the duplexer to accommodate both SDL carrier and the UL carrier in parallel. As a result, the switching solution is needed to facilitate the CA between such two bands. On the other hand, since the frequency of UL carrier in FDD cell and the SDL carrier are close to each other, the Rx antenna could serve the two DL carriers simultaneously, so even the switching solution is utilized, dual DL carriers are allowed in parallel.

Based on the analysis above, one major difference from R19 low band CA is dual Rx in two DL carriers  allowed in R20 case, so the switching happens between the below two statuses:

- Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

- Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1

2.2 RRM impact analysis

In last meeting, it has been approved that the applicability of switching pattern defined in R19 LB CA is fully reused for R20 LB CA, i.e. the switching pattern is applicable for the phase of SCell activation procedure and activated SCell. During the phase of deactivated SCell, UE is assumed as dwelling on the PCell. Based on this framework, we discuss the RRM impacts on both SCell and PCell.

2.2.1 Timing

Drawing the experience achieved in R19 LBCA, both the transmission timing for PCell and the MRTD between PCell and SCell should be identified for R20 LB CA as well.

Transmission timing for PCell

Regarding the transmission timing for PCell, after some analysis and discussion, it is decided to still reuse the legacy requirements as well as the prerequisite.

Impact on timing requirement for PCell(R19 LB CA) - Agreement in AH: - the existing Tx timing requirement for PCell with the same applicability condition (SSB is available in last 160ms) can apply for R19 LB CA via switching.

Based on specification in clause 7.1.1 in 38.133 as below, the reference cell used for providing DL frame could only be the SpCell for pTAG. So for the FDD plus SDL band combination, the FDD cell(PCell) is always the reference cell.

7.1 UE transmit timing 7.1.1 Introduction The UE shall have capability to follow the frame timing change of the reference cell in connected state or when transmiting PUSCH on CG resources for SDT in RRC_Inactive. The uplink frame transmission takes place before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. For serving cell(s) in pTAG, UE shall use the SpCell as the reference cell for deriving the UE transmit timing for cells in the pTAG. For serving cell(s) in sTAG, UE shall use any of the activated SCells as the reference cell for deriving the UE transmit timing for the cells in the sTAG. UE initial transmit timing accuracy and gradual timing adjustment requirements are defined in the following requirements. ... 7.1.2 Requirements - ... When the UL SCS is 120 kHz or smaller, the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. When the UL SCS is 480 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 80 ms. When the UL SCS is 960 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 40 ms. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus NTA+NTA offset×Tc. The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame used by the UE to determine downlink timing is received from the reference cell at the UE antenna. NTA for PRACH is defined as 0. - ...

Considering the DL carrier of PCell is always on only except the duration of switching gap, similar as legacy normal CA, no challenge for UE to receive SSB in last 160ms at the DL carrier of PCell, so similar as R19 LB CA, reuse the existing Tx timing requirement for PCell with the same prerequisite, i.e. SSB is available in last 160ms.

Proposal 1: Reuse the existing Tx timing requirement for PCell with the same prerequisite(SSB is available in last 160ms).

MRTD

The deployment of R20 LB CA is similar as R19 LB CA, co-located and synchronized network deployment for both carriers, and both carriers are in a single TAG. So it is straightforward to at least leverage the MRTD requirement defined in R19 LB CA as below, i.e. the maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us.

Agreement on Monday AH:(R19 LB CA) - The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us. - Whether or not above RTD can be used as side condition for SCell activation will be discussed in another issue.

Proposal 2: Reuse the MRTD requirement specified in R19 low band CA via switching.

2.2.2 SDL SCell related RRM requirements

Around the SDL SCell related RRM aspects, the progress was achieved in last meeting as below:

Agreement: - For L3 measurement and L1 measurement on activated SDL SCell, reuse the Rel-19 requirement as starting point. - For TCI state switch delay requirements, reuse the Rel-19 requirement as starting point. - For SDL SCell activation/deactivation delay requirements, reuse the Rel-19 requirement as starting point. - No new RRM requirement is expected for deactivated SCell measurement period requirements

So no need to further discuss the deactivated SCell measurement, we focus on the SCell activation/deactivation procedure and activated SCell measurement respectively.

SCell activation/deactivation

The switching pattern is applicable during the SCell activation procedure. We depict the switching between Case 1 and Case 3 for R 20 low band CA as Figure 1, and compared with the switching between Case 1 and Case 2 in R19 low band CA shown in Figure 2, the main difference lies in whether the FDD DL carrier is available during the period of SDL carrier ON. It can be seen that in Figure 1, even during the SDL carrier ON, PCell DL carrier is still applicable, so the legacy interruption on PCell should be reconsidered.

Regarding the enabling/disabling switching pattern, we believe all R19 agreements could be reused, including the SCell which is in activation procedure, deactivation procedure. There are still suspending issues are on-going discussing in R19 maintenance phase, we could wait and leverage the final conclusion.

Proposal 3 Reuse the application time of enabling/disabling the switching pattern defined in R19 LB CA.

Figure 1. Switching between Case 1 and Case 3 for R20 low band CA

Figure 2. Switching between Case 1 and Case 2 for R19 low band CA

In legacy, an interruption window on PCell and other active SCell was identified due to the target SCell activation/deactivation. The actual interruption is limited in the interruption window, the starting and ending of the interruption window are defined in clause 8.3 of TS 38.133. The interruption length is defined in clause 8.2.2.2.2 of TS 38.133.

When UE receives the SDL SCell activation command, based on the consensus on enabling of switching pattern achieved in R19 maintenance, UE apply the switching pattern at slot n+(THARQ+3ms+1ms)/slot-length for MAC CE based SCell activation or slot n+TRRC_Process /slot-length for RRC based direct SCell activation. After UE could apply the switching pattern, UE would judge whether it is during the PCell ON period(T1) or SDL SCell ON period(T2) currently. If it is during PCell ON period(T1), no interruption needed since UE has already dwell on PCell, and UE would not retune the RF to SCell until the switching gap. If it is during SCell ON period(T2), UE has to retune the duplexer/RF from PCell DL+UL to PCell DL+ SDL, which leads to interruption on SCell.

Proposal 4: Whether the SCell activation would lead to interruption,

-  The SCell activation would not lead to interruption if UE enables the switching pattern triggered by SCell activation command during PCell ON period.

-  The SCell activation would lead to interruption if UE enable the switching pattern triggered by SCell activation command during SCell ON period.

For the SCell deactivation, when we look at the above Figure 1, it is possible that the SDL SCell deactivation command is received or timer expires in the PCell ON duration(T1) or the SDL carrier ON duration(T2). Based on the consensus on disabling of the switching pattern achieved in R19 maintenance, for MAC CE triggered SCell deactivation, UE successfully loads and reads the switching pattern until slot n+ ((THARQ+3ms+1ms)/slot-length). For the timer based deactivation, still suspending. After UE loads and reads the switching pattern, UE needs to dwelll on PCell DL+UL since the SCell is deactivated. Similar as above analysis on the case of SCell activation, which also depends on currently it is in T1 or T2.

Proposal 5: Whether the SCell deactivation would lead to interruption,

-  The SCell deactivation would not lead to interruption if UE disables the switching pattern triggered by MAC CE or timer during PCell ON period.

-  The SCell deactivation would lead to interruption if UE disables the switching pattern triggered by MAC CE or timer during SCell ON period.

Another issue regarding the SDL SCell activation is whether to consider SSB-less operation. During the initial phase of R19 LB CA, the applicability of SSB-less was touched, while due to very limited TU and to control the workload, finally the R19 LB CA is assumed as SSB based. While in R20 LB CA, since we could leverage much experience from R19 and the TU allocated to R20 LB CA is kind of rich, we believe the support of SSB-less operation could be considered.

It should be noted that the side condition of RTD for SSB-less operation could reuse the existing requirement, i.e. the RTD between the SCell and the collocated PCell is within CP where CP is corresponding to the SCS of SSB-less SCell. For 15kHz SCS, the CP length is about 4.7us which is larger than 3us, so we can not see any challenge to support SSB-less operation for SDL SCell activation.

Proposal 6: Inter-band SSB-less operation could be considered for the SDL SCell activation to facilitate more efficient SCell activation.

Activated SDL SCell measurement

Both L3 and L1 measurements should be discussed. Both gap based and gap less L3 measurements are involved in the activated SDL SCell measurement.

For the gap based L3 measurement, similar as R19 LB CA, the legacy requirements of intra-frequency, inter-frequency and inter-RAT measurements could be reused for R20 LB CA.

For the gap-less L3 measurement, the methodology of Kp based measurement extension could be reused for R20 LB CA to address the case of partial available measurement occasion.

Proposal 7: For activated SDL SCell L3 measurement, reuse the requirements of gap based and gap less L3 measurements defined in R19 LB CA .

For the L1 measurement, both SSB based and CSI-RS based L1 RSRP/L1 SINR/BFD/CBD measurement/evaluation have been identified and reached consensus on the exact requirements in R19 LB CA. Similarly as L3 measurement, the scaling factor based measurement extension is applied to address the partial available measurement occasion issue. All these requirements could be reused for R20 LB CA.

Proposal 8: For activated SDL SCell L1 measurement, reuse the requirements of L1 RSRP/L1 SINR/BFD/CBD defined in R19 LB CA.

2.2.3 PCell related RRM requirements

For PCell measurement, it is kind of different from the R19 LB CA. As our analysis in section 2.1, even the switching solution is utilized, dual DL carriers are allowed in parallel. So no matter in the PCell ON period or SCell O period, PCell DL is always available, the PCell DL is only interrupted during the switching gap.

Observation 1: No matter in PCell ON period or SCell ON period, PCell DL carrier is always available only except the switching gap.

Starting from such observation, for PCell L3 gap-less measurement and L1 measurement, they are only impacted by the switching gap. As a result, the methodology of scaling factor based measurement extension could be reused but the details is different from R19 LB CA, the unavailable measurement occasions are the occasions colliding with the switching gap.

Proposal 9: For PCell L3 gap-less and L1 measurements, reuse the methodology of scaling factor based measurement extension, the unavailable measurement occasions are the occasions colliding with the switching gap.

- Conclusion

In this contribution, we have the following proposals on the RRM enhancements:

- For timing:

Proposal 1: Reuse the existing Tx timing requirement for PCell with the same prerequisite(SSB is available in last 160ms).

Proposal 2: Reuse the MRTD requirement specified in R19 low band CA via switching.

- For SDL SCell activation/deactivation:

Proposal 3 Reuse the application time of enabling/disabling the switching pattern defined in R19 LB CA.

Proposal 4: Whether the SCell activation would lead to interruption,

-  The SCell activation would not lead to interruption if UE enables the switching pattern triggered by SCell activation command during PCell ON period.

-  The SCell activation would lead to interruption if UE enable the switching pattern triggered by SCell activation command during SCell ON period.

Proposal 5: Whether the SCell deactivation would lead to interruption,

-  The SCell deactivation would not lead to interruption if UE disables the switching pattern triggered by MAC CE or timer during PCell ON period.

-  The SCell deactivation would lead to interruption if UE disables the switching pattern triggered by MAC CE or timer during SCell ON period.

Proposal 6: Inter-band SSB-less operation could be considered for the SDL SCell activation to facilitate more efficient SCell activation.

For activated SDL SCell measurement:

Proposal 7: For activated SDL SCell L3 measurement, reuse the requirements of gap based and gap less L3 measurements defined in R19 LB CA .

Proposal 8: For activated SDL SCell L1 measurement, reuse the requirements of L1 RSRP/L1 SINR/BFD/CBD defined in R19 LB CA.

For PCell measurement:

Observation 1: No matter in PCell ON period or SCell ON period, PCell DL carrier is always available only except the switching gap.

Proposal 9: For PCell L3 gap-less and L1 measurements, reuse the methodology of scaling factor based measurement extension, the unavailable measurement occasions are the occasions colliding with the switching gap.

- Reference

- RP-252953, New WID on enhancement of low NR band carrier aggregation via switching, Apple, AT&T, Anterix, Bell Mobility, BT plc, CATT, China Telecom, Ericsson, HiSilicon, Huawei, Intel Corporation, LGE, MediaTek, Murata, Nokia, Ofinno, OPPO, Qualcomm, Samsung, Spreadtrum, Skyworks Solutions Inc., TELUS, UNISOC, vivo, Xiaomi, ZTE

- R4-2514824, WF on NR_LBCA_Sw_enh_RRM, Samsung, RAN4#116bis meeting

- RP-242532, Motivation for a new work item on low band carrier aggregation via switching in Rel-19, AT&T, Anterix, Apple, Bell Mobility, Boost Mobile Network, BT plc, Ericsson, Google, Huawei, Murata, Nokia, OPPO, Qorvo, Qualcomm, Skyworks Solutions Inc., Sony, Southern Linc, TELUS, vivo, ZTE