# 3GPP TSG-RAN WG4 Meeting #117                                                                    R4-2521999

# Dallas, US,  November 17 - 21, 2025

Source:		Ericsson

Title:			RedCap UE RF requirment impact

Agenda item:		7.4.3.1

Document for:		Approval

# Introduction

In this paper, we present our general overview on (e)RedCap RF impact analysis.

# Discussion

In WF[1], the below remaining issues are stated:

Issue 4-1: Whether 3MHz UE CBW should be optional or mandatory in Rel-20 for (e)Redcap

FFS whether 3MHz should be optional or mandatory in Rel-20 for (e)Redcap

Issue 4-2: Release independent of 3MHz UE CBW for (e)Redcap

It is agreed 3MHz UE CBW is release independent from Rel-18 for (e)RedCap UE.

Issue 4-3: UE capability

FFS in next meeting what UE capabilities of (e)Redcap need to be updated to support 3MHz and sent LS to RAN2.

For issue 4-1, the channel bandwidth listed in clause 5.3.5 in TS 38.101-1 are mandatory unless indicated as optional according to the 38.306.  3MHz channel bandwidth for band n5, n12, n26, n28, n31, n72 ,n85 and n100 are optional while n87, n88, n106 and n110 are mandatory. This is for non-RedCap UE.  For (e)RedCap UE, we think the same rule should be applied.

For (e)Redcap UE, follow the non-Redcap UE to report the mandatory/optional support 3MHz, according to the clause 5.3.5 in TS 38.101-1.

For release independent issue 4-2, as stated in 38.307, the release independent for (e)RedCap UE is from Rel-17 for RedCap and Rel-18 for eRedCap UE:

Table 5.9-1 [2]: NR (e)RedCap operating band

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| (e)Redcap operating band | FDD | Rel-171 Rel-182 | Table B.4.18-1 |
| NOTE 1: This is applied to RedCap UE. NOTE 2: This is applied to eRedCap UE. |  |  |  |

And also regarding the 3MHz channel bandwidth support on Non-RedCap UE, it is Rel-18 stated in 38.307:

Table 5.1-3[2]: NR channel bandwidths

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the channel bandwidth was introduced) |
| --- | --- | --- | --- |
| NR channel bandwidths, all unless otherwise stated | FDD, TDD, SDL, SUL | Rel-15 | Table B.4.1-1, Table B.4.3-1 |
| Channel bandwidth of 3 MHz | FDD | Rel-18 | Table B.4.1-1 |
| Channel bandwidth of 40MHz for band n281 | FDD | Rel-16 | Table B.4.1-1 |
| Note 1: If UE reports the capability of support of 40MHz for band n28, the UE shall support the enhanced channel raster capability for n28. |  |  |  |

For (e) RedCap UE support the 3MHz channel bandwidth, the release independent could be earliest from Rel-18. However, there is RAN plenary decision that the (e) RedCap is not supported for 3MHz in Rel-18. This means 3MHz CBW support cannot be from Rel-18 due to this decision. In last meeting, RRM forum also discuss the release independent issue. As RRM has more TU and meeting cycles than RF, we suggest moving the release independent issue to RRM forum. There is also a need to update the 38.307 to add the additional table for the channel bandwidth of 3MHz release independent for (e) RedCap.

## 5.9	Additional operating bands and channel bandwidth for (e)Redcap for NR frequency range 1

Requirements for a (e)Redcap UE for additional operating bands within FR1 of TS 38.101-1 in Rel-P [2] are introduced via this clause.

Table 5.9-1: NR (e)RedCap operating band

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| (e)Redcap operating band | FDD | Rel-171 Rel-182 | Table B.4.18-1 |
| NOTE 1: This is applied to RedCap UE. NOTE 2: This is applied to eRedCap UE. |  |  |  |

Table 5.9-2: NR (e)RedCap channel bandwidth

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the channel bandwidth was introduced) |
| --- | --- | --- | --- |
| (e)Redcap Channel bandwidth of 3 MHz | FDD | [Rel-X]1 | Table B.4.18-1 |
| NOTE 1: This is applied to RedCap UE and eRedCap UE. |  |  |  |

For (e) RedCap UE support the 3MHz channel bandwidth, move the release independent to RRM forum and potential update the 38.307 to reflect the consensus later on.

As 3MHz channel bandwidth is introduced in Rel-20 for (e) RedCap UE, there will be Rel-17 RedCap and Rel-18 eRedCap UE without supporting 3MHz CBW deployed in network. Therefore, additional capability signalling will be needed for the (e) RedCap UE to report the 3MHz support in the bands where 3MHz CBW is optional.

For TN non-RedCap UE, there are two UE capability relating to support of 3MHz, however, these two signalling are not applicable to (e) RedCap UEs.  As the needed capability is not unique for RF, we suggest moving the capability to RRM forum and single LS from RAN4 to RAN2.

The capability signalling is needed but suggest moving the capability discussion to RRM as the capability is not unique for RF.

| support3MHz-ChannelBW-Asymmetric-r18 Indicates whether the UE supports 3 MHz channel bandwidth in uplink with larger than 3 MHz channel BW in DL, including short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS. This feature is supported for 15kHz SCS only (except for the PRACH formats with 1.25kHz SCS). It applies to bands where the UE indicates support for asymmetricBandwidthCombinationSet with 3 MHz UL according to clause 5.3.6 of TS 38.101-1 [2] for TN or clause 5.3.6 of TS 38.101-5 [34] for NTN. For NTN bands, a UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE 1: The UE supporting this feature supports configuration of 15 PRB UL BWP operation. NOTE 2: If the UE indicates support in asymmetricBandwidthCombinationSet for a 3MHz UL in a band according to clause 5.3.6 of 38.101-1 [2] for TN or clause 5.3.6 of 38.101-5 [34] for NTN, this feature shall be indicated for the band. | Band | No | FDD only | FR1 only |
| --- | --- | --- | --- | --- |
| support3MHz-ChannelBW-Symmetric-r18 Indicates whether the UE supports 3 MHz symmetric channel bandwidth in DL and UL, including the following functional components: - Reception of 12 PRB PBCH based on RB-level puncturing; - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS; - Reception of 15 PRB CORESET0. This feature is supported for 15kHz SCS only (except for the PRACH formats with 1.25kHz SCS). It is applicable when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in TS 38.101-1 [2] for TN or Table 5.4.3.3-1a of TS 38.101-5 [34] for NTN. For NTN bands, a UE supporting this feature shall also indicate the support of nonTerrestrialNetwork-r17. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 15 PRB BWP operation in DL and UL. | Band | No | FDD only | FR1 only |

# Conclusions

In this contribution, we present our general overview for 3Mhz (e)RedCap RF impact observations and proposals:

Proposal-1: For (e)Redcap UE, follow the non-Redcap UE to report the mandatory/optional support 3MHz, according to the clause 5.3.5 in TS 38.101-1.

Proposal-2: For (e) RedCap UE support the 3MHz channel bandwidth, move the release independent to RRM forum and potential update the 38.307 to reflect the consensus later on.

Proposal-3:The capability signalling is needed but suggest moving the capability discussion to RRM as the capability is not unique for RF.

# References

- R4-2515101,	Way Forward for [116bis][323] NR_less_than_5MHz_RedCap_UERF, Sony

- 38.307