3GPP TSG-RAN WG4 Meeting #117                                                                    R4-2522002

Dallas, US,  November 17 - 21, 2025

Source:		Ericsson

Title:			A-IoT coexisting simulation assumptions

Agenda item:		7.11.3

Document for:		Approval

# Introduction

In this paper, we present our views on Rel-20 A-IoT coexisting simulation assumptions.

# Discussion

There is open issue in WF[1] for performance metric:

Issue 3-1: performance metric

Agreement:

- For calibration purpose, consider coupling loss, SINR with ACI, and SINR without ACI

- For formal coexistence simulation,

  - Option 1: Use SINR degradation of [1dB]

  - Option 2: Use SNR from LLS

In Rel-19 coexisting study, the SNR degradation is used as performance metric due to no waveform design available. Considering the D2R and R2D waveform available from the Rel-19, RAN4 can use the SNR instead of SNR degradation and use the detection performance instead of the throughput in Rel-20 coexisting. Currently RAN1 is evaluating the link budget so SNR is simulated by companies.

The are two error case when using SINR degradation instead of the SNR decoding threshold. For first case,  if UE has SINR degradation of more than 1 dB but the SINR is greater than the SNR decoding threshold, there is no coexisting issue. For second case, if the SINR degradation is less than 1 dB but SINR is less than decoding threshold, UE cannot decode the signal.  These two error cases will be counted if only SINR is used as performance metric.

Last meeting, there are companies’ concerns that to align with the SNR threshold in RAN4 will add additional work load in group. We recognize this and suggest that each company can report their own SNR threshold as additional metric for coexisting, in addition to the SINR degradation.

Use the SNR from LLS  and SINR degradation as coexisting metric instead of the SNR degradation alone in Rel-20.

# Conclusions

In this contribution, we present our overview on the A-IoT coexisting simulation assumptions with below observation and proposals:

Proposal-1: Use the SNR from LLS  and SINR degradation as coexisting metric instead of the SNR degradation alone in Rel-20.

# References

- R4-2515152, WF for [116bis][321] Study of A-IoT for outdoor, LG