3GPP TSG-RAN WG4 Meeting #117                                                                    R4-2522004

Dallas, US,  November 17 - 21, 2025

Source:		Ericsson

Title:			TP to TS 38.195 for Receiver OOB, Spurious and intermodulation

Agenda item:		6.14.2

Document for:		Approval

# Introduction

In this paper, we present our TP for TS 38.195 for Rel-19 A-IoT conformance testing.

# Discussion

TP start here:

## 7.4	Out-of-band blocking

### 7.4.1	Definition and applicability

The out-of-band blocking characteristics is a measure of the receiver ability to receive a wanted signal at its assigned channel at the antenna connector for BS type 1-C in the presence of an unwanted interferer out of the operating band, which is a CW signal for out-of-band blocking.

### 7.4.2	Minimum requirement

The minimum requirements for BS type 1-C are in TS 38.194 [2], clause 7.4.2.

### 7.4.3	Test purpose

To verify that the BS type 1-C receiver dynamic range, the BLER performance shall fulfil the specified limit.

### 7.4.4	Method of test

#### 7.4.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC):

-	M; see clause [4.9.1]

#### 7.4.4.2	Procedure

The minimum requirement is applied to all connectors under test.

1)	Connect the connector under test to measurement equipment as shown in annex [D.2.5] for BS type 1-C.

2)	Set the signal generator for the wanted signal as defined in clause 7.4.5 to transmit as specified in table 7.4.5.1-1.

4)	Set the Signal generator for the interfering signal to transmit at the frequency offset and as specified in table 7.4.5.1-1. The CW interfering signal shall be swept with a step size of 1 MHz over than range 1 MHz to (FUL_low - ΔfOOB) MHz and (FUL_high + ΔfOOB) MHz to 12750 MHz.

5)	Measure the BLER performance according to annex A.1.

### 7.4.5	Test requirements

#### 7.4.5.1	General requirements

the BLER performance shall be 10% of the reference measurement channel as specified in annex A.1, with a wanted and an interfering signal coupled to BS type 1-C antenna connector using the parameters in table 7.4.5.1-1.

The reference measurement channel for the wanted signal is identified in clause 7.2.2 for each BS channel bandwidth and further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The out-of-band blocking requirement apply from 1 MHz to FUL,low - ΔfOOB and from FUL,high + ΔfOOB up to 12750 MHz, including the downlink frequency range of the FDD operating band for BS supporting FDD. The ΔfOOB for BS type 1-C is defined in table 7.3.2.2-0.

Minimum conducted requirement is defined at the antenna connector for BS type 1-C.

Table 7.4.5.1-1: Out-of-band blocking performance requirement for NR

| Wanted Signal mean power (dBm) | Interfering Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- |
| PREFSENS +6 dB (Note) | -15 | CW carrier |
| NOTE : PREFSENS depends also on the BS channel bandwidth as specified in Table 7.2.2-1. |  |  |

## 7.5	Receiver spurious emissions

### 7.5.1	Definition and applicability

The receiver spurious emissions power is the power of emissions generated or amplified in a receiver unit that appear at the antenna connector (for BS type 1-C). The requirements apply to all BS with separate RX and TX antenna connectors.

NOTE 1:	In normal operating condition, A-IoT BS is configured as HD-FDD operation.

### 7.5.2	Minimum requirement

The minimum requirements for BS type 1-C are in TS 38.194 [x], clause 7.5.2.

### 7.5.3	Test purpose

The test purpose is to verify the ability of the BS to limit the interference caused by receiver spurious emissions to other systems.

### 7.5.4	Method of test

#### 7.5.4.1	Initial conditions

Test environment: Normal; see annex [B.2].

RF channels to be tested for single carrier: M; see clause [4.9.1].

#### 7.5.4.2	Procedure

The minimum requirement is applied to all connectors under test,

1)	Connect the connector under test to measurement equipment as shown in annex [D.2.6] for BS type 1-C.

2)	Set the measurement equipment parameters as specified in Table 7.5.5.1-1.

The measurement device characteristics shall be:

-	Detection mode: True RMS.

The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in [Table 4.1.2.3-1].

4)	Measure the spurious emissions over each frequency range described in Table 7.5.5.1-1.

### 7.5.5	Test requirements

#### 7.5.5.1	Basic limits

The RX spurious emissions requirements for BS type 1-C are that for each antenna connector, the power of emissions shall not exceed basic limits specified in table 7.5.5.1-1.

Table 7.5.5.1-1: General BS receiver spurious emissions limits

| Spurious frequency range | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -57 dBm | 100 kHz | Note 1 |
| 1 GHz – 12.75 GHz | -47 dBm | 1 MHz | Note 1, Note 2 |
| NOTE 1: Measurement bandwidths as in ITU-R SM.329 [2], s4.1. NOTE 2: Upper frequency as in ITU-R SM.329 [2], s2.5 table 1. NOTE 3: Void NOTE 4: The frequency range from ΔfOBUE below the lowest frequency of the BS transmitter operating band to ΔfOBUE above the highest frequency of the BS transmitter operating band may be excluded from the requirement. ΔfOBUE is defined in clause 6.6.1. For multi-band connectors, the exclusion applies for all supported operating bands. NOTE 5: Void |  |  |  |

## 7.6	Receiver intermodulation

### 7.7.1	Definition and applicability

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency at the antenna connector for BS type 1-C in the presence of two interfering signals which have a specific frequency relationship to the wanted signal.

### 7.7.2	Minimum requirement

The minimum requirements for BS type 1-C are in TS 38.194 [2], clause 7.6.2.

### 7.7.3	Test purpose

The test purpose is to verify the ability of the BS receiver to inhibit the generation of intermodulation products in its non-linear elements caused by the presence of two high-level interfering signals at frequencies with a specific relationship to the frequency of the wanted signal.

### 7.7.4	Method of test

#### 7.7.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier (SC): M; see clause [4.9.1]

#### 7.7.4.2	Procedure

The minimum requirement is applied to all connectors under test.

1)	Connect the connector under test to measurement equipment as shown in annex D.2.7 for BS type 1-C.

2)	Set the signal generator for the wanted signal to transmit as specified in table 7.7.5-1.

3)	Set the signal generator for the interfering signal to transmit at the frequency offset and as specified in table 7.7.5-1.

4)	Measure the BLER performance according to annex A.1.

### 7.7.5	Test requirements

The BLER performance shall be 10% of the reference measurement channel as specified in annex A.1, with a wanted signal at the assigned channel frequency and two interfering signals coupled to the BS type 1-C antenna connector, with the conditions specified in Tables 7.7.5-1 for narrowband intermodulation performance. The reference measurement channel for the wanted signal is identified in tables 7.2.2-1 for each BS channel bandwidth and further specified in annex A.1. The characteristics of the interfering signal is further specified in annex D.

The receiver intermodulation requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth edges. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges.

Table 7.7.5-1: Narrowband intermodulation performance requirement for A-IoT Medium Range BS

| Channel bandwidth of the lowest/highest carrier received [kHz] | Wanted signal mean power [dBm] | Interfering signal mean power [dBm] | Interfering RB centre frequency offset from the lower/upper Base Station RF Bandwidth edg [kHz] | Type of interfering signal |
| --- | --- | --- | --- | --- |
| 200 | PREFSENS + 6dB (Note 1) | -53 | ±340 | CW |
|  |  | -53 | ±880 | 5 MHz E-UTRA signal, 1 RB (Note 2) |
| 3520 | PREFSENS + 6dB (Note 1) | -53 | ±270 | CW |
|  |  | -53 | ±780 | 3 MHz E-UTRA signal, 1 RB (Note 2) |
| NOTE 1: PREFSENS depends on the sub-carrier spacing as specified in Table 7.2.2-1. NOTE 2: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the lower/upper Base Station RF Bandwidth edge. |  |  |  |  |

# Conclusions

In this contribution, we present our TP to TS 38.195 above.

# References

- R4-2515058, WF for [116bis][314] A-IoT_BSCW, Huawei

- R4-2513774	TS skeleton for A-IOT BS conformance testing