3GPP TSG-RAN WG4 Meeting #117	R4-2522020

Dallas, USA, November 17th – 21st, 2025

Agenda Item:	8.12.2.2

Source:	Ericsson

Title:	On RRM specification improvement

Document for:	Approval

# Introduction

Rel-20 specification improvements were discussed in RAN4#116bis, with the agreements summarized in [4].

# Discussion

## Earlier work on specification quality

Some efforts on RRM specification quality improvement in RAN4 were made during Rel-19, based on a RAN task, which resulted in the agreements on basic editorial rules [2] and the entire specification clean up. Furthermore, on hierarchy of indent it was agreed [3] that a RAN2-like pseudo code approach will be tried out for R19 features with independent clauses, which seemed to work well.

- Proposal 1: Based on Rel-19 RRM specification quality improvement work, adapt the following for the 6G RRM specification:

  - Drafting rules agreed in [2],

  - RAN2 pseudo-code approach [3] for RRM requirements with multiple conditions and complex structures (simpler requirements can be exceptions from this rule – to be decided on the cases-by-case basis).

## Structural options and specification organization

### Requirements part

It is proposed to structure the new 6G specification by requirements types at the top level, similar to earlier releases. Some sections can be restructured though, e.g., signaling characteristics.

- Proposal 2: 6G requirements are organized by requirements types at the top level.

- Proposal 3: Section “Signalling characteristics” may be reorganized, e.g., split into two parts such as below:

  - Radio link maintenance procedures (RLM, CBD, BFD, CA/DC delays, active TCI state switching delays, etc.),

  - Radio link quality (interruptions).

- Proposal 4: Example requirements structure in the new 6G RRM specification can be as in the table below:

| Section correspondence between 5G and 6G RRM requirements |  |
| --- | --- |
| 5G RRM specification | 6G RRM specification |
| Scope | Scope |
| References | References |
| Definitions | Definitions |
| RRC_IDLE state mobility | RRC_IDLE state mobility |
| RRC_INACTIVE state mobility | RRC_INACTIVE state mobility |
| RRC_CONNECTED state mobility | RRC_CONNECTED state mobility |
| Timing | Timing |
| Signalling characteristics | Radio link maintenance procedures (RLM, CBD, BFD, CA/DC delays, active TCI state switching, etc.) |
|  | Radio link quality (interruptions) |
| Measurement procedure | Measurement procedure |
| Measurement performance requirements | Measurement performance requirements |

### Test cases part

#### Specification structure for test cases

To make it easier and more intuitive finding test cases of interest, it can be beneficial to have the specification structure for test cases similar to the requirements structure in the main part of the specification, at least at the top level (or maybe even at the second level).

- Proposal 5: Specification structure for 6G test cases follows the requirements structure in the main part of the specification, e.g.:

  - General

  - RRM Test Configurations

  - RRM Test Cases

    - Test cases for RRC_IDLE state mobility

    - Test cases for RRC_INACTIVE state mobility

    - Test cases for RRC_CONNECTED state mobility

    - Test cases for Timing

    - Test cases for Signalling characteristics

    - Test cases for Measurement procedure

    - Test cases for Measurement performance requirements.

Furthermore, in the test cases part of the TS 38.133 we also have some use cases at the top level, e.g., ATG, RedCap, positioning, etc. On one hand, this simplifies finding e.g. the positioning and NB-IoT test cases, but on the other hand this breaks the mapping between requirements section and test cases specification structure.

- Proposal 6: Test cases for specific applications or use cases can be in a separate section, but without breaking the main structure of test cases and the mapping between the core requirements sections and sections with test cases, e.g.:

  - Option 1: Test cases for specific applications/uses cases are added as separate sections after the main test cases hierarchy;

  - Option 2: Test cases for specific applications/uses cases are grouped at a next level of the test cases hierarchy (e.g., Test cases for RRC_CONNECTED state mobility -> Test cases for RRC_CONNECTED state mobility for RedCap).

#### Relation between requirements and test cases

The issue of unclear or non-straightforward relation between requirements and test cases has also been discussed under the specification improvement study for NR. As a possible solution to the issue, it was proposed to introduce a mapping table between requirements and test cases. It was realized, whoever, that such a table is expected to be very long, and also maintaining and keeping always up to date such a table will most likely require extra efforts and increase the number of CRs. Another approach could be to include a reference to the corresponding test cases in the corresponding requirement clause, e.g., in the text or as a new subclause.

- Proposal 7: Include a reference to the corresponding test cases in the corresponding requirement clause, e.g., in the text or as a new subclause.

#### Common configurations

We observed that many configuration parameters in test cases are repeated. It may simplify the specification if more such common configurations can be identified and collected in a common section, e.g., under A.3 (RRM test configurations) or similar.

- Proposal 8: If common configurations can be identified for different test cases, they can be collected in a common section, e.g., similar to A.3 (RRM test configurations).

## Principles for organizing the 6G Big CR work

The Big CR approach is very common now in RAN4 and used for most features. However, many more CRs are expected for the first version of the new 6G specification, which will not be feasible for a single Big CR.

- Proposal 9: Multiple Big CRs shall be allowed for the first version of the 6G RRM specification – the work split to be agreed and followed, based on the top and second section levels, depending on the amount of work.

- Proposal 10: Example of Big CR work split:

  - Option 1:

    - Big CR 1: Scope, References, Definitions

    - Big CR 2: RRC_IDLE state mobility

    - Big CR 3: RRC_INACTIVE state mobility

    - Big CR 4: RRC_CONNECTED state mobility

    - Big CR 5: Timing

    - Big CR 6: Signalling Characteristics/RLM, BM

    - Big CR 7: Signalling Characteristics/other

    - Big CR 8: Measurement procedure/general aspects

    - Big CR 9: Measurement procedure/Intra-frequency, inter-frequency

    - Big CR 10: Measurement procedure/Inter-RAT

# Summary

- Proposal 1: Based on Rel-19 RRM specification quality improvement work, adapt the following for the 6G RRM specification:

  - Drafting rules agreed in [2],

  - RAN2 pseudo-code approach [3] for RRM requirements with multiple conditions and complex structures (simpler requirements can be exceptions from this rule – to be decided on the cases-by-case basis).

- Proposal 2: 6G requirements are organized by requirements types at the top level.

- Proposal 3: Section “Signalling characteristics” may be reorganized, e.g., split into two parts such as below:

  - Radio link maintenance procedures (RLM, CBD, BFD, CA/DC delays, active TCI state switching delays, etc.),

  - Radio link quality (interruptions).

- Proposal 4: Example requirements structure in the new 6G RRM specification can be as in the table below:

| Section correspondence between 5G and 6G RRM requirements |  |
| --- | --- |
| 5G RRM specification | 6G RRM specification |
| Scope | Scope |
| References | References |
| Definitions | Definitions |
| RRC_IDLE state mobility | RRC_IDLE state mobility |
| RRC_INACTIVE state mobility | RRC_INACTIVE state mobility |
| RRC_CONNECTED state mobility | RRC_CONNECTED state mobility |
| Timing | Timing |
| Signalling characteristics | Radio link maintenance procedures (RLM, CBD, BFD, CA/DC delays, active TCI state switching, etc.) |
|  | Radio link quality (interruptions) |
| Measurement procedure | Measurement procedure |
| Measurement performance requirements | Measurement performance requirements |

- Proposal 5: Specification structure for 6G test cases follows the requirements structure in the main part of the specification, e.g.:

  - General

  - RRM Test Configurations

  - RRM Test Cases

    - Test cases for RRC_IDLE state mobility

    - Test cases for RRC_INACTIVE state mobility

    - Test cases for RRC_CONNECTED state mobility

    - Test cases for Timing

    - Test cases for Signalling characteristics

    - Test cases for Measurement procedure

    - Test cases for Measurement performance requirements.

- Proposal 6: Test cases for specific applications or use cases can be in a separate section, but without breaking the main structure of test cases and the mapping between the core requirements sections and sections with test cases, e.g.:

  - Option 1: Test cases for specific applications/uses cases are added as separate sections after the main test cases hierarchy;

  - Option 2: Test cases for specific applications/uses cases are grouped at a next level of the test cases hierarchy (e.g., Test cases for RRC_CONNECTED state mobility -> Test cases for RRC_CONNECTED state mobility for RedCap).

- Proposal 7: Include a reference to the corresponding test cases in the corresponding requirement clause, e.g., in the text or as a new subclause.

- Proposal 8: If common configurations can be identified for different test cases, they can be collected in a common section, e.g., similar to A.3 (RRM test configurations).

- Proposal 9: Multiple Big CRs shall be allowed for the first version of the 6G RRM specification – the work split to be agreed and followed, based on the top and second section levels, depending on the amount of work.

- Proposal 10: Example of Big CR work split:

  - Option 1:

    - Big CR 1: Scope, References, Definitions

    - Big CR 2: RRC_IDLE state mobility

    - Big CR 3: RRC_INACTIVE state mobility

    - Big CR 4: RRC_CONNECTED state mobility

    - Big CR 5: Timing

    - Big CR 6: Signalling Characteristics/RLM, BM

    - Big CR 7: Signalling Characteristics/other

    - Big CR 8: Measurement procedure/general aspects

    - Big CR 9: Measurement procedure/Intra-frequency, inter-frequency

    - Big CR 10: Measurement procedure/Inter-RAT

# References

[1] RP-250766, Lean and Streamlined 6G Standards, Apple, AT&T, BT, CMCC, DT, Ericsson, Huawei, Intel, KT, MediaTek, Nokia, NTT Docomo, Qualcomm, Reliance Jio, Samsung, Spark NZ, T-Mobile USA, Telstra, Verizon, Vodafone, ZTE, March 2025.

[2] R4-2416917, WF for [112bis][225] RRM_Spec_Improvement, Apple, Oct. 2024.

[3] R4-2420107, WF for [113][234] RRM_Spec_Improvement, Apple, Nov. 2024.

[4] R4-2514648, Way Forward for [116bis][111] 6G operation efficiency, CATT, Oct. 2025.