3GPP TSG-RAN WG4 Meeting #117                                                           R4-2522028

Dallas, USA, 17 – 21 November, 2025

Source:	Samsung

Title:	Discussion on RRM impacts on Rel-20 MIMO Phase 6

Agenda item:			7.10.3

Document for:	Discussion

- Introduction

In RAN#108, RAN plenary approved a new MIMO work item of Rel-20 MIMO Phase 6 in [1], which has been further revised in [2] in RAN#109.

The objectives of Rel-20 MIMO Phase 6 are described in [1] including:

The detailed objectives are as follows: RAN1: - On enhancing UL capacity and coverage, specify the following enhancements: - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification: - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible. - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots - On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

In RAN4#116bis meeting, WF in [3] has been approved. In this contribution, we further discuss on impacts to RRM requirements related to the above objectives.

- Discussion

2.1 Cross-slot SRS

For objective#1 in WID, the SRS further enhancement are primarily focus on two dimensions: 1) frequency domain enhancement; 2) time domain enhancement.

- Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible. - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots

In RAN4#116bis meeting, RAN4 discussed the RRM impacts by introducing cross-slot SRS. The issues are listed as below:

Issue 2-1-1: Whether to introduce additional RRM requirements of SRS antenna port switching for cross-slot SRS < Way forward > FFS the following options: - Option 1 (QC, Huawei, Ericsson): No RRM impact. - Option 2 (MTK, LG): Wait for further progress in RAN1. - MTK: wait for RAN1 conclusions and check whether the legacy interruption requirements are still applicable. - LG: Regarding cross-slot SRS, RAN4 to postpone the discussion until further progress in RAN1 - Option 3 (vivo): - RAN4 to define SRS AS requirements for the case for cross-slot SRS resource. - RAN4 start from Scenario 1 as agreed in RAN1, and whether to consider other scenarios depends on further conclusion in RAN1. - Option 4 (Samsung) - FFS on whether to extend the applicability of SRS antenna port switching requirements to the case of SRS resource(s) not only in last 6 symbols of a slot. Issue 2-1-2: Whether to introduce additional RRM requirements of SRS carrier switching for cross-slot SRS < Way forward > FFS the following options: - Option 1 (QC, MTK, LG): - RAN4 should wait for further RAN1 progress before RRM requirements for cross-slot SRS are discussed. - Option 2 (Huawei): - No RRM impact. - Option 3 (vivo): - RAN4 to define SRS AS requirements for the case for cross-slot SRS resource. - RAN4 start from Scenario 1 as agreed in RAN1, and whether to consider other scenarios depends on further conclusion in RAN1.

For time domain enhancement, the purpose it to introduce the “cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set”. There can be different time domain resource mapping in this enhancement including: 1) different SRS resources are in two adjacent slots. 2) one SRS resource is in two adjacent slots.

In RAN1#122bis meeting, RAN1 achieved the agreements:

Agreement To determine the time-domain location of a cross-slot SRS resource, - ‘l0’ is interpreted as the starting symbol in the starting slot for an SRS resource. - ‘l'+l0’ is interpreted as the symbol index in a slot or two consecutive S and U slots. - The restriction ‘loffset≥NsymbSRS-1’ is removed. Agreement: - For an aperiodic cross-slot SRS resource set in scenario 1, the slot offset of the SRS resource set refers to the first of the two slots spanned by the SRS resource set. - Note 1: Whether 0 or more SRS resource with time-domain resource entirely in the second slot (i.e. U slot) can be supported in scenario 1 is discussed separately. - Note 2: If the transmission of 0 or more SRS resource with time-domain resource entirely in the second slot (i.e. U slot) is supported, for the resource(s) in the second slot, whether/how to utilize the slot offset configured for the SRS resource set is discussed separately. - Note 3: 0 or more SRS resource with time-domain resource entirely in the first slot (i.e. S slot) can be supported in scenario 1.

From RRM requirements of Interruptions at NR SRS antenna port switching. In legacy RAN4 requirements for it, the common understanding is the components of the interruption time of SRS antenna port switching are: antenna switching time before and after SRS transmission; SRS transmission time of X symbols. And in the requirements, it is separated to 1 SRS symbol is configured or others (X=6 SRS symbols).

In existing RRM requirements, the SRS antenna port switching requirements are defined. However, the requirements are applicable by conditions.

The requirements in this clause are applicable to SRS antenna port switching on FR1 and SRS resource(s) is only configured within the last 6 symbols of a slot.

After Rel-16, the SRS are further enhanced in time domain which more symbols can be used without the limitation of “last 6 symbols”.

However, in RRM requirements, there is no further enhancement for “SRS antenna port switching” to other than “FR1 and SRS resource(s) is only configured within the last 6 symbols of a slot.”

In previous MIMO WI, NR_ENDC_RF_Ph4, and RAN-P RRM enh proposal, there were discussion on whether to extend existing SRS antenna port switching RRM requirements in MIMO WI or other common RRM enhancement WI.

According to the latest agreements in RAN1, the time-domain location of a cross-slot SRS resource can be extended for adjacent slots. The current SRS antenna port switching requirements to the case of SRS resource(s) cannot be applied in most cross-slot scenarios.

Observation 1: In SRS antenna port switching requirements, the applicability of the requirement is “on FR1 and SRS resource(s) is only configured within the last 6 symbols of a slot”, which cannot be applied in most cross-slot scenarios.

Proposal 2: RAN4 to discuss on extended the applicability of SRS antenna port switching requirements to the case of SRS resource(s) not only in last 6 symbols of a slot nor FR1 only.

2.2 DL CSI acquisition enhancement

For objective#2 in WID, the DL CSI acquisition enhancement are primarily focus on two dimensions: 1) Early SRS/CSI/CSI-RS triggering; 2) CSI-RS freq. density reduction.

- On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports

For 1) Early SRS/CSI/CSI-RS triggering, from the purpose, RAN1 is intended to introduce an early SRS/CSI/CSI_RS trigger mechanism for the UE transitioning from IDLE/INACTIVE to CONNECTED mode. It may work together with SCell activation/deactivation or SCell dormancy procedures.

In existing RRM requirements, RAN4 specified many RRM requirements related to such procedures, such as: the SCell activation delay or SCell deactivation delay or Interruption due to SCell dormancy. In RAN4 discussion and final requirements, RAN4 should specify the timeline of the procedures and conditions. In previous releases, RAN4 specified the Scell activation/deactivation delay in many WIs. For example, in Rel-16, the measurement in based on SSB. In Rel-17, Scell activation procedures can be enhanced by A-TRS (temporary RS) for intra-band CA. After that, Scell activation were also enhance in NES and for EMR and further enhanced in RRM phase 5.

In RAN1#122 meeting, RAN1 initially discussed for early triggering for both SRS-AS and CSI/CSI-RS in SCell activation procedure, the early agreements are shown as below:

Agreement: For early triggering of SRS-AS when SCell transition from deactivation to activation, support at least aperiodic SRS-AS transmission on a SCell, triggered based on legacy SCell activation command activating the SCell. - FFS: Timeline of the aperiodic SRS-AS transmission (requirement is up to RAN4) - FFS: How to trigger the aperiodic SRS-AS transmission based on legacy SCell activation command - FFS: Whether/what new information is provided in SCell activation command? Agreement: For early triggering of CSI/CSI-RS when SCell transition from deactivation to activation, support aperiodic CSI reporting for a SCell triggered based on legacy SCell activation command activating the SCell. - The aperiodic CSI reporting is assosicated with at least aperiodic CSI-RS for CSI on the SCell - FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI (requirement is up to RAN4) - FFS: How to trigger the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI based on legacy SCell activation command? - FFS: Whether/what new information is provided in SCell activation command? Agreement: For early triggering of SRS-AS/CSI-RS/CSI when UE transition from IDLE to CONNECTED mode, study the following at least three options for Step-1 and Step-2: - Option-1: NW can provide the resource/report configuration in SIBx based on only one UE capability assumption, and UE can report through MSG3 whether the resource/report configuration received in SIBx is supported. - Option-2: NW can provide the resource/report configuration(s) in SIBx based on one or multiple UE capability assumptions, and UE can report through MSG3 which resource/report configuration(s) received in SIBx is/are supported. - Option-3: NW can provide the resource/report configuration(s) in SIBx based on one or multiple UE capability assumptions, and UE can report through MSG3 the supported capability(s) of early SRS/CSI/CSI-RS triggering (e.g., whether to support this feature, max number of CSI-RS ports, xTyR for SRS-AS, max bandwidth of the CSI-RS/SRS-AS etc.). Note: The term “capability” or “UE capability” above does not mean legacy RRC based UE capability.

It can be observed that the early triggering of SRS-AS/CSI-RS/CSI may work together or with in the SCell activation procedure which mean there are RRM impacts by early SRS/CSI/CSI-RS triggering especially for SCell activation/deactivation delay/SCell dormancy.

In RAN4#116bis meeting, RAN4 discussed the above from three aspects.

2.2.1 Early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode

Issue 3-1-1: Timeline of early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode < Way forward > FFS the following proposal: - Proposal 1 (Ericsson): - RAN4 shall first confirm whether the UE can transmit or receive during the RRC setup delay. If the UE cannot, then RAN4 should clarify to RAN1 through LS that, as per the existing procedure, UE cannot transmit/receive from the RRCSetup reception until RRCSetup processing delay is complete. RRCSetup processing delay as per TS 38.331 is 10ms. - RAN4 shall first confirm whether the UE can transmit or receive during the RRC resume delay. If the UE cannot, then RAN4 should clarify to RAN1 that, as per the existing procedure, UE cannot transmit/receive from the RRCResume reception until RRCResume processing delay is complete. RRCResume processing delay as per TS 38.331 is 6 or 10ms based on different scenarios. Issue 3-1-2: Whether to introduce additional RRM requirements for early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode < Way forward > FFS the following options: - Option 1 (QC, Nokia): - RAN4 should wait for further progress in RAN1 for early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode. - Option 2 (MTK, Huawei) - Not to define any new RRM core requirements for early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode. - Option 2a (Apple): - RAN4 do not consider new timeline related requirements for early CSI triggering when UE transitions from IDLE to CONNECTED mode. - Option 2b (LGE): - Regarding CSI acquisition in RRC_IDLE to RRC_CONNECTED, RAN4 to postpone the discussion until any RRM impact is identified.

For the Issue 3-1-2, to be honest, after checking the RAN1 discussion and latest agreement, we don’t think the inputs can be sufficient enough for RAN4 discussion.

Proposal 3: Whether to introduce additional RRM requirements for early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode, defer the discussion and conclusion until further RAN1 progress.

But for the Issue 3-1-1, we think this issue is important for discussion. During the transition time from IDLE to CONNECTED mode, what is the typical UE behaviour? In previous RAN4 requirements, there is no clear consensus or agreement in the spec to specify the UE behaviour during RRC procedure delay.

Observation 2: No clear consensus or agreement in the spec to specify the UE behaviour during RRC procedure delay.

In typical UE implementation, the UE may be unable to transmit or receive any signal and there is no need to. Especially for the case involved RF changes, the UE need to receive and transmit the signal after the RF is ready and stable. However, this restriction will greatly reduce the benefit of Early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode. The receiving and transmission can only happen after RRC procedure delay (The time unit is usually >=10ms).

Observation 3: If UE cannot transmit or receive signals during the RRC procedure delay, the benefit for early triggering will be reduced a lot.

Proposal 4: RAN4 to achieve the consensus on whether UE can transmit and receive signals during RRC procedure delay. RAN4 to discuss whether there can be conditions for the case UE can transmit and receive signals during RRC procedure delay, take an example, if there is condition which does not involved RF changes.

    - Early SRS/CSI-RS/CSI triggering for SCell activation

In RAN1#122bis meeting, the agreement for early SRS/CSI-RS/CSI triggering for SCell activation is still open.

Agreement: On triggering mechanism for early aperiodic SRS-AS transmission on a SCell and early aperiodic CSI reporting for a SCell, based on the legacy SCell activation activating the SCell, down-select one from the followings in RAN1#123 meeting: - Alt-1 (Implicit mechanism): - For early aperiodic SRS-AS transmission, the SRS resource set(s) triggered for the SCell is determined according to RRC configuration. - For early aperiodic CSI reporting, the CSI report configuration(s) triggered for the SCell is determined according to RRC configuration - Alt-2 (Explicit mechanism): - For early aperiodic SRS-AS transmission, the SRS resource set(s) triggered for the SCell is determined according to an indication in SCell activation command. - For early aperiodic CSI reporting, the CSI report configuration(s) triggered for the SCell is determined according to an indication in SCell activation command.

In last RAN4#116bis, RAN4 discussed the applicable scenarios of early SRS/CSI-RS/CSI triggering for Scell activation:

Issue 3-2-1: Applicable scenarios of early SRS/CSI-RS/CSI triggering for SCell activation < Way forward > FFS the following proposals:

We observed there was similar discussio in RAN1 although there was no agreements in last RAN1#122bis meeting.

In previous releases in RAN4, we defined so many different Scell activation delay requiements in:

8.3.2 SCell Activation Delay Requirement for Deactivated SCell

8.3.2A	SCell Activation Delay Requirement for Deactivated SCell based on measurement in IDLE/INACTIVE mode

8.3.7 SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells

8.3.12 SCell Activation Delay Requirement for Deactivated PUCCH SCell

8.3.13 SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple Scells

8.3.16 Fast SCell Activation Delay Requirement for Deactivated SCell

8.3.17 SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation

8.3.18 SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells with L3 reporting

8.3.19 OD-SSB based SCell Activation Delay Requirement for Deactivated SCell

By our understanding, the motivation of early triggering described in WID includes SRS antenna switching, CSI-RS and CSI.

For the SCell Activation Delay Requirement for Deactivated PUCCH SCell, in existing RAN4 agreements in Rel-17 and existing requirements:

The above delay requirement shall apply provided that: - The target pathloss reference signal determined during PUCCH SCell activation is known otherwise longer activation time is expected if the pathloss reference signal is unknown; and - The RA on PUCCH SCell is not interrupted by the RA on PCell otherwise additional delay to activate the SCell is expected; and - No SRS carrier based switching or SRS antenna port switching occurs during the SCell activation procedure otherwise the PUCCH SCell activation delay can be extended.

Observation 4: For PUCCH SCell activation in Rel-17, No SRS carrier based switching or SRS antenna port switching occurs during the SCell activation procedure.

In Rel-17, the CSI-RS for tracking during the Scell activation has already been supported for fast Scell activation.

Proposal 5: RAN4 to prioritize the discussion on the Early SRS/CSI-RS/CSI triggering for SCell activation from below cases unless RAN1 achieves agreements on different cases:

- Rel-15 single SCell (non-PUCCH SCell) activation

- Rel-17 fast SCell activation

    - Early SRS/CSI-RS/CSI triggering when switching out of SCell dormancy

In latest WF in RAN4#116bis:

Issue 3-3-1: RRM impact due to early SRS/CSI-RS/CSI triggering when switching out of SCell dormancy < Way forward > FFS the following options: - Option 1 (MTK): - For early SRS/CSI/CSI-RS triggering when switching out of SCell dormancy, the legacy switching delay TdormantBWPswitchDelay and interruption requirements “Interruptions due to Active BWP switching Requirement” are applicable. - Option 2 (CATT, Huawei, LGE, Apple, Samsung, Nokia): - Wait for more RAN1 progress.

By checking RAN1 progress in last RAN1#122bis meeting,

Agreement: For early triggering of SRS-AS when SCell transition from a dormant BWP to a non-dormant BWP, support aperiodic SRS-AS transmission on the non-dormant BWP, triggered via a DCI indicating switching out of SCell dormancy. - Legacy DCI format(s) for switching out of SCell dormancy is reused without introducing new DCI field or resizing the existing DCI field - FFS: Which DCI format(s) from DCI format 0_1/0_3/1_1/1_3/2_6 is supported - FFS: Triggering mechanism(s) - FFS: Timeline of the aperiodic SRS-AS transmission (requirement is up to RAN4) Agreement: For early triggering of CSI/CSI-RS when SCell transition from a dormant BWP to a non-dormant BWP, support aperiodic CSI reporting for the non-dormant BWP, triggered via a DCI indicating switching out of SCell dormancy. - The aperiodic CSI reporting is associated with aperiodic CSI-RS for CSI on the non-dormant BWP - Legacy DCI format(s) for switching out of SCell dormancy is reused without introducing new DCI field or resizing the existing DCI field - FFS: Which DCI format(s) from DCI format 0_1/0_3/1_1/1_3/2_6 is supported - FFS: Triggering mechanism(s) - FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI (requirement is up to RAN4)

We don’t think the inputs can be sufficient for RAN4 discussion.

Proposal 6: Whether to have RRM impact due to early SRS/CSI-RS/CSI triggering when switching out of SCell dormancy, defer the discussion and conclusion until further RAN1 progress.

- Conclusion

In this contribution, we provide our proposals for overall RRM impacts of Rel-20 MIMO Phase 6:

Observation 1: In SRS antenna port switching requirements, the applicability of the requirement is “on FR1 and SRS resource(s) is only configured within the last 6 symbols of a slot”, which cannot be applied in most cross-slot scenarios.

Proposal 2: RAN4 to discuss on extended the applicability of SRS antenna port switching requirements to the case of SRS resource(s) not only in last 6 symbols of a slot nor FR1 only.

Proposal 3: Whether to introduce additional RRM requirements for early SRS/CSI-RS/CSI triggering during transition from IDLE/INACTIVE to CONNECTED mode, defer the discussion and conclusion until further RAN1 progress.

Observation 2: No clear consensus or agreement in the spec to specify the UE behaviour during RRC procedure delay.

Observation 3: If UE cannot transmit or receive signals during the RRC procedure delay, the benefit for early triggering will be reduced a lot.

Proposal 4: RAN4 to achieve the consensus on whether UE can transmit and receive signals during RRC procedure delay. RAN4 to discuss whether there can be conditions for the case UE can transmit and receive signals during RRC procedure delay, take an example, if there is condition which does not involved RF changes.

Observation 4: For PUCCH SCell activation in Rel-17, No SRS carrier based switching or SRS antenna port switching occurs during the SCell activation procedure.

Proposal 5: RAN4 to prioritize the discussion on the Early SRS/CSI-RS/CSI triggering for SCell activation from below cases unless RAN1 achieves agreements on different cases:

- Rel-15 single SCell (non-PUCCH SCell) activation

- Rel-17 fast SCell activation

Proposal 6: Whether to have RRM impact due to early SRS/CSI-RS/CSI triggering when switching out of SCell dormancy, defer the discussion and conclusion until further RAN1 progress.

- Reference

[1] RP-251856, New WID: NR MIMO Phase 6, Samsung (Moderator)

[2] RP-252936, Revised WI: NR MIMO Phase 6, MediaTek

[3] R4-2514825, WF on NR_MIMO_Ph6_RRM, MediaTek