3GPP TSG-RAN4 Meeting #117	R4-2522030

# Dallas, United States, 17th - 21st November, 2025

Agenda item:		8.11

Source:				Keysight Technologies UK Ltd

Title:					On UE 6G OTA Testability Aspects

Document for:		Approval

# Introduction

This contribution is addressing our continued thoughts about the 6G OTA testability aspects for UE testing, specifically the non-AI features, of the new 6G SI [1].

# Discussion

The following sections provide some additional views on OTA testability aspects for the different frequency ranges considered [1], i.e.,

From a technology perspective, the study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around ~7GHz), and FR2-1 (24.25 GHz – 52.6GHz).

specifically organized by topics addressed in either the WF [2] or topic summary [3] from the previous meeting. Additionally, various 6G OTA testing aspects are repeated from our initial contribution [4].

### Improved Testability of Conducted Requirements

The following items related to testability were captured in last meeting’s WF

Issue 1-2-1: Whether it is valuable to improve some conducted requirements to be verified via OTA approach Agreements: - RAN4 can further study the testability limitations of some conducted test cases, consider the following aspects as starting point, e.g., - FR1 conducted test cases already consider antenna-performance impacts but not verified via radiated approach in a case-by-case manner, e.g. MSD - FR1 test case simplification with both conducted and radiated considered, e.g., spurious emission - Discuss which conducted conformance testing could either yield testability issues, e.g., lack of physical Tx/Rx ports, and/or highly dynamic nature of multiple Tx/Rx operation, including antenna tuning for impedance and/or pattern..

As we outlined in our last contribution [4], we believe that the dynamic nature of multiple active receiver and transmitter operation will likely see an increase in conducted conformance test system complexity (including the need for complex vendor declarations and thus implementations in test system software, etc.). Additionally, the complexity of conducted testing has increased significantly in NR FR1 especially based on the available number of physical Tx and/or Rx ports of the UE, e.g., the number of UE Rx ports were increased from 4Rx (Rel-15) to 8Rx (Rel-18).

The following set of test cases are suitable candidates to be verified OTA rather than conducted for the reasons mentioned above: MSD, CA, multi-Tx/Rx test cases.

Proposal 1: Study the following test cases as suitable candidates to be verified OTA rather than conducted: MSD, CA, multi-Tx/Rx test cases

Some discussions were held last meeting whether the spurious emissions test cases could be simplified by moving them from conducted to radiated, as captured in the WF [2]. Given the regulatory nature of radiated spurious emissions test cases in various regions and thus the already existing requirements to test spurious emissions OTA using various regulatory bodies, a move from conducted to radiated might not be desirable to avoid duplicate testing, especially given the large increase in test time (conducted vs radiated).

Proposal 2: Hold off transitioning the spurious emissions test cases from conducted to radiated given the regulatory nature of these test cases to avoid duplicate testing

### Introduction of Realism by Transitioning Conducted TCs to OTA

Limited discussions were held last meeting to introduce more realism by transitioning conducted test cases to OTA; the main focus of the discussions were related to transitioning test cases from conducted to OTA due to potential testability concerns of conducted tests. However, we believe there is value to study the introduction of realism for select, existing conducted TCs when transitioned to OTA by assessing the end-to-end performance. Examples of such test cases could include the maximum sustained data rate which would indeed benefit from more realistic RF conditions. Select CSI reporting test cases could potentially also be candidates to be studied OTA.

Proposal 3: Study the transition of select conducted TCs to OTA to introduce realism

### Test methodologies for new 6GR frequencies

The following agreements were captured in the RAN4#116bis WF [2]

Issue 2-1-1: Testability for New Frequencies between FR1 and FR2 Agreements: RAN4 study the testability including conducted testing and radiated testing for the new frequencies. The following aspects can be considered: - The frequency range definition will be discussed under the spectrum agenda. - For conducted test, use existing test method as a starting point - For OTA test - Further study potential test methodologies including the full-package, i.e., supported frequency range, test setup, configuration, positioning, procedure, validation/calibration, quiet-zone/test zone, MU, testing time reduction

As outlined in our previous contribution, a key question for each of the new frequencies defined for 6G between FR1 and FR2 is whether they are closer to FR1 or FR2. This will have a significant impact on the decision towards radiated vs conducted requirements as the presence of a physical conducted port/connector will likely be dependent on that frequency range of operation.

More importantly, the actual frequencies of 6G operation between FR1 and FR2 will determine whether a new OTA test system might be needed (undesirable) or whether existing OTA test systems and methodologies could be leveraged, e.g., should existing FR1 test systems be upgraded to support an extended frequency range towards the top end or should existing FR2 test systems be upgraded to support an extended frequency range towards the bottom end? Generally, it is more straightforward to increase the upper frequency of existing test systems rather than decreasing the lower frequency of operation, e.g., due to absorber performance. Especially given the rather different nature of methodologies for FR1 (DFF with distributed-axes systems) and FR2 (IFF with primarily combined-axes systems), it would be desirable to decide on the baseline system/methodology early. Various suggestions on operating frequencies were made in contributions to the OTA testability and the spectrum agenda. At this point, it is too early to put together and review full packages for either new or updated existing OTA test systems until concrete decisions for the new frequency ranges between FR1 and FR2 in the OTA 6G Spectrum Agenda have been reached. Additionally, the specification of test methodologies and systems will have a dependency on new device types, discussed in the next section.

Proposal 4: Await concrete decisions in the OTA 6G Spectrum Agenda for the new frequency ranges between FR1 and FR2 as well as new device types for 6GR before studying either new or upgraded existing OTA test systems for the new 6GR frequencies.

### Testability for Different Device Types

The following agreements were captured in the RAN4#116bis WF [2]

Issue 4-1-1: OTA testability applicability for different UE types in 6GR day-1 Agreements: - RAN4 consider the study of OTA test system to better accommodate different UE types, new feature/functionalities, new performance metrics and test cases. - The UE types and form factor considered in testability can be further discussed in this SI. - RAN4 study the feasibility of developing a single system to cover above aspects.

While the primary focus for 6G conformance testing will likely continue with handheld UEs such as smartphones, other UE types are gaining visibility, e.g., UAVs (Uncrewed Aerial Vehicles), eVTOL (electric vertical take-off and landing), full vehicles, AR/VR/XR devices, vehicular UEs, VSATs, MU-MIMO base stations, etc. Given the very large size of some of these devices, it is certainly not realistic/feasible to consider a single system for 6G OTA testing, especially due to the increased path losses/reduced dynamic range of very large OTA systems and since most labs consider real estate within their facilities extremely important so that test coverage in a single facility can be maximized. Feedback from RAN4 is requested which device types shall be prioritized for 6G for OTA testing.

Proposal 5: Feedback from RAN4 is requested which new device types shall be prioritized for 6G for OTA testing

If some of the prioritized device types (from Proposal 5) are significantly larger than what existing conformance OTA test systems can readily support, RAN4 should consider testing the corresponding sub-assemblies/components with (ideally) the same OTA performance, e.g., replacing full vehicle testing with TCU+antenna+ground plane testing instead.

Proposal 6: If some of the prioritized device types (from Proposal 5) are significantly larger than what existing conformance OTA test systems can readily support, RAN4 should consider testing the corresponding sub-assemblies/components with (ideally) the same OTA performance

### Improved test methods and metric for DL MIMO OTA

The following agreements were captured in the RAN4#116bis WF [2]

Issue 8-1-1: RAN4 consider MIMO OTA for 6GR day-1 Agreements： RAN4 will study the dynamic MIMO OTA (at least dynamic channel model and link adaptation) for 6G. static MIMO is not precluded. The following can be considered as starting point: - 2D or 3D channel model - FFS phantom involved testing - FFS multi-TRP - FFS focus on FR1 as first priority - FR2 and new frequency range may also be considered

As mentioned in our previous contribution, industry’s interest in the range between FR1 and FR2 over the last few years especially focused on larger channel bandwidths and larger number of BS antenna array elements; for Multi User (MU)-MIMO, this was commonly reflected in their terminology such as Massive-MIMO, Giga-MIMO, eXtreme-MIMO, etc. The use of hundreds if not thousands of BS antenna array elements promised pencil beams to individual users including beam forming in AZ as well as EL. This could/should have test and test system implications for MIMO OTA testing for the range between FR1 and FR2 but likely also for the mid to high range of FR1, i.e., the simplification of setting the ZoA of the CDL channel models to 90° would no longer be representative/real life. It is therefore proposed to introduce realism to MIMO OTA testing for 6GR by introducing 3D channel models. One example extension of a 2D MPAC (with 2D channel models only) to a 3D MPAC (with 3D channel models) is illustrated in schematically in Figure 1.

Figure 1: Example extension of 2D MPAC (5G MIMO OTA, left) to 3D MPAC (6G MIMO OTA, right)

Proposal 7: Introduce realism to MIMO OTA testing for 6GR by introducing 3D channel models

The use of single-hand or potentially even dual-hand phantoms would also introduce more realism to MIMO OTA testing and potentially save some test time by avoiding different device rotations. A liaison with CTIA should be considered to introduce the appropriate phantoms developed in CTIA to 3GPP 6G OTA testing.

Proposal 8: Introduce realism to MIMO OTA testing for 6GR by focusing on testing with single-hand and potentially even dual-hand phantoms (following a liaison with CTIA)

It is proposed to focus MIMO OTA assessments in 6G for FR1 and the range between FR1 and FR2 on more realistic scenarios including dynamic channel models, link adaptation (similar to those studied in [13]) and forego the static MIMO approach which completely lacks realism given the static channel models and fixed RMCs.

Proposal 9: Focus MIMO OTA assessments in 6G for FR1 as well as the range between FR1 and FR2 on more realistic scenarios, namely dynamic MIMO OTA with dynamic channel models and link adaptation

The multi-TRP approach is an interesting idea to introduce additional realism to dynamic MIMO OTA. A very preliminary, high-level review revealed that that more efforts will be needed to define these test cases and that additional CE HW resources might be needed. It is requested that operators/OEMs/chipset vendors provide more information on the desired implementation details regarding Multi-TRP for MIMO OTA.

Proposal 10: Operators/OEMs/chipset vendors to provide detailed information on the desired implementation details regarding Multi-TRP for MIMO OTA

### Consideration of OTA Testability in Requirements Definition Phase

During the study and work-item phase of 5G NR FR2, test equipment and system vendors have repeatedly pointed out testability issues of select test cases which often were disregarded (“consider these requirements system level requirements”) and thus caused significant efforts in RAN5 to define relaxations as well as a list of test cases that are untestable due to the lack of dynamic range. In other instances, requirements were defined, e.g., various FR2 power classes or NTN VSAT (Ka/Ku bands), without appropriately defining the testability aspects whatsoever. It is therefore proposed to consider the testability aspects during the definition of 6G requirements.

Proposal 11: 6G OTA testability aspects need to be considered together with feature development going forward (regardless of frequency range) as important synchronizations between WIs and SIs and/or RAN4 and RAN5 resulted in unnecessary delays and confusion, e.g., low-UL/high-DL test cases, Ku/Ka bands, various FR2 power classes.

### Test Time and MU Considerations of FR2 OTA Testing

The two key concerns from industry regarding FR2 testability were related to the large MUs (in excess of 4-5dB) and the test time. The mm-wave frequency range will obviously come with larger test equipment and component uncertainties, and the OTA environment will have additional MU elements with non-insignificant MUs when compared to conducted environment. Whether significant reductions in MU due to test equipment will be possible for the FR2 range for 6G operation is questionable.

However, there is a potential to address the overall test time concerns and some MU impact for 6G FR2 OTA testing. Here, it might be worthwhile to briefly review the nature of classical conducted conformance and OTA test approaches: while conducted conformance test cases commonly check the pass/fail compliance to a given requirement, the OTA test cases generally assess the actual Tx/Rx end-to-end performance by leveraging a parametric test approach (“searches”). For receiver sensitivity assessments, for instance, an iterative search is performed in an OTA test to determine the sensitivity, EIS, that yield a specific TP threshold, e.g., Clause A.3.3.3 of [17]

EIS, or receiver sensitivity measurements, is defined as the minimum downlink signal power received at the UE antenna input required to provide a data throughput rate greater than or equal to 95% of the maximum throughput of the reference measurement channel (RMC) (the maximum throughput is per Appendix A of TS 38.521-1 [11]).

while in a conducted test the REFSENS power level is applied at the conducted port to assess whether the minimum TP threshold is reached/exceeded (PASS) vs not (FAIL), e.g., Clause 7.3 of [18]

## 7.3 Reference sensitivity ### 7.3.1 General The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

In other words, classical OTA testing generally allowed performance ranking of UEs, i.e., not just a pass/fail verdict.

For 5G NR FR2 OTA testing with active antenna arrays, it was decided to leverage the performance ranking and the parametric approaches for some of the key UE RF metrics, e.g., Maximum Output Power (MOP) and REFSENS by mandating beam peak searches with very fine measurement grids to identify the test direction that yields the best MOP or REFSENS performance of the UE. Trade-offs were required between selecting an acceptable uncertainty (to select the absolute best beam) and test time for the 3D searches; those trade-offs still resulted in significant MUs and test times just to be able to assess certain absolute performance Tx/Rx metrics (rather than the compliance with the pass/fail requirements). For 5G NR FR2, these beam peak searches were defined as pre-condition tests for a large number of UE RF, Demodulation, and RRM TCs that merely checked for compliance against pass/fail conditions. A reduction in MU and test time for 6G FR2 could be to avoid beam peak searches to determine the absolute best Tx/Rx performance directions (within a given uncertainty) and allowing testing in directions that pass the minimum requirements for MOP and REFSENS instead. For instance, such directions could be identified during the spherical coverage test cases that are also utilizing 3D searches but with a much coarser measurement grid compared to the beam peak searches. Whether the beam peak direction is absolutely required for some of the regulatory test cases is FFS.

Proposal 12: For 6G FR2 UE RF OTA testing, consider revisiting the need for beam peak searches to reduce MU and overall test time.

# Conclusion

The following observations and conclusions were made in this contribution.

Proposal 1: Study the following test cases as suitable candidates to be verified OTA rather than conducted: MSD, CA, multi-Tx/Rx test cases

Proposal 2: Hold off transitioning the spurious emissions test cases from conducted to radiated given the regulatory nature of these test cases to avoid duplicate testing

Proposal 3: Study the transition of select conducted TCs to OTA to introduce realism

Proposal 4: Await concrete decisions in the OTA 6G Spectrum Agenda for the new frequency ranges between FR1 and FR2 as well as new device types for 6GR before studying either new or upgraded existing OTA test systems for the new 6GR frequencies.

Proposal 5: Feedback from RAN4 is requested which new device types shall be prioritized for 6G for OTA testing

Proposal 6: If some of the prioritized device types (from Proposal 5) are significantly larger than what existing conformance OTA test systems can readily support, RAN4 should consider testing the corresponding sub-assemblies/components with (ideally) the same OTA performance

Proposal 7: Introduce realism to MIMO OTA testing for 6GR by introducing 3D channel models

Proposal 8: Introduce realism to MIMO OTA testing for 6GR by focusing on testing with single-hand and potentially even dual-hand phantoms (following a liaison with CTIA)

Proposal 9: Focus MIMO OTA assessments in 6G for FR1 as well as the range between FR1 and FR2 on more realistic scenarios, namely dynamic MIMO OTA with dynamic channel models and link adaptation

Proposal 10: Operators/OEMs/chipset vendors to provide detailed information on the desired implementation details regarding Multi-TRP for MIMO OTA

Proposal 11: 6G OTA testability aspects need to be considered together with feature development going forward (regardless of frequency range) as important synchronizations between WIs and SIs and/or RAN4 and RAN5 resulted in unnecessary delays and confusion, e.g., low-UL/high-DL test cases, Ku/Ka bands, various FR2 power classes.

Proposal 12: For 6G FR2 UE RF OTA testing, consider revisiting the need for beam peak searches to reduce MU and overall test time.

# References

- RP-251881, Study on 6G Radio, NTT DOCOMO (Moderator), 3GPP TSG RAN Meeting #108, June 2025

- R4-2514647, WF for [116bis][110] 6G testability and OTA, Feature lead (vivo), 3GPP TSG-RAN4 Meeting #116bis, October 2025

- R4-2514517, Topic summary for [116bis][110] 6G testability and OTA, Feature lead (vivo), 3GPP TSG-RAN4 Meeting #116bis, October 2025

- R4-2513347, On UE 6G OTA Testability Aspects, Keysight Technologies, 3GPP TSG-RAN4 Meeting #116-bis, October 2025

- R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4 Chair, 3GPP TS_RAN WG4 #116, August 2025

- 6GWS-250191, Keysight views on 6G testability, Keysight Technologies, 3GPP 6G Workshop, March 2025

- TS 38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone

- TS 38.101-4, User Equipment (UE) radio transmission and reception; Part 4: Performance requirements

- TS 38.133, Requirements for support of radio resource management

- TS 38.161, User Equipment (UE) TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements; Range 1 Standalone and Range 1 Interworking operation with other radios

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz

- TS 38.151, Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements

- TR 38.762, Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) dynamic test methodology for FR1 UEs

- TR 38.803, Study on new radio access technology: Radio Frequency (RF) and co-existence aspects

- TR 38.820, 7 - 24 GHz frequency range

- TS 38.101-3, User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios

- TS 38.561, User Equipment (UE) conformance specification; UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)

- TS 38.101-1, User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone