3GPP TSG-RAN WG4 Meeting #117				                           R4-2522046

Dallas, USA, Nov. 17-21, 2025

Source:	Sony

Title:	Further views on parameters related to UE channel bandwidth of 6GR

Agenda item:	8.2.3

Document for:	Approval

# Background

In RAN4#116bis, initial discussion on system parameters for 6GR from RAN4 side was carried out [1] and a WF was agreed [2].  In this contribution, we share our further views on parameters related to UE channel bandwidth of 6GR based on the scope that was agreed in the WF.

# FFT size

For FFT size, the following agreement was made in RAN4#116bis

- Consider FFT size, maximum Channel Bandwidth and numerology as a framework to have feasibility and complexity study from implementation perspective, especially for 8K or 16K FFT size considering the associated SCS and also the frequency ranges

- Provide RAN1 with early RAN4 feedback on the feasibility and trade-offs of the proposed FFT/CBW/SCS combinations to help guide their decisions.

For massive IoT devices, it is considered that the bandwidth required to be supported would be relatively small compared to broadband devices and there is no high demand on a larger FFT size. Therefore, assuming the same FFT size as legacy IoT devices, which would help reduce the R&D complexity of new 6G massive IoT devices and enable some hardware reuse, 2K FFT size can be assumed for massive IoT devices.

For broadband and ultra-broadband devices, to support the enlarged UE CBW mentioned above, it may be necessary to consider a larger FFT size than 5G NR. Therefore, it is proposed to consider an 8K FFT size as a baseline for further study.

Proposal 1: It is proposed to assume a 2k FFT for massive IoT devices, while an 8k FFT size as baseline for higher-capability device types while study the feasibility of supporting even higher FFT size.

# SCS Numerology

For SCS, the following agreement was made in RAN4#116bis

- Evaluate the following proposals regarding numerology from RAN4 perspective

  - "Single numerology" proposal

  - Frequency sub-range/Band specific SCS values proposal

    - Compare perf gain and implementation complexity for different SCS with same frequency range or specific band

    - Study numerology for SSB of initial cell search from RAN4 perspective

  - Other proposals not presented in this meeting are not precluded

- Provide RAN1 with early RAN4 feedback on the feasibility and trade-offs of the proposed FFT/CBW/SCS combinations to help guide their decisions.

In 5G NR, multiple numerologies were defined for each frequency band to offer sufficient flexibility for different usage scenarios. However, in practice, only a limited set of numerologies was deployed, and typically a single fixed numerology per band has been deployed. Supporting multiple numerologies in 5G NR places additional complexity in both device and network implementation, but without improving the actual field performance. Therefore, in 6GR, it is proposed to consider only a single numerology per band, e.g., a single SCS for each frequency band.

Proposal 2: It is proposed that 6GR adopts single numerology per frequency band.

Typically, the SCS is selected based on the frequency, with a larger SCS being used for higher frequencies to mitigate the increased phase noise and support a larger bandwidth with a given FFT size. As a starting point, it is proposed that 6GR should adopt 15 kHz SCS for frequency bands below 3 GHz, and 30 kHz for frequency bands above 3 GHz in FR1 and possible further extend the 30 kHz SCS to around 7 or 8 GHz. For frequency bands around 15 GHz, 60 kHz SCS may be more proper considering the increased phase noise.  For FR2-1, 120 kHz can be taken as a baseline.

Proposal 3: It is proposed that 6GR should adopt 15 kHz SCS for frequency bands below 3 GHz and 30 kHz for frequency bands above 3 GHz in FR1 and around 7 GHz, 60 kHz for frequencies around 15GHz and 120 kHz for FR2-1.

For the SCS design of SSB, it can be further discussed if the same SCS should be adopted as data channels. For example, in FR2-1, where a higher SCS, e.g., 240 kHz, may help to reduce the latency for initial access when a large number of analog beams needs to be swept for initial access. However, to facilitate the discussion, it is proposed to focus on such an issue after the baseline SCS values for each band and the basic design of SSB is determined.

Proposal 4: RAN4 further studies the SCS for SSB design, including whether it should be the same or not with the data channel, once the baseline SCS and basic SSB design is determined.

# UE CBW

UE CBW is a critical parameter that not only affects the cost of the baseband but also the RF design. The key design consideration is the minimum and maximum UE CBW that a device needs to support.

## Min UE CBW

For the minimum UE CBW, it is likely to be chosen between 3 MHz vs. 5MHz. Based on the lessons learned from 5G NR, the demand for 3MHz to allow the deployment of 6GR in narrower spectrum allocation would still be present. Meanwhile, we don't expect major impact on supporting the minimum bandwidth from the UE implementation aspect, and thus it should be feasible for all device types to support and operate with 3MHz as the minimum UE CBW. Therefore, for all device types in 6GR, a minimum UE CBW of 3MHz CBW should be supported in 6G day-1.

Proposal 5: For all device types in 6GR, they should be able to operate with a minimum UE CBW of 3MHz in FR1 in 6G day-1.

However, the key question is how would 6GR support 3MHz UE CBW, e.g., whether the system design would be optimized for 3MHz CBW or not. For an optimized system design for 3MHz UE CBW, the corresponding SSB BW needs to be reduced from 20 PRB in 5G NR to 15 PRB. In 5G NR, the SSB is simply punctured and thus a coverage loss is observed with 3MHz SSB design.

Observation 1: the key issue on minimum BW of 6GR is whether to optimize the system design based on 3MHz or 5MHz.

In 6GR, if it would be agreed to have a native design of SSB for 3MHz UE CBW, a longer duration e.g., a longer SSB sequence or higher repetition needs to be considered to ensure the coverage. In addition, how to handle the sync raster design with smaller SSB BW also needs to be considered if the same principle for sync raster design as in 5G NR would be adopted. Last but not least, the criticality of 3MHz to operators’ spectrum resources need to be taken into account as well. Hence, we propose that RAN4 focuses on deciding if 3MHz UE CBW shall be supported in an optimized manner or in a similar approach as in 5G NR based on the factors of SSB coverage, processing delay, sync raster and operator spectrum allocation.

Proposal 6: RAN4 focuses on deciding if 3MHz UE CBW shall be supported in an optimized manner or in a similar approach as in 5G NR with consideration of SSB coverage, processing delay, sync raster and operator spectrum allocation.

## Max UE CBW

The maximum UE CBW affects UE implementations in both RF and BB domains. Therefore, we consider that different device types may have different maximum UE CBW. The detailed proposal for device type has been presented in our accompanying paper in [3]

Proposal 7: RAN4 shall consider different max UE CBW for different device types.

### Max UE CBW for broadband and ultra-broadband devices

For broadband and ultra-broadband devices, a larger single CC UE CBW can be considered for 6GR, supporting higher data rates in the upper part of FR1 and “FR3”, compared to 5G NR. Considering the SCS and FFT size discussed in section 3 and 4, it is proposed that the maximum UE CBW with single CC for broadband devices is 100 MHz on FR1, “200” MHz in FR3 and 400 MHz in FR2-1. For ultra-broadband devices, it can be 200 MHz in FR1, 400 MHz in “FR3”, and 400 MHz in FR2-1 in 6GR.

Proposal 8: It is proposed that the maximum UE CBW with a single CC for Broadband device with formfactor limitations, e.g., smartphone, glass type XR devices, to be 100 MHz in FR1, 200 MHz in “FR3”, and 400 MHz in FR2-1. For ultra-broadband devices, it can be 200 MHz in FR1, 400 MHz in “FR3”, while keeping 400 MHz in FR2-1.

### Max UE CBW for Massive IoT devices

For massive IoT devices, a typical maximum UE CBW currently considered would be in the range of 5-20 MHz [1],[4]-[6]. From a DL perspective, a too small BW for massive IoT may limit the allocation of SIB and PUCCH, which may affect performance and frequency resources usage of broad band device types that are mentioned above, assuming the desirable scenario that massive IoT devices and broadband devices share the same SIB and PUCCH. On the other hand, a large DL BW for massive IoT, e.g., 20 MHz, may require an increased baseband processing capability and memory footprint to handle a larger data rate. Therefore, a balanced design with consideration of both the impact to broadband device types as well as the device complexity is needed to determine the bandwidth of the massive IoT devices.

Proposal 9: RAN4 to study the different max bandwidth of the massive IoT devices with consideration of both the impact to broadband device type as well as the device complexity.

One possible design could be that massive IoT devices can support a relatively wide bandwidth, especially in the DL direction, but with a peak date rate limitation similar to 5G NR eRedcap, to balance the impact to broadband devices while limiting the device complexity of 6G massive IoT.

Observation 2: A massive IoT device may support a relatively wide bandwidth, especially in the DL direction, but with a peak date rate limitation similar to 5G NR eRedcap, to balance the impact to broadband devices while limiting the device complexity of 6G massive IoT.

However, one particular issue related to the bandwidth of massive IoT, especially the UL bandwidth, is about enabling a true SAW-less design for 6G massive IoT devices. SAW-less HD-FDD design is a key consideration for reducing RF domain complexity compared to (e)Redcap in 5G NR [3][4][5], where the implementation of the SAW-less device is shown in Fig. 1.  With a fully SAW-less HD-FDD design:

- A true single SKU design for global operation is possible, allowing for true economies of scale.

- Additionally, a fully SAW-less HD-FDD design can achieve a reduced chip footprint and a smaller IoT module.

- Last but not least, the insertion loss on Tx and Rx chain can be reduced, and the power efficiency of the system can be enhanced.

A similar design principle has been widely adopted in 4G IoT, e.g., cat-M1 and NB-IoT, in commercial production, thanks to 4G-IoT's even smaller CBW. However, it is not the case in 5G NR as larger BW needs to be supported, even for (e)Redcap devices, which is one of the factors that creates a significant implementation feasibility difference between 4G IoT and 5G (e)Redcap devices. The UL CBW is particularly critical here, as SAW filters are usually required to suppress high-order intermodulation distortion (IMD) products to meet the spurious emission limits defined either in 3GPP or by regulators. Therefore, a slightly reduced bandwidth compared to 5G (e.g., 3 MHz or allowable output power backoff in a challenging scenario) would be required in the 6G design to enable a fully SAW-less HD-FDD implementation.

Observation 3: with 3MHz UL BW, a full SAW-less design of 6G massive IoT with improved power efficiency and reduced chipset footprint supporting a true single SKU design for global operation can be enabled.

Fig. 1. Comparison of massive IoT device with SAW filter/duplexer design vs a fully SAW-less design.

Additionally, uplink transmission, particularly for IoT devices, is power-limited and reducing UL bandwidth can lower PA, output power demand, thermal load, and Battery consumption. Meanwhile, a smaller bandwidth leads to a higher power spectral density (PSD) that can improve the network coverage This is especially beneficial for small IoT UEs under high load.

Observation 4: Smaller uplink BW can help IoT devices to reduce the power consumption while improving the coverage with an enhanced PSD.

Moreover, for IoT devices which mostly operate in lower bands, especially in sub 1GHz, the spectrum allocation for each operator is typically very limited anyway. Mandating low end IoT devices to support a larger bandwidth that it may never use doesn’t provide sufficient benefit in terms of cost and complexity. Therefore, it is proposed to consider at least allowing 3MHz maximum UE CBW either in RF or BB in UL as a possible configuration to enable SAW-less implementation in the 6G massive IoT design.

Proposal 10: For massive IoT devices, consider allowing UE to operate with 3 MHz as the maximum UE RF or BB CBW in UL as a possible configuration, at least for FDD bands in below 1GHz, to allow SAW-less implementation for 6G massive IoT devices.

Proposal 11: RAN4 can also consider different maximum BW for different frequency bands/sub frequency ranges.

It is also beneficial for the 6GR design to consider different values of max UE CBW in UL and DL directions, e.g., asymmetric UE CBW. In our understanding, this should be straightforward at least in FDD bands, where it is natural that the UE has different RF chains for UL and DL, thus there is no limitation due to RF re-tuning that would be needed if different UE CBW is configured for UL and DL.

Observation 5: it is feasible from RF aspect to support asymmetric UL and DL BW, including asymmetric maximum BW at least for FDD bands.

Proposal 12: The 6GR study shall consider the UL UE CBW is different to the DL UE CBW, including the maximum UE CBW.

# Conclusion

In this contribution, we share our initial view on the bandwidth and its related parameters of 6GR from the RAN4 perspective, where the following observations and proposals are given:

Observation 1: the key issue on minimum BW of 6GR is whether to optimize the system design based on 3MHz or 5MHz.

Observation 2: A massive IoT device may support a relatively wide bandwidth, especially in the DL direction, but with a peak date rate limitation similar to 5G NR eRedcap, to balance the impact to broadband devices while limiting the device complexity of 6G massive IoT.

Observation 3: with 3MHz UL BW, a full SAW-less design of 6G massive IoT with improved power efficiency and reduced chipset footprint supporting a true single SKU design for global operation can be enabled.

Observation 4: Smaller uplink BW can help IoT devices to reduce the power consumption while improving the coverage with an enhanced PSD.

Observation 5: it is feasible from RF aspect to support asymmetric BW, including asymmetric maximum BW at least for FDD bands.

Proposal 1: It is proposed to assume a 2k FFT for massive IoT devices, while an 8k FFT size as baseline for higher-capability device types while study the feasibility of supporting even higher FFT size.

Proposal 2: It is proposed that 6GR adopts single numerology per frequency band.

Proposal 3: It is proposed that 6GR should adopt 15 kHz SCS for frequency bands below 3 GHz and 30 kHz for frequency bands above 3 GHz in FR1 and around 7 GHz, 60 kHz for frequencies around 15GHz and 120 kHz for FR2-1.

Proposal 4: RAN4 further studies the SCS for SSB design, including whether it should be the same or not with the data channel, once the baseline SCS and basic SSB design is determined.

Observation 5: it is feasible from RF aspect to support asymmetric UL and DL BW, including asymmetric maximum BW at least for FDD bands.

Proposal 6: RAN4 focuses on deciding if 3MHz UE CBW shall be supported in a optimized manner or in a similar approach as in 5G NR with consideration of SSB coverage, processing delay, sync raster and operator spectrum allocation.

Proposal 7: RAN4 shall consider different max UE CBW for different device types.

Proposal 8: It is proposed that the maximum UE CBW with a single CC for Broadband device with formfactor limitations, e.g., smartphone, glass type XR devices, to be 100 MHz in FR1, 200 MHz in “FR3”, and 400 MHz in FR2-1. For ultra-broadband devices, it can be 200 MHz in FR1, 400 MHz in “FR3”, while keeping 400 MHz in FR2-1.

Proposal 9: RAN4 to study the different max bandwidth of the massive IoT devices with consideration of both the impact to broadband device type as well as the device complexity.

Proposal 10: For massive IoT devices, consider allowing UE to operate with 3 MHz as the maximum UE RF or BB CBW in UL as a possible configuration, at least for FDD bands in below 1GHz, to allow SAW-less implementation for 6G massive IoT devices.

Proposal 11: RAN4 can also consider different maximum BW for different frequency bands/sub frequency ranges.

Proposal 12: The 6GR study shall consider the UL UE CBW is different to the DL UE CBW, including the maximum UE CBW.

# References

- R4-2514508		Topic Summary for [116bis][101] 6G system parameter	Moderator (Huawei)

- R4-2514618		WF on [116bis][101] 6G system parameter	Huawei

- RP-251951		Minimum requirements for Massive Communication for IoT	Ericsson, AT&T, BT, CHTTL, Deutsche Telekom, FirstNet, Fujitsu, Intel Corporation, KDDI, KT Corp., NTT DOCOMO, OPPO, Semtech, Sequans, SK Telecom, Sony, Spark NZ, Telstra, Thales, T-Mobile USA, Verizon, Vodafone

- RP-252119		6G Massive IoT Requirements	Sony Europe Limited, Nordic Semiconductor ASA

- RP-252327		On 3GPP Support of Massive IoT in 6G	AT&T, Ericsson, FirstNet, KT Corp., MediaTek Inc., Nokia, Nordic Semi, NTT DOCOMO, Qualcomm, Rakuten, Sequans, Spark NZ, SONY, T-Mobile USA, Verizon