3GPP TSG-RAN WG4#117	R4-252060

Dallas, TX, USA, November 17th – 21st, 2025

Agenda item:	5.9.4

Source: 	Qualcomm

Title: 	About CA_n5A-n8A

Document for:	Approval

# Introduction

Considerations on CA_n5A-n8A are provided in this contribution. This contribution is submitted into both RRM and UE RF agendas as this covers both aspects.

- Discussion

UL CA_n5A-n8A (2UL/2DL) version of it has been discussed across two releases, Rel-18 and Rel-19. In RAN4#116, the discussion was transferred into RRM session to discuss RRM impacts. There was no conclusion in RRM session either.

WF from [1] RAN4#116 is as follows.

In our view, the challenges are multi-dimensional and cannot be easily addressed by just looking into e.g. RRM impacts, as the issues raised by companies are not in general properly addressed. It can be noted that given this topic keeps appearing in 3GPP, more and more internal discussions across different WG’s have probably been carried out.

The agreement in RAN4#116 was as follows.

Agreement:

For CA_n5-n8, in RAN4#116bis, an agenda item in RRM session will be introduced to discuss and determine if there is RRM impact. If RRM impact is identified and agreed, this topic will not be further pursued. If not, the discussion will continue in main session under R19 maintenance.

- When n5DL is operated, n8UL RF path needs to be switched off and the same for the other way around.

- Proponent companies are encouraged to bring detailed anlysis, including the possible gain/loss.

Before the conclusion on RRM impact is made, no further discussion is needed in the main session.

The most important aspect is the first bullet “When n5DL is operated, n8UL RF path needs to be switched off and the same for the other way around.”. We identified this issue several meetings ago, and we consider that is the key aspect to properly address.

In said RRC Case 1 (2UL/2DL), one must reconfigure RFFE between any n5DL reception and any n8 UL transmission, and vice versa. In other words, there must be an empty slot or slots depending on the reconfiguration time. In addition, UE must know when the empty slot/slot is/are. This matter was partially addressed in [2] as there is empty subframe #9 and #19 in the example scheduling pattern figure.

This scheduling restriction was proposed to be handled by scheduling restriction in RAN4 specification “RAN4 to capture the following NW scheduling restriction or similar in RF spec: UE is not scheduled to transmit on n8_UL in slots where UE is configured or scheduled to receive on n5_DL”

In our view, the scheduling restriction would require definition of NW and UE behavior and possibly some switching pattern definition in RAN1. To us, e.g. wording “DCI on n8_DL schedules n8_UL for subframe #9 which triggers to switch from 1UL/2DL to 2UL/1DL” is clearly something which cannot be done just in RAN4. It would be entirely new thing if UE is expected to do some reconfiguration based on scheduling only. In our view, this is clearly UE behavior related matter and should hence be addressed in RAN1 specifications.

Furthermore, some proponent companies have proposed to introduce a new UE capability “supportOfnon-ConcurrentInterbandULDL-r19” which in our view would be totally against 3GPP principles as for every UE capability the expected NW and UE behavior should be clearly specified. Just stating the obvious, NW and UE behavior cannot be just captured in RAN2 UE capability.

Recap of the discussion in RRM session during RAN4 #116bis

Based on online and offline discussions, the following observations were acknowledged by the companies:

Performance Gain Analysis

The analysis provided by the proponents appears incorrect and requires re-assessment under more realistic assumptions for the following reasons:

- RF Switching Constraint: There should be no DL Rx/UL Tx during RF switching.

- UL Scheduling Limitation: Even if 18 slots are available for UL scheduling (excluding 2 slots impacted by RF switching) on UL band n5, the effective number of UL slots for UL scheduling is limited by the available DL slots on band 5, which is 9, unless both UE and NW support the optional feature of TB-over-multi-slot.

  - This feature is not commercially available as far as we know.

  - Therefore, the analysis does not seem practical or field-relevant.

  - This also contradicts the proponents’ argument that 2UL/2DL CA for CA_n5-n8 is simple and readily deployable in the field.

Specification Impact

- There are potential impacts on specifications beyond RF requirements and UE capability.

- Scheduling Restrictions: These should be explicitly stated in the 3GPP specification, with TS 38.213 being an appropriate place. For example:

  - Clarify from which slot the scheduling restriction and RF switching should begin upon UL scheduling on n8.

  - Clearly state that the UE is not expected to be scheduled with UL on n8 in slots where DL measurement resources are present, as well as in the slot immediately before and after those.

- Scheduling restrictions need to be enforced by explicit statements somewhere in the 3GPP spec (TS 38.214 seems appropriate).

- UE behavior during the RA procedure, particularly RAR monitoring, needs to be specified. This likely requires additions in the RAN1 and/or RAN2 specifications.

Observation 1: In order to make UL CA_n5A-n8A feasible, there must be an empty slot/slots between any n5DL and n8UL in RRC Case 1, and vice versa

Observation 2: UE must know the location for the empty slot/slots

Observation 3: RAN4 cannot alone define the location of the empty slot/slots, instead it must be NW and UE behavior aspect and maybe could even require switching pattern (RAN1)

Observation 4: UE capability cannot be even considered for this case, unless expected NW and UE behavior is known

Observation 5: The following observations were acknowledged by the companies in RRM session at RAN4 #116bis:

- The performance gain analysis provided by the proponents appears incorrect and requires re-assessment under more realistic assumptions for the following reasons:

  - RF Switching Constraint: There should be no DL Rx/UL Tx during RF switching.

  - UL Scheduling Limitation: Even if 18 slots are available for UL scheduling (excluding 2 slots impacted by RF switching) on UL band n5, the effective number of UL slots for UL scheduling is limited by the available DL slots on band 5, which is 9, unless both UE and NW support the optional feature of TB-over-multi-slot.

    - This feature is not commercially available as far as we know.

    - Therefore, the analysis does not seem practical or field-relevant.

    - This also contradicts the proponents’ argument that 2UL/2DL CA for CA_n5-n8 is simple and readily deployable in the field.

- There are potential impacts on specifications beyond RF requirements and UE capability.

  - Scheduling Restrictions: These should be explicitly stated in the 3GPP specification, with TS 38.213 being an appropriate place. For example:

    - Clarify from which slot the scheduling restriction and RF switching should begin upon UL scheduling on n8.

    - Clearly state that the UE is not expected to be scheduled with UL on n8 in slots where DL measurement resources are present, as well as in the slot immediately before and after those.

  - Scheduling restrictions need to be enforced by explicit statements somewhere in the 3GPP spec (TS 38.214 seems appropriate).

  - UE behavior during the RA procedure, particularly RAR monitoring, needs to be specified. This likely requires additions in the RAN1 and/or RAN2 specifications.

Proposal 1: There are implications to other WG’s, so as per WF R4-2507910, this UL CA is not pursued in Rel-19

Proposal 2: RAN4 shall not proceed with defining UL CA_n5A-n8A unless UE and NW behavior is clearly assured through proper specification support from RAN1 and RAN2.

Proposal 3: If proponent companies are still keen to specify this UL CA_n5A-n8A, it would need to be discussed in RAN Plenary and RAN1 should be involved in the work

- Conclusion

Considerations on CA_n5A-n8A are provided in this contribution are provided in this contribution with the following observations and proposals.

Observation 1: In order to make UL CA_n5A-n8A feasible, there must be an empty slot/slots between any n5DL and n8UL in RRC Case 1, and vice versa

Observation 2: UE must know the location for the empty slot/slots

Observation 3: RAN4 cannot alone define the location of the empty slot/slots, instead it must be NW and UE behavior aspect and maybe could even require switching pattern (RAN1)

Observation 4: UE capability cannot be even considered for this case, unless expected NW and UE behavior is known

Observation 5: The following observations were acknowledged by the companies in RRM session at RAN4 #116bis:

- The performance gain analysis provided by the proponents appears incorrect and requires re-assessment under more realistic assumptions for the following reasons:

  - RF Switching Constraint: There should be no DL Rx/UL Tx during RF switching.

  - UL Scheduling Limitation: Even if 18 slots are available for UL scheduling (excluding 2 slots impacted by RF switching) on UL band n5, the effective number of UL slots for UL scheduling is limited by the available DL slots on band 5, which is 9, unless both UE and NW support the optional feature of TB-over-multi-slot.

    - This feature is not commercially available as far as we know.

    - Therefore, the analysis does not seem practical or field-relevant.

    - This also contradicts the proponents’ argument that 2UL/2DL CA for CA_n5-n8 is simple and readily deployable in the field.

- There are potential impacts on specifications beyond RF requirements and UE capability.

  - Scheduling Restrictions: These should be explicitly stated in the 3GPP specification, with TS 38.213 being an appropriate place. For example:

    - Clarify from which slot the scheduling restriction and RF switching should begin upon UL scheduling on n8.

    - Clearly state that the UE is not expected to be scheduled with UL on n8 in slots where DL measurement resources are present, as well as in the slot immediately before and after those.

  - Scheduling restrictions need to be enforced by explicit statements somewhere in the 3GPP spec (TS 38.214 seems appropriate).

  - UE behavior during the RA procedure, particularly RAR monitoring, needs to be specified. This likely requires additions in the RAN1 and/or RAN2 specifications.

Proposal 1: There are implications to other WG’s, so as per WF R4-2507910, this UL CA is not pursued in Rel-19

Proposal 2: RAN4 shall not proceed with defining UL CA_n5A-n8A unless UE and NW behavior is clearly assured through proper specification support from RAN1 and RAN2.

Proposal 3: If proponent companies are still keen to specify this UL CA_n5A-n8A, it would need to be discussed in RAN Plenary and RAN1 should be involved in the work

- Reference

[1] R4-2507910, WF on simultaneous Rx/Tx requirements, Huawei

[2] R4-2514184, “Discussion on RRM impacts for CA_n5_n8”, Huawei, HiSilicon