3GPP TSG RAN WG4 Meeting #117	R4-2522108

Dallas, US, November 17 - 21, 2025

Title: 	Introduction of US 4.9GHz band system parameters

Agenda item:	5.11.2

Source:	Huawei, HiSilicon

Document for:	Discussion

# Introduction

To introduce US 4.9GHz band the system parameters are discussed in this tdoc. All the Tables refer to TS 38.101-1.

# Discussion

## System parameters

Operating band

A new TDD band should be created as shown in table below:

NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n7917 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |
| n114 | 4940 MHz – 4990 MHz | 4940 MHz – 4990 MHz | TDD |
| NOTE 17: For this band, CORESET#0 values from Table 13-5 or Table 13-6 in [8, TS 38.213] are applied regardless of the minimum channel bandwidth. |  |  |  |

It should be mentioned that n79 was tagged with Note 17. However with the minimum CBW is 10 MHz per WID[1], the CORESET#0 configurations of TS 38.213 Table 13-5/6 would be very limited, then it would be better not to mention NOTE 17 for this band, which would allow using Table 13-1/2/3/4 of TS 38.213 for CORESET#0 configurations.

Proposal 1: update Table 5.2-1 as:

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n114 | 4940 MHz – 4990 MHz | 4940 MHz – 4990 MHz | TDD |

No need to use NOTE 17 for n114.

Proposal 2: Update Table 5.3.5-1 as:

Table 5.3.5-1 Channel bandwidths for each NR band

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n114 | 15 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |

Channel raster (NR- ARFCN):

The channel raster range is given in table below:

Proposal 3: Update Table 5.4.2.3-1 as:

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n114 | 15 | 729334 – <1> – 732666 | 729334 – <1> – 732666 |
|  | 30 | 729334 – <2> – 732666 | 729334 – <2> – 732666 |

Synchronization raster:

The following GSCN range can be used for n114.

Proposal 4: Update Table 5.4.3.3-1 as:

Table 5.4.3.3-1: Applicable SS raster entries per operating band for above 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n114 | 30 kHz | Case C | 8850 – <1> – 88877 |

# Conclusions

Proposal 1: update Table 5.2-1 as:

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n114 | 4940 MHz – 4990 MHz | 4940 MHz – 4990 MHz | TDD |

No need to use NOTE 17 for n114.

Proposal 2: Update Table 5.3.5-1 as:

Table 5.3.5-1 Channel bandwidths for each NR band

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n114 | 15 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |

Proposal 3: Update Table 5.4.2.3-1 as:

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n114 | 15 | 729334 – <1> – 732666 | 729334 – <1> – 732666 |
|  | 30 | 729334 – <2> – 732666 | 729334 – <2> – 732666 |

Proposal 4: Update Table 5.4.3.3-1 as:

Table 5.4.3.3-1: Applicable SS raster entries per operating band for above 3 MHz channel bandwidth

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n114 | 30 kHz | Case C | 8850 – <1> – 88877 |

# Reference

[1]	“RP-252929, Revised WID on Introduction of NR TDD 4.9GHz band for US operation, Fujitsu.”