3GPP TSG-RAN WG4 Meeting #117                                            R4-2522137

Dallas, USA, Nov 17 – 21, 2025

Source: 	Huawei, HiSilicon

Title: 	Initial coexistence simulation results for outdoor AIoT

Agenda Item:	7.11.3

Document for:	Approval

# Introduction

The simulation assumptions for calibration are agreed in WF [1]. Some initial results are presented in the following.

For calibration purpose, the coupling loss between NR UE and BS or between AIoT device and reader as well as the intra-system SINR are the key metrics for companies to report and align. The SINR including both intra- and inter-system interference is reported for information purpose at this meeting.

# Discussion

## Simulation assumptions

As per WF [1], the following cases are to be evaluated. Due to limited time, we have only simulated standalone mode with frequency reuse of 1:1.

Table 1: Evaluation cases

| Operation mode | Device type | Evaluation case number | Aggressor | Victim | Direction |
| --- | --- | --- | --- | --- | --- |
| Standalone | Device 2b | 1 | Device | NR UL | D2R |
|  |  | 2 | NR UL | reader | D2R |
|  |  | 3 | Reader | NR DL | R2D |
|  |  | 4 | NR DL | device | R2D |
|  | Device C | 5 | Device | NR UL | D2R |
|  |  | 6 | NR UL | reader | D2R |
|  |  | 7 | Reader | NR DL | R2D |
|  |  | 8 | NR DL | device | R2D |
| In-band | Device 2b | 9 | Device | NR UL | D2R |
|  |  | 10 | NR UL | reader | D2R |
|  |  | 11 | Reader | NR DL | R2D |
|  |  | 12 | NR DL | device | R2D |
|  | Device C | 13 | Device | NR UL | D2R |
|  |  | 14 | NR UL | reader | D2R |
|  |  | 15 | Reader | NR DL | R2D |
|  |  | 16 | NR DL | device | R2D |

To calculate the interference between NR and AIoT systems, the ACLR and ACS parameters as listed in Table 2 are used in the simulations.

Table 2: ACLR and ACS parameters for the simulation (credit to VIVO for the block diagrams)

| Aggressor | Victim | ACLR | ACS |
| --- | --- | --- | --- |
| Device D2R | NR UL | For UE3, 25/20dB are used as the effective ACLR for device C and 2b, respectively. | 46dB as effective ACS is used for all UEs. |
| NR UL | Reader D2R | ACLRe = X + 10*log10(BWUE/BWD2R) The scaling factor of 10log10(3.3M/15k) = 23dB is used for all UEs. For UE1/UE2, the ACLRe=30+23=53dB For UE3, the ACLRe = 43+23=66dB | ACSe = X + 10*log10(BWUE/BWD2R) for UE3 No interference from UE1/UE2 The ACS (and ACLR) for AIoT is likely to be defined over the channel BBW (e.g. 180 or 200kHz). Hence, for UE3 apply the scaling factor of 10log10(10M/3/180k)=12dB, i.e. effective ACS for UE3=46+12=58dB. |
| Reader R2D | NR DL | For reader, the effective ACLR is set to 45dB. | For UE, the ACS is set to 33dB. |
| NR DL | Device R2D | ACLRe = X + 10*log10(BWUE/BWR2D) The scaling factor is 17dB, resulting in ACLR=45+17=62dB. | ACSe = X + 10*log10(BWUE/BWR2D) Following the above 2-step model, the effective ACS for the 10MHz NR interference is set to 35/30 + 17dB for device C and 2b, respectively. |

## Simulation results

Some simulation results are plotted in Table 3. Note that the device always transmits with max power (i.e., 5dBm for device C) for calibration as per WF [1].

Between NR UE and BS, minimum coupling loss of 70dB is applied. Between AIoT device and reader, minimum distance of 35 meters is applied.

Table 3: Simulation results for case 5, 6, 7, 8 (i.e., SA, device C, frequency reuse 1:1, device always Tx at 5dBm)

| Aggressor | Victim | Coupling Loss | SINR |
| --- | --- | --- | --- |
| Device D2R | NR UL |  |  |
| Reader R2D | NR DL |  |  |
| NR UL | Reader D2R |  |  |
| NR DL | Device R2D |  |  |

## ACIR modelling

NR and AIoT systems have different channel bandwidths. The former is wideband and the latter is narrowband. Hence, there’re two types of ACIR modelling.

The first one is for the case when the wideband system interferes with the narrowband system. For example,

Table 4: Effective ACLR and ACS when NR BS interferes with AIoT device

| ACLRe = X + 10*log10(BWUE/BWR2D) . | ACSe = X + 10*log10(BWUE/BWR2D) |
| --- | --- |

For this case, there seems to be consensus that a bandwidth scaling factor is applied to derive the effective ACLR and ACS.

The second one is for the case when the narrowband system interferes with the wideband system. For example,

Table 5: Effective ACLR and ACS when AIoT BS interferes with NR UE

With the two-step ACLR model, the effective ACLR can be calculated as:

1aclre=1aclr1+1aclr2BWvic-BWaggBWagg

Since the victim BW is much larger than the aggressor (10MHz vs 180kHz), maybe a three-step ACLR model is more suitable. For example, ACLR1=X dB, ACLR2 = X+13 dB, and ACLR3 = X+20 dB.

- RAN4 to discuss and agree on the ACIR model for coex simulations by considering:

- BW scaling factor when NR system interferes with AIoT;

- two-step or three-step ACLR model when AIoT system interferes with NR.

# Conclusion

This contribution provides some initial simulation results for calibration purpose. And we make the following proposal for formal coexistence simulations:

- RAN4 to discuss and agree on the ACIR model for coex simulations by considering:

- BW scaling factor when NR system interferes with AIoT;

- two-step or three-step ACLR model when AIoT system interferes with NR.

# References

[1]: R4-2515152 WF for [116bis][321] Study of A-IoT for outdoor, LGE, RAN4#116bis