3GPP TSG-RAN4 Meeting #117	R4-2522246

# Dallas, USA, 17th Nov 2025 – 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6273 | rev | - | Current version: | 18.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_ATG-Perf) Io value correction in Table A.19.4.5.1.1-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-Perf |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Incorrect Io value in Table A.19.4.5.1.1-3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Io value in Table A.19.4.5.1.1-3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test parameters |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.19.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <<Start of Change1>>

### A.19.4.5	Pathloss reference signal switching delay

#### A.19.4.5.1	MAC-CE based pathloss reference signal switch delay

##### A.19.4.5.1.1	Test Purpose and Environment

The purpose of this test is to verify the MAC-CE based pathloss reference signal switch delay requirement defined in clause 8.14D.

The supported test configurations are shown in table A.19.4.5.1.1-1. The test scenario comprises of one cell (Cell 1) as given in table A.19.4.5.1.1-2. Cell-specific parameters of the cell are specified in table A.19.4.5.1.1-3 below.

The test consists of 3 successive time periods, with duration of T1, T2 and T3, respectively.

Prior to the start of the time duration T1,

-	UE is connected to Cell 1 on radio channel 1.

-	UE shall be fully synchronized to SSB #0.

During T1,

-	The UE shall track SSB #1 so that SSB #1 as a pathloss reference signal is known to the UE.

Time period T2 starts when the UE is configured of the power headroom reporting functionality by upper layers by the test equipment and the UE shall transmit a PHR during T2.

During T2,

-	UE is configured with a phr-ProhibitTimer timer value for Cell 1.

-	UE is configured with a phr-Tx-PowerFactorChange value for Cell 1.

During T3,

Time period T3 starts when a PDSCH carrying MAC-CE activation for pathloss reference signal switch, sent from the test equipment to the UE to swicth the pathloss reference signal from SSB 0 to SSB 1, is received at the UE side in Cell 1’s slot # denoted i. The UE shall switch its pathloss reference signal to the target one and send PHR.

The UE shall be able to apply the target pathloss reference signal of the serving cell on which pathloss reference signal switch occurs no later than the slot i + THARQ+ 3 ms + 5*Ttarget_PL-RS + 2 msNR slot length as defined in clause 8.14.  The UE shall be able to apply old pathloss reference signals until the slot i + THARQ+ 3Nslotsubframe,µ as defined in clause 8.14.

The test equipment verifies the pathloss RS switch time by counting the slots from the time when the pathloss RS switch command is transmitted till a PHR is received during T3.

UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

The specific gNB reference location is emulated by test system.

Table A.19.4.5.1.1-1: MAC-CE based pathloss reference signal switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.19.4.5.1.1-2: General test parameters for MAC-CE based pathloss reference signal switch in SA

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Active PCell |  |  | Cell 1 |  |
| RF Channel Number |  |  | 1 |  |
| Duplex mode | Config 1 |  | FDD |  |
|  | Config 2, 3 |  | TDD |  |
| DL initial BWP configuration | Config 1, 2, 3 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration | Config 1, 2, 3 |  | DLBWP.1.1 |  |
| UL initial BWP configuration | Config 1, 2, 3 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration | Config 1, 2, 3 |  | ULBWP.1.1 |  |
| TDD Configuration | Config 1 |  | Not Applicable |  |
|  | Config 2 |  | TDDConf.1.1 |  |
|  | Config 3 |  | TDDConf.2.1 |  |
| CORESET Reference Channel | Config 1 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| SSB Configuration | Config 1 |  | SSB.1 FR1 |  |
|  | Config 2 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB.2 FR1 |  |
| SMTC Configuration | Config 1, 2 |  | SMTC.1 |  |
|  | Config 3 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing | Config 1, 2 |  | 15 kHz |  |
|  | Config 3 |  | 30 kHz |  |
| SSB index assigned as pathloss RS |  |  | 0 in T1, 0 in T2, 1 in T3 |  |
| OCNG parameters |  |  | OP.1 |  |
| CP length |  |  | Normal |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |  |
| DRX |  |  | OFF |  |
| Gap pattern ID |  |  | gp0 |  |
| phr-ProhibitTimer |  | sub frame | 0 |  |
| phr-Tx-PowerFactorChange |  | dB | 5 |  |
| phr-PeriodicTimer |  | sub frame | infinity |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| T1 |  | s | 2 |  |
| T2 |  | s | 2 |  |
| T3 |  | s | 0.2 |  |

Table A.19.4.5.1.1-3: NR Cell specific test parameters for MAC-CE based pathloss reference signal switch in SA

| Parameter |  |  |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T3 |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  | dB | 4 |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  | dB | 0 |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  | dB | 0 |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  | dB |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  | dB |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  | dB |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  | dB |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  | dB |  |  |  |
| SSB with index 0 |  |  |  | dB | 7 |  |  |
|  |  | Config 1, 2, 3 |  | dBm/15 kHz | -101 |  |  |
|  |  |  |  | dB | 7 |  |  |
|  | SS-RSRP Note 4 |  | Config 1, 2 | dBm/ SCS | -94 |  |  |
|  |  |  | Config 3 |  | -91 |  |  |
| SSB with index 1 |  |  |  | dB | -3 |  |  |
|  |  | Config 1, 2, 3 |  | dBm/15 kHz | -101 |  |  |
|  |  |  |  | dB | -3 |  |  |
|  | SS-RSRP Note 4 |  | Config 1, 2 | dBm/ SCS | -104 |  |  |
|  |  |  | Config 3 |  | -101 |  |  |
| SSB with index 0 Io Note 5 | Config 1, 2 |  |  | dBm | -65.3/9.36 MHz |  |  |
|  | Config 3 |  |  |  | -59.2/38.16 MHz |  |  |
| SSB with index 1 Io Note 5 | Config 1, 2 |  |  | dBm | -71.28/9.36 MHz |  |  |
|  | Config 3 |  |  |  | -65.18/38.16 MHz |  |  |
| Propagation condition | Config 1, 2 |  |  |  | AWGN +220 Hz |  |  |
|  | Config 3 |  |  |  | AWGN +500 Hz |  |  |
| NOTE 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: The signal contains PDCCH for UEs other than the device under test as part of OCNG. NOTE 3: SNR levels correspond to the signal to noise ratio over the SSS REs. NOTE 4: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. NOTE 5: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. |  |  |  |  |  |  |  |

##### A.19.4.5.1.2	Test Requirements

During T3, the UE shall start to send the PHR for PCell no later than the slot i + THARQ+ 3 ms + 5*Ttarget_PL-RS + 2 msNR slot length.

During T3, the UE shall start to send the PHR for PCell no earlier than the slot i + THARQ+ 3Nslotsubframe,µ.

Where, THARQ is the timing between pathloss reference MAC-CE activation command and acknowledgement as specified in [7], Ttarget_PL-RS is the periodicity of the target pathloss reference signal which is SSB in this test.

During T3, UE shall send L1-RSRP report with measurement results for both SSB0 and SSB1.

All of the above test requirements shall be fulfilled in order for the observed pathloss RS switch delay to be counted as correct.

The rate of correct events observed during repeated tests shall be at least 90 %.

NOTE:	The UE shall be given proper uplink transmission grant during T2 and T3.

### <<End of Change1>>