3GPP TSG-RAN WG4 Meeting #117	      R4-2522255

Dallas, USA, 17th – 21st of November, 2025

Agenda item:			10.6.1

Source:	THALES, SES, EUTELSAT Group, ESA, HISPASAT, XIAOMI, CATT, ZTE, EchoStar, Airbus, Gilat, Aalyria, Fraunhofer IIS, Gatehouse Satcom, Novamint, Lockheed Martin, Tejas Networks, JSAT, MITRE, Kratos, SHARP, BMW, CSCN

Title:	Maintenance - Correction of Mobile VSAT with NGSO for the Ka-band

Document for:	Discussion and Decision

# Introduction

Rel-18 specifications for the Ka-Band VSAT have accommodated mobile VSAT with GSO only connectivity, as it can be seen below for mobile VSAT type 4 and 5 (table from TS 38.101-5 Rel-18 version 18.11.0).

Table 9.2.1.0-1: The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. |
|  | 52 | Mobile VSAT communicating with GSO with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. |  |  |

However, when Ku-band VSAT was introduced as part of Rel-19, the table from TS 38.101-5 (version 19.2.0) was updated as follows:

Table 9.2.1.0-1: The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 43 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52, 3 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 64 | Mobile VSAT communicating with LEO only with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. NOTE 3: GSO support is applicable to bands n512, n511, n510, n509, n508 and n248, n247; LEO support is only applicable to bands n509, n508 and n248, n247. NOTE 4: The NTN VSAT type 6 is only applicable to bands n509, n508 and n248, n247. |  |  |

There are therefore several differences between Rel-18 and Rel-19 mobile VSAT types as follows:

|  | VSAT type 4 | VSAT type 5 | VSAT type 6 |
| --- | --- | --- | --- |
| Ku-band (from Rel-19) | GSO and NGSO | GSO and NGSO | NGSO only |
| Ka-band (from Rel-18) | GSO (currently) | GSO (currently) | N/A |
| VSAT type | Mechanical steering | Electronic steering | Electronic steering |

With the following frequency bands (see for instance Rel-19 TS 38.101-5, version 19.2.0) for the Ka-band and Ku-band operating in FR2-NTN:

Table 5.2.3-1: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n5121 | 27500 MHz - 30000 MHz | 17300 MHz - 20200 MHz | FDD |
| n5112 | 28350 MHz - 30000 MHz | 17300 MHz - 20200 MHz | FDD |
| n5103 | 27500 MHz - 28350 MHz | 17300 MHz - 20200 MHz | FDD |
| n509 | 14000 MHz - 14500 MHz | 10700 MHz – 127504,5 MHz | FDD |
| n508 | 13750 MHz - 14000 MHz | 10700 MHz – 127504,5 MHz | FDD |
| NOTE 1: This band is applicable in the countries subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the countries subject to or referring to FCC 47 CFR part 25. NOTE 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202). NOTE 4: For Region 2, the downlink is limited to 10700 – 12700 MHz. NOTE 5: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT. |  |  |  |

The purpose of this contribution is to introduce the concept of the Mobile VSAT with NGSO for the Ka-band, VSAT types 4 & 5.

# Assumptions & Analysis

Relevant EN specifications for TS 38.101-5 Ka-band are:

1/ For GSO:

- EN 303 978 - V2.1.2, “Satellite Earth Stations and Systems (SES); Harmonised Standard for Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in geostationary orbit, operating in the 27,5 GHz to 30,0 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU”.

- EN 301 459 - V2.1.1, “Satellite Earth Stations and Systems (SES); Standard for Satellite Interactive Terminals (SIT) and Satellite User Terminals (SUT) transmitting towards satellites in geostationary orbit, operating in the 29,5 GHz to 30,0 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU”.

- EN 301 360 - V2.1.1, “Satellite Earth Stations and Systems (SES); Harmonised Standard for Satellite interactive Terminals (SIT) and Satellite User Terminals (SUT) transmitting towards satellites in geostationary orbit, operating in the 27,5 GHz to 29,5 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU”.

2/ For NGSO:

- EN 303 979 - V2.1.2, “Satellite Earth Stations and Systems (SES); Harmonised Standard for Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in non-geostationary orbit, operating in the 27,5 GHz to 29,1 GHz and 29,5 GHz to 30,0 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU”.

- EN 303 699 - V1.1.1, “Satellite Earth Stations and Systems (SES); Fixed stations communicating with nongeostationary satellite systems in the 20 GHz and 30 GHz FSS bands; Harmonised Standard for access to radio spectrum”.

However, only the GSO references have been integrated so far as part of Rel-18. Therefore, the EIRP limits specified by the European Harmonised Standards [1,2] have now to be integrated as part of 3GPP Specification [3], i.e. the EIRP off-axis mask specified in the 38.101-5 for terminals 4 and 5 should be re-used by NGSO VSATs to allow a smooth integration of GSO terminals as NGSO.

The following assumptions can be considered for Ka-band:

- For VSAT Types 4 and 5 when connecting to Ka-band NGSO we assume the same EIRPmax as for the Type 3 VSAT devices.

- We are assuming that through specific configuration (less antenna elements and/or less power), it is possible to use Type 5 as Type 3 VSAT for NGSO connectivity.

Similar assumptions were actually used for the Ku-band introduction in Rel-19 as it could be seen below for Note 2 and Note 3 below:

Table 9.2.1.0-1: The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 43 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52, 3 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 64 | Mobile VSAT communicating with LEO only with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. NOTE 3: GSO support is applicable to bands n512, n511, n510, n509, n508 and n248, n247; LEO support is only applicable to bands n509, n508 and n248, n247. NOTE 4: The NTN VSAT type 6 is only applicable to bands n509, n508 and n248, n247. |  |  |

Observation 1: Note 2 is mentioning “NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction.” This note was added to the Ku-band specification without verification (and also exists for the Ka-band from the beginning) and is essential since is up to implementation that VSAT type 5 comply with the OFF-axis EIRP mask.

Observation 2: Ku-band VSAT type 4 has no OFF-axis EIRP mask limitation with respect to LEO connectivity. This indicates that Ka-band VSAT type 4 should follow the same reasoning (since precedent exists).

Observation 3: After a very careful analysis of Rel-19 specification for TS 38.101-5, it has been noticed that no differentiation was carried out between VSATs Type 4/5 connecting to GSO and connecting to LEO for Ku-band, since worst case was considered for terminals capable of connecting to both GSO and LEO: same EIS, same EIRPmin and same EIRPmax. Same reasoning could be also therefore used for Ka-band.

In 2024, Eutelsat Group provided precise regulatory status explanation for bands above 10 GHz post WRC-23. With respect to Ka-band (17.7 to 30 GHz) is mentioned (see R4-2400516, Eutelsat Group, “Regulatory status of NTN in bands above 10 GHz post WRC-23”):

The almost complete Rel-18 specifications for NTN in bands above 10 GHz cover deployment requirements and use cases for the Ka band within the scope of the ITU-R following WRC-19. This did not permit what the ITU-R terms as non-GSO ESIM (Earth Station In Motion) – which 3GPP refer to as “mobile VSAT”. Non-GSO ESIM in Ka was reviewed at WRC-23 under agenda item 1.16 and resolution COM5/3 (WRC-23) updates use of the frequency bands 17.7-18.6 GHz, 18.8-19.3 GHz, 19.7-20.2 GHz (space-to-Earth) and 27.5-29.1 GHz and 29.5-30 GHz (Earth-to-space). The resolution adds aeronautical and maritime ESIMs communicating with non-geostationary space stations in the fixed-satellite service [2]. In [3] we propose that the Rel-18 standards gap of Mobile VSAT to NGSO should be addressed in the Rel-19 RAN4 non spectrum package, now that the regulatory blocker has been removed. With the following references: [2] World Radiocommunication Conference 2023 (WRC-23) Provisional Final Acts (ITU WRC, Dubai 2023) [3] R4-2400511 Motivation to support mobile VSAT in NGSO deployment scenarios (Eutelsat Group, RAN4#110, Athens)

Observation 4: Potentially, if required, a limitation could be introduced for NGSO connectivity with respect to frequency bands being mentioned at the WRC-23: 17.7-18.6 GHz, 18.8-19.3 GHz, 19.7-20.2 GHz (Space-to-Earth, or DownLink) and 27.5-29.1 GHz and 29.5-30 GHz (Earth-to-Space, or Uplink).

Observation 5: The TDoC provides a correction without any requirement change, therefore a CR would be possible. Otherwise, companies could agree to use a TEI.

Proposal 1: Since upon WRC-23 there were no regulatory limitations for Ka-band mobile VSAT with NGSO and there is no new ETSI EN impacted requirements (since GSO requirements can be reused for LEO), it is recommended to deal with this issue during maintenance phase.

Proposal 2: For Ka-band mobile VSAT type 5, use Note 2 from Ku-band “NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction.”

Proposal 3: For Ka-band mobile VSAT type 5, remove Note 3 from Ku-band “GSO support is applicable to bands n512, n511, n510, n509, n508 and n248, n247; LEO support is only applicable to bands n509, n508 and n248, n247.”

Proposal 4: Potentially, RAN4 could update the Rel-18 as follows through maintenance CR:

Table 9.2.1.0-1: The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. |  |  |

Proposal 5: RAN4 shall update the Rel-19 as follows through maintenance CR:

Table 9.2.1.0-1: The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 43 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52, 3 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 643 | Mobile VSAT communicating with LEO only with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. NOTE 3: GSO support is applicable to bands n512, n511, n510, n509, n508 and n248, n247; LEO support is only applicable to bands n509, n508 and n248, n247. NOTE 43: The NTN VSAT type 6 is only applicable to bands n509, n508 and n248, n247. |  |  |

Proposal 5 (alternative): Another way of proceeding without removing Note 3 but simply replace the content with a restriction, RAN4 shall update the Rel-19 as follows through maintenance CR:

Table 9.2.1.0-1: The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 43 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52, 3 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 64 | Mobile VSAT communicating with LEO only with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. NOTE 3: LEO support for bands n512, n511, n510 exclude 18.6-18.8 GHz in DL and 29.1-29.5 GHz in UL. GSO support is applicable to bands n512, n511, n510, n509, n508 and n248, n247; LEO support is only applicable to bands n509, n508 and n248, n247. NOTE 4: The NTN VSAT type 6 is only applicable to bands n509, n508 and n248, n247. |  |  |

Proposal 6: Mobile VSAT type 6 for Ka-bands can be handled as part of Rel-20 NTN enhancement WI.

Proposal 7: RAN4 confirms that RRM work for Ku-band under Rel-19 with respect to Mobile VSAT NTN types 4 and 5 is also applicable to Ka-band.

Proposal 8: RAN4 to confirm one of the two possible strategies for introduction of Mobile VSAT for the Ka-band:

1/ TEI for Rel-18 (based on Proposal 4), OR

2/ UE maintenance under Rel-19 based on Proposal 5 (alternative).

# Conclusion

Observation 1: Note 2 is mentioning “NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction.” This note was added to the Ku-band specification without verification (and also exists for the Ka-band from the beginning) and is essential since is up to implementation that VSAT type 5 comply with the OFF-axis EIRP mask.

Observation 2: Ku-band VSAT type 4 has no OFF-axis EIRP mask limitation with respect to LEO connectivity. This indicates that Ka-band VSAT type 4 should follow the same reasoning (since precedent exists).

Observation 3: After a very careful analysis of Rel-19 specification for TS 38.101-5, it has been noticed that no differentiation was carried out between VSATs Type 4/5 connecting to GSO and connecting to LEO for Ku-band, since worst case was considered for terminals capable of connecting to both GSO and LEO: same EIS, same EIRPmin and same EIRPmax. Same reasoning could be also therefore used for Ka-band.

Observation 4: Potentially, if required, a limitation could be introduced for NGSO connectivity with respect to frequency bands being mentioned at the WRC-23: 17.7-18.6 GHz, 18.8-19.3 GHz, 19.7-20.2 GHz (Space-to-Earth, or DownLink) and 27.5-29.1 GHz and 29.5-30 GHz (Earth-to-Space, or Uplink).

Observation 5: The TDoC provides a correction without any requirement change, therefore a CR would be possible. Otherwise, companies could agree to use a TEI.

Proposal 1: Since upon WRC-23 there were no regulatory limitations for Ka-band mobile VSAT with NGSO and there is no new ETSI EN impacted requirements (since GSO requirements can be reused for LEO), it is recommended to deal with this issue during maintenance phase.

Proposal 2: For Ka-band mobile VSAT type 5, use Note 2 from Ku-band “NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction.”

Proposal 3: For Ka-band mobile VSAT type 5, remove Note 3 from Ku-band “GSO support is applicable to bands n512, n511, n510, n509, n508 and n248, n247; LEO support is only applicable to bands n509, n508 and n248, n247.”

Proposal 4: Potentially, RAN4 could update the Rel-18 as follows through maintenance CR:

Table 9.2.1.0-1: The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. |  |  |

Proposal 5: RAN4 shall update the Rel-19 as follows through maintenance CR:

Table 9.2.1.0-1: The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 43 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52, 3 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 643 | Mobile VSAT communicating with LEO only with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. NOTE 3: GSO support is applicable to bands n512, n511, n510, n509, n508 and n248, n247; LEO support is only applicable to bands n509, n508 and n248, n247. NOTE 43: The NTN VSAT type 6 is only applicable to bands n509, n508 and n248, n247. |  |  |

Proposal 5 (alternative): Another way of proceeding without removing Note 3 but simply replace the content with a restriction, RAN4 shall update the Rel-19 as follows through maintenance CR:

Table 9.2.1.0-1: The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 43 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52, 3 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 64 | Mobile VSAT communicating with LEO only with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. NOTE 3: LEO support for bands n512, n511, n510 exclude 18.6-18.8 GHz in DL and 29.1-29.5 GHz in UL. GSO support is applicable to bands n512, n511, n510, n509, n508 and n248, n247; LEO support is only applicable to bands n509, n508 and n248, n247. NOTE 4: The NTN VSAT type 6 is only applicable to bands n509, n508 and n248, n247. |  |  |

Proposal 6: Mobile VSAT type 6 for Ka-bands can be handled as part of Rel-20 NTN enhancement WI.

Proposal 7: RAN4 confirms that RRM work for Ku-band under Rel-19 with respect to Mobile VSAT NTN types 4 and 5 is also applicable to Ka-band.

Proposal 8: RAN4 to confirm one of the two possible strategies for introduction of Mobile VSAT for the Ka-band:

1/ TEI for Rel-18 (based on Proposal 4), OR

2/ UE maintenance under Rel-19 based on Proposal 5 (alternative).

# References

- ETSI EN 303 699 V1.1.1 (2021-02)

- ETSI EN 303 979 V2.1.1 (2016-05)

- 3GPP TS 38.101-5