3GPP TSG-WG4 Meeting #117	R4-2522362

# Dallas, USA, Nov. 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6225 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.133 on positioning reporting delay requirements in RRC_INACTIVE state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_air-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To add positioning reporting delay requirement in RRC_INACTIVE state. - The agreement in ad-hoc session (R4-25011891) are applied in this CR. Issue 3-1: Reporting Delay Requirements for case 1 Agreement: Take the framework of the existing reporting delay requirements - Reporting delay includes measurement delay, inference delay and the time needed until the UE send the report - Check until next meeting on the number of samples needed for the measurements - The agreements in ad-hoc session (R4-2514632) are applied in this draftCR. Issue 3-1: Requirements for case 1 To be clarified in the requirements: - Request location information message should refer to “case 1” positiniong - Reuse the legacy RSTD measurement delay for all the cases (with gap, w/o gap, PRS aggregation) - Beam sweeping factor for FR2 including sweeping reduction factor - Take the legacy values - Rx TEG: check on signaling, take the legacy(used in RSTD measurements) approach if it is signaled - Cover all RRC states CR structure: - Introduce new clause for AI/ML based positioning case 1 - Reference existing requirements wherever they are reused to avoid duplicating the same requirement Introduce requirements for each RRC state in the corresponding clauses: Draft for RRC idle: Nokia Draft for RRC connected: E/// Draft for RRC inactive: Nokia |  |  |  |  |  |  |  |  |
| Summary of change: |  | - New clauses for positioning reporting delay requirements are introduced in RRC_INACTIVE state. - Change 1 addresses the delay requirement in RRC_INACTIVE. - Case 1 is initiated when NR-DL-AIML-RequestLocationInformation message from LMF via LPP. - Positioning reporting delay includes three components: measurement delay, inference delay and the time needed until the UE sends the report. - No gap is configurable in RRC_INACTIVE. Meausrement period in this clause is not impacted by a gap. - Measurement period follows either cluase 5.6.2.5 ‘measurement period requirements’ or clause 5.6.2.6 ‘measurement period requirement with bandwidth aggregation.’ |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Positioning reporting delay is not properly defined in RRC_INACTIVE state. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clauses: 5.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2521902 |  |  |  |  |  |  |  |  |

## <START OF CHANGE 1>

#### 5.X	Position Reporting Requirements for UE-based direct AI/ML Positioning

#### 5.X.1	Introduction

The reporting delay requirements in clause 5.X shall apply provided the UE supports DL AI/ML positioning specified in TS 38.305 [22] as indicated in NR-DL-AIML-ProvideCapabilities, and has received NR-DL-AIML-RequestLocationInformation message from LMF via LPP requesting UE to report its location based on DL AI/ML positioning [34].

The position reporting delay is the sum of measurement delay, inference delay, and the time needed until the UE is ready to send the position report to LMF and is defined as the time between the moment when the position report is triggered and the moment when the UE is ready to start to transmit the position report over the air interface. The UE shall be able to send the positioning response within

Tposition_reporting_delay=Tmeasurement_delay+Tinference_delay,

where Tmeasurement_delay is either the measurement period defined in clause 5.X.2 or the measurement period with bandwidth aggregation as defined in clause 5.X.3, and the inference delay, Tinference_delay, is the summation of inference delay across the number of configured PFLs where inference delay per PFL is reported by UE for FG 59-3 and indicated to LMF. For bandwidth aggregation, the inference delay is the sum of inference delays across all effective PFLs where the UE performs aggregated measurements, with inference delay per effective PFL is equal to each per-PFL inference delay reported by the UE for FG 59-3 and indicated to the LMF.

When performing measurements in RRC_INACTIVE state, if the UE supports reporting of NR positioning location information via SDT, the UE may be able to report the positioning while it remains in RRC_INACTIVE state; otherwise, the UE will transition to RRC_CONNECTED state prior to transmitting the positioning report. Therefore, the positioning reporting delay in RRC_INACTIVE state excludes all of the following:

-	additional delay caused by other LPP signalling on the DCCH,

-	delay uncertainty introduced when inserting the measurement report in the TTI of the uplink DCCH, equal to 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration,

-	any delay caused by unavailability of UL resources to transmit the measurement report,

-	any transmission delay needed by SDT,

-	the time needed to transition to RRC_CONNECTED state to report the measurements.

#### 5.X.2	Measurements Period Requirements

After receiving both NR-DL-AIML-ProvideAssistanceData message and NR-DL-AIML-RequestLocationInformation  message from the LMF via LPP [34], the UE shall be able to measure PRS resources within the measurement period Tmeasurement_delay to produce inputs for its AI/ML model to infer its location for reporting. The measurement delay when UE performs PRS measurement within the measurement gap is defined as:

Tmeasurement_delay,Total=i=1LTmeasurement_delay,i+ L-1*maxTeffect,i

Where:

-	i is the index of positioning frequency layer,

-	L is total number of positioning frequency layers, and

-	Teffect,i is the periodicity of the PRS measurement in positioning frequency layer i

Tmeasurement_delay,i is the measurement period for PRS measurement in positioning frequency layer i as specified below:

Tmeasurement_delay,i= Kcarrier_PRS* NRx,TEG,i* NRxBeam,i*NPRS,islotN'*Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast,i ,

Where:

-	NRxBeam,i is the UE Rx beam sweeping factor:

-	NRxBeam,i = 1 if positioning frequency layer i is in FR1. If positioning frequency layer i is in FR2, NRxBeam,i equals to the value as UE reported in supportedLowerRxBeamSweepingFactor-FR2 if the capability is reported by the UE for the band containing positioning frequency layer i, and LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-AIML-RequestLocationInformation.

-	NRxBeam,i equals to 8, otherwise.

-	Kcarrier_PRS is a scaling factor for PRS-based NR positioning measurements in RRC_INACTIVE. If the UE supports parallelPRS-MeasRRC-Inactive-r17, Kcarrier_PRS = 1; otherwise,

-	If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRS=Kcarrier+1, where Kcarrier is defined in clause 4.2.2.4

-	If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS=Nlayers+1, where Nlayers is defined in clause 4.2.2.7.

-	NRx,TEG,i is the Rx TEG specific scaling factor:

-	NRx,TEG,i = P, where P is the number of UE Rx TEGs configured by LMF for PRS measurement depending on the UE capability regarding the number of UE-RxTEG(s) that is supported by UE and reported for FG 59-6

-	NPRS,islot is the maximum number of DL PRS resources in positioning frequency layer i configured in a slot.

-	Lavailable_PRS,i is the time duration of available PRS in positioning frequency layer i to be measured within TPRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	Nsample is the number of PRS samples, where

-	Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbour cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbour cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

-	Tlast,i is the measurement duration for the last PRS sample in positioning frequency layer i, including the sampling time and processing time, Tlast,i = Ti + Tavailable_PRS,i ,

-	Teffect,i is the periodicity of the PRS measurement in positioning frequency layer i defined as:

Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i ,

where:

-	Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34],

-	Tavailable_PRS,i=LCMTPRS,i,TDRX, the least common multiple between TPRS,i and TDRX

-	TDRX is defined as following:

-	TDRX is DRX cycle length, defined in clause 7.1 TS 38.304, when no extended DRX (eDRX) cycle is configured

-	TDRX is defined as T in clause 7.1 TS 38.304 when RAN eDRX <= 10.24 s and CN eDRX <= 10.24 s

-	TDRX is the maximum of the T inside and outside of the CN PTW, where T inside and outside of the CN PTW are defined in clause 7.1 TS 38.304, when RAN eDRX <= 10.24 s and CN eDRX > 10.24 s

-	 TDRX is the maximum of the DRX cycles within the CN PTW and the RAN PTW when RAN eDRX > 10.24 s

-	TPRS,i is the periodicity of DL PRS resource with muting on positioning frequency layer i.

If more than one PRS periodicities are configured in positioning frequency layer i, the least common multiple of PRS periodicities TperPRS with muting among all DL PRS resource sets in the positioning frequency layer is used to derive TPRS,i, where,

-	TperPRS with muting=Nmuting*TperPRS, is the PRS periodicity with muting per PRS resource,

-	TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

-	Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where

-	TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

-	{N,T} is the UE capability combination per band for RRC_INACTIVE state where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSymbols-r17 in TS 37.355 [34],  T (ms) corresponds to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34], for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34],

-	N’ is UE capability for number of DL PRS resources that it can process in a slot in RRC_INACTIVE state as indicated by maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive-r17 specified in TS 37.355 [34].

If the following conditions are met, the time Tmeasurement_delay,Total starts from the first DL PRS resource(s) instances inside a PTW after both the NR-DL-AIML-ProvideAssistanceData message and NR-DL-AIML-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

-	UE is configured with RAN eDRX > 10.24 s, and

-	periodic PRS measurement reporting is configured, and

-	RAN eDRX cycle is smaller or equal to the PRS measurement reporting periodicity configured via higher layer parameter reportingInterval in TS 37.355 [34], and

-	there is one or more PRS resources occurring in PTW.

Otherwise, the time Tmeasurement_delay,Total starts from the first DL PRS resource(s) after both the NR-DL-AIML-ProvideAssistanceData message and NR-DL-AIML-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

NOTE 1:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

If the DRX cycle is reconfigured during the measurement period, then the measurement period can be longer.

The measurement requirements do not apply to any PRS resource that always collides with other higher-priority DL signals/channels.

Longer measurement period is expected when there are collisions between PRS resources and other higher-priority DL signals/channels.

If Kcarrier_PRS changes for any PFL during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 5.X.2 do not apply if the PRS configuration given by higher layer parameters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-AIML-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If cell re-selection occurs while measurements are being performed, then the UE shall continue and complete the on-going measurements after the cell selection is completed. The measurement period can be longer.

If the RRC state transition occurs from RRC_INACTIVE to RRC_CONNECTED state during the measurement period then the UE shall continue the measurement in the RRC_CONNECTED state. The measurement period can be longer.

#### 5.X.3		Measurements Period Requirements with Bandwidth Aggregation

When UE physical layer receives last of NR-DL-AIML-ProvideAssistanceData message, NR-DL-AIML-RequestLocationInformation message from LMF via LPP [34] with an indication of PRS resources in multiple PFLs that can be aggregated in nr-DL-PRS-AggregationInfo, the UE shall be able to measure PRS resources within the measurement period Tmeasurement_delay to produce inputs for its AI/ML model to infer its location for reporting. The measurement delay when UE performs PRS measurement by aggregating PRS resources in multiple PFLs is defined as:

Tmeasurement_delay= Tmeasurement_delay, aggr+ Tmeasurement_delay,non-aggr + Tmargin

where:

-	Tmeasurement_delay, aggr is the total measurement period for aggregated measurements as defined in this clause, and

-	Tmeasurement_delay,non-aggr is the total measurement period for non-aggregated measurements as defined in this clause, and

-	When both Tmeasurement_delay, aggr  and Tmeasurement_delay,non-aggr are non-zero,  then Tmargin=Tmargin'; otherwise Tmargin=0, where Tmargin'=maxTeffect,j, and the maximum Teffect,j is across all the PFL combination(s) and non-aggregated PFL(s) configured for positioning measurements.

Tmeasurement_delay,non-aggr is zero if every resource set on every PFL is linked for aggregation to at least one other resource set on another PFL. Otherwise, Tmeasurement_delay,non-aggr is as defined in clause 5.X.2 with the following exceptions,

-	only PFLs containing resource set(s) not linked to any other resource set(s) are considered in L,

-	on each PFL i, only resource set(s) not linked to any other resource set(s) are considered in Tavailable_PRS,i, NPRS,islot and Lavailable_PRS,i,

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by supportedDL-PRS-ProcessingSamples-RRC-Inactive specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples,

-	L-1*maxTeffect,i is replace by L-1*Tmargin'.

Tmeasurement_delay,aggr is zero if no resource sets on any PFL are linked for aggregation with other resource sets on other PFLs. Otherwise, Tmeasurement_delay,aggr is defined as

Tmeasurement_delay,aggr= m=1MTmeasurement_delay,aggr,m+ M-1*Tmargin'    ,

where:

-	m is the index of PFL combination,

-	M is total number of PFL combinations,

-	Teffect,aggr,m is the periodicity of the PRS measurement in PFL combination m as defined in this clause,

-	Tmeasurement_delay,aggr,m is the measurement period for PRS measurement in PFL combination m as specified below.

Tmeasurement_delay,aggr,m=Kcarrier,aggr*NRx,TEG,aggr,m*NRxBeam,aggr,m*NPRS,aggr,mslotNaggr,m'*LavailablePRS,aggr,mNaggr,m*Nsample-1*Teffect,aggr,m+Tlast,aggr,m ,

where:

-	Kcarrier,aggr is a scaling factor for PRS measurements in RRC_INACTIVE,

-	Kcarrier,aggr=1 if the UE supports parallelPRS-MeasRRC-Inactive-r17,

-	Otherwise,

-	If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier,aggr=Kcarrier+1, where Kcarrier is defined in clause 4.2.2.4

-	If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier,aggr=Nlayers+1, where Nlayers is defined in clause 4.2.2.7.

-	NRx,TEG,aggr,m is the scaling factor for PRS measurement with multiple Rx TEGs.

-	NRx,TEG,aggr,m = P, where P is the number of UE Rx TEGs configured by LMF for PRS measurement depending on the UE capability regarding the number of UE-RxTEG(s) that is supported by UE and reported for FG 59-6

-		NRxBeam,aggr,m is a scaling factor for PRS measurements with multiple Rx beams, and is defined as

-	NRxBeam,aggr,m = 1 if PFL combination m is in FR1,

-	NRxBeam,aggr,m equals to 8 if PFL combination m is in FR2 unless specified otherwise.

-	NPRS,aggr,mslot is the maximum number of DL PRS resources in PFL combination m configured in a slot, and only the PRS resources in resource set(s) linked to other resource set in PFL combination m are counted

-	Naggr,m' is the UE capability on maximum number of DL PRS resources that can be processed in a slot for PFL combination m as indicated by maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR1-r18 for FR1 and maxNumOfAggregatedDL-PRS-ResourcePerSlot-FR2-r18 for FR2 specified in TS 37.355 [34].

-	LavailablePRS,aggr,m is the time duration of available PRS resources in PFL combination m to be measured during TPRS,aggr,m, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of LavailablePRS,aggr,m, only unmuted PRS resources in resource set(s) linked to other resource set in PFL combination m and that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	Naggr,m is the UE capability on duration of DL PRS resources in ms for PFL combination m as indicated by prs-durationOfTwoPRS-BWA-ProcessingSymbolsN-r18 or prs-durationOfThreePRS-BWA-ProcessingSymbolsN-r18 specified in TS 37.355 [34].

-	Nsample is number of PRS measurement samples,

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-IdleAndInactive specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples,

-	Nsample= 4 otherwise.

-	Teffect,aggr,m is the periodicity of the PRS measurement in PFL combination m,

Teffect,aggr,m=Taggr,mTavailable,PRS,aggr,m*Tavailable,PRS,aggr,m

-	Taggr,m is the UE capability on time for processing of DL PRS resources in ms for PFL combination m as indicated by prs-durationOfTwoPRS-BWA-ProcessingSymbolsT-r18 or prs-durationOfThreePRS-BWA-ProcessingSymbolsT specified in TS 37.355 [34].

-	Tavailable,PRS,aggr,m=LCMTPRS,aggr,m,TDRX, the least common multiple between TPRS,aggr,m and the DRX cycle length TDRX, where TPRS,aggr,m is the periodicity of DL PRS resource with muting on PFL combination m.

-	If more than one PRS periodicities are configured in PFL combination m, the least common multiple of PRS periodicities TperPRS with muting among all DL PRS resource sets that are linked to other resource set in PFL combination m, is used to derive TPRS,aggr,m, and for each applicable PRS resource set,

-	TperPRS with muting=Nmuting*TperPRS, is the PRS periodicity with muting per PRS resource, and

-	TperPRS is the periodicity of PRS resource set given by the higher-layer parameter DL-PRS-Periodicity, and

-	Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where 	TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

-	Tlast,aggr,m is the measurement duration for the last PRS sample in PFL combination m, including the sampling time and processing time, Tlast,aggr,m=Taggr,m+Tavailable,PRS,aggr,m.

The requirements in this clause for aggregated measurements apply provided that the linked PRS resource sets on multiple PFLs for aggregated measurements are transmitted by the TRP using single Tx chain as defined in clause 5.1.6.5.3 in TS 38.214 [26].

If PRS resources in one or more of the aggregated PFLs in PFL combination m are dropped because of collision with other signals, Tmeasurement_delay,aggr,m can be longer than specified above.

The time Tmeasurement_delay,Total starts from the first DRX cycle containing the DL PRS resource(s) in the assistance data after both the NR-DL-AIML-ProvideAssistanceData message and NR-DL-AIML-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

NOTE:	No separate requirement on non-aggregated measurement based on Tmeasurement_delay,non-aggr or on aggregated measurement based on Tmeasurement_delay,aggr is applied when both Tmeasurement_delay, aggr and Tmeasurement_delay,non-aggr are non-zero.

If the DRX cycle is reconfigured during the measurement period, then the measurement period can be longer.

The measurement requirements do not apply to any PRS resource that always collides with other higher-priority DL signals/channels.

Longer measurement period is expected when there are collisions between PRS resources and other higher-priority DL signals/channels.

If Kcarrier_PRS changes for any PFL or any PFL combination during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 5.X.3 do not apply if the PRS configuration given by higher layer parameters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-AIML-ProvideCapabilities.

If cell re-selection occurs while measurements are being performed, then the UE shall continue and complete the on-going measurements after the cell selection is completed. The measurement period can be longer.

If the RRC state transition occurs from RRC_INACTIVE to RRC_CONNECTED state during the measurement period, then the UE shall continue the measurement in the RRC_CONNECTED state. The measurement period can be longer.

## <END OF CHANGE 1>