RAN4 AI-ML positioning capabilities:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_AIML_Air | 59-3 | Inference time per positioning frequency layer for AI-ML positioning case 1 | The maximum required inference time per configured positioning frequency layer for AI-ML case 1 | 58-2-1 | n/a | n/a | Network will not know how much time UE takes to infer its co-ordinates | Per hand | n/a | n/a | n/a | Candidate values: {1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 14, 16, 18, 20} ms Need for location server to know if the feature is supported | Optional with capability signalling |
| 59. NR_AIML_Air | 59-4 | PRS measurement for reduced sample for AI-ML positioning case 1 | The capability to support reporting UE co-ordinate based on measuring on 1 or 2 samples of a DL PRS resource set in AI-ML positioning case 1 | 58-2-1 | n/a | n/a | UE will not support reduced number of measurement samples for AI-ML positioning case 1 | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| 59. NR_AIML_Air | 59-5 | Support of lower RX beam sweeping factor for AI-ML positioning case 1 | 1. Support of the lower RX beam sweeping factor than 8 for FR2 in AI-ML positioning case 1 2. Number of RX beam sweeping factor in AI-ML positioning case 1 | 58-2-1 | n/a | n/a | UE will not support lower RX beam sweeping factor than 8 for FR2 in AI-ML positioning case 1 | Per band | n/a | n/a | n/a | Component 2: candidate values {1, 2, 4, 6} Need for location server to know if the feature is supported | Optional with capability signaling |
| 59. NR_AIML_Air | 59-6 | UE-RxTEGs for AI-ML positioning case 1 | 1. Support of UE-RxTEGs for UE-based AI-ML positioning case 1 2. The maximum number of UE-RxTEG(s), which is supported and reported by UE for UE-based AI-ML positioning case 1 | 58-2-1 | n/a | n/a | Network will not know how many RxTEGs UE supports to measure for AI-ML positioning case 1 | Per band | n/a | n/a | n/a | Component 2 candidate values: {1, 2, 3, 4} Need for location server to know if the feature is supported | Optional with capability signaling |