3GPP TSG-RAN WG4 Meeting #117	R4-2521477

# Dallas, USA, 17th - 21th, Nov, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.181 | CR |  | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on SAN PUSCH requirement with inter-slot OCC in TS 38.181 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Perf |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-19 NR NTN enhancement WI, RAN4 has introduced NR PUSCH requirement with inter-slot OCC feature for DFT-s-OFDM waveform. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add new manufacturer declarations for inter-slot OCC featrure in section - Add new sections for NR PUSCH requirement with inter-slot OCC feature for DFT-s-OFDM waveform for both conducted performance and radiated performance - Add the measurement of performance requirements in section 4.6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NR PUSCH requirement with inter-slot OCC feature for DFT-s-OFDM waveform can not be verfied correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6, 8.2.6, 11.2.6, C3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2521477 |  |  |  |  |  |  |  |  |

==============First change==============

4.6	Manufacturer declarations

The following SAN declarations listed in table 4.6-1, when applicable to the SAN under test, are required to be provided by the manufacturer for the conducted requirements testing of the SAN type 1-H, and radiated requirements testing of SAN type 1-H, SAN type 1-O and SAN type 2-O.

Table 4.6-1 Manufacturers declarations for SAN type 1-H conducted test requirements, and for SAN type 1-H, SAN type 1-O and SAN type 2-O radiated test requirements

| Declaration identifier | Declaration | Description | Applicability (Note 1) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | SAN type 1-H (Note 2) | SAN type 1-O | SAN type 2-O |
| D.1 | Coordinate system reference point | Location of coordinated system reference point in reference to an identifiable physical feature of the SAN enclosure. | x | x | x |
| D.2 | Coordinate system orientation | Orientation of the coordinate system in reference to an identifiable physical feature of the SAN enclosure. | x | x | x |
| D.3 | Beam identifier | A unique title to identify a beam, e.g. a, b, c or 1, 2, 3. The vendor may declare any number of beams with unique identifiers. The minimum set to declare for conformance, corresponds to the beams at the reference beam direction with the highest intended EIRP, and covering the properties listed below: 1) A beam with the narrowest intended BeWθ and narrowest intended BeWϕ possible when narrowest intended BeWθ is used. 2) A beam with the narrowest intended BeWϕ and narrowest intended BeWθ possible when narrowest intended BeWϕ is used. 3) A beam with the widest intended BeWθ and widest intended BeWϕ possible when widest intended BeWθ is used. 4) A beam with the widest intended BeWϕ and widest intended BeWθ possible when widest intended BeWϕ is used. 5) A beam which provides the highest intended EIRP of all possible beams. When selecting the above five beam widths for declaration, all beams that the SAN is intended to produce shall be considered, including beams that during operation may be identified by any kind of cell or UE specific reference signals, with the exception of any type of beam that is created from a group of transmitters that are not all phase synchronised. (Note 3) | x | x | x |
| D.4 | Operating bands and frequency ranges | List of NR operating band(s) supported by the SAN and if applicable, frequency range(s) within the operating band(s) that the SAN can operate in. Supported bands declared for every beam for SAN type 1-O (D.3), or every TAB connector for SAN type 1-H. (Note 4) | c | x | x |
| D.5 | SAN requirements set | Declaration of one of the NR satellite access node requirement's set as defined for SAN type 1-H, or SAN type 1-O. | c | x | x |
| D.6 | SAN class | Declared as GEO SAN, or LEO SAN. | c | x | x |
| D.7 | SAN channel band width and SCS support | SAN supported SCS and channel bandwidth per supported SCS. Declared for each beam for SAN type 1-O (D.3) or each TAB connector for SAN type 1-H, and each operating band (D.4). | c | x | x |
| D.8 | OTA peak directions set reference beam direction pair | The beam direction pair, describing the reference beam peak direction and the reference beam centre direction. Declared for every beam (D.3). | x | x | x |
| D.9 | OTA peak directions set | The OTA peak directions set for each beam. Declared for every beam (D.3). | x | x | x |
| D.10 | OTA peak directions set maximum steering direction(s) | The beam direction pair(s) corresponding to the following points: 1) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive Φ direction, while the θ value being the closest possible to the reference beam centre direction. 2) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative Φ direction, while the θ value being the closest possible to the reference beam centre direction. 3) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the positive θ direction, while the Φ value being the closest possible to the reference beam centre direction. 4) The beam peak direction corresponding to the maximum steering from the reference beam centre direction in the negative θ direction, while the Φ value being the closest possible to the reference beam centre direction. The maximum steering direction(s) may coincide with the reference beam centre direction. Declared for every beam (D.3). | x | x | x |
| D.11 | Rated beam EIRP (Prated,c,EIRP) | The rated EIRP level per carrier (Prated,c,EIRP) at the beam peak direction associated with a particular beam direction pair for each of the declared maximum steering directions (D.10), as well as the reference beam direction pair (D.8). Declared for every beam (D.3). (Note 11) | x | x | x |
| D.12 | Beamwidth | The beamwidth for the reference beam direction pair and the four maximum steering directions. Declared for every beam (D.3). | x | x | x |
| D.13 | Equivalent beams | List of beams which are declared to be equivalent. Equivalent beams imply that the beams are expected to have identical OTA peak directions sets and intended to have identical spatial properties at all steering directions within the OTA peak directions set when presented with identical signals. All declarations (D.4 – D.12) made for the beams are identical and the transmitter unit, RDN and antenna array responsible for generating the beam are of identical design. | x | x | x |
| D.14 | Parallel beams | List of beams which have been declared equivalent (D.13) and can be generated in parallel using independent RF power resources. Independent power resources mean that the beams are transmitted from mutually exclusive transmitter units. | x | x | x |
| D.15 | Number of carriers at maximum TRP | The number of carriers per operating band the SAN is capable of generating at maximum TRP declared for every beam (D.3). | n/a | x | x |
| D.16 | Maximum Satellite Access Node RF Bandwidth | Maximum Satellite Access Node RF Bandwidth in the operating band, declared for each supported operating band for each beam for SAN type 1-O, or for each TAB connector for SAN type 1-H (D.4). (Note 10) | c | x | x |
| D.17 | Total RF bandwidth (BWtot) | Total RF bandwidth BWtot of transmitter and receiver, declared per the band combinations (D.42). | c | x | x |
| D.18 | Contiguous spectrum operation support | Ability of SAN to support contiguous frequency distribution of carriers when operating multi-carrier in an operating band. Declared for each single-band RIB for SAN type 1-O or each single-band connector for SAN type 1-H, for each operating band. | c | x | x |
| D.19 | OSDD identifier | A unique identifier for the OSDD. | x | x | n/a |
| D.20 | OSDD operating band support | Operating band supported by the OSDD, declared for every OSDD (D.19). (Note 5) | x | x | n/a |
| D.21 | OTA sensitivity supported SAN channel bandwidth and SCS | The SAN supported SCS and channel bandwidth per supported SCS by each OSDD. | x | x | n/a |
| D.22 | Redirection of receiver target support | Ability to redirect the receiver target related to the OSDD. | x | x | n/a |
| D.23 | Minimum EIS for FR1 (EISminSENS) | The minimum EISminSENS requirement (i.e. maximum allowable EIS value) applicable to all sensitivity RoAoA per OSDD. Declared per NR supported channel BW for the OSDD (D.19). The lowest EIS value for all the declared OSDD's is called minSENS, while its related range of angles of arrival is called minSENS RoAoA. (Note 6) | x | x | n/a |
| D.24 | Receiver target reference direction Sensitivity Range of Angle of Arrival | The sensitivity RoAoA associated with the receiver target reference direction (D.26) for each OSDD. | x | x | n/a |
| D.25 | Receiver target redirection range | For each OSDD the associated union of all the sensitivity RoAoA achievable through redirecting the receiver target related to the OSDD. | x | x | n/a |
| D.26 | Receiver target reference direction | For each OSDD an associated direction inside the receiver target redirection range (D.25). (Note 7) | x | x | n/a |
| D.27 | Conformance test directions sensitivity RoAoA | For each OSDD that includes a receiver target redirection range, four sensitivity RoAoA comprising the conformance test directions (D.28). | x | x | n/a |
| D.28 | Conformance test directions | For each OSDD four conformance test directions. If the OSDD includes a receiver target redirection range the following four directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the receiver target redirection range, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the receiver target redirection range, while φ value being the closest possible to the receiver target reference direction. If an OSDD does not include a receiver target redirection range the following 4 directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the sensitivity RoAoA, while θ value being the closest possible to the receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the sensitivity RoAoA, while φ value being the closest possible to the receiver target reference direction. | x | x | n/a |
| D.29 | OTA coverage range | Declared as a single range of directions within which selected TX OTA requirements are intended to be met. (Note 8) | x | x | x |
| D.30 | OTA coverage range reference direction | The direction describing the reference direction of the OTA converge range (D.29). (Note 9) | x | x | x |
| D.31 | OTA coverage range maximum directions | The directions corresponding to the following points: 1) The direction determined by the maximum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA coverage range, while θ value being the closest possible to the OTA coverage range reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA coverage range, while φ value being the closest possible to the OTA coverage range reference direction. | x | x | x |
| D.32 | The rated carrier OTA SAN power, Prated,c,TRP | Prated,c,TRP is declared as TRP OTA power per carrier, declared per supported operating band. (Note 11) | n/a | x | x |
| D.33 | Rated transmitter TRP, Prated,t,TRP | Rated total radiated output power. Declared per supported operating band. (Note 11) | n/a | x | x |
| D.34 | Rated carrier output power (Prated,c,TABC) | Conducted rated carrier output power, per single band connector. Declared per supported operating band, per TAB connector for SAN type 1-H. (Note 11) | c | n/a | n/a |
| D.35 | Rated total output power (Prated,t,TABC) | Conducted total rated output power. Declared per supported operating band, per TAB connector for SAN type 1-H. (Note 11) | c | n/a | n/a |
| D.36 | Single band connector | List of single-band connector for the supported operating bands (D.4). | c | n/a | n/a |
| D.37 | Equivalent connectors | List of TAB connector of SAN type 1-H, which have been declared equivalent. Equivalent connectors imply that the TAB connector of SAN type 1-H, are expected to behave in the same way when presented with identical signals under the same operating conditions. All declarations made for the TAB connector of SAN type 1-H are identical and the transmitter unit and/or receiver unit driving TAB connector of SAN type 1-H are of identical design. | c | n/a | n/a |
| D.38 | Single-band RIB | List of single-band RIB for the supported operating bands (D.4). | n/a | x | x |
| D.39 | Single or multiple carrier | SAN capability to operate with a single carrier (only) or multiple carriers. Declared per supported operating band, per RIB for SAN type 1-O or per TAB connector for SAN type 1-H. | c | x | x |
| D.40 | Maximum number of supported carriers per operating band | Maximum number of supported carriers. Declared per supported operating band, per RIB for SAN type 1-O or per TAB connector for SAN type 1-H. (Note 10) | c | x | x |
| D.41 | Maximum supported power difference between carriers | Maximum supported power difference between carriers in each supported operating band. Declared per operating band (D.4), per RIB for SAN type 1-O or per TAB connector for SAN type 1-H. | c | x | x |
| D.42 | Operating band combination support | List of operating bands combinations supported by single-band RIB(s) of SAN type 1-O, or single-band connector(s) of SAN type 1-H. | c | x | x |
| D.43 | OTA REFSENS RoAoA | Range of angles of arrival associated with the OTA REFSENS. | n/a | x | x |
| D.44 | OTA REFSENS receiver target reference direction | Reference direction inside the OTA REFSENS RoAoA (D.43). | n/a | x | x |
| D.45 | OTA REFSENS conformance test directions | The following four OTA REFSENS conformance test directions shall be declared: 1) The direction determined by the maximum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the OTA REFSENS receiver target reference direction. 2) The direction determined by the minimum φ value achievable inside the OTA REFSENS RoAoA, while θ value being the closest possible to the OTA REFSENS receiver target reference direction. 3) The direction determined by the maximum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the OTA REFSENS receiver target reference direction. 4) The direction determined by the minimum θ value achievable inside the OTA REFSENS RoAoA, while φ value being the closest possible to the OTA REFSENS receiver target reference direction. | n/a | x | x |
| D.46 | Relation between supported maximum RF bandwidth, number of carriers and Rated maximum TRP | If the rated transmitter TRP and total number of supported carriers are not simultaneously supported, the manufacturer shall declare the following additional parameters: - The reduced number of supported carriers at the rated transmitter TRP; - The reduced total output power at the maximum number of supported carriers. | n/a | x | x |
| D.47 | Relation between supported maximum RF bandwidth, number of carriers and Rated total output power | If the rated total output power and total number of supported carriers are not simultaneously supported, the manufacturer shall declare the following additional parameters: - The reduced number of supported carriers at the rated total output power; - The reduced total output power at the maximum number of supported carriers. | c | n/a | n/a |
| D.48 | TAB connectors used for performance requirement testing | To reduce test complexity, declaration of a representative (sub)set of TAB connectors to be used for performance requirement test purposes. At least one TAB connector mapped to each demodulation branch is declared. | c | n/a | n/a |
| D.49 | Prated,c,sys,GEO | The sum of Prated,c,TABC for all TAB connectors for a single carrier of the SAN GEO class. | c | n/a | n/a |
| D.50 | Prated,c,TABC,GEO | The rated carrier output power per TAB connector of the SAN GEO class. | c | n/a | n/a |
| D.51 | Prated,c,sys,LEO | The sum of Prated,c,TABC for all TAB connectors for a single carrier of the SAN LEO class. | c | n/a | n/a |
| D.52 | Prated,c,TABC,LEO | The rated carrier output power per TAB connector of the SAN LEO class. | c | n/a | n/a |
| D.53 | EIS REFSENS for FR2 (EISREFSENS_50M) | The EISREFSENS_50M level applicable in the OTA REFSENS RoAoA, (used as a basis for the derivation of the FR2 EISREFSENS for other channel bandwidths supported by SAN). (Note 12) | n/a | n/a | x |
| D.54 | SAN test conditions: Barometric pressure | Range of barometric pressure values for the Satellite Payload RF (SPRF) testing. | x | x |  |
| D.55 | SAN test conditions: Temperature | Range of temperature values for SPRF testing. | x | x |  |
| D.56 | SAN test conditions: Relative humidity | Range of relative humidity values for SPRF testing. | x | x |  |
| D.57 | SAN test conditions: Vibration | Range of vibration values for SPRF testing. | x | x |  |
| D.58 | SAN test conditions: Additional conditions | Additional (e.g. mission-specific) conditions for the SPRF testing. | x | x |  |
| D.59 | SAN test conditions: power supply | Manufacturer declaration related to the power supply. | x | x |  |
| D.60 | NB-IoT in-band operation | Manufacturer shall declare the support of NB-IoT operation in NTN NR in-band and the number of supported NB-IoT carriers in total and for each supported band, frequency range and channel bandwidth. | x | x |  |
| D.61 | NB-IoT sub-carrier spacing | If the SAN supports NB-IoT operation in NTN NR in-band, manufacturer shall declare if it supports 15 kHz sub-carrier spacing, 3.75 kHz sub-carrier spacing, or both for NPUSCH. | x | x |  |
| D.62 | NB-IoT power dynamic range | If manufacturer declares support for NB-IoT operation in NTN NR in-band, manufacturer shall declare the maximum power dynamic range it could support with a minimum of +6dB or +3dB as specified in clause 6.3.4 of TS 38.108 [2] (Note 13). | x | x |  |
| D.100 | PUSCH mapping type | Declaration of the supported PUSCH mapping type as specified in TS 38.211 [8], i.e., type A, type B or both. | c | x | n/a |
| D.101 | PUCCH format | Declaration of the supported PUCCH format(s) as specified in TS 38.211 [8], i.e., format 0, format 1, format 2, format 3, format 4. | c | x | x |
| D.102 | PRACH format and SCS | Declaration of the supported PRACH format(s) as specified in TS 38.211 [8], i.e., format: 0, 2, B4, C2. Declaration of the supported SCS(s) per supported PRACH format with short sequence, as specified in TS 38.211 [8], i.e., 15 kHz, 30 kHz or both. | c | x | x |
| D.103 | Additional DM-RS for PUCCH format 3 | Declaration of the supported additional DM-RS for PUCCH format 3: without additional DM-RS, with additional DM-RS or both. | c | x | x |
| D.104 | Additional DM-RS for PUCCH format 4 | Declaration of the supported additional DM-RS for PUCCH format 4: without additional DM-RS, with additional DM-RS or both. | c | x | x |
| D.105 | PUCCH multi-slot | Declaration of multi-slot PUCCH support. | c | x | n/a |
| D.106 | MCS index table 3 | Declaration of support MCS index table 3 as specified in TS 38.214 [9]. | c | x |  |
| D.107 | PUSCH repetition type A | Declaration of support PUSCH repetition type A | c | x |  |
| D.108 | PUSCH DM-RS bundling | Declaration of supported SCS and FDD PUSCH DM-RS bundling, i.e., {15 kHz, 30 kHz} | c | x | n/a |
| D.109 | PUSCH with inter-slot OCC | Declaration of supported PUSCH with inter-slot OCC for DFT-s-OFDM waveform | c | x | n/a |
| NOTE 1: Manufacturer declarations applicable per SAN requirement set were marked as "x" or "c". Manufacturer declarations not applicable per SAN requirement set were marked as "n/a". NOTE 2: For SAN type 1-H, the only radiated declarations are related to EIRP and EIS requirements. For declarations marked as 'c', related conducted declarations apply, and for declarations marked as 'x', related radiated declarations apply. NOTE 3: Depending on the capability of the system some of these beams may be the same. For those same beams, testing is not repeated. NOTE 4: These operating bands are related to their respective single-band RIBs, or single-band TAB connectors. NOTE 5: As each identified OSDD has a declared minimum EIS value (D.23), multiple operating band can only be declared if they have the same minimum EIS declaration. NOTE 6: If the SAN type 1-H or SAN type 1-O is not capable of redirecting the receiver target related to the OSDD then there is only one RoAoA applicable to the OSDD. NOTE 7: For an OSDD without receiver target redirection range, this is a direction inside the sensitivity RoAoA. NOTE 8: OTA coverage range is used for conformance testing of such TX OTA requirements as occupied bandwidth, frequency error or EVM. NOTE 9: The OTA coverage reference direction may be the same as the Reference beam direction pair (D.8) but does not have to be. NOTE 10: Parameters for contiguous spectrum operation in the operating band are assumed to be the same unless they are separately declared. When separately declared, they shall still use the same declaration identifier. NOTE 11: If a SAN is capable of 64QAM DL operation then up to two rated output power declarations may be made. One declaration is applicable when configured for 64QAM transmissions, and the other declaration is applicable when not configured for 64QAM transmissions. NOTE 12: Although EISREFSENS_50M level is based on a reference measurement channel with BWChannel = 50 MHz, it does not imply that SAN has to support 50 MHz channel bandwidth. NOTE 13: This manufacturer may declare two values, one with a minimum of +6dB and the other with a minimum of +3dB. |  |  |  |  |  |

==============Second change==============

8.2.5	Performance requirements for PUSCH with DMRS bundling

<Unchanged part skipped>

8.2.6	Performance requirements for PUSCH with inter-slot OCC

8.2.6.1	Definition and applicability

The performance requirement of PUSCH with inter-slot OCC is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions.

Which specific test(s) are applicable to SAN is based on the test applicability rules defined in clause [8.1.2.x].

8.2.6.2	Minimum Requirement

The minimum requirement is in TS 38.108 [2] clause [8.2.x].

8.2.6.3	Test Purpose

The test shall verify the receiver's ability to achieve throughput under multipath fading propagation conditions for a given SNR with inter-slot OCC.

8.2.6.4	Method of test

8.2.6.4.1	Initial Conditions

Test environment: Normal, see Annex B.2.

RF channels to be tested for single carrier: M; see clause 4.9.1.

8.2.6.4.2	Procedure

1)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to all SAN TAB connectors for diversity reception via a combining network as shown in annex D.6 for SAN type 1-H.

2)	Adjust the AWGN generator, according to the SCS and channel bandwidth, defined in table 8.2.2.4.2-1.

Table 8.2.6.4.2-1: AWGN power level at the SAN input

| Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15 | 5 | -86.5 dBm / 4.5MHz |
| 30 | 10 | -83.6 dBm / 8.64MHz |
| NOTE: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |

3)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameters in table 8.2.6.4.2-2.

Table 8.2.6.4.2-2: Test parameters for testing PUSCH with inter-slot OCC

| Parameter |  | Value |  |
| --- | --- | --- | --- |
| Transform precoding |  | Enabled |  |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence (Note 1) | 0,2,3,1 |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS position | pos1 |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |
|  | DM-RS port(s) | UE0 | UE1 |
|  |  | {0} | {1} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |  |
| Time domain resource assignment | PUSCH mapping type | A, B |  |
|  | Start symbol | 0 |  |
|  | Allocation length | 14 |  |
|  | PUSCH aggregation factor | n2 |  |
| Frequency domain resource assignment | RB assignment | 15 kHz SCS: 25 PRBs in the middle of the test bandwidth 30kHz SCS: 24PRBs in the middle of the test bandwidth |  |
|  | Frequency hopping | Disabled |  |
| Frequency offset (Hz) | Offset for multiplexed UEs in OCC group | 200 |  |
| Orthogonal cover codes | OCC length | 2 |  |
|  | OCC index and corresponding code | 0 | 1 |
|  |  | {1,1} | {1, -1} |
| Code block group based PUSCH transmission |  | Disabled |  |
| NOTE 1: The same RV sequence is applied within inter-slot OCC |  |  |  |

4)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex G.

5)	Adjust the equipment so that required SNR specified in table 8.2.2.6-1 to 8.2.2.6-4 is achieved at the SAN input.

6)	For each of the reference channels in table 8.2.2.6-1 to 8.2.2.6-4 applicable for the base station, measure the throughput per each UE.

8.2.6.5	Test Requirement

The throughput measured per UE according to clause 8.2.6.4.2 shall not be below the limits for the SNR levels specified in table 8.2.6.5-1 to 8.2.6.5-4.

Table 8.2.6.5-1: Test requirements for PUSCH with 70% of maximum throughput per UE, Type A, 5 MHz channel bandwidth, FR1-NTN, 15 kHz SCS

| Number of TX antennas | Number of Rx antennas | Cyclic prefix | Propagation conditions (annex XX) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | OCC index | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3A-3 | pos1 | 0 | TBD |
|  |  | Normal |  | 70 % | G-FR1-NTN-A3A-3 | pos1 | 1 | TBD |
| 1 | 2 | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3A-3 | pos1 | 0 | TBD |
|  |  | Normal |  | 70 % | G-FR1-NTN-A3A-3 | pos1 | 1 | TBD |

Table 8.2.6.5-2: Test requirements for PUSCH with 70% of maximum throughput per UE, Type A, 10 MHz channel bandwidth, FR1-NTN, 30 kHz SCS

| Number of TX antennas | Number of Rx antennas | Cyclic prefix | Propagation conditions (annex XX) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | OCC index | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3A-4 | pos1 | 0 | TBD |
|  |  | Normal |  | 70 % | G-FR1-NTN-A3A-4 | pos1 | 1 | TBD |
| 1 | 2 | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3A-4 | pos1 | 0 | TBD |
|  |  | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3A-4 | pos1 | 1 | TBD |

Table 8.2.6.5-3: Test requirements for PUSCH with 70% of maximum throughput per UE, Type B, 5 MHz channel bandwidth, FR1-NTN, 15 kHz SCS

| Number of TX antennas | Number of Rx antennas | Cyclic prefix | Propagation conditions (annex XX) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | OCC index | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3A-3 | pos1 | 0 | TBD |
|  |  | Normal |  | 70 % | G-FR1-NTN-A3A-3 | pos1 | 1 | TBD |
| 1 | 2 | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3A-3 | pos1 | 0 | TBD |
|  |  | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3A-3 | pos1 | 1 | TBD |

Table 8.2.6.5-4: Test requirements for PUSCH with 70% of maximum throughput per UE, Type B, 10 MHz channel bandwidth, FR1-NTN, 30 kHz SCS

| Number of TX antennas | Number of Rx antennas | Cyclic prefix | Propagation conditions (annex XX) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | OCC index | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3A-4 | pos1 | 0 | TBD |
|  |  | Normal |  | 70 % | G-FR1-NTN-A3A-4 | pos1 | 1 | TBD |
| 1 | 2 | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3A-4 | pos1 | 0 | TBD |
|  |  | Normal |  | 70 % | G-FR1-NTN-A3A-4 | pos1 | 1 | TBD |

==============Third change==============

11.2.5	Performance requirements for PUSCH with inter-slot OCC

<Unchanged part skipped>

11.2.6	Performance requirements for PUSCH with inter-slot OCC

11.2.6.1	Definition and applicability

The performance requirement of PUSCH with inter-slot OCC is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions.

Which specific test(s) are applicable to SAN is based on the test applicability rules defined in clause 11.1.X.

11.2.6.2	Minimum Requirement

For SAN type 1-O, the minimum requirement is in TS 38.108 [2], clause 11.2.2.X

11.2.6.3	Test purpose

The test shall verify the receiver's ability to achieve throughput under multipath fading propagation conditions for a given SNR with inter-slot OCC.

11.2.6.4	Method of test

11.2.6.4.1	Initial conditions

Test environment: Normal, see annex B.2.

RF channels to be tested for single carrier: M, see clause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.44 in table 4.6-1).

11.2.6.4.2	Procedure

1)	Place the SAN with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex D.7.

2)	Align the manufacturer declared coordinate system orientation of the SAN with the test system.

3)	Set the SAN in the declared direction to be tested.

4)	Connect the SAN tester generating the wanted signal, multipath fading simulators and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.3. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A, and according to additional test parameters listed in table 11.2.6.4.2-1.

Table 11.2.6.4.2-1: Test parameters for testing PUSCH with inter-slot OCC

| Parameter |  | Value |  |
| --- | --- | --- | --- |
|  |  | SNR type 1-O |  |
| Transform precoding |  | Enabled |  |
| HARQ | Maximum number of HARQ transmissions | 4 |  |
|  | RV sequence (Note 1) | 0,2, 3,1 |  |
| DM-RS | DM-RS configuration type | 1 |  |
|  | DM-RS duration | single-symbol DM-RS |  |
|  | Additional DM-RS position | pos1 |  |
|  | Number of DM-RS CDM group(s) without data | 2 |  |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |  |
|  | DM-RS port(s) | UE0 | UE1 |
|  |  | {0} | {1} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |  |
| Time domain resource assignment | PUSCH mapping type | A, B |  |
|  | Start symbol | 0 |  |
|  | Allocation length | 14 |  |
|  | PUSCH aggregation factor | n2 |  |
| Frequency domain resource assignment | RB assignment | 15 kHz SCS: 25 PRBs in the middle of the test bandwidth 30kHz SCS: 24PRBs in the middle of the test bandwidth |  |
|  | Frequency hopping | Disabled |  |
| Orthogonal cover codes | OCC length | 2 |  |
|  | OCC index and corresponding code | 0 | 1 |
|  |  | {1,1} | {1, -1} |
| Code block group based PUSCH transmission |  | Disabled |  |
| NOTE 1: The same RV sequence is applied within inter-slot OCC. |  |  |  |

6)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex J.

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the SAN receiver is as specified in clause 11.2.6.5.1 for SAN type 1-O, and that the SNR at the SAN receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 11.2.6.4.2-2.

Table 11.2.6.4.2-2: AWGN power level at the SAN input

| SAN Type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| 1-O (NOTE 2) | 15 | 5 | -86.5 - ΔOTAREFSENS dBm / 4.5 MHz |
|  | 30 | 10 | -83.6 - ΔOTAREFSENS dBm / 8.64 MHz |
| NOTE 1: ΔOTAREFSENS as declared in D.43 in table 4.6-1 and clause 7.1. NOTE 2: The AWGN power level contains an AWGN offset of 15dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 15dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |  |

8)	For reference channels applicable to the SAN, measure the throughput.

11.2.6.5	Test Requirement

11.2.6.5.1	Test requirement for SAN type 1-O

The throughput measured according to clause 11.2.6.4.2 shall not be below the limits for the SNR levels specified in table 11.2.6.5.1-1 ~ 11.2.6.5.1.4.

Table 11.2.6.5.1-1: Test requirements for PUSCH with 70% of maximum throughput per UE, Type A, 5 MHz channel bandwidth, FR1-NTN, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions (annex XX) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | OCC index | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3A-3 | pos1 | 0 | TBD |
|  |  | Normal |  | 70 % | G-FR1-NTN-A3A-3 | pos1 | 1 | TBD |
| 1 | 2 | Normal |  | 70 % | G-FR1-NTN-A3A-3 | pos1 | 0 | TBD |
|  |  | Normal |  | 70 % | G-FR1-NTN-A3A-3 | pos1 | 1 | TBD |

Table 11.2.5.5.1-2: Test requirements for PUSCH with 70% of maximum throughput per UE, Type A, 10 MHz channel bandwidth, FR1-NTN, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions (annex XX) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | OCC index | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3A-4 | pos1 | 0 | TBD |
|  |  | Normal |  | 70 % | G-FR1-NTN-A3A-4 | pos1 | 1 | TBD |
| 1 | 2 | Normal |  | 70 % | G-FR1-NTN-A3A-4 | pos1 | 0 | TBD |
|  |  | Normal |  | 70 % | G-FR1-NTN-A3A-4 | pos1 | 1 | TBD |

Table 11.2.5.5.1-3: Test requirements for PUSCH with 70% of maximum throughput per UE, Type B, 5 MHz channel bandwidth, FR1-NTN, 15 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions (annex XX) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | OCC index | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3A-3 | pos1 | 0 | TBD |
|  |  | Normal |  | 70 % | G-FR1-NTN-A3A-3 | pos1 | 1 | TBD |
| 1 | 2 | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3A-3 | pos1 | 0 | TBD |
|  |  | Normal |  | 70 % | G-FR1-NTN-A3A-3 | pos1 | 1 | TBD |

Table 11.2.5.5.1-4: Test requirements for PUSCH with 70% of maximum throughput per UE, Type B, 10 MHz channel bandwidth, FR1-NTN, 30 kHz SCS

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions (annex XX) | Fraction of maximum throughput | FRC (annex G) | Additional DM-RS position | OCC index | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3A-4 | pos1 | 0 | TBD |
|  |  | Normal |  | 70 % | G-FR1-NTN-A3A-4 | pos1 | 1 | TBD |
| 1 | 2 | Normal | NTN-TDLC5-200 Low | 70 % | G-FR1-NTN-A3A-4 | pos1 | 0 | TBD |
|  |  | Normal |  | 70 % | G-FR1-NTN-A3A-4 | pos1 | 1 | TBD |

==============Fourth change==============

# A.3A	Fixed Reference Channels for performance requirements (QPSK, R=99/1024)

The parameters for the reference measurement channel are specified in table A.3A-1 for FR1-NTN PUSCH performance requirements:

-	FRC parameters are specified in table A.3A-1 for FR1-NTN PUSCH with transform precoding disabled, additional DM-RS position = pos1 and 1 transmission layer.

The parameters for the reference measurement channel are specified in table A.3A-2 for FR2-NTN PUSCH performance requirements:

-	FRC parameters are specified in table A.3A-2 for FR2-NTN PUSCH with transform precoding disabled, additional DM-RS position = pos1 and 1 transmission layer.

The parameters for the reference measurement channel are specified in table A.3A-3 for FR1-NTN PUSCH performance requirements:

-	FRC parameters are specified in table A.3A-3 for FR1-NTN PUSCH with transform precoding enabled, additional DM-RS position = pos1 and 1 transmission layer.

Table A.3A-1: FRC parameters for FR1-NTN PUSCH performance requirements, transform precoding disabled, additional DM-RS position = pos1 and 1 transmission layer (QPSK, R=99/1024)

| Reference channel | G-FR1-NTN-A3A-1 | G-FR1-NTN-A3A-2 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 30 |
| Allocated resource blocks | 25 | 24 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 |
| MCS table | 64QAMLowSE | 64QAMLowSE |
| Modulation | QPSK | QPSK |
| Code rate (Note 2) | 99/1024 | 99/1024 |
| Payload size (bits) | 704 | 672 |
| Transport block CRC (bits) | 16 | 16 |
| Code block CRC size (bits) | - | - |
| Number of code blocks - C | 1 | 1 |
| Code block size including CRC (bits) (Note 2) | 720 | 688 |
| Total number of bits per slot | 7200 | 6912 |
| Total symbols per slot | 3600 | 3456 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos1, and l0= 2 and l= 11 for PUSCH mapping type A, and l0= 0 and l= 10 for PUSCH type B, as per table 6.4.1.1.3-3 of TS 38.211 [8]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [7]. |  |  |

Table A.3A-2: FRC parameters for FR2-NTN PUSCH performance requirements, transform precoding disabled, additional DM-RS position = pos1 and 1 transmission layer (QPSK, R=99/1024)

| Reference channel | G-FR2-NTN-A3A-1 |
| --- | --- |
| Subcarrier spacing (kHz) | 120 |
| Allocated resource blocks | 32 |
| CP-OFDM Symbols per slot (Note 1) | 8 |
| MCS table | 64QAMLowSE |
| Modulation | QPSK |
| Code rate (Note 2) | 99/1024 |
| Payload size (bits) | 608 |
| Transport block CRC (bits) | 16 |
| Code block CRC size (bits) | - |
| Number of code blocks - C | 1 |
| Code block size including CRC (bits) (Note 2) | 624 |
| Total number of bits per slot | 6144 |
| Total symbols per slot | 3072 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos1, and l0= 0 and l = 8 for PUSCH mapping type B, as per table 6.4.1.1.3-3 of TS 38.211 [8]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [7]. |  |

Table A.3A-3: FRC parameters for FR2-NTN PUSCH performance requirements, transform precoding enabled, additional DM-RS position = pos1 and 1 transmission layer (QPSK, R=99/1024)

| Reference channel | G-FR1-NTN-A3A-3 | G-FR1-NTN-A3A-4 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 30 |
| Allocated resource blocks | 25 | 24 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 |
| MCS table | 64QAMLowSE | 64QAMLowSE |
| Modulation | QPSK | QPSK |
| Code rate (Note 2) | 99/1024 | 99/1024 |
| Payload size (bits) | 704 | 672 |
| Transport block CRC (bits) | 16 | 16 |
| Code block CRC size (bits) | - | - |
| Number of code blocks - C | 1 | 1 |
| Code block size including CRC (bits) (Note 2) | 720 | 688 |
| Total number of bits per slot | 7200 | 6912 |
| Total symbols per slot | 3600 | 3456 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos1, and l0= 0 and l = 10 for PUSCH mapping type B, as per table 6.4.1.1.3-3 of TS 38.211 [8]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [7]. |  |  |

==============Fifth change==============

C.3	Measurement of performance requirements

Table C.3-1: Derivation of Test Requirements (Performance tests)

| Test | Minimum Requirement in TS 38.108 [2] | Test Tolerance (TT) | Test requirement in the present document |
| --- | --- | --- | --- |
| 8.2.1 Performance requirements for PUSCH with transform precoding disabled | SNRs as specified | [0.6] dB for 1Tx cases | Formula: SNR + TT T-put limit unchanged |
| 8.2.2 Performance requirements for PUSCH with transform precoding enabled | SNRs as specified | [0.6] dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.3 Performance requirements for UL timing adjustment | SNRs as specified | [0.3] dB for AWGN | Formula: SNR + TT T-put limit unchanged |
| 8.2.4 Performance requirements for PUSCH repetition Type A | SNRs as specified | [0.6] dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.5 Performance requirements for PUSCH with DM-RS bundling | SNRs as specified | [0.6] dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.6 Performance requrments for PUSCH with inter-slot OCC | SNRs as specified | [0.6] dB | Formula: SNR + TT T-put limit unchanged |
| 8.3.1 Performance requirements for PUCCH format 0 | SNRs as specified | [0.6] dB | Formula: SNR + TT False ACK limit unchanged Correct ACK limit unchanged |
| 8.3.2 Performance requirements for PUCCH format 1 | SNRs as specified | [0.6] dB | Formula: SNR + TT False ACK limit unchanged False NACK limit unchanged Correct ACK limit unchanged |
| 8.3.3 Performance requirements for PUCCH format 2 | SNRs as specified | [0.6] dB | Formula: SNR + TT False ACK limit unchanged Correct ACK limit unchanged UCI BLER limit unchanged |
| 8.3.4 Performance requirements for PUCCH format 3 | SNRs as specified | [0.6] dB | Formula: SNR + TT UCI BLER limit unchanged |
| 8.3.5 Performance requirements for PUCCH format 4 | SNRs as specified | [0.6] dB | Formula: SNR + TT UCI BLER limit unchanged |
| 8.4.1 PRACH false alarm probability and missed detection | SNRs as specified | [0.6] dB for fading cases [0.3] dB for AWGN cases | Formula: SNR + TT PRACH false detection limit unchanged PRACH detection limit unchanged |

Table C.3-2: Derivation of test requirements (FR1-NTN OTA performance tests)

| Test | Minimum Requirement in TS 38.108 [2] | Test Tolerance (TTOTA) | Test requirement in the present document |
| --- | --- | --- | --- |
| 11.2.1 Performance requirements for PUSCH with transform precoding disabled | See clause 11.2.1.1 | [0.6] dB | Formula: SNR + TTOTA T-put limit unchanged |
| 11.2.2 Performance requirements for PUSCH with transform precoding enabled | See clause 11.2.1.2 | [0.6] dB | Formula: SNR + TTOTA T-put limit unchanged |
| 11.2.3 Performance requirements for UL timing adjustment | See clause 11.2.1.3 | [0.3] dB for AWGN cases | Formula: SNR + TTOTA T-put limit unchanged |
| 11.2.4 Performance requirements for PUSCH repetition Type A | See clause 11.2.1.4 | [0.6] dB | Formula: SNR + TTOTA BLER limit unchanged |
| 11.2.5 Performance requirements for PUSCH with DM-RS bundling | See clause 11.2.1.5 | [0.6] dB | Formula: SNR + TTOTA BLER limit unchanged |
| 11.2.6 Performance requirements for PUSCH with inter-slot OCC | See clause 11.2.1.6 | [0.6] dB | Formula: SNR + TTOTA BLER limit unchanged |
| 11.3.1 Performance requirements for PUCCH format 0 | See clause 11.3.1.1 | [0.6] dB | Formula: SNR + TTOTA False ACK limit unchanged Correct ACK limit unchanged |
| 11.3.2 Performance requirements for PUCCH format 1 | See clause 11.3.1.3 | [0.6] dB | Formula: SNR + TTOTA False ACK limit unchanged False NACK limit unchanged Correct ACK limit unchanged |
| 11.3.3 Performance requirements for PUCCH format 2 | See clause 11.3.1.4 | [0.6] dB | Formula: SNR + TTOTA False ACK limit unchanged Correct ACK limit unchanged UCI BLER limit unchanged |
| 11.3.4 Performance requirements for PUCCH format 3 | See clause 11.3.1.5 | [0.6] dB | Formula: SNR + TTOTA UCI BLER limit unchanged |
| 11.3.5 Performance requirements for PUCCH format 4 | See clause 11.3.1.6 | [0.6] dB | Formula: SNR + TTOTA UCI BLER limit unchanged |
| 11.4.1 PRACH false alarm probability and missed detection | See clause 11.4.1 | [0.6] dB for fading cases [0.3] dB for AWGN cases | Formula: SNR + TTOTA PRACH False detection limit unchanged PRACH detection limit unchanged |
| NOTE: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

==============End of change==============