3GPP TSG-RAN WG4 Meeting #116	R4-2511873

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:			7.24.3.2

Source:	Samsung

Title:	TP to TR 38.774 on FR2 UE RF of LP-WUR

Document for:	Approval

# 1  Introduction

In this paper, we provide text proposal to TR 38.774 on FR2 UE RF of LP-WUR focusing on FR2 simulation assumption.

# 2  Text propsoal

## <<Start of Change>>

## 6.2	Simulation assumption

### 6.2.1	General

To evaluate the RF performance of LP-WUS/WUR, the link level simulation is performed in this WI. This clause captures the simulation assumption for LP-WUS UE RF and BS RF analysis.

### 6.2.2	Simulation parameters

For RF performance analysis, the following simulation assumptions are considered.

Table 6.2.2-1 General Link level simulation assumption for FR1 LP-WUR

| Attributes | Assumptions |
| --- | --- |
| Waveform | OOK-4 waveform |
| Center frequency | 900MHz, 2.6GHz and 3.5GHz |
| Channel structure | 5 bits |
| RM coding | 8/16/32bit for OOK |
| CRC | NO |
| Chip rate | M=4 |
| overlaid OFDM sequence | 4 candidates overlaid sequences for M=4 |
| WUS duration | OOK：RM coding length*2/M OFDM：ceil(5/log2(4)/(M/2)) |
| Coding | 1/2 rate Manchester coding |
| Time error | 0/0.9 us |
| Residual Frequency error | 0/10/20 ppm |
| SCS | 30kHz |
| UE Channel BW | 20MHz (51 RB)-case 1 10MHz (24 RB)-case 2 5MHz (11 RB)-case 3 |
| WUS RB | - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases |
| Position within channel | - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 505kHz for 5MHz, 665kHz for 10MH, 805kHz for 20Mhz - For interference cell2 5MHz: fixed 505kHz |
| Guard RB | For ASCS: 0 or 1RB on each side of LP-WUS bandwidth For ACS: 1/2/3/4 RB |
| Filter | 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded The filter bandwidth is adapted with actual WUS RBs, for 5MHz case |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; EPRE of PDSCH /EPRE of in-band LP-WUS = [20~33] dB NOTE: decide the interference level depending on SNR |
| Wanted signal level | For ACS, REFSENS + 14 dB for LP-WUS as baseline, different wanted power level can be also considered |
| Sampling rate | 7.68MHz |
| ADC bit width | 4/8 bits ADC for ASCS/ACS Encourage companies to provide simulation results with both options for comparison |
| Phase noise | Reciprocal mixing can be optionally considered for ASCS/ACS. Note that actual implementation choice of phase noise is driven by jammer requirement and power consumption target. |
| Non-linearities | Not modelled |
| Power boosting | EPRE ratio: 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation |
| Channel Model | AWGN |
| Performance metric | - 1% MDR - false alarm rate 1% as side condition |
| Note: The detailed Receiver algorithm is UE implementation . |  |

Table 6.2.2-2 General Link level simulation assumption for FR2 LP-WUR

| Attributes | Assumptions |  |  | Notes |
| --- | --- | --- | --- | --- |
| Case name (waveform) | OOK-1 waveform |  | OOK-4 waveform |  |
| Center frequency | 24300MHz |  |  | n258 example band |
| WUS structure | Total 8/16/32 bits |  |  | (placeholder for RAN1 decision) |
| Chip rate | M=1 | M=1/2/4 |  |  |
| Coding | 1/2 rate Manchester coding |  |  |  |
| Time error | up to UE implementation |  |  | Difficult to separate time and frequency errors in realistic receivers |
| Residual Frequency error | 0/10/20 ppm |  |  |  |
| SCS | 120kHz |  |  |  |
| UE Channel BW | 200MHz (132 RB)-case 1 100MHz (66 RB)-case 2 |  |  |  |
| WUS RB | - Fixed 11RB |  |  | Based on agreement in RAN1#118-Bis |
| Position within channel | - Center for ASCS, edge for ACS [assume no ASCS impact] |  |  |  |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - Refer to 5.3.3 in TS38.101-2 |  |  |  |
| Guard RB | - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth - For ACS: 1/2/3/4 RB |  |  |  |
| Filter | - 3th/5th Order lowpass Butterworth - Other order lowpass filter is not precluded |  |  |  |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |  |  |  |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; NOTE: decide the interference level depending on SNR |  |  |  |
| Wanted signal level | For ACS, REFSENS + 14 dB for LP-WUS |  |  |  |
| Sampling rate | TBD |  |  |  |
| ADC bit width | 4/8 bits ADC for ASCS/ACS Encourage companies to provide simulation results with both options for comparison |  |  |  |
| Phase noise | FFS |  |  |  |
| Non-linearities | Not modelled |  |  |  |
| Power boosting | 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation |  |  |  |
| Channel Model | Option 1: TDL-C 100 Option 2: AWGN Note: encourage companies to provide simulation results with both options |  |  | The procedure for using CDL channel models in demod analysis is not fully established. |

## <<End of Change>>