3GPP TSG-WG4 Meeting # 116	R4-2512154

# Bengaluru, IN, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 5937 | rev | 1 | Current version: | 18.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.133 corrections to performance requirement for NR positioning (Rel. 18) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Perf |  |  |  |  |  | Date: |  |  | 2025-08-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - There is square bracket around SRS transmission timing for the applicability of UE Rx-Tx time difference measurement. - Incorrect use of symbol for millisecond. - IEs in report mapping clauses for UL-RTOA and gNB Rx-Tx time difference measurement are not aligned with the IEs in NRPPa specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Square bracket around SRS transmission timing for the applicability of UE Rx-Tx time difference measurement is removed. - Symbol for millisecond is rectified from “msec” to “ms”. - IEs in report mapping clauses for UL-RTOA and gNB Rx-Tx time difference measurement are updated to align with the IEs in NRPPa specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - SRS transmission timing for the applicability of UE Rx-Tx time difference measurement remains ambiguous. - Symbol for millisecond is not correct. - Report mapping of UL-RTOA and gNB Rx-Tx time difference measurement for new k values is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.25A.2, 10.1.44.2, 10.1A.18.2, 13.1.1, 13.1.1A, 13.2.1, and 13.2.1A. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2510756 |  |  |  |  |  |  |  |  |

## START OF CHANGE 1

#### 10.1.25A.2	Measurement Accuracy Requirements

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall not apply, if:

NTA_offset defined in table 7.1.2-2 changes during the UE Rx-Tx measurement period or

if the uplink transmission timing changes during the UE Rx-Tx measurement period due to the network-configured Timing Advance.

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that:

-	The UE transmits SRS within the range from -160 ms to 160 ms of at least one DL PRS resource of each of the TRPs in the assistance data.

-	PRS resources linked for aggregation saftisfy all the conditions specified in TS 38.214 [26] clause 5.1.6.5.3.

-	the spacing between the center frequencies of adjacent PFLs containing PRS resources linked for aggregation does not exceed the nominal channel spacing for intra-band contiguous CA defined in TS 38.101-1 [18], clause 5.4A.1 for FR1 and in TS 38.101-2 [19], clause 5.4A.1 for FR2-1.

If the uplink transmission timing changes during the UE Rx-Tx measurement period due to the autonomous timing adjustment defined in clause 7.1.2 then:

-	UE Rx-Tx measurement accuracy requirements shall apply for a cell, which is also the downlink reference cell (defined in clause 7.1.1) for SRS transmission even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

-	UE Rx-Tx measurement accuracy requirements shall not apply for a cell, which is not the downlink reference cell (defined in clause 7.1.1) for SRS transmission, if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

When a serving cell change occurs during the UE Rx-Tx measurement period, the UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx measurement.

The accuracy requirements in table 10.1.25A.2-1 for FR1 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

PRP|dBm according to annex B.2.14 for a corresponding Band.

AWGN propagation condition.

Table 10.1.25A.2-1: UE Rx-Tx time difference measurement accuracy in FR1 in AWGN

| Accuracy for 2 PFLs | Accuracy for 3 PFLs | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRS Ês/Iot | PRS bandwidth per PFL | PRS SCS | PRS resource repetition (TrepPRS*LPRS/KcombPRSNote 3 | NR operating band groupsNote 2 | IoNote 4 range |  |
|  |  |  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | TcNote 5 | dB | PRB | kHz |  |  | dBm / SCSPRS | dBm/BW |
| ±12 + | ±10 + | -3 | ≥104 | 15 | ≥1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B | -126.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C | -126 |  |
|  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 |  |
|  |  |  |  |  |  | NR_FDD_FR1_F | -124.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -124 |  |
|  |  |  |  |  |  | NR_FDD_FR1_H | -123.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_N | -120.5 |  |
| ±6 + | ±3 + |  | ≥132 | 30 | ≥1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B | -123.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C | -123 |  |
|  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 |  |
|  |  |  |  |  |  | NR_FDD_FR1_F | -121.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -121 |  |
|  |  |  |  |  |  | NR_FDD_FR1_H | -120.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_N | -117.5 |  |
| ±3 + | ±2 + |  | ≥272 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ±6 + | ±3 + |  | ≥64 | 60 | ≥1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B | -120.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C | -120 |  |
|  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 |  |
|  |  |  |  |  |  | NR_FDD_FR1_F | -118.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -118 |  |
|  |  |  |  |  |  | NR_FDD_FR1_H | -117.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_N | -114.5 |  |
| ±3 + | ±2 + |  | ≥ 132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ±14+ | ±13+ | -13 | ≥104 | 15 | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ±8+ | ±4+ |  | ≥132 | 30 | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ±4+ | ±3+ |  | ≥272 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ±9+ | ±4+ |  | ≥ 64 | 60 | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ±4+ | ±3+ |  | ≥ 132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest PRB number for the corresponding SCS. NOTE 7: is the margin determined from table 10.1.25A.2-5, where the PRS BW refers to the sum of the PRS PRB numbers across the aggregated PFLs. |  |  |  |  |  |  |  |  |

The accuracy requirements in table 10.1.25A.2-1a for FR1 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

PRP|dBm according to annex B.2.14 for a corresponding Band.

Number of measurement samples is less than 4.

AWGN propagation condition.

Table 10.1.25A.2-1a: UE Rx-Tx time difference measurement accuracy in FR1 in AWGN with reduced measurement samples

| Accuracy for 2 PFLs | Accuracy for 3 PFLs | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRS Ês/Iot | PRS bandwidth per PFL | PRS SCS | PRS resource repetition (TrepPRS*LPRS/KcombPRSNote 3 | NR operating band groupsNote 2 | IoNote 4 range |  |
|  |  |  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | TcNote 5 | dB | PRB | kHz |  |  | dBm / SCSPRS | dBm/BW |
| ±11+ | ±9+ | 0 | ≥104 | 15 | ≥1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B | -126.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C | -126 |  |
|  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 |  |
|  |  |  |  |  |  | NR_FDD_FR1_F | -124.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -124 |  |
|  |  |  |  |  |  | NR_FDD_FR1_H | -123.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_N | -120.5 |  |
| ±5+ | ±3+ |  | ≥132 | 30 | ≥1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B | -123.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C | -123 |  |
|  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 |  |
|  |  |  |  |  |  | NR_FDD_FR1_F | -121.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -121 |  |
|  |  |  |  |  |  | NR_FDD_FR1_H | -120.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_N | -117.5 |  |
| ±3+ | ±2+ |  | ≥272 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ±5+ | ±3+ |  | ≥64 | 60 | ≥1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B | -120.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C | -120 |  |
|  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 |  |
|  |  |  |  |  |  | NR_FDD_FR1_F | -118.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -118 |  |
|  |  |  |  |  |  | NR_FDD_FR1_H | -117.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_N | -114.5 |  |
| ±3+ | ±2+ |  | ≥ 132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ±14+ | ±13+ | -6 | ≥104 | 15 | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ±9+ | ±4+ |  | ≥132 | 30 | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ±4+ | ±3+ |  | ≥272 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ±9+ | ±4+ |  | ≥ 64 | 60 | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ±4+ | ±3+ |  | ≥ 132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest PRB number for the corresponding SCS. NOTE 7: is the margin determined from table 10.1.25A.2-5, where the PRS BW refers to the sum of the PRS PRB numbers across the aggregated PFLs. |  |  |  |  |  |  |  |  |

The accuracy requirements in table 10.1.25A.2-2 for FR1 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

PRP|dBm according to annex B.2.14 for a corresponding Band.

Fading propagation condition.

Table 10.1.25A.2-2: UE Rx-Tx time difference measurement accuracy in FR1 for fading channel

| Accuracy for 2 PFLs | Accuracy for 3 PFLs | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRS Ês/Iot | PRS bandwidth per PFL | PRS SCS | PRS resource repetition (TrepPRS*LPRS/KcombPRSNote 3 | NR operating band groupsNote 2 | IoNote 4 range |  |
|  |  |  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | TcNote 5 | dB | PRB | kHz |  |  | dBm / SCSPRS | dBm/BW |
| ±40+ | ±38+ | -3 | ≥104 | 15 | ≥1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B | -126.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C | -126 |  |
|  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 |  |
|  |  |  |  |  |  | NR_FDD_FR1_F | -124.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -124 |  |
|  |  |  |  |  |  | NR_FDD_FR1_H | -123.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_N | -120.5 |  |
| ±33+ | ±32+ |  | ≥132 | 30 | ≥1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B | -123.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C | -123 |  |
|  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 |  |
|  |  |  |  |  |  | NR_FDD_FR1_F | -121.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -121 |  |
|  |  |  |  |  |  | NR_FDD_FR1_H | -120.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_N | -117.5 |  |
| ±33+ | ±30+ |  | ≥272 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ±35+ | ±34+ |  | ≥64 | 60 | ≥1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B | -120.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C | -120 |  |
|  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 |  |
|  |  |  |  |  |  | NR_FDD_FR1_F | -118.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -118 |  |
|  |  |  |  |  |  | NR_FDD_FR1_H | -117.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_N | -114.5 |  |
| ±32+ | ±31+ |  | ≥ 132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ±44+ | ±39+ | --13 | ≥104 | 15 | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ±34+ | ±34+ |  | ≥132 | 30 | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ±33+ | ±30+ |  | ≥272 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ±35+ | ±37+ |  | ≥ 64 | 60 | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ±33+ | ±34+ |  | ≥ 132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest PRB number for the corresponding SCS. NOTE 7: is the margin determined from table 10.1.25A.2-5, where the PRS BW refers to the sum of the PRS PRB numbers across the aggregated PFLs. |  |  |  |  |  |  |  |  |

The accuracy requirements in table 10.1.25A.2-3 for FR2 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

PRP|dBm according to annex B.2.14 for a corresponding Band.

AWGN propagation condition.

Table 10.1.25A.2-3: UE Rx-Tx time difference measurement accuracy in FR2 in AWGN

| Accuracy for 2 PFLs | Accuracy for 3 PFLs | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRS Ês/Iot | PRS bandwidth per PFL | PRS SCS | PRS resource repetition(TrepPRS*LPRS/KcombPRSNote 3 | IoNote 4 range |  |
|  |  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | TcNote 5 | dB | PRB | kHz |  | dBm / SCSPRS | dBm/BWChannel |
| ±6+ | ±3+ | -3 | ≥64 | 60 | ≥1 | Same value as PRP in table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ±3+ | ±2+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| ±3+ | ±2+ |  | ≥64 | 120 | ≥1 | Same value as PRP in table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ±2+ | ±1+ |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| ±9+ | ±4+ | -13 | ≥64 | 60 | ≥1 | NOTE 6 | NOTE 6 |
| ±4+ | ±3+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| ±4+ | ±3+ |  | ≥64 | 120 | ≥1 | NOTE 6 | NOTE 6 |
| ±2+ | ±1+ |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest PRB number for the corresponding SCS. NOTE 7: is the margin determined from table 10.1.25A.2-6, where the PRS BW refers to the sum of the PRS PRB numbers across the aggregated PFLs. |  |  |  |  |  |  |  |

The accuracy requirements in table 10.1.25A.2-3a for FR2 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

PRP|dBm according to annex B.2.14 for a corresponding Band.

Number of measurement samples is less than 4.

AWGN propagation condition.

Table 10.1.25A.2-3a: UE Rx-Tx time difference measurement accuracy in FR2 in AWGN with reduced measurement samples

| Accuracy for 2 PFLs | Accuracy for 3 PFLs | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRS Ês/Iot | PRS bandwidth per PFL | PRS SCS | PRS resource repetition(TrepPRS*LPRS/KcombPRSNote 3 | IoNote 4 range |  |
|  |  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | TcNote 5 | dB | PRB | kHz |  | dBm / SCSPRS | dBm/BWChannel |
| ±5+ | ±3+ | 0 | ≥64 | 60 | ≥1 | Same value as PRP in table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ±3+ | ±2+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| ±3+ | ±2+ |  | ≥64 | 120 | ≥1 | Same value as PRP in table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ±2+ | ±1+ |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| ±9+ | ±4+ | -6 | ≥64 | 60 | ≥1 | NOTE 6 | NOTE 6 |
| ±4+ | ±3+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| ±4+ | ±3+ |  | ≥64 | 120 | ≥1 | NOTE 6 | NOTE 6 |
| ±2+ | ±1+ |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest PRB number for the corresponding SCS. NOTE 7: is the margin determined from table 10.1.25A.2-6, where the PRS BW refers to the sum of the PRS PRB numbers across the aggregated PFLs. |  |  |  |  |  |  |  |

The accuracy requirements in table 10.1.25A.2-4 for FR2 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

PRP|dBm according to annex B.2.14 for a corresponding Band.

Fading propagation condition.

Table 10.1.25A.2-4: UE Rx-Tx time difference measurement accuracy in FR2 for fading channel

| Accuracy for 2 PFLs | Accuracy for 3 PFLs | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRS Ês/Iot | PRS bandwidth per PFL | PRS SCS | PRS resource repetition(TrepPRS*LPRS/KcombPRSNote 3 | IoNote 4 range |  |
|  |  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | TcNote 5 | dB | PRB | kHz |  | dBm / SCSPRS | dBm/BWChannel |
| ±57+ | ±38+ | -3 | ≥64 | 60 | ≥1 | Same value as PRP in table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ±37+ | ±32+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| ±44+ | ±37+ |  | ≥64 | 120 | ≥1 | Same value as PRP in table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ±38+ | ±38+ |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| ±73+ | ±43+ | -13 | ≥64 | 60 | ≥1 | NOTE 6 | NOTE 6 |
| ±46+ | ±35+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| ±56+ | ±42+ |  | ≥64 | 120 | ≥1 | NOTE 6 | NOTE 6 |
| ±43+ | ±47+ |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest PRB number for the corresponding SCS. NOTE 7: is the margin determined from table 10.1.25A.2-6, where the PRS BW refers to the sum of the PRS PRB numbers across the aggregated PFLs. |  |  |  |  |  |  |  |

Table 10.1.25A.2-5: Margin for UE Rx-Tx time difference measurement accuracy in FR1

| Min(PRS BW, SRS BW) (PRB) |  |  | Margin (Tc Note 1) |
| --- | --- | --- | --- |
| SCS = 15 kHz | SCS = 30 kHz | SCS = 60 kHz |  |
| ≥ 104 | N/A | N/A | 56 |
| N/A | ≥ 132 | ≥ 64 | 24 |
| N/A | N/A | ≥ 132 | 24 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: If SRS and PRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |  |

Table 10.1.25A.2-6: Margin for UE Rx-Tx time difference measurement accuracy in FR2

| Min(PRS BW, SRS BW) (PRB) |  | Margin (Tc Note 1) |
| --- | --- | --- |
| SCS = 60 kHz | SCS = 120 kHz |  |
| ≥ 64 | N/A | 32 |
| ≥ 132 | ≥ 64 | 24 |
| N/A | ≥ 128 | 20 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: If SRS and PRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |

The relative accuracy of UE Rx-Tx measurement in this clause is defined as accuracy of the difference between two UE Rx-Tx measurements.

The relative accuracy requirements in table 10.1.25A.2-7 for FR1 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

PRP|dBm according to annex B.2.14 for a corresponding Band.

AWGN propagation condition.

The two UE Rx-Tx time difference measurements are associated with the same RxTx TEG.

Table 10.1.25A.2-7: UE Rx-Tx time difference relative measurement accuracy in FR1 in AWGN with TEG reporting

| Accuracy for 2 PFLs | Accuracy for 3 PFLs | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRS Ês/Iot | PRS bandwidth per PFL Note 1 | PRS SCS | PRS resource repetition (TrepPRS*LPRS/KcombPRSNote 2 | NR operating band groupsNote 4 | IoNote 3 range |  |
|  |  |  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | TcNote 5 | dB | PRB | kHz |  |  | dBm / SCSPRS | dBm/BW |
| ±20+ Δ | ±16+Δ | (PRS Ês/Iot)j ≥-6 (PRS Ês/Iot)i ≥-13 | ≥104 | 15 | ≥1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_B | -126.5 |  |
|  |  |  |  |  |  | NR_TDD_FR1_C | -126 |  |
|  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 |  |
|  |  |  |  |  |  | NR_FDD_FR1_F | -124.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -124 |  |
|  |  |  |  |  |  | NR_FDD_FR1_H | -123.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_N | -120.5 |  |
| ±11+Δ | ±6+Δ |  | ≥132 | 30 | ≥1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_B | -123.5 |  |
|  |  |  |  |  |  | NR_TDD_FR1_C | -123 |  |
|  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 |  |
|  |  |  |  |  |  | NR_FDD_FR1_F | -121.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -121 |  |
|  |  |  |  |  |  | NR_FDD_FR1_H | -120.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_N | -117.5 |  |
| ±6+Δ | ±4+Δ |  | ≥272 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ±11+Δ | ±6+Δ |  | ≥64 | 60 | ≥1 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  |  | NR_FDD_FR1_B | -120.5 |  |
|  |  |  |  |  |  | NR_TDD_FR1_C | -120 |  |
|  |  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 |  |
|  |  |  |  |  |  | NR_FDD_FR1_F | -118.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G | -118 |  |
|  |  |  |  |  |  | NR_FDD_FR1_H | -117.5 |  |
|  |  |  |  |  |  | NR_FDD_FR1_N | -114.5 |  |
| ±5+Δ | ±4+Δ |  | ≥ 132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of resource j and resource i. NOTE 2: Minimum number of PRS resource repetitions among resource j and resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest PRB number for the corresponding SCS. NOTE 7: Δ is the value of the timing error margin for the RxTx TEG, reported via nr-UE-RxTxTEG-TimingErrorMargin. Δ cannot be larger than the sum of the margins in table 10.1.25A.2-5 (dependent on PRS/SRS BW) for any pair of individual UE Rx-Tx time difference measurements associated with the RxTx TEG. |  |  |  |  |  |  |  |  |

The relative accuracy requirements in table 10.1.25A.2-8 for FR2 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

PRP|dBm according to annex B.2.14 for a corresponding Band.

AWGN propagation condition.

the two UE Rx-Tx time difference measurements are associated with the same RxTx TEG.

Table 10.1.25A.2-8: UE Rx-Tx time difference relative measurement accuracy in FR2 in AWGN with TEG reporting

| Accuracy for 2 PFLs | Accuracy for 3 PFLs | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRS Ês/Iot | PRS bandwidth per PFL Note 1 | PRS SCS | PRS resource repetition(TrepPRS*LPRS/KcombPRSNote 2 | IoNote 3 range |  |
|  |  |  |  |  |  | Minimum Io | Maximum Io |
| TcNote 5 | TcNote 5 | dB | PRB | kHz |  | dBm / SCSPRS | dBm/BWChannel |
| 11+Δ | 6+Δ | (PRS Ês/Iot)j ≥-6 (PRS Ês/Iot)i ≥-13 | ≥64 | 60 | ≥1 | Same value as PRP in table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 5+Δ | 4+Δ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| 6+Δ | 4+Δ |  | ≥64 | 120 | ≥1 | Same value as PRP in table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 3+Δ | 2+Δ |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| NOTE 1: Minimum PRS bandwidth per PFL, which is minimum of the PRS bandwidths of resource j and resource i. NOTE 2: Minimum number of PRS resource repetitions among resource j and resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest PRB number for the corresponding SCS. NOTE 6: Δ is the value of the timing error margin for the RxTx TEG, reported via nr-UE-RxTxTEG-TimingErrorMargin. Δ cannot be larger than the sum of the margins in table 10.1.25A.2-6 (dependent on PRS/SRS BW) for any pair of individual UE Rx-Tx time difference measurements associated with the RxTx TEG. |  |  |  |  |  |  |  |

## END OF CHANGE 1

## START OF CHANGE 2

#### 10.1.44.2	Measurement Accuracy Requirements

The relative accuracy of DL RSCP measurement in this clause is defined as accuracy of the difference between two DL RSCP measurements, each based on single measurement sample in single PFL.

The requirements in this clause are derived based on AWGN channel and based on two-tap channel defined in 38.101-4 [21] annex B.2.4 (a = 1, τd=0.45 µs and fD=5 Hz).

The DL RSCP relative measurement accuracy requirements in this clause shall not apply, if:

-	NTA_offset defined in table 7.1.2-2 changes during the DL RSCP with UE Rx-Tx measurement period, or

-	if the uplink transmission timing changes during the DL RSCP with UE Rx-Tx measurement period due to the network-configured Timing Advance.

The DL RSCP relative measurement accuracy requirements in this clause shall apply provided that:

-	The UE transmits SRS within the range from -160 ms to 160 ms of at least one DL PRS resource of each of the TRPs in the assistance data.

If the uplink transmission timing changes during the DL RSCP with UE Rx-Tx measurement period due to the autonomous timing adjustment defined in clause 7.1.2 then:

-	DL RSCP and UE Rx-Tx measurement accuracy requirements shall apply for a cell, which is also the downlink reference cell (defined in section 7.1.1) for SRS transmission.

-	UE Rx-Tx measurement accuracy requirements shall not apply for a cell, which is not the downlink reference cell (defined in section 7.1.1) for SRS transmission.

When a serving cell change occurs during the DL RSCP with UE Rx-Tx measurement period, UE Rx-Tx measurement accuracy requirements and DL RSCP measurement requirements do not apply.

The relative DL RSCP accuracy requirements defined in clause 10.1.44 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled for FR1.

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled for FR2.

-	Conditions for DL RSCP measurements are fulfilled according to annex B.2.14 for a corresponding Band for each relevant PRS resource configured for measurement.

-	DL RSCP measurements to derive the relative accuracy are performed on PRS resources within the same set of symbols.

-	DL RSCPD measurements are based on PRS resources with comb size other than 12.

The accuracy requirement for relative DL-RSCP shall be within ±(X+Y) degree.

The values of X for accuracy requirements for relative DL-RSCP measurement in FR1 are defined in table 10.1.44.2-1 for AWGN channel and in table 10.1.44.2-2 for two-tap channel and is derived assuming no carrier frequency offset at TRP.

The values of X for accuracy requirements for relative DL-RSCP measurement in FR2 are defined in table 10.1.44.2-3 for AWGN channel and in table 10.1.44.2-4 for two-tap channel and is derived assuming no carrier frequency offset at TRP.

Y is 14 degrees for FR1 and 28 degrees for FR2.

Table 10.1.44.2-1: DL RSCP relative accuracy in FR1 for AWGN channel

| Accuracy (PRS Ês/Iot)ref ≥ -3dB (PRS Ês/Iot)i ≥ -13dB | Accuracy (PRS Ês/Iot)ref ≥ 0dB (PRS Ês/Iot)i ≥ -6dB | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |  |
|  |  |  |  |  | NR operating band groups Note 4 | Minimum Io | Maximum Io |
| degree | degree | kHz | PRB |  |  | dBm/SCS | dBm/BWChannel |
| 14 | 4 | 15 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B | -126.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C | -126 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -124.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -123.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -120.5 | -50 |
| 9 | 4 |  | ≥ 52 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 6 | 3 |  | ≥ 104 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 14 | 5 | 30 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B | -123.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C | -123 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -121.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -120.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -117.5 | -50 |
| 9 | 3 |  | ≥ 48 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 7 | 3 |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 14 | 4 | 60 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B | -120.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C | -120 | -50 |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -50 |
|  |  |  |  |  | NR_FDD_FR1_N | -114.5 | -50 |
| 10 | 4 |  | ≥ 64 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 7 | 3 |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. |  |  |  |  |  |  |  |

Table 10.1.44.2-2: DL RSCP relative accuracy in FR1 for two-tap channel

| Accuracy (PRS Ês/Iot)ref ≥ 0dB (PRS Ês/Iot)i ≥ -6dB | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |  |
|  |  |  |  | NR operating band groups Note 4 | Minimum Io | Maximum Io |
| degree | kHz | PRB |  |  | dBm/SCS | dBm/BWChannel |
| 9 | 15 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -50 |
|  |  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B | -126.5 | -50 |
|  |  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C | -126 | -50 |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -50 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -50 |
|  |  |  |  | NR_FDD_FR1_F | -124.5 | -50 |
|  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -124 | -50 |
|  |  |  |  | NR_FDD_FR1_H | -123.5 | -50 |
|  |  |  |  | NR_FDD_FR1_N | -120.5 | -50 |
| 7 |  | ≥ 52 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 5 |  | ≥ 104 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 7 | 30 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -124 | -50 |
|  |  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B | -123.5 | -50 |
|  |  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C | -123 | -50 |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -50 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -50 |
|  |  |  |  | NR_FDD_FR1_F | -121.5 | -50 |
|  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -121 | -50 |
|  |  |  |  | NR_FDD_FR1_H | -120.5 | -50 |
|  |  |  |  | NR_FDD_FR1_N | -117.5 | -50 |
| 5 |  | ≥ 48 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 7 |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 7 | 60 | ≥ 24 | ≥ 4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  | NR_FDD_FR1_B, NR_TDD_FR1_B | -120.5 | -50 |
|  |  |  |  | NR_FDD_FR1_C, NR_TDD_FR1_C | -120 | -50 |
|  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -50 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -50 |
|  |  |  |  | NR_FDD_FR1_F | -118.5 | -50 |
|  |  |  |  | NR_FDD_FR1_G, NR_TDD_FR1_G, NR_SDL_FR1_G | -118 | -50 |
|  |  |  |  | NR_FDD_FR1_H | -117.5 | -50 |
|  |  |  |  | NR_FDD_FR1_N | -114.5 | -50 |
| 9 |  | ≥ 64 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| 6 |  | ≥ 132 | ≥ 1 | Note 5 | Note 5 | Note 5 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. NOTE 5: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest PRB number for the corresponding SCS. |  |  |  |  |  |  |

Table 10.1.44.2-3: DL RSCP relative accuracy in FR2 for AWGN channel

| Accuracy (PRS Ês/Iot)ref ≥ -3dB (PRS Ês/Iot)i ≥ -13dB | Accuracy (PRS Ês/Iot)ref ≥ 0dB (PRS Ês/Iot)i ≥ -6dB | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |
|  |  |  |  |  | Minimum Io | Maximum Io |
| degree | degree | kHz | PRB |  | dBm/SCS | dBm/BWChannel |
| 14 | 4 | 60 | ≥ 24 | ≥ 4 | Same value as PRS_RP in table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 8 | 4 |  | ≥ 64 | ≥ 1 | Note 4 | Note 4 |
| 6 | 3 |  | ≥ 132 | ≥ 1 | Note 4 | Note 4 |
| 16 | 5 | 120 | ≥ 32 | ≥ 4 | Same value as PRS_RP in table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 9 | 4 |  | ≥ 64 | ≥ 1 | Note 4 | Note 4 |
| 7 | 3 |  | ≥ 128 | ≥ 1 | Note 4 | Note 4 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. |  |  |  |  |  |  |

Table 10.1.44.2-4: DL RSCP relative accuracy in FR2 for two-tap channel.

| Accuracy (PRS Ês/Iot)ref ≥ 0dB (PRS Ês/Iot)i ≥ -6dB | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | PRS SCS | PRS bandwidth Note 1 | PRS resource repetition (TrepPRS*LPRS/KcombPRS) Note 2 | Io Note 3 range |  |
|  |  |  |  | Minimum Io | Maximum Io |
| degree | kHz | PRB |  | dBm/SCS | dBm/BWChannel |
| 8 | 60 | ≥ 24 | ≥ 4 | Same value as PRS_RP in table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 5 |  | ≥ 64 | ≥ 1 | Note 4 | Note 4 |
| 3 |  | ≥ 132 | ≥ 1 | Note 4 | Note 4 |
| 9 | 120 | ≥ 32 | ≥ 4 | Same value as PRS_RP in table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| 6 |  | ≥ 64 | ≥ 1 | Note 4 | Note 4 |
| 5 |  | ≥ 132 | ≥ 1 | Note 4 | Note 4 |
| NOTE 1: Minimum PRS bandwidth, which is minimum of the PRS bandwidths of the reference resource and the measured neighbour resource i. NOTE 2: Minimum number of PRS resource repetitions among the reference resource and the measured neighbour resource i. TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeN defined in TS 37.355 [34], respectively. NOTE 3: Io is assumed to have constant EPRE across the bandwidth. NOTE 4: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest PRB number for the corresponding SCS. |  |  |  |  |  |

## END OF CHANGE 2

## START OF CHANGE 3

#### 10.1A.18.2	Measurement Accuracy Requirements

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall not apply, if:

-	NTA_offset defined in table 7.1A.2-2 changes during the UE Rx-Tx measurement period or

-	if the uplink transmission timing changes during the UE Rx-Tx measurement period due to the network-configured Timing Advance.

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that:

-	The UE transmits SRS within the range from -160 ms to 160 ms of at least one DL PRS resource of each of the TRPs in the assistance data.

If the uplink transmission timing changes during the UE Rx-Tx measurement period due to the autonomous timing adjustment defined in clause 7.1A.2 then:

-	UE Rx-Tx measurement accuracy requirements shall apply for a cell, which is also the downlink reference cell (defined in clause 7.1A.1) for SRS transmission even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

-	UE Rx-Tx measurement accuracy requirements shall not apply for a cell, which is not the downlink reference cell (defined in clause 7.1A.1) for SRS transmission, if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

When a serving cell change occurs during the UE Rx-Tx measurement period, the UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx measurement.

The relative accuracy of UE Rx-Tx measurement in this clause is defined as accuracy of the difference between two UE Rx-Tx measurements.

## END OF CHANGE 3

## START OF CHANGE 4

## 13.1	UL-RTOA

### 13.1.1	Report mapping

The reporting range of UL Relative Time of Arrival (UL-RTOA), as defined in Clause 5.2.2 of TS 38.215 [4], is defined from -985024Tc to +985024Tc. The reporting resolution is uniform across the reporting range and is defined as T = Tc2k where k is selected by gNB from the set {-6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5}.

Tc is defined in TS 38.211 [6].

LMF provides a recommended resolution parameter, timingReportingGranularityFactor or timingReportingGranularityFactorExtended [35]. gNB selects parameter k based on timingReportingGranularityFactor or timingReportingGranularityFactorExtended [35] and informs the LMF.

The mapping of measured quantity for each reporting resolution (k) is defined in table 13.1.1-1 to table 13.1.1-12.

Table 13.1.1-1: Measurement report mapping for k=0

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_0000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_0000001 | -985024 UL_RTOA < -985023 | Tc |
| UL_RTOA_0000002 | -985023 UL_RTOA < -985022 | Tc |
|  |  | … |
| UL_RTOA_0985023 | -2 UL_RTOA < -1 | Tc |
| UL_RTOA_0985024 | -1 UL_RTOA 0 | Tc |
| UL_RTOA_0985025 | 0 < UL_RTOA 1 | Tc |
| UL_RTOA_0985026 | 1 < UL_RTOA 2 | Tc |
| UL_RTOA_0985027 | 2 < UL_RTOA 3 | Tc |
| … | … | … |
| UL_RTOA_1970048 | 985023 < UL_RTOA 985024 | Tc |
| UL_RTOA_1970049 | 985024 < UL_RTOA | Tc |

Table 13.1.1-2: Measurement report mapping for k=1

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_000001 | -985024 UL_RTOA < -985022 | Tc |
| UL_RTOA_000002 | -985022 UL_RTOA < -985020 | Tc |
|  |  | … |
| UL_RTOA_492511 | -4 UL_RTOA < -2 | Tc |
| UL_RTOA_492512 | -2 UL_RTOA 0 | Tc |
| UL_RTOA_492513 | 0 < UL_RTOA 2 | Tc |
| UL_RTOA_492514 | 2 < UL_RTOA 4 | Tc |
| UL_RTOA_492515 | 4 < UL_RTOA 6 | Tc |
| … | … | … |
| UL_RTOA_985024 | 985022 < UL_RTOA 985024 | Tc |
| UL_RTOA_985025 | 985024 < UL_RTOA | Tc |

Table 13.1.1-3: Measurement report mapping for k=2

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_000001 | -985024 UL_RTOA < -985020 | Tc |
| UL_RTOA_000002 | -985020 UL_RTOA < -985018 | Tc |
|  |  | … |
| UL_RTOA_246255 | -8 UL_RTOA < -4 | Tc |
| UL_RTOA_246256 | -4 UL_RTOA 0 | Tc |
| UL_RTOA_246257 | 0 < UL_RTOA 4 | Tc |
| UL_RTOA_246258 | 4 < UL_RTOA 8 | Tc |
| UL_RTOA_246259 | 8 < UL_RTOA 12 | Tc |
| … | … | … |
| UL_RTOA_492512 | 985020 < UL_RTOA 985024 | Tc |
| UL_RTOA_492513 | 985024 < UL_RTOA | Tc |

Table 13.1.1-4: Measurement report mapping for k=3

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_000001 | -985024 UL_RTOA < -985016 | Tc |
| UL_RTOA_000002 | -985016 UL_RTOA < -985008 | Tc |
|  |  | … |
| UL_RTOA_123127 | -16 UL_RTOA < -8 | Tc |
| UL_RTOA_123128 | -8 UL_RTOA 0 | Tc |
| UL_RTOA_123129 | 0 < UL_RTOA 8 | Tc |
| UL_RTOA_123130 | 8 < UL_RTOA 16 | Tc |
| UL_RTOA_123131 | 16 < UL_RTOA 24 | Tc |
| … | … | … |
| UL_RTOA_246256 | 985016 < UL_RTOA 985024 | Tc |
| UL_RTOA_246257 | 985024 < UL_RTOA | Tc |

Table 13.1.1-5: Measurement report mapping for k=4

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_000001 | -985024 UL_RTOA < -985008 | Tc |
| UL_RTOA_000002 | -985008 UL_RTOA < -984992 | Tc |
|  |  | … |
| UL_RTOA_061563 | -32 UL_RTOA < -16 | Tc |
| UL_RTOA_061564 | -16 UL_RTOA 0 | Tc |
| UL_RTOA_061565 | 0 < UL_RTOA 16 | Tc |
| UL_RTOA_061566 | 16 < UL_RTOA 32 | Tc |
| UL_RTOA_061567 | 32 < UL_RTOA 48 | Tc |
| … | … | … |
| UL_RTOA_123128 | 985008 < UL_RTOA 985024 | Tc |
| UL_RTOA_123129 | 985024 < UL_RTOA | Tc |

Table 13.1.1-6: Measurement report mapping for k=5

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_00000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_00001 | -985024 UL_RTOA < -984992 | Tc |
| UL_RTOA_00002 | -984992 UL_RTOA < -984960 | Tc |
|  |  | … |
| UL_RTOA_30781 | -64 UL_RTOA < -32 | Tc |
| UL_RTOA_30782 | -32 UL_RTOA 0 | Tc |
| UL_RTOA_30783 | 0 < UL_RTOA 32 | Tc |
| UL_RTOA_30784 | 32 < UL_RTOA 64 | Tc |
| UL_RTOA_30785 | 64 < UL_RTOA 96 | Tc |
| … | … | … |
| UL_RTOA_61564 | 984992 < UL_RTOA 985024 | Tc |
| UL_RTOA_61565 | 985024 < UL_RTOA | Tc |

Table 13.1.1-7: Measurement report mapping for k=-1

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_0000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_0000001 | -985024 UL_RTOA < -985023.5 | Tc |
| UL_RTOA_0000002 | -985023.5 UL_RTOA < -985023 | Tc |
|  |  | … |
| UL_RTOA_1970048 | -0.5 UL_RTOA < 0 | Tc |
| UL_RTOA_1970049 | 0 UL_RTOA < 0.5 | Tc |
| … | … | … |
| UL_RTOA_3940095 | 985023 UL_RTOA < 985023.5 | Tc |
| UL_RTOA_3940096 | 985023.5 UL_RTOA < 985024 | Tc |
| UL_RTOA_3940097 | 985024 UL_RTOA | Tc |

Table 13.1.1-8: Measurement report mapping for k=-2

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_0000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_0000001 | -985024 UL_RTOA < -985023.75 | Tc |
| UL_RTOA_0000002 | -985023.75 UL_RTOA < -985023.5 | Tc |
|  |  | … |
| UL_RTOA_3940096 | -0.25 UL_RTOA < 0 | Tc |
| UL_RTOA_3940097 | 0 UL_RTOA < 0.25 | Tc |
| … | … | … |
| UL_RTOA_7880191 | 985023.5 UL_RTOA < 985023.75 | Tc |
| UL_RTOA_7880192 | 985023.75 UL_RTOA < 985024 | Tc |
| UL_RTOA_7880193 | 985024 UL_RTOA | Tc |

Table 13.1.1-9: Measurement report mapping for k=-3

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_00000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_00000001 | -985024 UL_RTOA < -985023.875 | Tc |
| UL_RTOA_00000002 | -985023.875 UL_RTOA < -985023.75 | Tc |
|  |  | … |
| UL_RTOA_07880192 | -0.125 UL_RTOA < 0 | Tc |
| UL_RTOA_07880193 | 0 UL_RTOA < 0.125 | Tc |
| … | … | … |
| UL_RTOA_15760383 | 985023.75 UL_RTOA < 985023.875 | Tc |
| UL_RTOA_15760384 | 985023.875 UL_RTOA < 985024 | Tc |
| UL_RTOA_15760385 | 985024 UL_RTOA | Tc |

Table 13.1.1-10: Measurement report mapping for k=-4

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_0000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_0000001 | -985024 UL_RTOA < -985023.9375 | Tc |
| UL_RTOA_0000002 | -985023.9375 UL_RTOA < -985023.875 | Tc |
|  |  | … |
| UL_RTOA_15760384 | -0.0625 UL_RTOA < 0 | Tc |
| UL_RTOA_15760385 | 0 UL_RTOA < 0.0625 | Tc |
| … | … | … |
| UL_RTOA_31520767 | 985023.875 UL_RTOA < 985023.9375 | Tc |
| UL_RTOA_31520768 | 985023.9375 UL_RTOA < 985024 | Tc |
| UL_RTOA_31520769 | 985024 UL_RTOA | Tc |

Table 13.1.1-11: Measurement report mapping for k=-5

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_00000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_00000001 | -985024 UL_RTOA < -985023.9688 | Tc |
| UL_RTOA_00000002 | -985023.9688 UL_RTOA < -985023.9375 | Tc |
|  |  | … |
| UL_RTOA_31520768 | -0.0312 UL_RTOA < 0 | Tc |
| UL_RTOA_31520769 | 0 UL_RTOA < 0.0312 | Tc |
| … | … | … |
| UL_RTOA_63041535 | 985023.9375 UL_RTOA < 985023.9688 | Tc |
| UL_RTOA_63041536 | 985023.9688 UL_RTOA < 985024 | Tc |
| UL_RTOA_63041537 | 985024 UL_RTOA | Tc |

Table 13.1.1-12: Measurement report mapping for k=-6

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| UL_RTOA_000000000 | -985024 > UL_RTOA | Tc |
| UL_RTOA_000000001 | -985024 UL_RTOA < -985023.9844 | Tc |
| UL_RTOA_000000002 | -985023.9844 UL_RTOA < -985023.9688 | Tc |
|  |  | … |
| UL_RTOA_063041536 | -0.0156 UL_RTOA < 0 | Tc |
| UL_RTOA_063041537 | 0 UL_RTOA < 0.0156 | Tc |
| … | … | … |
| UL_RTOA_126083071 | 985023.9688 UL_RTOA < 985023.9844 | Tc |
| UL_RTOA_126083072 | 985023.9844 UL_RTOA < 985024 | Tc |
| UL_RTOA_126083073 | 985024 UL_RTOA | Tc |

### 13.1.1A	Additional Path Report Mapping for UL-RTOA

The reporting range of additional path reporting for UL Relative Time of Arrival (UL-RTOA), as defined in Clause 5.2.2 of TS 38.215 [4], is defined from -8175Tc to +8175Tc. The reporting resolution is uniform across the reporting range and is defined as T = Tc2k where k is selected by gNB from the set {-6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5}.

Tc is defined in TS 38.211 [6].

LMF provides a recommended resolution parameter, timingReportingGranularityFactor or timingReportingGranularityFactorExtended [35]. gNB selects parameter k based on timingReportingGranularityFactor or timingReportingGranularityFactorExtended [35] and informs the LMF.

The mapping of measured quantity for each reporting resolution (k) is defined in Table 13.1.1A-1 to Table 13.1.1A-12.

Table 13.1.1A-1: Measurement report mapping for k=0

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174 | Tc |
| path_00002 | -8174 path < -8173 | Tc |
|  |  | … |
| path_08175 | -1 path < 0 | Tc |
| path_08176 | 0 path < 1 | Tc |
| … | … | … |
| path_ 16349 | 8173 path < 8174 | Tc |
| path_ 16350 | 8174 path < 8175 | Tc |
| path_ 16351 | 8175 path | Tc |

Table 13.1.1A-2: Measurement report mapping for k=1

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8175 | Tc |
| path_0001 | -8175 path < -8173 | Tc |
| path_0002 | -8173 path < -8171 | Tc |
|  |  | … |
| path_4088 | -1 path < 1 | Tc |
| … | … | … |
| path_8174 | 8171 path < 8173 | Tc |
| path_8175 | 8173 path < 8175 | Tc |
| path_8176 | 8175 path | Tc |

Table 13.1.1A-3: Measurement report mapping for k=2

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8174 | Tc |
| path_0001 | -8174 path < -8170 | Tc |
| path_0002 | -8170 path < -8166 | Tc |
|  |  | … |
| path_2044 | -2 path < 2 | Tc |
| … | … | … |
| path_4086 | 8166 path < 8170 | Tc |
| path_4087 | 8170 path < 8174 | Tc |
| path_4088 | 8174 path | Tc |

Table 13.1.1A-4: Measurement report mapping for k=3

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8172 | Tc |
| path_0001 | -8172 path < -8164 | Tc |
| path_0002 | -8164 path < -8156 | Tc |
|  |  | … |
| path_1022 | -4 path < 4 | Tc |
| … | … | … |
| path_2042 | 8156 path < 8164 | Tc |
| path_2043 | 8164 path < 8172 | Tc |
| path_2044 | 8172 path | Tc |

Table 13.1.1A-5: Measurement report mapping for k=4

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8168 | Tc |
| path_0001 | -8168 path < -8152 | Tc |
| path_0002 | -8152 path < -8136 | Tc |
|  |  | … |
| path_511 | -8 path < 8 | Tc |
| … | … | … |
| path_1020 | 8136 path < 8152 | Tc |
| path_1021 | 8152 path < 8168 | Tc |
| path_1022 | 8168 path | Tc |

Table 13.1.1A-6: Measurement report mapping for k=5

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000 | path < -8160 | Tc |
| path_001 | -8160 path < -8128 | Tc |
| path_002 | -8128 path < -8096 | Tc |
|  |  | … |
| path_256 | 0 path < 32 | Tc |
| … | … | … |
| path_509 | 8096 path < 8128 | Tc |
| path_510 | 8128 path < 8160 | Tc |
| path_511 | 8160 path | Tc |

Table 13.1.1A-7: Measurement report mapping for k=-1

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174.5 | Tc |
| path_00002 | -8174.5 path < -8174 | Tc |
|  |  | … |
| path_16350 | -0.5 path < 0 | Tc |
| path_16351 | 0 path < 0.5 | Tc |
| … | … | … |
| path_32699 | 8174 path < 8174.5 | Tc |
| path_32700 | 8174.5 path < 8175 | Tc |
| path_32701 | 8175 path | Tc |

Table 13.1.1A-8: Measurement report mapping for k=-2

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174.75 | Tc |
| path_00002 | -8174.75 path < -8174.5 | Tc |
|  |  | … |
| path_32700 | -0.25 path < 0 | Tc |
| path_32701 | 0 path < 0.25 | Tc |
| … | … | … |
| path_65399 | 8174.5 path < 8174.75 | Tc |
| path_65400 | 8174.75 path < 8175 | Tc |
| path_65401 | 8175 path | Tc |

Table 13.1.1A-9: Measurement report mapping for k=-3

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000000 | path < -8175 | Tc |
| path_000001 | -8175 path < -8174.875 | Tc |
| path_000002 | -8174.875 path < -8174.75 | Tc |
|  |  | … |
| path_065400 | -0.125 path < 0 | Tc |
| path_065401 | 0 path < 0.125 | Tc |
| … | … | … |
| path_130799 | 8174.75 path < 8174.875 | Tc |
| path_130800 | 8174.875 path < 8175 | Tc |
| path_130801 | 8175 path | Tc |

Table 13.1.1A-10: Measurement report mapping for k=-4

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000000 | path < -8175 | Tc |
| path_000001 | -8175 path < -8174.9375 | Tc |
| path_000002 | -8174.9375 path < -8174.875 | Tc |
|  |  | … |
| path_130800 | -0.0625 path < 0 | Tc |
| path_130801 | 0 path < 0.0625 | Tc |
| … | … | … |
| path_261599 | 8174.875 path < 8174.9375 | Tc |
| path_261600 | 8174.9375 path < 8175 | Tc |
| path_261601 | 8175 path | Tc |

Table 13.1.1A-11: Measurement report mapping for k=-5

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000000 | path < -8175 | Tc |
| path_000001 | -8175 path < -8174.9688 | Tc |
| path_000002 | -8174.9688 path < -8174.9375 | Tc |
|  |  | … |
| path_261600 | -0.0312 path < 0 | Tc |
| path_261601 | 0 path < 0.0312 | Tc |
| … | … | … |
| path_523199 | 8174.9375 path < 8174.9688 | Tc |
| path_523200 | 8174.9688 path < 8175 | Tc |
| path_523201 | 8175 path | Tc |

Table 13.1.1A-12: Measurement report mapping for k=-6

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000000 | path < -8175 | Tc |
| path_0000001 | -8175 path < -8174.9844 | Tc |
| path_0000002 | -8174.9844 path < -8174.9688 | Tc |
|  |  | … |
| path_0523200 | -0.0156 path < 0 | Tc |
| path_0523201 | 0 path < 0.0156 | Tc |
| … | … | … |
| path_1046399 | 8174.9688 path < 8174.9844 | Tc |
| path_1046400 | 8174.9844 path < 8175 | Tc |
| path_1046401 | 8175 path | Tc |

## END OF CHANGE 4

## START OF CHANGE 5

## 13.2	gNB Rx-Tx time difference

### 13.2.1	Report mapping

The reporting range of gNB Rx-Tx time difference, as defined in Clause 5.2.3 of TS 38.215 [4], is defined from -985024Tc to +985024Tc. The reporting resolution is uniform across the reporting range and is defined as T = Tc2k where k is selected by gNB from the set {-6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5}.

Tc is defined in TS 38.211 [6].

LMF provides a recommended resolution parameter, timingReportingGranularityFactor or timingReportingGranularityFactorExtended [35]. gNB selects parameter k based on timingReportingGranularityFactor or timingReportingGranularityFactorExtended [35] and informs the LMF.

The mapping of measured quantity for each reporting resolution (k) is defined in table 13.2.1-1 to table 13.2.1-12.

Table 13.2.1-1: Measurement report mapping for k=0

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_0000000 | -985024 > RX-TX | Tc |
| RX-TX_0000001 | -985024 RX-TX < -985023 | Tc |
| RX-TX_0000002 | -985023 RX-TX < -985022 | Tc |
|  |  | … |
| RX-TX_0985023 | -2 RX-TX < -1 | Tc |
| RX-TX_0985024 | -1 RX-TX 0 | Tc |
| RX-TX_0985025 | 0 < RX-TX 1 | Tc |
| RX-TX_0985026 | 1 < RX-TX 2 | Tc |
| RX-TX_0985027 | 2 < RX-TX 3 | Tc |
| … | … | … |
| RX-TX_1970048 | 985023 < RX-TX 985024 | Tc |
| RX-TX_1970049 | 985024 < RX-TX | Tc |

Table 13.2.1-2: Measurement report mapping for k=1

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_000000 | -985024 > RX-TX | Tc |
| RX-TX_000001 | -985024 RX-TX < -985022 | Tc |
| RX-TX_000002 | -985022 RX-TX < -985020 | Tc |
|  |  | … |
| RX-TX_492511 | -4 RX-TX < -2 | Tc |
| RX-TX_492512 | -2 RX-TX 0 | Tc |
| RX-TX_492513 | 0 < RX-TX 2 | Tc |
| RX-TX_492514 | 2 < RX-TX 4 | Tc |
| RX-TX_492515 | 4 < RX-TX 6 | Tc |
| … | … | … |
| RX-TX_985024 | 985022 < RX-TX 985024 | Tc |
| RX-TX_985025 | 985024 < RX-TX | Tc |

Table 13.2.1-3: Measurement report mapping for k=1

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_000000 | -985024 > RX-TX | Tc |
| RX-TX_000001 | -985024 RX-TX < -985020 | Tc |
| RX-TX_000002 | -985020 RX-TX < -985018 | Tc |
|  |  | … |
| RX-TX_246255 | -8 RX-TX < -4 | Tc |
| RX-TX_246256 | -4 RX-TX 0 | Tc |
| RX-TX_246257 | 0 < RX-TX 4 | Tc |
| RX-TX_246258 | 4 < RX-TX 8 | Tc |
| RX-TX_246259 | 8 < RX-TX 12 | Tc |
| … | … | … |
| RX-TX_492512 | 985020 < RX-TX 985024 | Tc |
| RX-TX_492513 | 985024 < RX-TX | Tc |

Table 13.2.1-4: Measurement report mapping for k=3

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_0000 | -985024 > RX-TX | Tc |
| RX-TX_0001 | -985024 RX-TX < -985016 | Tc |
| RX-TX_0002 | -985016 RX-TX < -985008 | Tc |
|  |  | … |
| RX-TX_123127 | -16 RX-TX < -8 | Tc |
| RX-TX_123128 | -8 RX-TX 0 | Tc |
| RX-TX_123129 | 0 < RX-TX 8 | Tc |
| RX-TX_123130 | 8 < RX-TX 16 | Tc |
| RX-TX_123131 | 16 < RX-TX 24 | Tc |
| … | … | … |
| RX-TX_246256 | 985016 < RX-TX 985024 | Tc |
| RX-TX_246257 | 985024 < RX-TX | Tc |

Table 13.2.1-5: Measurement report mapping for k=4

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_000000 | -985024 > RX-TX | Tc |
| RX-TX_000001 | -985024 RX-TX < -985008 | Tc |
| RX-TX_000002 | -985008 RX-TX < -984992 | Tc |
|  |  | … |
| RX-TX_061563 | -32 RX-TX < -16 | Tc |
| RX-TX_061564 | -16 RX-TX 0 | Tc |
| RX-TX_061565 | 0 < RX-TX 16 | Tc |
| RX-TX_061566 | 16 < RX-TX 32 | Tc |
| RX-TX_061567 | 32 < RX-TX 48 | Tc |
| … | … | … |
| RX-TX_123128 | 985008 < RX-TX 985024 | Tc |
| RX-TX_123129 | 985024 < RX-TX | Tc |

Table 13.2.1-6: Measurement report mapping for k=5

| Reported Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_00000 | -985024 > RX-TX | Tc |
| RX-TX_00001 | -985024 RX-TX < -984992 | Tc |
| RX-TX_00002 | -984992 RX-TX < -984960 | Tc |
|  |  | … |
| RX-TX_30781 | -64 RX-TX < -32 | Tc |
| RX-TX_30782 | -32 RX-TX 0 | Tc |
| RX-TX_30783 | 0 < RX-TX 32 | Tc |
| RX-TX_30784 | 32 < RX-TX 64 | Tc |
| RX-TX_30785 | 64 < RX-TX 96 | Tc |
| … | … | … |
| RX-TX_61564 | 984992 < RX-TX 985024 | Tc |
| RX-TX_61565 | 985024 < RX-TX | Tc |

Table 13.2.1-7: Measurement report mapping for k=-1

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_0000000 | -985024 > RX-TX | Tc |
| RX-TX_0000001 | -985024  RX-TX < -985023.5 | Tc |
| RX-TX_0000002 | -985023.5  RX-TX < -985023 | Tc |
|  |  | … |
| RX-TX_1970048 | -0.5  RX-TX < 0 | Tc |
| RX-TX_1970049 | 0  RX-TX < 0.5 | Tc |
| … | … | … |
| RX-TX_3940095 | 985023  RX-TX < 985023.5 | Tc |
| RX-TX_3940096 | 985023.5  RX-TX < 985024 | Tc |
| RX-TX_3940097 | 985024  RX-TX | Tc |

Table 13.2.1-8: Measurement report mapping for k=-2

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_0000000 | -985024 > RX-TX | Tc |
| RX-TX_0000001 | -985024  RX-TX < -985023.75 | Tc |
| RX-TX_0000002 | -985023.75  RX-TX -985023.5 | Tc |
|  |  | … |
| RX-TX_3940096 | -0.25  RX-TX < 0 | Tc |
| RX-TX_3940097 | 0  RX-TX < 0.25 | Tc |
| … | … | … |
| RX-TX_7880191 | 985023.5  RX-TX < 985023.75 | Tc |
| RX-TX_7880192 | 985023.75  RX-TX < 985024 | Tc |
| RX-TX_7880193 | 985024  RX-TX | Tc |

Table 13.2.1-9: Measurement report mapping for k=-3

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_0000000 | -985024 > RX-TX | Tc |
| RX-TX_0000001 | -985024  RX-TX < -985023.875 | Tc |
| RX-TX_0000002 | -985023.875  RX-TX -985023.75 | Tc |
|  |  | … |
| RX-TX_7880192 | -0.125  RX-TX < 0 | Tc |
| RX-TX_7880193 | 0  RX-TX < 0.125 | Tc |
| … | … | … |
| RX-TX_15760383 | 985023.75  RX-TX < 985023.875 | Tc |
| RX-TX_15760384 | 985023.875  RX-TX < 985024 | Tc |
| RX-TX_15760385 | 985024  RX-TX | Tc |

Table 13.2.1-10: Measurement report mapping for k=-4

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_0000000 | -985024 > RX-TX | Tc |
| RX-TX_0000001 | -985024  RX-TX < -985023.9375 | Tc |
| RX-TX_0000002 | -985023.9375  RX-TX -985023.875 | Tc |
|  |  | … |
| RX-TX_15760384 | -0.0625  RX-TX < 0 | Tc |
| RX-TX_15760385 | 0  RX-TX < 0.0625 | Tc |
| … | … | … |
| RX-TX_31520767 | 985023.875  RX-TX < 985023.9375 | Tc |
| RX-TX_31520768 | 985023.9375  RX-TX < 985024 | Tc |
| RX-TX_31520769 | 985024  RX-TX | Tc |

Table 13.2.1-11: Measurement report mapping for k=-5

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_00000000 | -985024 > RX-TX | Tc |
| RX-TX_00000001 | -985024  RX-TX < -985023.9688 | Tc |
| RX-TX_00000002 | -985023.9688  RX-TX -985023.9375 | Tc |
|  |  | … |
| RX-TX_31520768 | -0.0312  RX-TX < 0 | Tc |
| RX-TX_31520769 | 0  RX-TX < 0.0312 | Tc |
| … | … | … |
| RX-TX_63041535 | 985023.9375  RX-TX < 985023.9688 | Tc |
| RX-TX_63041536 | 985023.9688  RX-TX < 985024 | Tc |
| RX-TX_63041537 | 985024  RX-TX | Tc |

Table 13.2.1-12: Measurement report mapping for k=-6

| Reported Quantity Value | Measured Quantity Value | Unit |
| --- | --- | --- |
| RX-TX_00000000 | -985024 > RX-TX | Tc |
| RX-TX_00000001 | -985024  RX-TX < -985023.9844 | Tc |
| RX-TX_00000002 | -985023.9844  RX-TX -985023.9688 | Tc |
|  |  | … |
| RX-TX_63041536 | -0.0156  RX-TX < 0 | Tc |
| RX-TX_63041537 | 0  RX-TX < 0.0156 | Tc |
| … | … | … |
| RX-TX_126083071 | 985023.9688  RX-TX < 985023.9844 | Tc |
| RX-TX_126083072 | 985023.9844  RX-TX < 985024 | Tc |
| RX-TX_126083073 | 985024  RX-TX | Tc |

### 13.2.1A	Additional Path Report Mapping for gNB Rx-Tx

The reporting range of additional path for gNB Rx-Tx time difference, as defined in Clause 5.2.3 of TS 38.215 [4], is defined from -8175Tc to 8175Tc. The reporting resolution is uniform across the reporting range and is defined as T = Tc2k where k is selected by gNB from the set {-6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5}.

Tc is defined in TS 38.211 [6].

LMF provides a recommended resolution parameter, timingReportingGranularityFactor or timingReportingGranularityFactorExtended [35]. gNB selects parameter k based on timingReportingGranularityFactor or timingReportingGranularityFactorExtended [35] and informs the LMF.

The mapping of measured quantity for each reporting resolution (k) is defined in table 13.2.1A-1 to table 13.2.1A-12.

Table 13.2.1A-1: Measurement report mapping for k=0

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174 | Tc |
| path_00002 | -8174 path < -8173 | Tc |
|  |  | … |
| path_08175 | -1 path < 0 | Tc |
| path_08176 | 0 path < 1 | Tc |
| … | … | … |
| path_ 16349 | 8173 path < 8174 | Tc |
| path_ 16350 | 8174 path < 8175 | Tc |
| path_ 16351 | 8175 path | Tc |

Table 13.2.1A-2: Measurement report mapping for k=1

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8175 | Tc |
| path_0001 | -8175 path < -8173 | Tc |
| path_0002 | -8173 path < -8171 | Tc |
|  |  | … |
| path_4088 | -1 path < 1 | Tc |
| … | … | … |
| path_8174 | 8171 path < 8173 | Tc |
| path_8175 | 8173 path < 8175 | Tc |
| path_8176 | 8175 path | Tc |

Table 13.2.1A-3: Measurement report mapping for k=2

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8174 | Tc |
| path_0001 | -8174 path < -8170 | Tc |
| path_0002 | -8170 path < -8166 | Tc |
|  |  | … |
| path_2044 | -2 path < 2 | Tc |
| … | … | … |
| path_4086 | 8166 path < 8170 | Tc |
| path_4087 | 8170 path < 8174 | Tc |
| path_4088 | 8174 path | Tc |

Table 13.2.1A-4: Measurement report mapping for k=3

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8172 | Tc |
| path_0001 | -8172 path < -8164 | Tc |
| path_0002 | -8164 path < -8156 | Tc |
|  |  | … |
| path_1022 | -4 path < 4 | Tc |
| … | … | … |
| path_2042 | 8156 path < 8164 | Tc |
| path_2043 | 8164 path < 8172 | Tc |
| path_2044 | 8172 path | Tc |

Table 13.2.1A-5: Measurement report mapping for k=4

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_0000 | path < -8168 | Tc |
| path_0001 | -8168 path < -8152 | Tc |
| path_0002 | -8152 path < -8136 | Tc |
|  |  | … |
| path_511 | -8 path < 8 | Tc |
| … | … | … |
| path_1020 | 8136 path < 8152 | Tc |
| path_1021 | 8152 path < 8168 | Tc |
| path_1022 | 8168 path | Tc |

Table 13.2.1A-6: Measurement report mapping for k=5

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000 | path < -8160 | Tc |
| path_001 | -8160 path < -8128 | Tc |
| path_002 | -8128 path < -8096 | Tc |
|  |  | … |
| path_256 | 0 path < 32 | Tc |
| … | … | … |
| path_509 | 8096 path < 8128 | Tc |
| path_510 | 8128 path < 8160 | Tc |
| path_511 | 8160 path | Tc |

Table 13.2.1A-7: Measurement report mapping for k=-1

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174.5 | Tc |
| path_00002 | -8174.5 path < -8174 | Tc |
|  |  | … |
| path_16350 | -0.5 path < 0 | Tc |
| path_16351 | 0 path < 0.5 | Tc |
| … | … | … |
| path_32699 | 8174 path < 8174.5 | Tc |
| path_32700 | 8174.5 path < 8175 | Tc |
| path_32701 | 8175 path | Tc |

Table 13.2.1A-8: Measurement report mapping for k=-2

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_00000 | path < -8175 | Tc |
| path_00001 | -8175 path < -8174.75 | Tc |
| path_00002 | -8174.75 path < -8174.5 | Tc |
|  |  | … |
| path_32700 | -0.25 path < 0 | Tc |
| path_32701 | 0 path < 0.25 | Tc |
| … | … | … |
| path_65399 | 8174.5 path < 8174.75 | Tc |
| path_65400 | 8174.75 path < 8175 | Tc |
| path_65401 | 8175 path | Tc |

Table 13.2.1A-9: Measurement report mapping for k=-3

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000000 | path < -8175 | Tc |
| path_000001 | -8175 path < -8174.875 | Tc |
| path_000002 | -8174.875 path < -8174.75 | Tc |
|  |  | … |
| path_065400 | -0.125 path < 0 | Tc |
| path_065401 | 0 path < 0.125 | Tc |
| … | … | … |
| path_130799 | 8174.75 path < 8174.875 | Tc |
| path_130800 | 8174.875 path < 8175 | Tc |
| path_130801 | 8175 path | Tc |

Table 13.2.1A-10: Measurement report mapping for k=-4

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000000 | path < -8175 | Tc |
| path_000001 | -8175 path < -8174.9375 | Tc |
| path_000002 | -8174.9375 path < -8174.875 | Tc |
|  |  | … |
| path_130800 | -0.0625 path < 0 | Tc |
| path_130801 | 0 path < 0.0625 | Tc |
| … | … | … |
| path_261599 | 8174.875 path < 8174.9375 | Tc |
| path_261600 | 8174.9375 path < 8175 | Tc |
| path_261601 | 8175 path | Tc |

Table 13.2.1A-11: Measurement report mapping for k=-5

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000000 | path < -8175 | Tc |
| path_000001 | -8175 path < -8174.9688 | Tc |
| path_000002 | -8174.9688 path < -8174.9375 | Tc |
|  |  | … |
| path_261600 | -0.0312 path < 0 | Tc |
| path_261601 | 0 path < 0.0312 | Tc |
| … | … | … |
| path_523199 | 8174.9375 path < 8174.9688 | Tc |
| path_523200 | 8174.9688 path < 8175 | Tc |
| path_523201 | 8175 path | Tc |

Table 13.2.1A-12: Measurement report mapping for k=-6

| Reported Quantity Value, path_i | Measured Quantity Value, path | Unit |
| --- | --- | --- |
| path_000000 | path < -8175 | Tc |
| path_000001 | -8175 path < -8174.9844 | Tc |
| path_000002 | -8174.9844 path < -8174.9688 | Tc |
|  |  | … |
| path_523200 | -0.0156 path < 0 | Tc |
| path_523201 | 0 path < 0.0156 | Tc |
| … | … | … |
| path_1046399 | 8174.9688 path < 8174.9844 | Tc |
| path_1046400 | 8174.9844 path < 8175 | Tc |
| path_1046401 | 8175 path | Tc |

## END OF CHANGE 5