3GPP TSG-WG4 Meeting # 116	R4-2512287

# Bangaluru, India  Aug. 25th – Aug. 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft CR | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on CJT calibration reporting for Delay offset, Frequency offset |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Core |  |  |  |  |  | Date: |  |  | 2025-08-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Core requirements are agreed to be supported for the CJTC reporting. The requirement needs to be added to spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following sections are added - Introduction - Requirements applicability - Measurement Reporting Requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec. is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.x, 9.x.1, 9.x.2, 9.x.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4- 2510704 |  |  |  |  |  |  |  |  |

---------------------- Start of changes ---------------------------

## 9.X	CJT calibration reporting for Delay offset and Frequency offset

### 9.X.1	Introduction

When configured with a CSI-ReportConfig where reportQuantity is set to 'CJTC-Dd', or ' CJTC -F', the UE shall perform CJT calibration measurements for delay offset, and frequency offset, respectively, on the configured CSI-RS resources. The UE shall measure all CSI-RS resources indicated by the higher layer parameter resourcesForChannelMeasurement.

For CJT reporting types (CJTC-Dd, CJTC-F), the UE shall be capable of measuring NTRP ∈ {2, 3, 4} CSI-RS Resource Sets within a single CSI Resource Setting, all of which shall be associated with the higher layer parameter trs-Info.

The UE shall report the configured measurement quantity and transmit aperiodic CSI reports according to the CSI-ReportConfig for the active BWP.

### 9.X.2	Requirements applicability

The requirements in clause 9 apply, provided:

-	The CSI-RS resources configured for CJTC measurements are measurable.

A CSI-RS resource configured for CJTC report shall be considered measurable when for each relevant CSI-RS the following conditions are met:

-	CJTC reporting related side conditions given in clauses B,

-	CSI-RS Ês/Iot is greater than or equal to the level described in clause B.

CJTC requirements are defined for periodic measurement resources and aperiodic reporting resources.

### 9.X.3	Measurement Reporting Requirements

The UE shall send CJTC reports only for report configurations configured for the active BWP.

The UE shall report the quantised CJTC value as a n-bit value ranging from 0 to 2n-1 with the step size as described in clause 10; where the n is determined based on higher layer parameter.

#### 9.X.3.1	Aperiodic Reporting

Reported CJTC measurements contained in aperiodic triggered but measured over periodic resources shall meet the accuracy requirements specified in clause 10.

The UE shall only send aperiodic CJTC measurement reports, if a DCI trigger has been received.

After the UE receives CSI request in DCI with (reportQuantity set to ‘ cjtc-Dd’, ‘ cjtc-F’), the UE shall transmit the aperiodic CJTC reporting on PUSCH over the air interface at the time specified according to relevant clause in [26].

--------------------END OF CHANGES--------------------------