3GPP TSG-RAN Working Group 4 (Radio) Meeting #116-bis	R4-2514109

# Prague, Czech Republic, 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.105 | draftCR | xxxx | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 37.105: Simplification of tables for additional requirements for co-existence and co-location |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-10-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN plenary tasked RAN4 to propose a shorter format of the co-existence and co-location requirements (RAN#106 meeting, see RP-243288). The CR scope is to reduce the size of the requirement tables for additional transmitter spurious emission and receiver blocking requirements for co-existence and co-location. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The tables are restructured to include the general requirement level and exceptions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Very long tables that need maintenance over many BS specifications every time a new band is introduced or a correction is made. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.7.6.3.3, 9.7.6.3.4, 9.7.6.4.3, 9.7.6.4.4, 10.6.2.2, 10.6.3, 10.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------Start change---

##### 9.7.6.3.3	Additional spurious emissions requirements

The TRP of any spurious emission shall not exceed the limits of table 9.7.6.3.3-1 for a AAS BS where requirements for co-existence with the system listed in the first column apply. For a multi-band RIB, the exclusions and conditions in the notes column of table 9.7.6.3.3-1 apply for each supported operating band.

Table 9.7.6.3.3-1 OTA AAS BS Spurious emissions limits for UTRA FDD BS in geographic coverage area of systems operating in other frequency bands

| System type to co-exist with | Frequency range for co-existence requirement | Basic limits (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| GSM850 or CDMA850 | 869 - 894 MHz | -51 | 100kHz | NOTE 1 |
|  | 824 - 849 MHz | -55 |  |  |
| GSM900 | 921 - 960 MHz | -51 |  |  |
|  | 876 - 915 MHz | -55 |  |  |
| DCS1800 | 1805 - 1880 MHz | -41 |  |  |
|  | 1710 - 1785 MHz | -55 |  |  |
| PCS1900 | 1930 - 1990 MHz | -41 |  |  |
|  | 1850 - 1910 MHz | -55 |  |  |
| Other system type | Frequency range of downlink operating band of the BS to co-exist with (NOTE 7, 8) | -52 | 1MHz | NOTE 1 |
|  | Frequency range of uplink operating band of the BS to co-exist with (NOTE 8) | -49 |  | NOTE 1, 4, 5, 6 |

NOTE 1:	As defined in the scope for spurious emissions in this clause, except for the cases where the noted requirements apply to a BS operating in Band n28, the co-existence requirements in table 9.7.6.3.3-1 do not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see TS 37.104 [5], clause 4.5). Emission limits for this excluded frequency range may be covered by local or regional requirements.

NOTE 2:	Table 9.7.6.3.3-1  assumes that two operating bands, where the frequency ranges in TS 37.104 [5], clause 4.5, would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	TDD base stations deployed in the same geographical area, that are synchronized and use the same or adjacent operating bands can transmit without additional co-existence requirements. For unsynchronized base stations, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 4:	For Band 28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with E-UTRA Band 27 UL operating band, where requirement applies 4 MHz above the Band n28 downlink operating band.

NOTE 5:	For Band 29 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with UTRA Band XII, E-UTRA Band 12 or NR Band n12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL or NR Band n85 UL operating band, where requirement applies 1 MHz below the Band n29 downlink operating band.

NOTE 6:	For Band 67 BS, specific solutions may be required to fulfil the spurious emissions limits for BS co-existence with E-UTRA Band 28 or NR Band n28 UL operating band or NR Band n83 UL operating band, where requirement applies for 703 MHz to 736 MHz.

NOTE 7:	Does not apply for co-existence with standalone downlink bands (SDO) defined in TS 36.104, table 5.5-1.

NOTE 8:	Frequency range of UTRA, E-UTRA and NR bands, as described in as described in TS 25.104, TS 36.104 and TS 38.104.

|  | - |  |  |  |
| --- | --- | --- | --- | --- |
|  | - |  |  |  |
|  | - |  |  |  |
|  | - |  |  |  |
|  | - |  |  |  |

The following requirement may be applied for the protection of PHS in geographic areas in which both PHS and UTRA FDD are deployed. This requirement is also applicable at specified frequencies falling between 12.5MHz below the first carrier frequency used and 12.5MHz above the last carrier frequency used.

The TRP of any spurious emission shall not exceed:

Table 9.7.6.3.3-2: AAS BS OTA Spurious emissions limits for BS in geographic coverage area of PHS

| Band | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 1884.5 - 1915.7 MHz | -35 dBm | 300 kHz |  |

Table 9.7.6.3.3-3: Void

NOTE:	This requirement for the frequency range 2610-2615 MHz may be applied to geographic areas in which both UTRA-TDD and UTRA-FDD are deployed.

The following requirement shall be applied to AAS BS operating in Bands XIII and XIV to ensure that appropriate interference protection is provided to 700 MHz public safety operations. This requirement is also applicable at specified frequencies falling between 12.5 MHz below the first carrier frequency used and 12.5 MHz above the last carrier frequency used.

The TRP of any spurious emission shall not exceed:

Table 9.7.6.3.3-4: AAS BS OTA Spurious emissions limits

| Operating Band | Band | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| XIII | 763 - 775 MHz | -40 dBm | 6.25 kHz |  |
| XIII | 793 - 805 MHz | -40 dBm | 6.25 kHz |  |
| XIV | 769 - 775 MHz | -40 dBm | 6.25 kHz |  |
| XIV | 799 - 805 MHz | -40 dBm | 6.25 kHz |  |

The following requirement shall be applied to AAS BS operating in Bands XXVI to ensure that appropriate interference protection is provided to 800 MHz public safety operations. This requirement is also applicable at specified frequencies falling between 12.5 MHz below the first carrier frequency used and 12.5 MHz above the last carrier frequency used.

The TRP of any spurious emission shall not exceed:

Table 9.7.6.3.3-5: AAS BS OTA Spurious emissions limits

| Operating Band | Band | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| XXVI | 851 - 859 MHz | -7 dBm | 100 kHz | Applicable for offsets > 37.5kHz from the channel edge |

##### 9.7.6.3.4	Co-location with other base stations

###### 9.7.6.3.4.1	General

These requirements may be applied for the protection of other BS receivers when GSM900, DCS1800, PCS1900, GSM850, CDMA850, UTRA FDD, UTRA TDD, E-UTRA BS and/or NR BS are co-located with a BS.

The requirements assume with base stations of the same class.

NOTE:	For co-location with UTRA, the requirements are based on co-location with UTRA FDD or TDD base stations.

The requirements are co-location emission requirements are specified as the power sum of the supported polarization(s) at the co-location reference antenna conducted output(s).

###### 9.7.6.3.4.2	Minimum Requirement

The output of the co-location reference antenna of any spurious emission shall not exceed the limits of table 9.7.6.3.4.2-1 for a AAS BS where requirements for co-location with a BS type listed in the first column apply, depending on the declared Base Station class. For a multi-band RIB, the exclusions and conditions in the Notes column of table 9.7.6.3.4.2-1 apply for each supported operating band.

Table 9.7.6.3.4.2-1: UTRA AAS BS OTA Spurious emissions limits for AAS BS co-located with another BS

| Frequency range of uplink operating band of the | System type to co-locate with | Basic limits (dBm/100kHz) |  |  | Notes |
| --- | --- | --- | --- | --- | --- |
| co-located BS |  | WA BS | MR BS | LA BS |  |
| 824 - 849 MHz | GSM850 or CDMA850 | -98 | -91 | -70 | NOTE 1 |
| 876 - 915 MHz | GSM900 | -98 | -91 | -70 |  |
| 1710 - 1785 MHz | DCS1800 | -98 | -91 | -80 |  |
| 1850 - 1910 MHz | PCS1900 | -98 | -91 | -80 |  |
| UL frequency range of n51, n91, n93 | NR | N/A | N/A | -88 |  |
| UL frequency range of n53 | NR | N/A | -91 | -88 |  |
| UL frequency range of n100, n101 | NR | -96 | N/A | N/A |  |
| UL frequency range of n96, n102 | NR | N/A | -90 | -87 |  |
| UL frequency range of n104 | NR | -95 | -90 | -87 |  |
| Frequency range of uplink operating band of the co-located BS | Other (NOTE 4, 5) | -96 | -91 | -88 |  |

NOTE 1:	As defined in the scope for spurious emissions in this subclause, the co-location requirements in table 9.7.6.3.4.2-1 do not apply for the 10 MHz frequency range immediately outside the BS transmit frequency range of a downlink operating band (see subclause 9.7.1). The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30 dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [12].

NOTE 2:	Table 9.7.6.3.4.2-1 assumes that two operating bands, where the corresponding BS transmit and receive frequency ranges in subclause 9.7.1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-location requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	Co-located TDD base stations that are synchronized and using the same or adjacent operating band can transmit without special co-locations requirements. For unsynchronized base stations, special co-location requirements may apply that are not covered by the 3GPP specifications.

NOTE 4:	Does not apply for co-location with V2X operation defined in TS 36.104, table 5.5-1.

NOTE 5:	UTRA, E-UTRA, NR as described in TS 25.104, TS 36.104 and TS 38.104.

----------- end of change -------

----------- start of change ----- OTA SINGLE RAT E-UTRA--

##### 9.7.6.4.3	Additional spurious emissions requirements

###### 9.7.6.4.3.1	General

These requirements may be applied for the protection of system operating in frequency ranges other than the BS downlink operating band. The limits may apply as an optional protection of such systems that are deployed in the same geographical area as the BS, or they may be set by local or regional regulation as a mandatory requirement for an operating band. It is in some cases not stated in the present document whether a requirement is mandatory or under what exact circumstances that a limit applies, since this is set by local or regional regulation. An overview of regional requirements in the present document is given in subclause 4.5.

Some requirements may apply for the protection of specific equipment (UE, MS and/or BS) or equipment operating in specific systems (GSM/EDGE, CDMA, UTRA, E-UTRA, NR, etc.) as listed below.

All additional spurious requirements are TRP unless otherwise stated.

###### 9.7.6.4.3.2	Minimum Requirement

The TRP of any spurious emission shall not exceed the limits of table 9.7.6.4.3.2-1 for an AAS BS where requirements for co-existence with the frequency rande and system identified in the first two columns apply. For a multi-band RIB, the exclusions and conditions in the notes column of table 9.7.6.4.3.2-1 apply for each supported operating band.

Table 9.7.6.4.3.2-1: AAS BS OTA Spurious emissions limits for co-existence with systems operating in other frequency bands

|  | - |  |  |  |
| --- | --- | --- | --- | --- |
|  | - |  |  |  |
|  | - |  |  |  |
|  | - |  |  |  |
|  | - |  |  |  |
|  |  |  |  | - |

| System type to co-exist with | Frequency range for co-existence requirement | Basic limits (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| GSM850 or CDMA850 | 869 - 894 MHz | -48 | 100kHz | NOTE 1, 3 |
|  | 824 - 849 MHz | -52 |  |  |
| GSM900 | 921 - 960 MHz | -48 |  |  |
|  | 876 - 915 MHz | -52 |  |  |
| DCS1800 | 1805 - 1880 MHz | -38 |  |  |
|  | 1710 - 1785 MHz | -52 |  |  |
| PCS1900 | 1930 - 1990 MHz | -38 |  |  |
|  | 1850 - 1910 MHz | -52 |  |  |
| Other system type | Frequency range of downlink operating band of the BS to co-exist with (NOTE 8, 9) | -43 | 1MHz | NOTE 1, 3 |
|  | Frequency range of uplink operating band of the BS to co-exist with (NOTE 9) | -40 |  | NOTE 1, 3, 5, 6, 7 |

NOTE 1:	As defined in the scope for spurious emissions in this subclause, except for the cases where the noted requirements apply to a BS operating in Band 25, Band 27, Band 28 or Band 29, the co-existence requirements in table 9.7.6.2.3.2-1 do not apply for the ΔfOBUE frequency range immediately outside the downlink operating band (see subclause 9.7.1). Emission limits for this excluded frequency range may be covered by local or regional requirements.

NOTE 2:	Table 9.7.6.2.3.2-1 assumes that two operating bands, where the frequency ranges in subclause 9.7.1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	For the protection of DCS1800, UTRA Band III or E-UTRA Band 3 in China, the frequency ranges of the downlink and uplink protection requirements are 1805 – 1850 MHz and 1710 – 1755 MHz respectively.

NOTE 4:	TDD base stations deployed in the same geographical area, that are synchronized and use the same or adjacent operating bands can transmit without additional co-existence requirements. For unsynchronized base stations (except in Band 46, or in Band 49), special co-existence requirements may apply that are not covered by the 3GPP specifications.

NOTE 6:	For Band 28 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with Band 27 UL operating band, where requirement applies 4 MHz above the Band n28 downlink operating band..

NOTE 7:	For Band 29 BS, specific solutions may be required to fulfil the spurious emissions limits for BS for co-existence with UTRA Band XII or E-UTRA Band 12 UL operating band, E-UTRA Band 17 UL operating band or E-UTRA Band 85 UL operating band, where requirement applies 1 MHz below the Band 29 downlink operating band.

NOTE 7:	For NR Band n67 BS, specific solutions may be required to fulfil the spurious emissions limits for NR BS co-existence with E-UTRA Band 28 or NR Band n28 UL operating band or NR Band n83 UL operating band, where requirement applies for 703 MHz to 736 MHz.

NOTE 8:	Does not apply for co-existence with standalone downlink bands (SDO) defined in TS 36.104, table 5.5-1.

NOTE 9:	Frequency range of UTRA, E-UTRA and NR bands, as described in TS 25.104, TS 36.104 and TS 38.104.

The following requirement may be applied for the protection of PHS. This requirement is also applicable at specified frequencies falling between ΔfOBUE below the lowest BS transmitter frequency of the downlink operating band and ΔfOBUE above the highest BS transmitter frequency of the downlink operating band.

The TRP of any spurious emission shall not exceed:

Table 9.7.6.4.3.2-2: AAS BS OTA Spurious emissions limits for BS for co-existence with PHS

| Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 1884.5 - 1915.7 MHz | -32 dBm | 300 kHz | Applicable for co-existence with PHS system operating in 1884.5-1915.7MHz |
| NOTE: The requirement is not applicable in China. |  |  |  |

The following requirement shall be applied to AAS BS operating in Bands 13 and 14 to ensure that appropriate interference protection is provided to 700 MHz public safety operations. This requirement is also applicable at the frequency range from ΔfOBUE below the lowest frequency of the BS downlink operating band up to ΔfOBUE above the highest frequency of the BS downlink operating band.

The TRP of any spurious emission shall not exceed:

Table 9.7.6.4.3.2-3: AAS BS OTA Spurious emissions limits for protection of 700 MHz public safety operations

| Operating Band | Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 13 | 763 - 775 MHz | -37 dBm | 6.25 kHz |  |
| 13 | 793 - 805 MHz | -37 dBm | 6.25 kHz |  |
| 14 | 769 - 775 MHz | -37 dBm | 6.25 kHz |  |
| 14 | 799 - 805 MHz | -37 dBm | 6.25 kHz |  |

The following requirement shall be applied to AAS BS operating in Band 26 to ensure that appropriate interference protection is provided to 800 MHz public safety operations. This requirement is also applicable at the frequency range from ΔfOBUE below the lowest frequency of the BS downlink operating band up to ΔfOBUE above the highest frequency of the BS downlink operating band.

The TRP of any spurious emission shall not exceed:

Table 9.7.6.4.3.2-4: AAS BS OTA Spurious emissions limits for protection of 800 MHz public safety operations

| Operating Band | Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 26 | 851 - 859 MHz | -4 dBm | 100 kHz | Applicable for offsets > 37.5kHz from the channel edge |

Table 9.7.6.4.3.2-5: Void

The following requirement may apply to AAS BS operating in Band 30 in certain regions. This requirement is also applicable at the frequency range from ΔfOBUE below the lowest frequency of the BS downlink operating band up to ΔfOBUE above the highest frequency of the BS downlink operating band.

The TRP of any spurious emission shall not exceed:

Table 9.7.6.4.3.2-6: Additional AAS BS OTA Spurious emissions limits for Band 30

| Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 2200MHz – 2345MHz | -36 dBm | 1 MHz |  |
| 2362.5MHz – 2365MHz | -16 dBm | 1 MHz |  |
| 2365MHz – 2367.5MHz | -31 dBm | 1 MHz |  |
| 2367.5MHz – 2370MHz | -33 dBm | 1 MHz |  |
| 2370MHz – 2395MHz | -36 dBm | 1 MHz |  |

The following requirement may apply to AAS BS operating in Band 48 in certain regions. The TRP of any spurious emission shall not exceed:

Table 9.7.6.4.3.2-7: Additional AAS BS OTA Spurious emissions limits for Band 48

| Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- |
| 3530MHz – 3720MHz | -16 dBm | 1 MHz | Applicable 10MHz from the assigned channel edge |
| 3100MHz – 3530MHz 3720MHz – 4200MHz | -31 dBm | 1 MHz |  |

In addition to the requirements in subclauses 9.7.6.2.1, 9.7.6.2.2 and above in the present subclause, the AAS BS may have to comply with the applicable emission limits established by FCC Title 47 [15], when deployed in regions where those limits are applied, and under the conditions declared by the manufacturer.

Table 9.7.6.4.3.2-8: Void

The following requirement shall be applied to AAS BS operating in Bands 13 and 14 to ensure that appropriate interference protection is provided to 700 MHz public safety operations. This requirement is also applicable at the frequency range from ΔfOBUE below the lowest frequency of the BS downlink operating band up to ΔfOBUE above the highest frequency of the BS downlink operating band.

The TRP of any spurious emission shall not exceed:

Table 9.7.6.4.3.2-9: AAS BS OTA Spurious emissions limits for protection of 700 MHz public safety operations

| Operating Band | Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 13 | 763 - 775 MHz | -37 dBm | 6.25 kHz |  |
| 13 | 793 - 805 MHz | -37 dBm | 6.25 kHz |  |
| 14 | 769 - 775 MHz | -37 dBm | 6.25 kHz |  |
| 14 | 799 - 805 MHz | -37 dBm | 6.25 kHz |  |

The following requirement shall be applied to AAS BS operating in Band 26 to ensure that appropriate interference protection is provided to 800 MHz public safety operations. This requirement is also applicable at the frequency range from ΔfOBUE below the lowest frequency of the BS downlink operating band up to ΔfOBUE above the highest frequency of the BS downlink operating band.

The TRP of any spurious emission shall not exceed:

Table 9.7.6.4.3.2-10: AAS BS OTA Spurious emissions limits for protection of 800 MHz public safety operations

| Operating Band | Frequency range | Maximum Level | Measurement Bandwidth | Notes |
| --- | --- | --- | --- | --- |
| 26 | 851 - 859 MHz | -13 dBm | 100 kHz | Applicable for offsets > 37.5kHz from the channel edge |

The following requirement may apply to BS operating in Band 54 in certain regions, to be used together with other information about the site installation to verify the compliance.

The level of emissions in the 1541 – 1650 MHz band, measured in measurement bandwidth according to table 9.7.6.4.3.2-11 shall not exceed the maximum TRP limits indicated in the table.

Table 9.7.6.4.3.2-11: Emissions levels for protection of the 1541-1650 MHz band

| Operating Band | Frequency range | Declared emission level (dBW) (Measurement bandwidth = 1 MHz) | Declared emission level (dBW) of discrete emissions of less than 700 Hz bandwidth (Measurement bandwidth = 1 kHz) | Declared emission level (dBW) of discrete emissions of less than 2 kHz bandwidth (Measurement bandwidth = 1 kHz) |
| --- | --- | --- | --- | --- |
| 54 | 1541 - 1559 MHz | PEIRP – 17 dBi + 9 dB |  | PEIRP – 17 dBi + 9 dB |
|  | 1559 - 1610 MHz | PEIRP – 17 dBi + 9 dB | PEIRP – 17 dBi + 9 dB |  |
|  | 1610 - 1650 MHz | PEIRP – 17 dBi + 9 dB | PEIRP – 17 dBi + 9 dB |  |

Note:	The regional requirements specified in attachment to the FCC reference document, 0007135419,  are defined in terms of EIRP (effective isotropic radiated power), which is dependent on both the BS emissions at the antenna connector and the deployment (including antenna gain and feeder loss). The method outlined in annex B1 indicates how the limit in table 9.7.6.4.3.2-11 demonstrates compliance to the regional requirement.

##### 9.7.6.4.4	Co-location with other base stations

###### 9.7.6.4.4.1	General

These requirements may be applied for the protection of other BS receivers when GSM900, DCS1800, PCS1900, GSM850, CDMA850, UTRA FDD, UTRA TDD E-UTRA BS and/or NR BS are co-located with a BS.

The requirements assume with base stations of the same class.

NOTE:	For co-location with UTRA, the requirements are based on co-location with UTRA FDD or TDD base stations.

The requirement is a co-location requirement. The power levels are specified at the co-location reference antenna output.

###### 9.7.6.4.4.2	Minimum Requirement

The power sum of any spurious emission is specified over all supported polarizations of the co-location reference antenna and shall not exceed the limits of table 9.7.6.4.4.2-1 for a AAS BS where requirements for co-location with a frequency range and conditions identified in the first two columns apply, depending on the declared Base Station class. For a multi-band RIB , the exclusions and conditions in the notes column of table 9.7.6.4.4.2-1 apply for each supported operating band.

Table 9.7.6.4.4.2-1: AAS BS OTA Spurious emissions limits for AAS BS co-located with another BS

| System type to co-exist with | Frequency range for co-existence requirement | Basic limits (dBm) | Measurement bandwidth | Notes |
| --- | --- | --- | --- | --- |
| GSM850 or CDMA850 | 869 - 894 MHz | -57 | 100kHz | NOTE 1, 3 |
|  | 824 - 849 MHz | -61 |  |  |
| GSM900 | 921 - 960 MHz | -57 |  |  |
|  | 876 - 915 MHz | -61 |  |  |
| DCS1800 | 1805 - 1880 MHz | -47 |  |  |
|  | 1710 - 1785 MHz | -61 |  |  |
| PCS1900 | 1930 - 1990 MHz | -47 |  |  |
|  | 1850 - 1910 MHz | -61 |  |  |
| Other system type | Frequency range of downlink operating band of the BS to co-exist with (NOTE 8, 9) | -52 | 1MHz | NOTE 1, 3 |
|  | Frequency range of uplink operating band of the BS to co-exist with (NOTE 9) | -49 |  | NOTE 1, 3, 5, 6, 7 |

NOTE 1:	As defined in the scope for spurious emissions in this subclause, the co-location requirements in table 9.7.6.4.4.2-1 do not apply for the ΔfOBUE frequency range immediately outside the BS transmit frequency range of a downlink operating band (see subclause 9.7.1). The current state-of-the-art technology does not allow a single generic solution for co-location with other system on adjacent frequencies for 30 dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [12].

NOTE 2:	Table 9.7.6.4.4.2-1 assumes that two operating bands, where the corresponding BS transmit and receive frequency ranges in subclause 9.7.1 would be overlapping, are not deployed in the same geographical area. For such a case of operation with overlapping frequency arrangements in the same geographical area, special co-location requirements may apply that are not covered by the 3GPP specifications.

NOTE 3:	Co-located TDD base stations that are synchronized and using the same or adjacent operating band can transmit without special co-locations requirements. For unsynchronized base stations, special co-location requirements may apply that are not covered by the 3GPP specifications.

| Frequency range for | Condition | Basic limits |  |  | Measurement | Note |
| --- | --- | --- | --- | --- | --- | --- |
| co-location requirement |  | WA BS (dBm) | MR BS (dBm) | LA BS (dBm) | bandwidth |  |
| Frequency range of co-located uplink operating band | Co-located with GSM850, CDMA850 or GSM900 | -98 | -91 | -70 | 100 kHz | Note 1 |
|  | Co-located with DCS1800 or PCS1900 | -98 | -91 | -80 | 100 kHz |  |
|  | 51, n91, n93 | N/A | N/A | -88 | 100 kHz |  |
|  | 53 | N/A | -91 | -88 | 100 kHz |  |
|  | n100, n101 | -96 | N/A | N/A | 100 kHz |  |
|  | n96, n102 | N/A | -90 | -87 | 100 kHz |  |
|  | n104 | -95 | -90 | -87 | 100 kHz |  |
|  | Other | -96 | -91 | -88 | 100 kHz |  |
| NOTE 1: As defined in the scope for spurious emissions in this clause, the co-location requirements in table 9.7.6.4.4.2-1 do not apply for the frequency range extending ΔfOBUE immediately outside the BS transmit frequency range of a downlink operating band (see table 4.5-1 in TS 37.104 [5]). |  |  |  |  |  |  |

----------- end of change -------

----------- start of change ---RX OTA Blocking----

#### 10.6.2.2	Co-location minimum requirement

This additional blocking requirement may be applied for the protection of AAS BS receivers when E-UTRA BS, NR BS, UTRA BS, CDMA BS or GSM/EDGE BS operating in a different frequency band are co-located with an AAS BS.

The requirement is a co-location requirement. The interferer power levels are specified at the co-location reference antenna conducted input. The interfering power is specified per supported polarization.

The requirement is valid over minSENS RoAoA.

When the wanted and an interfering signal using the parameters in table 10.6.2.2-1, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in 3GPP TS 36.104 [8], subclause 7.2.1.

-	For any UTRA FDD carrier, the BER shall not exceed 0,001 for the reference measurement channel defined in 3GPP TS 25.104 [6], subclause 7.2.1.

Table 10.6.2.2-1: OTA Blocking requirement for co-location with BS in other frequency bands

| Frequency range of interfering signal | Wanted signal mean power (dBm) | Interfering signal mean power for WA BS (dBm) | Interfering signal mean power for MR BS (dBm) | Interfering signal mean power for LA BS (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| Frequency range of co-located downlink operating band | EISminSENS + 6 dB (Note 1) | +46 | +38 | +24 | CW carrier |
| NOTE 1: EISminSENS depends on the RAT, BS class and on the BS channel bandwidth, see clauses 10.2 and 10.3. NOTE 2: Except for a BS operating in Band 13, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the ΔfOOB immediately outside any of the supported uplink operating band. For a BS operating in band 13 the requirements do not apply when the interfering signal falls within the frequency range 768 - 797 MHz. NOTE 4: In China, the blocking requirement for co-location with DCS1800 and Band III BS is only applicable in the frequency range 1805 - 1850 MHz. NOTE 5: For an AAS BS operating in band 11, 21, or 74 the requirement for co-location with Band 32 applies for interfering signal within the frequency range 1475.9 - 1495.9 MHz. |  |  |  |  |  |

### 10.6.3	Minimum requirement for single RAT UTRA operation

#### 10.6.3.1	General minimum requirement

In addition to the following in-band and narrowband requirements, the general minimum requirements relating to out of band blocking defined for MSR in subclause 10.6.2.1 shall also be applied for single RAT UTRA operation.

The minimum requirement for in-band blocking and narrowband blocking UTRA operation is defined below:

The requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges applicable to each RIB.

For RIB supporting operation in non-contiguous spectrum, the requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least 15MHz. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap and is equal to -7.5MHz/+7.5MHz, respectively.

For a RIB supporting operation in non-contiguous spectrum the narrowband blocking requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least 400kHz or 600kHz, depending on the operating band. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap and is equal to -200kHz/+200kHz or -300kHz/+300kHz, respectively.

For multi-band RIBs the requirement in the in-band blocking frequency range applies for each supported operating band. The requirement applies in addition inside any Inter RF Bandwidth gap, in case Inter RF Bandwidth gap size is at least 15MHz. The interfering signal offset is defined relative to lower/upper Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap and is equal to -7.5MHz/+7.5MHz, respectively.

For multi-band RIBs the narrowband blocking requirement applies in addition inside any Inter RF Bandwidth gap, in case the Inter RF Bandwidth gap size is at least 400kHz or 600kHz, depending on the operating band. The interfering signal offset is defined relative to lower/upper Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap and is equal to -200kHz/+200kHz or -300kHz/+300kHz, respectively.

For the wanted and interfering signal at the RIB, using the parameters in tables 10.6.4.1-1 and 10.6.4.1-2, the following requirements shall be met:

-	For any UTRA FDD carrier, the BER shall not exceed 0,001 for the reference measurement channel defined in 3GPP TS 25.104 [6], subclause 7.2.1.

The OTA levels are applied referenced to 2 antenna gain offsets ΔOTAREFSENS and ΔminSENS.

Table 10.6.3.1-1: In-band blocking requirement for Single RAT UTRA AAS BS

| Base Station Type | Mean power of interfering signal [dBm] | Wanted Signal mean power [dBm] | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| Wide Area BS | -40- ΔOTAREFSENS | EISREFSENS + 6 dB | ±10MHz | WCDMA signal (NOTE 1) |
|  | -40 – ΔminSENS | EISminSENS + 6 dB |  |  |
| Medium Range BS | -35 - ΔOTAREFSENS | EISREFSENS + 6 dB |  |  |
|  | -35 – ΔminSENS | EISminSENS + 6 dB |  |  |
| Local Area BS | -30 - ΔOTAREFSENS | EISREFSENS + 6 dB |  |  |
|  | -30 – ΔminSENS | EISminSENS + 6 dB |  |  |
| NOTE 1: The characteristics of the W-CDMA interference signal are specified in Annex C of TS 25.104 [6]. NOTE 2: For multi-band RIBs, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, and not in the in-band blocking frequency range of an adjacent or overlapping operating band, the wanted Signal mean power is equal to -119.6 - ΔOTAREFSENS dBm or -119.6 - ΔminSENS dBm as appropriate. |  |  |  |  |

NOTE:	Table 10.6.4.1 assumes that two operating bands, where the downlink frequencies (see subclause 4.6) of one band would be within the in-band blocking region of the other band, are not deployed  in the same geographical area.

Table 10.6.3.1-2: Blocking performance requirement (narrowband) for Single RAT UTRA AAS BS

| Base Station Type | Mean power of interfering signal [dBm] | Wanted Signal mean power [dBm] | Minimum Offset of Interfering Signal | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| Wide Area BS | -47- ΔOTAREFSENS | EISREFSENS + 6 dB | ±2.7 MHz (NOTE 2) ±2.8 MHz (NOTE 3) | GMSK modulated (NOTE 1) |
|  | -47 – ΔminSENS | EISminSENS + 6 dB |  |  |
| Medium Range BS | -42 - ΔOTAREFSENS | EISREFSENS + 6 dB |  |  |
|  | -42 – ΔminSENS | EISminSENS + 6 dB |  |  |
| Local Area BS | -37 - ΔOTAREFSENS | EISREFSENS + 6 dB |  |  |
|  | -37 – ΔminSENS | EISminSENS + 6 dB |  |  |
| NOTE 1: GMSK modulation as defined in TS 45.004 [26]. NOTE 2: applies for bands II,IV,V,VIII,X,XII,XIV,XXV,XXVI NOTE 3: applies for bands III,VIII |  |  |  |  |

#### 10.6.3.2	Co-location minimum requirement

This additional blocking requirement may be applied for the protection of AAS BS receivers when E-UTRA BS, NR BS, UTRA BS, CDMA BS or GSM/EDGE BS operating in a different frequency band are co-located with an AAS BS.

The requirement is a co-location requirement. The interferer power levels are specified at the co-location reference antenna conducted input. The interfering power is specified per supported polarization.

The requirement is valid over minSENS RoAoA.

When the wanted and an interfering signal using the parameters in table 10.6.2.2-1 for co-location with UTRA or E-UTRA systems and table 10.6.3.2-1 for co-location with GSM systems, the following requirements shall be met:

-	For any UTRA FDD carrier, the BER shall not exceed 0,001 for the reference measurement channel defined in 3GPP TS 25.104 [6], subclause 7.2.1.

Table 10.6.3.2-1: UTRA additional OTA blocking requirement for co-location with BS in other frequency bands

| Frequency range of interfering signal | Wanted signal mean power (dBm) | Interfering signal mean power for WA BS (dBm) | Interfering signal mean power for MR BS (dBm) | Interfering signal mean power for LA BS (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| Frequency range of co-located downlink operating band | EISminSENS + 6 dB (Note 1) | +46 | +38 | +24 | CW carrier |
| NOTE 1: EISminSENS depends on the BS class and on the BS channel bandwidth, see clause 10.2. NOTE 2: Except for a BS operating in Band 13, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the ΔfOOB immediately outside any of the supported uplink operating band. For a BS operating in band 13 the requirements do not apply when the interfering signal falls within the frequency range 768 - 797 MHz. NOTE 4: In China, the blocking requirement for co-location with DCS1800 and Band III BS is only applicable in the frequency range 1805 - 1850 MHz. NOTE 5: For an AAS BS operating in band 11, 21, or 74 the requirement for co-location with Band 32 applies for interfering signal within the frequency range 1475.9 - 1495.9 MHz. |  |  |  |  |  |

### 10.6.4	Minimum requirement for single RAT E-UTRA operation

#### 10.6.4.1	General minimum requirement

In addition to the following in-band and narrowband requirements, the general minimum requirements relating to out of band blocking defined for MSR in subclause 10.6.2.1 shall also be applied for single RAT E-UTRA operation.

The minimum requirement for in-band blocking E-UTRA operation is defined below:

The requirement is applicable outside the Base Station RF Bandwidth or Radio Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or Radio Bandwidth edges applicable to each RIB.

For RIB supporting operation in non-contiguous spectrum, the requirement applies in addition inside any sub-block gap, in case the sub-block gap size is at least 15 MHz. The interfering signal offset is defined relative to the sub-block edges inside the sub-block gap.

For multi-band RIBs, the requirement applies in addition inside any Inter RF Bandwidth gap, in case the gap size is at least 15 MHz. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges inside the Inter RF Bandwidth gap.

For the wanted and interfering signal at the RIB, using the parameters in tables 10.6.4.1-1 and 10.6.4.1-2, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in 3GPP TS 36.104 [8], subclause 7.2.1.

The OTA levels are applied referenced to 2 antenna gain offsets ΔOTAREFSENS and ΔminSENS.

For multi-band RIBs, the requirement applies according to table 10.6.4.1-1 for the in-band blocking frequency ranges of each supported operating band.

Table 10.6.4.1-1: In-band blocking requirement for single RAT E-UTRA

| Base Station Type | Mean power of interfering signal [dBm] | Wanted Signal mean power [dBm] (NOTE 1,2) | Type of Interfering Signal | Interfering signal centre frequency minimum offset from the Base Station RF Bandwidth edge or edge of sub-block inside a gap [MHz] |
| --- | --- | --- | --- | --- |
| Wide Area BS | -43 - ΔOTAREFSENS | EISREFSENS + 6 dB | See table 10.6.4.1-2 | See table 10.6.4.1-2 |
|  | -43 – ΔminSENS | EISminSENS + 6 dB |  |  |
| Medium Range BS | -38 - ΔOTAREFSENS | EISREFSENS + 6 dB |  |  |
|  | -38 – ΔminSENS | EISminSENS + 6 dB |  |  |
| Local Area BS | -35 - ΔOTAREFSENS | EISREFSENS + 6 dB |  |  |
|  | -35 – ΔminSENS | EISminSENS + 6 dB |  |  |
| NOTE 1: EISREFSENS and EISminSENS depend on the RAT, the BS class and on the channel bandwidth, see subclauses 10.3 and 10.2. NOTE 2: For multi-band RIBs, in case of interfering signal that is not in the in-band blocking frequency range of the operating band where the wanted signal is present, and not in the in-band blocking frequency range of an adjacent or overlapping operating band, the wanted signal mean power is equal to EISREFSENS +1.4 dB or EISminSENS +1.4 dB as appropriate. |  |  |  |  |

Table 10.6.4.1-2: Interfering signals for single RAT E-UTRA in-band blocking performance requirement

| E-UTRA channel BW of the lowest/highest carrier received [MHz] | Interfering signal centre frequency minimum offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of interfering signal |
| --- | --- | --- |
| 1.4 | ±2.1 | 1.4 MHz E-UTRA signal |
| 3 | ±4.5 | 3 MHz E-UTRA signal |
| 5 | ±7.5 | 5 MHz E-UTRA signal |
| 10 | ±7.5 | 5 MHz E-UTRA signal |
| 15 | ±7.5 | 5 MHz E-UTRA signal |
| 20 | ±7.5 | 5 MHz E-UTRA signal |
| 20 | ±30 | 20 MHz E-UTRA signal |

#### 10.6.4.2	Co-location minimum requirement

This additional blocking requirement may be applied for the protection of AAS BS receivers when E-UTRA BS, NR BS, UTRA BS, CDMA BS or GSM/EDGE BS operating in a different frequency band are co-located with an AAS BS.

The requirement is a co-location requirement. The interferer power levels are specified at the co-location reference antenna conducted input. The interfering power is specified per supported polarization.

The requirement is valid over minSENS RoAoA.

When the wanted and an interfering signal using the parameters in table 10.6.2.2-1 for co-location with UTRA or E-UTRA systems and table 10.6.4.2-1 for co-location with GSM systems, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channel defined in 3GPP TS 36.104 [8], subclause 7.2.1.

Table 10.6.4.2-1: E-UTRA additional OTA blocking requirement for co-location with BS in other frequency bands

| Frequency range of interfering signal | Wanted signal mean power (dBm) | Interfering signal mean power for WA BS (dBm) | Interfering signal mean power for MR BS (dBm) | Interfering signal mean power for LA BS (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| Frequency range of co-located downlink operating band | EISminSENS + 6 dB (Note 1) | +46 | +38 | +24 | CW carrier |
| NOTE 1: EISminSENS depends on the BS class and on the BS channel bandwidth, see clause 10.2. NOTE 2: Except for a BS operating in Band 13, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the ΔfOOB immediately outside any of the supported uplink operating band. For a BS operating in band 13 the requirements do not apply when the interfering signal falls within the frequency range 768 - 797 MHz. NOTE 4: In China, the blocking requirement for co-location with DCS1800 and Band III BS is only applicable in the frequency range 1805 - 1850 MHz. NOTE 5: For an AAS BS operating in band 11, 21, or 74 the requirement for co-location with Band 32 applies for interfering signal within the frequency range 1475.9 - 1495.9 MHz. |  |  |  |  |  |

----------- end of change -------