3GPP TSG- RAN4 Meeting #116bis	R4-2514837

# Prague, Czech Republic, Oct. 13-17, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on RRM requirements for supporting intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NonCol_intraB_ENDC_NR_CA_Ph2-Core |  |  |  |  |  | Date: |  |  | 2025-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Scheduling availability of UE performing radio link monitoring requirements, the cases are specified for UE supporting intraBandNR-CA-non-collocated-r19 are not comprehensive. |  |  |  |  |  |  |  |  |
| s |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction on scheduling restriction requirements in clause 8.1.7.2 for UE supporting intraBandNR-CA-non-collocated-r19. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The scheduling restriction for supporting intra-band non-collocated NR-CA is not comprehensive. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.7.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1

For UEs which support simultaneousRxDataSSB-DiffNumerology [14] there are no restrictions on scheduling availability due to radio link monitoring based on SSB as RLM-RS. For UEs which do not support simultaneousRxDataSSB-DiffNumerology [14] the following restrictions apply due to radio link monitoring based on SSB as RLM -RS.

-	The UE is not expected to transmit PUCCH, PUSCH or SRS or receive PDCCH, PDSCH or CSI-RS for tracking or CSI-RS for CQI on SSB symbols to be measured for radio link monitoring.

When intra-band carrier aggregation in FR1 is performed, the scheduling restrictions on FR1 serving PCell or PSCell applies to all serving cells in the same band on the symbols that fully or partially overlap with the restricted symbols.

When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18, there are no scheduling restrictions on FR1 serving cell(s) configured on the non-contiguous CC(s) in the same band if nonCollocatedTypeNR-CA-r18 is not provided. Otherwise, the scheduling restrictions on FR1 serving PCell or PSCell applies to all serving cells in the same band on the symbols that fully or partially overlap with the restricted symbols if nonCollocatedTypeNR-CA-r18 is provided.

When intra-band non-contiguous carrier aggregation is configured for a UE supporting intraBandNR-CA-non-collocated-r19,

1> 	when UE is configured with maxMIMO-Layers with value equal to 4 for the configured FR1 intra-band non-contiguous CA

2> 	if nonCollocatedTypeNR-CA-v1900 is “type 4”

3>	there are no scheduling restrictions on FR1 serving cell(s) configured on the non-contiguous CC(s) in the same band

2> 	if nonCollocatedTypeNR-CA-v1900 is “type 1”

3>	the scheduling restrictions on FR1 serving PCell or PSCell apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

2> else if nonCollocatedTypeNR-CA-v1900 is not provided

3>	the scheduling restrictions on FR1 serving PCell or PSCell apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

1>	when UE is configured with maxMIMO-Layers with value equal to 2 for the configured FR1 intra-band non-contiguous CA

2> if nonCollocatedTypeNR-CA-r18 is not provided

3> there are no scheduling restrictions on FR1 serving cell(s) configured on the non-contiguous CC(s) in the same band

2>else

3> the scheduling restrictions on FR1 serving PCell or PSCell apply to all serving cells in the same band on the symbols that fully or partially overlap with restricted symbols.

When inter-band FR1-FR1 NR-DC is configured, there are no scheduling restrictions on FR1 serving cell(s) in the bands due to radio link monitoring performed on FR1 serving PCell or PSCell in different bands.

<Unchanged Text Skipped>