# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2515068

# Prague, CZ, 13th – 17th October, 2025

Title:	WF on [116bis][313] 5G_Broadcast_GSO

Agenda item:			6.5.1

Source:	Astrum Mobile, ZTE, Qualcomm

Document for:	Approval

- Topic #1: SAN conformance requirements

Issue 1-1-1: Motivation for conformance test specification for 36.181

- Agreement:

  - RAN4 recommends specifying SAN conformance requirements for LTE 5G broadcast over GSO

    - RAN4 can start SAN conformance related discussion from RAN4#117

    - WID needs to be updated in RAN#110 to include TS 36.181 as impacted specification.

Issue 1-1-2: SAN conformance requirements

- Agreement:

  - Take the Table 1 and 2 as baseline.

  - RAN4 to further investigate possible required changes for TS 36.181

Table 1 General requirements for NTN broadcast band

| Requirements in TS 36.181 | Expected impact |
| --- | --- |
| 4 General test conditions and declarations |  |
| 4.1 Measurement uncertainties and test requirements | 1. Need to add max test system uncertainty with 100kHz for 10MHz occupied bandwidth in Table 4.1.2.2-1. 2. Need to add 9MHz AWGN bandwidth in Table 4.1.2.3-1. |
| 4.2 Requirement reference points | No specification impact. |
| 4.3 Satellite Access Node classes | No specification impact. |
| 4.4 Regional requirements | No specification impact. |
| 4.5 SAN configurations | No specification impact. |
| 4.6 Manufacturer declarations | No specification impact. |
| 4.7 Test configurations | The carrier bandwidth in section 4.7.3.1 needs to be updated. |
| 4.8 Applicability of requirements | No specification impact. |
| 4.9 RF channels and test models | The carrier bandwidth in section 4.9.2.2 needs to be updated. |
| 4.10 Requirements for contiguous and non-contiguous spectrum | No specification impact. |
| 4.11 Requirements for SAN capable of multi-band operation | No specification impact. |
| 4.12 Reference coordinate system | No specification impact. |
| 4.13 Format and interpretation of tests | No specification impact. |

Table 2 RF requirements for NTN broadcast band

| Requirements in TS 36.181 | Expected impact |
| --- | --- |
| 6 Conducted transmitter characteristics |  |
| 6.1 General | Need to add BOG. |
| 6.2 Satellite Access Node output power | No specification impact. |
| 6.3 Output power dynamics |  |
| 6.3.1 General | No specification impact. |
| 6.3.2 RE power control dynamic range | No specification impact. |
| 6.3.3 Total power dynamic range | Need to add 10MHz SAN channel bandwidth in Table 6.3.3.5-1. |
| 6.4 Transmit ON/OFF power | No specification impact. |
| 6.5 Transmitted signal quality |  |
| 6.5.1 General | No specification impact. |
| 6.5.2 Frequency error | No specification impact. |
| 6.5.3 Modulation quality | Need to add EVM requirement for BOG in section 6.5.3.5. |
| 6.5.4 Time alignment error | No specification impact. |
| 6.5.5 DL RS power | Need to specify that this requirement is not applicable to BOG. |
| 6.6 Unwanted emissions |  |
| 6.6.1 General | No specification impact. |
| 6.6.2 Occupied bandwidth | Need to add 10MHz in Table 6.6.2.4.2-1. |
| 6.6.3 Adjacent Channel Leakage Power Ratio (ACLR) | Need to add 10MHz in Table 6.6.3.5-1. |
| 6.6.4 Out-of-band emissions | No specification impact. |
| 6.6.5 Transmitter spurious emissions | No specification impact. |
| 6.7 Transmitter intermodulation | No specification impact. |

- Topic #2: UE demodulation requirements

Issue 2-1-1: Test parameters

- Agreement:

  - Take the parameters listed below as baseline to define PMCH demodulation requirements.

    - SCS: 15kHz

    - Channel bandwidth: 10MHz

    - Modulation: Rank 1 with QPSK, Rank 1 with 16QAM

    - Antenna configuration: 1x2 low

    - BLER test point: 1% of BLER.

    - Other test parameters in Table 3

Table 3: Test Parameters for Testing

| Parameter |  | Unit | Test 1-4 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 (Note 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| Bandwidth |  | MHz | 10MHz |
| Note 1: . |  |  |  |

Issue 2-1-2: Fixed reference channel

- Agreement:

  - Take the following fixed reference channel in Table 4 as baseline for 5G broadcast over GSO UE demodulation requirements definition, which is for 15kHz subcarrier spacing for MBMS dedicated cell.

Table 4 Fixed Reference Channel for PMCH Receiver Requirements (15 kHz SCS)

| Parameter | PMCH (15 kHz SCS) |  |  |
| --- | --- | --- | --- |
|  | Unit | Value |  |
| Reference channel |  | R.x.1 | R.x.2 |
| PMCH bandwidth | MHz | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 |
| Allocated subframes per Radio Frame(Note1) |  | 10 | 10 |
| Modulation |  | QPSK | 16QAM |
| Target Coding Rate |  | 1/3 | 1/2 |
| Information Bit Payload (Note 2) |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 3624 | 9912 |
| Number of Code Blocks per Sub-Frame (Note 3) |  | 1 | 2 |
| Binary Channel Bits Per Subframe |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 10200 | 20400 |
| MBMS UE Category |  | ≥ 1 | ≥ 1 |
| Note 1: For FDD mode, all 10 subframes are available for MBMS, in line with TS 36.331. Note 2: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211. Note 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |

Issue 2-1-3: Propagation condition

- Agreement:

  - RAN4 to use the following NTN-TDLC (DS=5ns) channel model parameters as the propagation condition for 5G broadcast over GSO demodulation requirements definition.

Table 5: Delay profiles for 5G broadcast over GSO channel models

| Type | Model | Delay spread (r.m.s.) | Delay resolution |
| --- | --- | --- | --- |
| LOS | NTN-TDLC5 | 5 ns | 5 ns |

Table 6 NTN-TDLC5 (DS = 5 ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.6 | LOS path |
|  | 0 | -8.9 | Rayleigh |
| 2 | 60 | -21.5 | Rayleigh |
| Note 1: Tap #1 follows a Rician distribution. |  |  |  |

Table 7: Channel model parameters for 5G broadcast over GSO channel models

| Combination name | Model | Maximum Doppler frequency |
| --- | --- | --- |
| NTN-TDLC5-5 | NTN-TDLC5 | 5 Hz |