3GPP TSG-RAN WG4 Meeting #117		    								               	R4-2522232

Dallas, USA, November 17-21, 2025

Agenda item:			7.8.1

Source:	Moderator (Apple)

Title:	Topic summary for [117][320] NR_low_band_CA_switching

Document for:	Information

# Introduction

This document summarizes the contributions submitted for the general and RF related considerations on the enhancement of low NR band carrier aggregation via switching work item (AI 7.8.2 and 7.8.3).

Moderator’s note: proposals related to the possible coexistence issue (see Issue 7 in the WF from RAN4 #116bis [R4-2515156]) are handled in the NR_LBCA_Sw maintenance discussion thread (thread number 114).

## Summary of company contributions

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2522232 | Moderator (Apple) | Topic summary for [117][320] NR_low_band_CA_switching |
| R4-2520600 | Apple | (NR_LBCA_Sw_enh-Core) CR for 38.768 on enhanced LB CA via switching scenarios The new work item on the enhancement of low NR band carrier aggregation via switching was approved in RAN #109 in RP-252953. The work item will continue to capture technical background in TR38.768. This draft CR provides the work item description for the enhancement and introduces the relevant operator-provided scenarios. Change 1: capture the work item description on the enhancement of low NR band carrier aggregation via switching (Clause 1) Change 2: capture the operator-provided scenarios (Clause 4) |
| R4-2520092 | CATT | Discussion on R20 low bands CA via switching Proposal 1: for both CA_n12-n29 and CA_n28-n67, if RAN4 allow Type 1 UE (two DL concurrent inter-band carriers fully shared one Rx chain) for UE implementation, a new UE capability [Inter-band_NRCA_SharedRxChain] which is similar to the intra-band non-collocated CA is needed. Proposal 2: for CA_n28-n67, the reference RF architectures in figure 2 and figure 3 can be considered for different DL n28 frequency location. Proposal 3: its RAN4s common understanding that the switching gaps in both FDD DL band and SDL band are needed when the status is switching from Dual-DL FDD+SDL bands to UL/DL FDD band. That means UE cant maintain the DL reception in DL FDD band during the switching gap from Dual-DL FDD+SDL bands to UL/DL FDD band. Proposal 4-1: RAN4 consider to introduce the following calculation of switching gaps for the switching scenario from Dual-DL FDD+SDL bands to UL/DL FDD band. Proposal 4-2: FFS on the switching gaps calculation of switching scenario from UL/DL FDD band to Dual-DL FDD+SDL bands. Proposal 5: to design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together. Proposal 6: RAN4 can further discuss the relationship between R19 and R20 low band switching based on the following two alternatives. Alternative 1: There is no fallback behaviour between R19 and R20 low band switching Alternative 2: There is a fallback behaviour between R19 and R20 low band switching. |
| R4-2520253 | Huawei, HiSilicon | On remaining UE RF issues for NR_LBCA_Sw_enh Observation 3: Network can know whether TA for a UE is large or not and the UEs switching period via its UE capability so that the network can provide the UE with sufficient switching gap (symbols). Proposal 1: Discuss the actual detailed UE RF requirements, RAN1 impact and relationship and/or fallback behavior b/w Rel-19 & Rel-20 after obtaining the conclusion for BS-to-BS and UE-to-UE co-existence issue which may or may not impact on UE RF requirements. Proposal 2: No more discussion is needed for the case in Observation 3 under this WI. Proposal 3: Discuss relationship and/or fallback behavior between Rel-19 NR_LBCA_Sw and Rel-20 NR_LBCA_Sw_enh after all the remaining issues are resolved. |
| R4-2520582 | Apple | On RF architectures and requirements for LB CA via switching with 2DL Proposal 1: The following two-antenna based RF architecture can be used for low band CA via switching with dual-downlink. Proposal 2: The following diagram can be used for the FDD (UL+DL) to SDL+FDD DL transition with the switching gap on the switched-from carrier: Proposal 3: The following diagram can be used for the SDL+FDD DL to FDD (UL+DL) transition with the switching gap on the switched-from carrier: Proposal 4: Re-use the ON/OFF mask from TR38.768 for the Rel-20 ON/OFF mask as showing below: Proposal 5: For CA_12A-n29A and CA_n28A-n67A, no change on the REFSENS is expected and the single-carrier REFSENS values apply for the two carriers as it is the case for the legacy inter-band and intra-band contiguous CA requirements. Proposal 6: For CA_28F-n67A, RAN4 could consider specifying RIB,c to accommodate potential higher insertion losses for the combined filter relative to the single-band filters, depending on the outcome of the feasibility study of the combined n28DL + n67 filter. Proposal 10: We suggest defining a a new capability based on the Rel-19 capability as shown in the table below: |
| R4-2520737 | vivo | Discussion on LBCA R20 Proposal 1: For LBCA switching for the dual DL scenario, considering sufficient isolation of UL and DL the operation of n29 and n28 should be restricted to a certain range, Proposal 2: Considering the limitation of wider DL filters in suppressing closer UL, the REFSENS would be impacted and should be firstly studied, the performance of ACS, IBB and OBB could be further analysed based on relaxation of REFSENS. Proposal 3: For the architecture of LO sharing between two DL CCs, the upper limit of two DL CC received power spectral density imbalance should be studied. Proposal 4: When the LO is shared between two DL CCs, further discussion is needed on whether to define a new requirement or test case for REFSENS degradation due to image interference. Proposal 5: RAN4 requirement development should only consider to separate the RX chains for the two DL CCs after LNA. |
| R4-2520754 | Samsung | Views on UE RF requirements for LB CA switching Proposal 1: The definition of no-gap impact based on the frequency separation between the carriers and the maximum aggregated CBW that a single low-band Rx chain is required to process should be applicable to both filter options. Proposal 2: RAN4 should define a unified set of RF requirements that does not depend on a single, rigid filter realization Proposal 3: It is recommended to introduce a maximum allowable separation and/or maximum aggregated channel bandwidth ensuring all existing single-carrier Rx requirements even in the dual-DL switching configuration, without introducing new numeric requirements or complex inter-band coupling terms for dual-DL relaxations. Proposal 4: From the RF requirements viewpoint, Rel-19 and Rel-20 LB CA via switching should share a common structure and philosophy. Proposal 5: No fundamentally new class of RF requirement is considered necessary only for Rel-20 dual DL switching configurations to keep the specification consistent and manageable across releases, while providing sufficient flexibility to accommodate various UE architectures and deployment choices in the low-band region. |
| R4-2521133 | Ericsson | More on low-band CA via switching with dual downlink Observation 1: a single Rx filter appears feasible for supporting the example band combinations CA_n12A-n29A and CA_n28A-n67A with dual DL conditioned on a restriction to the restriction of n28 operation to the range 703-733/758-788 MHz relevant for operations in ITU-R Region 1. Observation 2: the standard ACS, in-band and out-of-band blocking requirements for inter-band CA also apply for LB CA implemented via switching unless otherwise stated. Observation 3: notwithstanding the deployment constraint no in-gap interference between DL carriers is expected listed in the Rel-20 WI objectives, there may still be in-band/gap blockers (other operators) for the example band combinations. Observation 4: protection of band n29/n67 from interferers in n12/n28 (703-733 MHz for n28) down to the standard -50 dBm/MHz is not feasible, a relaxed emission limit could be considered noting that n29/n67 only supports Scells. Observation 5: the CA_n5A-n29A switching band combination can be supported with dual downlink, a triplexer supporting this combination appears feasible from a filter capability standpoint. Observation 6: a first assessment of antenna performance for the CA_n5A-n29A combination with support of dual downlink shows a degradation of n5 total antenna efficiency of 2-3 dB and 4 dB for the n29 with a single-tuning state for a single antenna as compared to an implementation with two-state antenna tuning and LB switching. Further optimization of n5 (Pcell) uplink performance with a single tuning state at the expense of n29 (Scell) downlink performance could possibly enable implementation as a standard DL-only CA configuration without the performance degradation of semi-static switching, however. Proposal 1: the selectivity for the example band combinations is verified using requirements for intra-band non-contiguous CA for TDD bands with in-gap and out-of-gap interferers just as for EN-DC configurations for partially overlapping TDD bands for which co-located and synchronized network deployment for both carriers is also assumed. The selectivity requirements should be specified with regard to the aggregated CA bandwidth for both single-and dual Rx architectures with due account of the switching (performance for the DL-only carrier only measured during the HD-FDD operation). |
| R4-2521276 | ZTE Corporation, Sanechips | On R20 low-low band CA via switching Proposal 1: With a high performance SAW filter, it is feasible to use a combined band n28+n67 DL filter to support 60MHz channel bandwidth and the full band n28 implementation Proposal 2: Two separate filters for the two DL band is needed for the LBCA switching scenario with large separation between two DL bands. Proposal 3: For FDD carrier with Tx/Rx, the existing REFSENS requirements can be reused. For FDD carrier with Rx only, relax REFSENS requirements should be consider without Tx impacts. Proposal 4: For SDL carrier, the existing REFSENS requirements can be reused. Proposal 5: For SDL+FDD carrier use wideband receiver filter, the reference sensitivity is defined to be met with all downlink component carriers active without the uplink carriers active. Proposal 6: To limit the n28 operation to the Region 1 range 703-733/758-788 for CA_n28A-n67A. Proposal 7: For CA_n12A-n29A and CA_n28A-n67A, two new NOTEs in out of band blocking Table 7.6.3-2 in TS38.101-1 should be added to include whole DL frequency range of the two DL bands. Proposal 8: Inform RAN2 that RAN4 introduce new dual DL switching case for signalling updating. |
| R4-2521519 | LG Electronics | Discussion on low NR band carrier aggregation via switching Proposal 1: It is necessary to put gap between UL and SDL band. FFS on gap size Proposal 2: it is necessary to handle RF requirements (e.g., Emission, blocking and reference sensitivity requirement) after solving co-existence issue. |
| R4-2521572 | Nokia | On the issues for NR_LBCA_Sw_enh Proposal 1: For BS to BS co-existence arranging a guard band for CA_n12-n29 between n12-UL and n29-SDL should be studied. Proposal 2: Any new band combination in the future with carriers close to each other, only use cases where there is room for both a 3GPP defined channel bandwidth and sufficient guard band towards the uplink shall be considered. |
| R4-2522254 | Skyworks Solutions, Inc. | On band n28 and n67 co-banding for CA_n28-n67 LBLB switching Proposal: · To finalize the study of CA_n28A-n67 support with release 20 operation, the combined n28A+n67 filter feasibility should be finalized · Given the hardware cost and challenge of a filter or a four low-band antenna solution, R20 based switching support of CA_n28A-n67 performance gains should be evaluated against supporting it with the R19 switching solution. |

# Topic #1: General

## Summary of open issues

Issue 1-1: Enhanced low band switching scenarios

- Proposal: CR for 38.768 on enhanced LB CA via switching scenarios [Apple R4-2520600]

  - Change 1: capture the work item description on the enhancement of low NR band carrier aggregation via switching (Clause 1)

  - Change 2: capture the operator-provided scenarios (Clause 4)

- Recommended WF: This CR is based on the draft CR endorsed last meeting and seems agreeable

Issue 1-2: Relationship and/or fallback behavior between Rel-19 low band switching and Rel-20 low band switching

- Option 1: Discuss relationship and/or fallback behavior between Rel-19 NR_LBCA_Sw and Rel-20 NR_LBCA_Sw_enh after all the remaining issues are resolved. [Huawei R4-2520253]

  - From UE RF perspective, it would be possible for UE supporting Rel-20 NR_LBCA_Sw_enh to support Rel-19 NR_LBCA_Sw. It’s noted that actual UE’s performance for the UE, e.g., REFSENS etc. during Rel-19 NR_LBCA_Sw operation, may be different from a UE supporting only Rel-19 NR_LBCA_Sw due to Rx filter design difference.

  - At this moment, however, it is not clear if Case 3 (dualDL) has impact on RAN1 specification

- Option 2: RAN4 can further discuss the relationship between R19 and R20 low band switching based on the following two alternatives: [CATT R4-2520092]

  - Alternative 1: There is no fallback behaviour between R19 and R20 low band switching. Network can use the same IE (LowBandCA-Switching-r19 -> switchingPattern-r19 / gapDurationPCelltoSCell-r19 / gapDurationSCelltoPCell-r19) for both R19 and R20 low band switching. But UE need an additional UE capability to indicate whether it support R20 LB CA switching or not.

  - Alternative 2: There is a fallback behaviour between R19 and R20 low band switching. If UE indicate the support of R20 LB CA switching. That means UE also need to support R19 LB CA switching. Thus, additional IE is need for network to indicate whether the switching pattern is for R19 LB CA switching or R20 LB CA switching.

- Option 3: From the RF requirements viewpoint, Rel-19 and Rel-20 LB CA via switching should share a common structure and philosophy. [Samsung R4-2520754]

  - No fundamentally new class of RF requirement is considered necessary only for Rel-20 dual DL switching configurations to keep the specification consistent and manageable across releases, while providing sufficient flexibility to accommodate various UE architectures and deployment choices in the low-band region

- Recommended WF: Discuss further

# Topic #2: Reference architecture

## Summary of open issues

Issue 2-1: Feasibility of a combined band n28+n67 DL filter, considering the supported bandwidth and the full band n28 implementation

- Option 1: Given the hardware cost and challenge of a filter or a four low-band antenna solution, R20 based switching support of CA_n28A-n67 performance gains should be evaluated against supporting it with the R19 switching solution [Skyworks R4-2522254]

  - Even if a combined n28A+n67 filter is proven feasible, the support of switched CA_n28A-n67 with require redundant hardware to support other band n28 ranges and combinations impacting the solution size and cost.

  - In the case where such a filter proves not feasible, too poor performance or too costly, the only other alternative is a solution with 4 low-band antennas

- Option 2: For CA_n28A-n67A, RAN4 could consider specifying ΔRIB,c to accommodate potential higher insertion losses for the combined filter relative to the single-band filters, depending on the outcome of the feasibility study of the combined n28DL + n67 filter [Apple R4-2520582]

- Option 3: for CA_n28-n67, the following reference RF architectures can be considered for different DL n28 frequency location. [CATT R4-2520092].

Left figure for CA_n28-n67 with n28 DL carrier located in 768MHz~788MHz

Right figure for CA_n28-n67 with n28 DL carrier located in 758MHz~768MHz

- Recommended WF: Discuss further (please note that this issue is related to Issue 2-2)

Issue 2-2: Whether a restriction of n28 operation to the Region 1 range 703-733/758-788 MHz is relevant for the purpose of agreeing a reference architecture when deriving minimum requirements for the feature in the CA_n28A-n67A band combination

- Option 1: Infeasible: Full band filter (738-803 MHz); Feasible: A portion of n28 DL + n67 (738-788 MHz) [Huawei R4-2520253]

- Option 2: Single filters based on combining the two DLs can be used to simultaneously receive the two bands for CA_n12A-n29A and CA_n28A-n67A when the UE is in state#3, assuming that n28 is restricted to be operated in the range 703-733 / 758-788 (Region 1) [Apple R4-2520582, vivo R4-2520737, Ericsson R4-2521133]

- Option 3: RAN4 should define a unified set of RF requirements that does not depend on a single, rigid filter realization [Samsung R4-2520754, ZTE R4-2521276]

  - For the specific CA_n28A-n67A case, the recent discussion on RF filter feasibility has highlighted at least two reasonable options for the combined DL filter:

    - a full-band filter that covers the entire 738-803 MHz span; and

    - a more limited span, e.g., 738-788 MHz combined with a restricted Region-1 definition for n28.

  - A high performance SAW filter would be needed to support SDL n67+ full n28 DL. With a high performance SAW filter, it is feasible to use a combined band n28+n67 DL filter to support 60MHz channel bandwidth and the full band n28 implementation

- Option 4: a word-wide skew would have to implement band n28 with 3 different filter schemes in both the main and diversity path [Skyworks R4-2522254]

  - A band 28A/B or full duplexer and DL filter for amin and diversity path for stand alone

  - A band n28A+n67 DL filter in main and diversity path to support switched CA_n28-n67

  - A band n20/n28A triplexer in main path and a n28A+n20 filter in diversity path to support CA_n20-n28

  - Also note that band n28B may be combined with n26 for China or Japan

- Recommended WF: Discuss further based on the following Moderator observations:

  - It appears more companies are converging toward Option 2, and it would help to double-check the proposed frequency restriction range from Option 1

  - Proponents of Option 3 are requested to check whether a general requirement based on Option 2 could harm the possible UE implementation they envisioned

  - RAN4 should discuss how to accommodate the concerns raised in Option 4: for example, by identifying the factors that can impact UE RF requirements, such as REFSENS, ACS, blocking, etc.

Issue 2-3: Front end filter architecture assumption for the case of other LB-LB CA combinations where two DL bands are separated

- Option 1: Two separate filters for the two DL band is needed for the LBCA switching scenario with large separation between two DL bands [ZTE R4-2521276]

- Recommended WF: Check whether Option 1 is agreeable

Issue 2-4: Other aspects related to UE reference architecture

- Option 1: For LBCA switching for the dual DL scenario, considering sufficient isolation of UL and DL the operation of n29 should be restricted to a certain range, and further study is needed for CA_n12A-n29A [vivo R4-2520737]

- Option 2: The CA_n5A-n29A switching band combination can be supported with dual downlink, a triplexer supporting this combination appears feasible from a filter capability standpoint [Ericsson R4-2521133]

- Option 3: for both CA_n12-n29 and CA_n28-n67, if RAN4 allow Type 1 UE (two DL concurrent inter-band carriers fully shared one Rx chain) for UE implementation, a new UE capability [Inter-band_NRCA_SharedRxChain] which is similar to the intra-band non-collocated CA is needed [CATT R4-2520092]

- Recommended WF: Discuss further

# Topic #3: Switching period aspects

## Summary of open issues

Issue 3-1: Whether/what additional clarifications are needed for the time mask considering DL CA configuration of Case 3

- Option 1: Updates are needed for the big picture of the switching time mask and the related switching gap calculations (please refer to the contribution for detailed proposals) [CATT R4-2520092, Apple R4-2520582]

- Option 2: It is premature to discuss whether or what additional clarifications are needed without knowing RAN1 impact due to the introduction of dualDL (Case 3) [Huawei R4-2520253]

- Recommended WF: Discuss further

Issue 3-2: How to verify REFSENS requirements and switching pattern

- Option 1: design a new DL RMC to verify the REFSENS requirements and the DL receiving performance for UE before and/or after the Switching gap together [CATT R4-2520092]

- Recommended WF: Discuss further

# Topic #4: Rx requirement aspects

## Summary of open issues

Issue 4-1: Summary of impacted Rx requirements

- Option 1: consider the following list [LGE R4-2521519]:

  - Inter-band CA operating bands (already define in Rel-19)

  - NR CA configurations and bandwidth combinations (already define in Rel-19)

  - ∆TIB,c for Inter-band CA (No need to define in Rel-20)

  - Transmit ON/OFF time mask for inter-band CA (reuse in Rel-19)

  - ∆RIB,c due to CA

  - Reference sensitivity exceptions due to cross band isolation for CA

  - Blocking requirement

- Option 2: It is recommended to introduce a maximum allowable separation and/or maximum aggregated channel bandwidth ensuring all existing single-carrier Rx requirements even in the dual-DL switching configuration, without introducing new numeric requirements or complex inter-band coupling terms for dual-DL relaxations [Samsung R4-2520754]

  - The existing low-band UE Rx requirements can still be reused without any relaxation, provided that the aggregated CBW and the frequency separation between the carriers are kept within reasonable bounds under the “no-gap impact” condition

- Recommended WF: Discuss further

Issue 4-2: REFSENS and ΔRIB,c

- Option 1: Reuse single-carrier REFSENS and specify ΔRIB,c when needed [Apple R4-2520582, ZTE R4-2521276]

  - Option 1a: For CA_n28A-n67A, RAN4 could consider specifying ΔRIB,c to accommodate potential higher insertion losses for the combined filter relative to the single-band filters, depending on the outcome of the feasibility study of the combined n28DL + n67 filter.

  - Option 1b: For FDD carrier with Tx/Rx, the existing REFSENS requirements can be reused. For FDD carrier with Rx only, relax REFSENS requirements should be consider without Tx impacts. For SDL carrier, the existing REFSENS requirements can be reused. For SDL+FDD carrier use wideband receiver filter, the reference sensitivity is defined to be met with all downlink component carriers active without the uplink carriers active

- Option 2: Considering the limitation of wider DL filters in suppressing closer UL, the REFSENS would be impacted and should be firstly studied [vivo R4-2520737]

- Recommended WF: Discuss further

Issue 4-3: ACS and blocking

- Option 1: Reuse single-carrier ACS [Apple R4-2520582]

- Option 2: Considering the limitation of wider DL filters in suppressing closer UL, the performance of ACS, IBB and OBB could be further analysed based on relaxation of REFSENS. [vivo R4-2520737]

- Option 3: For CA_n12A-n29A and CA_n28A-n67A, two new NOTEs in out of band blocking Table 7.6.3-2 in TS38.101-1 should be added to include whole DL frequency range of the two DL bands [ZTE R4-2521276]

- Recommended WF: Discuss further

# Annex: References (for information)

Please refer to the latest approved WID in RP-252953

The core part of WI: - Specify the necessary UE RF and RRM core requirements to allow switching between {case 1, case 3} to enable low band carrier aggregation via switching for the dual downlink scenario between FDD band and SDL band [RAN4]. - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1. - Example band combinations: CA_n12A-n29A and CA_n28A-n67A, additional FDD+SDL band combinations can be added in the specific basket WI after this WI is completed. - Reuse the time mask requirements defined in Rel-19 for low-band CA via switching as much as possible - Note: Case 2 is specified in Rel-19 WID of NR_LBCA_Sw - Reuse the semi-static switching mechanism based on RRC configuration defined in Rel-19 for low-band CA via switching - Consider the following deployment constraints: - The carrier frequency for all cases is <1GHz. - Co-located and synchronized network deployment for both carriers. - Both carriers are in a single TAG. - SCS 15Khz on both carriers. - No in-gap interference between DL carriers is expected Note 1: Reuse Rel-19 LB-CA RAN4 requirements as much as possible.

Agreements from RAN4 #116bis [R4-2515156]

Issue 1: Work scope clarification - Do not consider the introduction of higher order CA with the LB CA switching as one of fallback configurations and/or different device types of LB CA switching (e.g., con-current DL CA, or FDD+FDD CA switching with concurrent DL CA) as RAN4 did in Rel-19 - Restrict the scope to two-band combinations consisting of one FDD DL carrier and one SDL carrier Issue 2: Common understanding of the UE reference architecture RAN4 has reached the common understanding of the following aspects related to the UE reference architecture: - The main path antenna is shared across Case 1 and Case 3 switching states - The diversity path antenna is shared across Case 1 and Case 3 switching states - UE needs to switch duplex/RF filter in the front-end across Case 1 and Case 3 switching states - An example of the front end filter architecture for the case of adjacent bands is provided below for information Issue 3: Open issues related to the UE reference architecture - Further discuss the feasibility of a combined band n28+n67 DL filter, considering the supported bandwidth and the full band n28 implementation - Whether a restriction of n28 operation to the Region 1 range 703-733/758-788 MHz is relevant for the purpose of agreeing a reference architecture when deriving minimum requirements for the feature in the CA_n28A-n67A band combination - Study careful feasibility of UE Rx filter for n28 DL+ n67 for two cases, one is full band filter (738-803 MHz) and the other is a portion of n28 DL + n67 (738-788 MHz) depending on the outcome of the above - Further discuss front end filter architecture for the case of other LB-LB CA combinations where two DL bands are separated Issue 4: Switching period for switching between Case 1 and Case 3 - Introduce new UE capability to support LB CA via switching between {case 1, case 3} with Dual DL with multiple values for switching period to accommodate different UE implementations - Switching period capability values {35us, 70us, 140us} - The capability shall be per band pair per band combination Issue 5: Common understanding of the switching time mask for switching between Case 1 and Case 3 - Reuse the Rel-19 ON/OFF time mask figure for Rel-20 LB CA switching Issue 6: Open issues related to the switching time mask for switching between Case 1 and Case 3 - Whether/what additional clarifications are needed for the time mask considering DL CA configuration of Case 3 - Whether confirmation by RAN1 is needed on any assumptions made by RAN4 - How to verify for conformance that the UL transient period precedes the first UL symbol after the switch and that it does not include the switching period is challenging using the existing time-mask tests Issue 7: Possible co-existence issues - The preliminary analysis of possible co-existence issues is summarized in the table below for information only (the intention is to give companies a starting point for their analysis) - Companies are encouraged to provide their analysis of the issue in the future meetings to allow RAN4 to confirm the co-existence scenarios and to make progress toward identifying possible mitigation - It is recommended to arrange future discussion of this issue in the Rel-19 maintenance agenda for the NR_LBCA_Sw work item Scenario; Type of interference; CA_n12-n29; CA_n28-n67. UE to UE (UE “A” is transmitting FDD UL and UE “B” is receiving both SDL & FDD DL in dual DL operation); Co-channel; UE “B” may face heavy co-channel interference issue if UE “B” is receiving signal at the first half part of n29 spectrum and UE “A” is transmitting signal at the upper side of n12 UL spectrum. This could be acceptable, given that n29 is supplemental carrier.; The situation of CA_n28-n67 is severer than that of CA_n12-n29. If, however, RAN4 assumes that n28 spectrum is restricted to 703-733 MHz UL and 758-788 MHz for the LB CA, it may be feasible to mitigate the situation thanks to 5 MHz guard band. This also mitigates the challenges in developing UE Rx filter to cover both the entire n28 and n67, since relative frequency ratio of around 8.4% (100*(65/770.5)) decreases to around 6.6% (100*(50/762)). Blocking; Since Rx filter covering both n29 and n12 DL cannot cut n12 UL power out if n12 UL is used at the upper side of n12, both n29 & n12 DL of UE “B” may face a blocking issue. Given that dual DL operation assumes that n12 DL (PCell) is always available, the blocking issue should be carefully handled, though those issues including co-channel are UE to UE so that the actual impact depends on various conditions such as distance between UEs, UL power, RBs position, received DL power, etc.; The situation of CA_n28-n67 is severer than that of CA_n12-n29. Similar to Co-channel interference, 5 MHz guard band can help UE “B” cut the power of n28 UL out. BS to UE / UE to BS; Given that co-location is assumed, all those wouldn’t have an issue.. BS to BS; Co-channel interference and blocking; Since the assumption is collocated scenario, this will cause a big impact on BS performance to receive n12 UL from UEs due to co-channel and blocking from n29 BS DL transmission, given that the guard band is only 1MHz.; The situation of CA_n28-n67 is severer than that of CA_n12-n29. Similar to UE to UE co-existence for CA_n28-n67, 5 MHz guard band can help n28 BS cut the power of n67 BS out and n67 BS cut the interference over n28 UL out.. Issue 8: Impact on RX requirements - Further discuss REFSENS, ΔRIB,c, ACS, in-band blocking, out of band blocking Issue 9: Related to configuration - Further discuss the relationship and/or fallback behavior between Rel-19 low band switching and Rel-20 low band switching