3GPP TSG-RAN WG4 Meeting # 117	R4-2522834

# Dallas, US, November 17th- 21st ,2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR |  | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on 38.101-4 Introducing multiple PMI requirements with 64TX Enhanced TypeII Codebook (2Rx, FDD) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-10-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN4 shall introduce multiple PMI requirements with 64Tx Enhanced TypeII codebook for 2Rx FDD |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce multiple PMI requirements with 64Tx Enhanced TypeII codebook for 2Rx FDD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.1.12(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.3	Reporting of Precoding Matrix Indicator (PMI)

6.3.1	1RX requirements

6.3.2	2RX requirements

6.3.2.1	FDD

6.3.2.1.12	Multiple PMI with 64TX Enhanced Type II Codebook

For the parameters specified in Table 6.3.2.1.12-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.12-2.

Table 6.3.2.1.12-1: Test parameters (dual-layer)

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| Subcarrier spacing |  | kHz | 15 |  |
| Duplex Mode |  |  | FDD |  |
| Propagation channel |  |  | TDLA30-5 |  |
| Antenna configuration |  |  | XP Medium 64 x 2 (N1,N2) = (8,4) |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(5) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 5,(7) |  |
|  | CSI-RS periodicity and offset | slot | 5/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource ID |  | Resource #1 | Resource #2 |
|  | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 32 | 32 |
|  | CDM Type |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17,(2, 4, 6, 8) | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17,(2, 9) | Row 17,(5, 12) |
|  | CSI-RS interval and offset | slot | Not configured | Not configured |
|  | aperiodicTriggeringOffset |  | 0 | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (5,7) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Wideband |  |
| Sub-band Size |  | RB | 8 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 7 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | etypeII-r19 |  |
|  | paramCombination-r19 |  | 6 (L =4, pν =1/2, β=1/2) |  |
|  | R(numberOfPMISubbandsPerCQISubband-r19) |  | 1 |  |
|  | (CodebookConfig-N1, CodebookConfig-N2) |  | (8,4) |  |
|  | (CodebookConfig-O1, CodebookConfig-O2) |  | (4,4) |  |
|  | (valueOfX1-typeII-CBSR-r19, valueOfX2-typeII-CBSR-r19) |  | Not configured |  |
|  | typeII-CBSR-r19 |  | Not configured |  |
|  | RI Restriction (typeII-RI-Restriction-r19) |  | 0010 |  |
|  | portMappingMethod |  | method1 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 11 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | TBD |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-7), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

Table 6.3.2.1.12-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| [SNR(dB) corresponding to 90% of maximum throughput] | TBD |