3GPP TSG-RAN WG4 Meeting #117											R4-2522838

Dallas, USA, November 17th – 21st, 2025

Agenda item:			6.18.1

Source:	Nokia

Title:	WF on NR_XR_Ph3_RRM

Document for:	Approval

# Topic #1: RRM core requirements maintenance

### Issue 1-1-1: Formulas for LTM

<Agreement>

- Formulas for measurement delay extension of L1/L2 triggered mobility in FR2-1 are

| Condition | TL1-RSRP_Measurement_Period_SSB_Inter |
| --- | --- |
| No DRX | Max(Treport, Ceil(Kgap (M*N +Lcancel)) Max(MGRP, SSB period) CSSFinter) |
| DRX cycle ≤ 320 ms | Max(Treport, Ceil(1.5 * Kgap (M*N +Lcancel)) Max(MGRP, SSB period, DRX cycle) CSSFinter) |
| DRX cycle > 320 ms | Ceil(Kgap (M*N +Lcancel)) DRX cycle CSSFinter |

# Topic #2: RRM performance requirements

## Sub-topic 2-1 – Test case list

### Issue 2-1-1: Scenarios for the definition of the test cases

<Agreement>

- this issue is closed.

### Issue 2-1-2: Test target – accuracy requirements

<Agreement>

- Define test case for accuracy requirements for FR1 inter-frequency RSRP.

### Issue 2-1-3: Test target – processing timeline requirements

<Agreement>

- For the Toffset between DCI and start of MG, define a single test case that uses the time offset of 5 ms or 3 ms with UE capability used as a condition.

### Issue 2-1-4: List of test cases – accuracy requirements

<Way foward>

- Option 1: Define accuracy test cases

  - Option 1a: TC at least for FR2-1

  - Option 1b: 1 TC for FR1 and 1 TC for FR2-1

  - Option 1c: Define accuracy requirements for one or both of:

    - SS-RSRP in FR1 and FR2-1

    - SS-RSRQ in FR1 and FR2-1

- Option 2: Do not define test cases for accuracy.

- Option 3: Accuracy is also verified in the measurement procedure test case.

## Sub-topic 2-2 – Test case configuration

### Issue 2-2-1: SSB time index detection

<Agreement>

- For measurement procedure, define test cases without SSB time index detection.

## Sub-topic 2-3 – MG cancellation model

### Issue 2-3-1: Cancelation model for XR test cases

<Agreement>

- Measurement gap cancelation is determined randomly by the test equipment.

### Issue 2-3-2: Cancelation ratio for test cases

<Agreement>

- [20, 30, 40] % of the configured gaps are cancelled, depending on the total number of configured gaps.

## Sub-topic 2-4 – DCI aspects

### Issue 2-4-1: Types of DCI with cancellation indication

<Agreement>

- only DCI 1-1 is used

### Issue 2-4-2: Behaviour of Option 1 and Option 2 for indication of value ‘0’

<Agreement>

- The gap cancellation is verified with a single DCI indication for each cancelled gap.

## Sub-topic 2-5 – CR drafting

### Issue 2-5-1: How to capture gap cancellation model

<Agreement>

- Repeated in every test case.

### Issue 2-5-2: How to include configuration tables based on TC of other releases

<Agreement>

- No repetition of tables, use reference to the test cases which were used as a baseline for the test, and add only different parameters/configurations.

### Issue 2-5-3: Other aspects of CR drafting

<Way forward>

- Companies are encouraged to bring proposals on

  - Exact text for capturing measurement gap cancellation.

  - Exact test for verification that the UE is scheduled during the cancelled gap.

  - Other aspects that may need to be aligned among test cases.