3GPP TSG-RAN WG4 Meeting #116	R4-2511777

# Bangalore, India, 25th – 29th August 2025

Title: 	WF on FDD PC2 duty cycle and measurement channel

Agenda Item:	4.2.2

Source: 	Qualcomm Incorporated

Document for:	Approval

PC2 FDD duty cycle and measurement channel

Background:

In 38.101-1 the maximum output power and configured maximum output power requirements specify a power reduction for PC2 UEs when default duty cycle of 50% is exceeded, unless support for larger duty cycle is signalled. However, no RMC with reduced duty cycle is specified.

On the other hand, there is also conflict between existing TS 38.101-1 and TR 38.861 regarding the applicability of duty cycle based SAR solution to FDD. The conclusion of TR 38.861 is that duty cycle solution is not consensus.  specification error correction in TS 38.101-1 and RAN2 capability description may be required.

Way forward: In future meetings to ensure ran4 specification is correct, choose one of the following alternatives

-	Option 1: Introduce new RMC with up to 50% duty cycle for PC2 FDD, starting point is R4-2511382

-	Option 2: Restrict duty cycle applicability only to TDD, changes needed in 6.2.1 and 6.2.4.

-	Option 3: Reflect and correct the RAN4 specification based on the consensus of the TR 38.861 firstly.