3GPP TSG-RAN WG4 Meeting #116	rev R4-2509892

Bengaluru, India, 25th -29th August, 2025

Source:	Huawei, Hisilicon

Title:	Draft TP to TR 38.719-02-01 include CA_n8-n50

Agenda item:			6.3.4(NR_CADC_R19_2BDL_yBUL)

Document for:	Approval

# Introduction

The revised basket WID was approved in RAN #108 meeting [1]. This contribution provides a text proposal for TR 38.719-02-01 to include CA_n8A-n50A.

# References

[1] RP-251806, Revised WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells, Ericsson, ZTE, Huawei, HiSilicon, June,2025, RAN#108

# Text Proposal

<Start of Text Proposal>

## 5.x	CA_n8-n50

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1:  CA band combination of band n8+n50

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n50 | 1432 MHz – 1517 MHz | 1432 MHz – 1517 MHz | TDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n8+n50

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n50A | CA_n8A-n50A | n8 | n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n50 | n50 channel bandwidths in Table 5.3.5-1 |  |

#### 5.x.1.3	UE co-existence studies for 1 band UL

Table 5.x.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n8-n50.

Table 5.x.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | n8 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 880 | 1760 | 2640 | 3520 | 4400 |  |
| n50 | fLow | fHigh | 915 | 1830 | 2745 | 3660 | 4575 |  |
| DL1 | 1432 | 1517 | N/A | - | - | - | - | UL harmonic |
| DL23 | 2864 | 3034 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 4296 | 4551 | - | - | N/A | - | N/A |  |
| DL4 | 5728 | 6068 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 7160 | 7585 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | There is no collision detected with both harmonic and harmonic mixing. |  |  |  |  |  |
| UL/DL harmonics |  | n50 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow | 1432 | 2864 | 4296 | 5728 | 7160 |  |
| n8 | fLow | fHigh | 1517 | 3034 | 4551 | 6068 | 7585 |  |
| DL1 | 925 | 960 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1850 | 1920 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2775 | 2880 | - | D | N/A | - | N/A |  |
| DL4 | 3700 | 3840 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 4625 | 4800 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Direct hit n50 UL2 / n8 DL3. |  |  |  |  |  |

Based on Table 5.x.1.3-1:

-	There is direct hit n50 UL2 / n8 DL3

Table 5.x.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n8A-n50A.

Table 5.x.1.3-2: Cross-band isolation analysis

| Bands | n8 |  | n50 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 880 | 915 | 1432 | 1517 |
| fDL (MHz) | 925 | 960 | 1432 | 1517 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 20 | 5 | 60 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 860 | 935 | 1372 | 1577 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 840 | 955 | 1312 | 1637 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 820 | 975 | 1252 | 1697 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 800 | 995 | 1192 | 1757 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 780 | 1015 | 1132 | 1817 |
| Analysis | There is no overlap of the band n8 ACLR range with the band n50 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n50 so no cross-band MSD is needed. |  | There is no overlap of the band n50 ACLR range with the band n8 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n8 so no cross-band MSD is needed. |  |

Based on Table 5.x.1.3-2:

There is no uplink band overlapping with the downlink in ACLR range. The existing diplexer isolation should provide sufficient rejection.

#### 5.x.1.4	∆TIB,c and ∆RIB,c values

For CA_n8-n50, the TIB,c and RIB,c of CA_n8-n75 can be reused, and values are given in the tables below.

Table 5.x.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n8-n50 | 0.3 | - |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.x.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n8-n50 | - | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

#### 5.x.1.5	REFSENS requirements

The MSD for H2 caused by n50 UL falls into n8 DL are proposed in the table 5.x.1.5-1:

Table 5.x.1.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic mixing from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n50 | n8 | 5 | 15 | 12 | 5 | 10 | NOTE 2 | UL2/DL3 direct-hit |
| NOTE 2: The requirements should be verified for DL NR-ARFCN of the Victim (lower) band (superscript LB) such that and with the UL carrier frequency and the channel bandwidth configured in the higher band, both in MHz. |  |  |  |  |  |  |  |  |

#### 5.x.1.6	OOB blocking exception requirements

There is no need for OOB blocking exception.

### 5.x.2	Specific for 2 bands UL CA

There is no need for OOB blocking exception.

#### 5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n8A-n50A | 23 | +2/-3 |

#### 5.x.2.2	UE co-existence studies for 2 bands UL

Table 5.x.2.2-1 lists Band n8 + Band n50 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.x.2.2-1: Band n8 and Band n50 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 517 | 637 | 2312 | 2432 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 243 | 398 | 1949 | 2154 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3192 | 3347 | 3744 | 3949 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1123 | 1313 | 3381 | 3671 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 1274 | 1034 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4072 | 4262 | 5176 | 5466 |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 4624 | 4864 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5188 | 4813 | 2228 | 2003 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2791 | 2466 | 119 | 394 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6608 | 6983 | 4952 | 5177 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6056 | 6381 | 5504 | 5779 |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.x.2.2-1, no IMD fall into Rx frequencies of bands n8 or band n50.

For CA_n8-n50, although PHS frequency range 1884.5-1915.7MHz is as pretected band of band n8, due to this combination will not be deployed in the PHS region, there is no need to add PHS frequency range as protected band for CA_n8-n50. Thus no protected bands are needed.

#### 5.x.2.3	REFSENS requirements

There is no need for new REFSENS requirements.

<End of Text Proposal>