3GPP TSG-RAN WG4 Meeting #116	R4-2413518

Bengaluru (Bangalore), India, 25th – 29th August, 2025

Title: 	WF on introduction of PC 1.5 and 2 in bands n100 and n101

Agenda Item:	6.1

Source: 	Nokia

Document for:	Approval

# Agreements:

# Issue 1: Reference PA design

# Use PC1 PA as reference.

# Issue 2: ACLR requirements

# Add a new 37 dB ACLR table for PC 1.5 and PC 2 in bands n100 and n101.

Table 6.5.2.4.1-3: NR ACLR requirement for PC1.5 and PC2 in bands n100 and n101

|  | Power class 1.5 | Power class 2 |
| --- | --- | --- |
| NR ACLR | 37 dB | 37 dB |
| NOTE: PC2 in Band n100 and PC2 and PC1.5 in Band n101 is allowed only for FRMCS cab-radio’s, i.e. user equipment installed on trains with external antenna on top of the train roof at approximately 4m over ground, that also support PC1. |  |  |

# Issue 3: MPR requirements

# TBD between the options below:

# Option 1: Use PC1 MPR.

# Option 2: Use PC1.5 and PC2 MPR.

# Issue 4: A-MPR requirements

# No new A-MPR.

# Issue 5: MSD requirements

# No MSD.