3GPP TSG-RAN WG4 Meeting #116										                     R4-2512133

Bengaluru, India, 25 – 29 August, 2025

Agenda item:			7.21.4

Source:	Huawei, HiSilicon

Title:	WF on RRM requirements for NR_duplex_evo

Document for:	Approval

Agreements

# Topic #1: UE-to-UE CLI handling

    - Sub-topic 1-1: General issues

Issue 1-1-1: Requirement applicability for basic UE operation

- Agreement

  - For UE not supporting simultaneous DL reception in 2 DL subbands, if RAN1 agrees such basic UE operation exists in maintenance phase, RAN4 will further the corresponding requirements in maintenance phase.

Issue 1-1-2: L1 CLI measurement in non-SBFD symbols

- Agreement (Monday online session)

  - Define requirements for L1 UE-to-UE CLI measurements based on valid symbol type.

- Agreement

  - Re-use NR-U approach for defining measurement period extension due to invalid symbol type.

  - Detailed requirements are discussed directly in draftCRs.

Sub-topic 1-2: L1-SRS-RSRP

Issue 1-2-1: Collision between Method #1 and Method #2

- Agreement (Tuesday AH session)

  - UE does not expect to be configured with L1-CLI-RSSI measurements resources within DL subband(s) and L1-SRS-RSRP measurement resources on the same symbol.

  - Note: this can be revised if RAN1 have different agreement for this Collision between Method#1 and Method#2.

Sub-topic 1-3: L1-CLI-RSSI

Issue 1-3-1: Measurement period

- Agreement (Tuesday AH session)

  - Based on 1 sample if timeRestrictionForChannelMeasurement is configured;

  - Based on 3 samples if timeRestrictionForChannelMeasurement is not configured.

Issue 1-3-2: Measurement restriction

- Agreement

  - Measurement requirements do not apply for L1-CLI-RSSI when CLI resources and resources for L1/L3 RRM measurements overlap in time for FR1

# Topic #2: SBFD operation impacts

Sub-topic 2-1: RRM impacts of SBFD operation

Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain

- Agreements (Monday online session)

  - For UE supporting simultaneous DL reception in 2 DL subbands,

    - For measurement period:

      - For UE indicating CSI processing time scaled by 2,

        - Add a lower bound of 8ms on top of existing measurement period, if the CSI-RS resource is across 2 DL subbands and UE indicates CSI processing time is scaled by 2.

      - For UE not indicating CSI processing time scaled by 2,

        - Lower bound is not needed and existing measurement period applies

    - Further discuss the measurement accuracy requirements in Perf part of the WI

      - For UE indicating CSI processing time scaled by 2

        - Option 1: existing accuracy requirements based on 48 RB for Case 3 apply.

        - Option 2: accuracy requirements based on 24 RB for Case 3 apply.

      - For UE not indicating CSI processing time scaled by 2

        - Option 1: accuracy requirements based on 24 RB for Case 3 apply.

  - For UE not supporting simultaneous DL reception in 2 DL subbands, if RAN1 agrees such basic UE operation exists in maintenance phase, RAN4 will further the corresponding requirements in maintenance phase.

Issue 2-1-1d: Lmax in NR-U approach

- Agreements

  - Option 1: define Lmax and related UE behaviour

    - For L1-RSRP and L1-SINR, UE shall/may restart the measurement when L exceeds Lmax.

    - For RLM and BFD, UE shall/may not send any in-sync or out-of-sync indication to higher layers when L exceeds Lmax.

    - For CBD, UE shall/may assume no new candidate beams are found for this evaluation period when L exceeds Lmax.

    - Lmax = 7 for Max(TDRX,TCSI-RS) ≤ 40ms, Lmax=5 for 40ms < Max(TDRX, TCSI-RS) ≤ 320ms, Lmax=3 for TDRX > 320ms

  - Option 2: define absolute time as upper bound for measurement and related UE behaviour

    - For L1-RSRP and L1-SINR, UE shall/may restart the measurement when the measurement period exceeds 5s.

    - For RLM and BFD, UE shall/may not send any in-sync or out-of-sync indication to higher layers when the evaluation period exceeds 5s.

    - For CBD, UE shall/may assume no new candidate beams are found for this evaluation period when the evaluation period exceeds 5s.

  - Option 3: define Lmax as applicability condition of the requirements

    - The requirements for L1-RSRP/L1-SINR/RLM/BFD/CBD apply provided that L does not exceed Lmax

    - Lmax = 7 for Max(TDRX,TCSI-RS) ≤ 40ms, Lmax=5 for 40ms < Max(TDRX, TCSI-RS) ≤ 320ms, Lmax=3 for TDRX > 320ms

Issue 2-1-2: Requirements for RACH

- Agreements

  - Discuss whether and how to update RACH requirements based on RAN2 spec in maintenance phase.

Issue 2-1-3: SCell activation

- Agreements (Monday online session)

  - The existing SCell activation delay requirement apply when activating a SBFD SCell. Further discuss whether any clarification is needed in the CR.

  - Note: the agreement can be revised if not aligned with RAN1 existing and future agreements.

Issue 2-1-4: Condition for SSB based measurement with basic UE operation

- Agreements

  - For UE not supporting simultaneous DL reception in 2 DL subbands, if RAN1 agrees such basic UE operation exists in maintenance phase, RAN4 will further the corresponding requirements in maintenance phase.

# Potential issues for further discussion

Topic #3: Performance part

Sub-topic 3-1: Accuracy requirements

Issue 3-1-1: L1-SRS-RSRP – side condition on time offset

Issue 3-1-2: L1-SRS-RSRP – side condition on SRS BW

Issue 3-1-3: L1-SRS-RSRP – side condition on SRS Es/Iot

Issue 3-1-4: L1-SRS-RSRP – accuracy requirements

Issue 3-1-5: L1-CLI-RSSI – accuracy requirements

Issue 3-1-6: L1-RSRP and L1-SINR – accuracy requirements

Sub-topic 3-2: Report mapping

Issue 3-2-1: Absolute report mapping

Issue 3-2-2: Differential report mapping

Sub-topic 3-3: Test cases

Issue 3-3-1: General test setup

Issue 3-3-2: Test case list

Issue 3-3-3: Test case details and test purpose

Issue 3-3-4: Test case scenarios

Issue 3-3-5: Test applicablity