3GPP TSG-RAN WG4 Meeting #116		R4-25xxxxx

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.29.1

Source:			Moderator (Qualcomm Incorporated)

Title:				WF for [116][229] IoT_NTN_TDD

Document for:	Information

# RRM Core Requirements

## Issue 0-A: Measurement resources

Agreement (online - Wed):

- RRM measurement requirements should be defined based on NRS rather than NSSS for the detected cells.

- For cell measurement in connected mode (8.14A) and RLM (7.23A), the requirement and corresponding test case should be defined such that the capability of NRS monitoring on DL NB-IoT subframes during ongoing UL transmissions is not mandatory.

## Issue 0-B: DRX

Tentative Agreement (ad-hoc):

- No RRM requirements in Rel-19 when DRX cycle is equal to or larger than 5.12s

## Issue 1: Cell reselection (4.6A.2)

Agreement (online - Wed):

- No RRM requirements are defined for scenarios involving WUS.

- No RRM requirements are defined for scenarios involving PUR.

- No RRM requirements are defined for eDRX.

- Define RRC Idle measurement requirements for cell reselection based on Table 4.6A.2.2-1 and Table 4.6A.2.5-1, for intra-frequency and inter-frequency scenarios, respectively.

- Tdetect,NB_Intra -NC, Tmeasure,NB_Intra -NC and Tevaluate, NB_intra -NC for intra-frequency based on Table 4.6A.2.2-1

| DRX cycle length [s] | Tdetect,NB_Intra_NC [s] (number of DRX cycles) | Tmeasure,NB_Intra_NB_NC [s] (number of DRX cycles) | Tevaluate,NB_intra_NB_NC [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 5.12 (16) | 1.28 (4) | 2.56 (8) |
| 0.64 | 5.12 (8) | 1.28 | 2.56 (4) |
| 1.28 | 5.12 (4) | 1.28 (1) | 2.56 (2) |
| 2.56 | 10.24 (4) | 2.56 (1) | 5.12 (2) |
| 5.12 | No requirement in Rel-19 | No requirement in Rel-19 | No requirement in Rel-19 |
| 10.24 | No requirement in Rel-19 | No requirement in Rel-19 | No requirement in Rel-19 |

  - Tdetect,NB_Inter_NC, Tmeasure,NB_Inter_NC and Tevaluate,NB_Inter_NC for inter-frequency based on Table 4.6A.2.5-1

| DRX cycle length [s] | Tdetect,NB_Inter_ NC [s] (number of DRX cycles) | Tmeasure,NB_Inter_ NC [s] (number of DRX cycles) | Tevaluate,NB_Inter_ NC [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 5.12 (16) | 1.28 (4) | 2.56 (8) |
| 0.64 | 5.12 (8) | 1.28 | 2.56 (4) |
| 1.28 | 5.12 (4) | 1.28 (1) | 2.56 (2) |
| 2.56 | 10.24 (4) | 2.56 (1) | 5.12 (2) |
| 5.12 | No requirement in Rel-19 | No requirement in Rel-19 | No requirement in Rel-19 |
| 10.24 | No requirement in Rel-19 | No requirement in Rel-19 | No requirement in Rel-19 |

## Issue 2: Cell measurement in connected mode (8.14A)

Agreement (online - Wed):

- For connected mode channel quality report, reuse the existing requirement

- Intra-frequency serving cell measurement requirement

  - When DRX is not in use

    - Reuse the existing requirements

  - When DRX is in use

    - Reuse the existing value of Tmeasure_intra (s) = 5 DRX cycles

    - No requirement in Rel-19 when DRX cycle is 5.12s and larger.

- Intra-satellite neighbor cell measurement requirements

  - When DRX is not in use:

    - Tdetect_intra_NB1-NC and Tmeasure _intra_NB1-NC are

      - 1.89 sec and 800 ms

  - When DRX is in use:

      - Tdetect_intra_NB1-NC = 10 DRX cycles

      - Tmeasure _intra_NB1-NC = 2 DRX cycles

      - No requirement in Rel-19 when DRX cycle is 5.12s and larger.

      - Scale Tdetect_intra_NB1-NC and Tmeasure _intra_NB1-NC by a factor of 2, when the UE measures cells across 3 or more different patterns of DL opportunities

- Inter-satellite measurements mode and inter-frequency measurement requirement

  - Reuse the existing requirement

  - No requirement in Rel-19 when DRX cycle is 5.12s and larger.

## Issue 3: RRC re-establishment (6.5A)

Agreement (online - Wed):

- Tsearch_NB1-NC,i in RRC re-establishment requirement is

  - If the target cell is known, then Tsearch_NB1-NC,i = 0 ms. If the target cell is unknown, Tsearch_NB1-NC,i = Ksatellite,i *2.52 sec.

  - FFS the definition of Ksatellite,i

## Issue 4: RRC Connection redirection to Non-Anchor Carrier (6.9A)

Agreement (ad-hoc):

- Tperiod_DL_bitmap: It is hardcoded as zero in RRM spec.

To be further discussed and concluded in RAN4#116-bis:

- Change ‘110 ms’ in the following bullets to an integer number of 90ms + DL-to-UL gap within the 90 ms TDD pattern, where the exact value is TBD upon completion of RAN1 spec.

  - TRRC_procedure_delay: It is the RRC procedure for processing the received message “CarrierConfigDedicated-NB”. It shall be less than 110 ms.

  - When the NPUSCH ACK transmission for the received RRC message takes longer than 110ms, the overall RRC connection redirection delay may be extended.

- TBD for the values of 10 ms, 40 ms, and 8 ms in the following bullets, and whether to retain the DL-Bitmap-NB IE upon completion of RAN1 and RAN2 spec:

  - TDL-UL switch: It is the time between the end of the last subframe in the repetition period of NPDCCH received on the non-anchor carrier and the start of the first subframe in the repetition period of the corresponding NPUSCH transmitted on the non-anchor carrier.

    - Option 1: TDL-UL switch is 8 ms.

    - Option 2: TDL-UL switch is 50 ms (THALES, Iridium)

## Issue 5: RLM (7.23A)

Agreement (online - Wed):

- Rmax

  - For the RLM requirement definition, the applicable Rmax should be limited to [2]. Additionally, the ‘Maximum NPDCCH Repetition Level’ should be modified from Rmax/4 in the existing NB-NTN requirements to Rmax/2.

Tentative Agreement (ad-hoc):

- Rmax

  - The RLM requirements in terms of RLF evaluation period are applicable only when the configured Rmax is equal to 2.

  - Rmax is configurable by NW.

- OOS and IS evaluation periods

  - When DRX is not in use,

    - 800 ms and 200 ms for OOS and IS, respectively.

  - When DRX is in use,

    - 8 DRX cycles for 0.256 ≤ DRX cycle ≤ 1.024

    - 4 DRX cycles for 1.024 < DRX cycle ≤ 3.072

    - 3 DRX cycle for 4.096 < DRX cycle < 5.12

## Issue 7: LS from RAN1 on precompensation

Moderator’s note:

- It is under discussion over email thread #316