3GPP TSG-RAN WG4 Meeting #116	R4-2512603

Bengalura, India, Aug 25th – 29th, 2025

Title: 	Way Forward for [116][335] A-IoT_demod

Agenda Item:	7.22.1

Source: 	Huawei, CMCC

Document for:	Approval

1 General

Issue 1-1: Deployment and topology

- Agreement

- The assumed deployment and topology D1T1-B for A-IoT demodulation requirements discussion in R19

  - No need to capture it in RAN4 specification

Issue 1-2: Operation mode

- Agreement

- Standalone and in-band operation for Device 1

  - Same demodulation performance requirements for standalone and in-band operation.

  - How to capture it in the specification can be discussed during the CR drafting if necessary

- Standalone operation for BS

2 Device 1 demodulation requirements

Issue 2-1: Whether to define PRDCH demodulation requirements for Ambient-IoT

- Way forward

- Option 1: Define PRDCH demodulation requirement.

- Option 2: Not define PRDCH demodulation requirements for Ambient-IoT considering that it can be verified by the RF REFSENS requirement

If RAN4 agreed to introduce PRDCH demodulation requirements, further discuss the following open issues:

Issue 2-2: Test methodology

- Agreement

- 10% BLER with BLER=1- Number of responsed Message 1Number of transmitted Message 0  or BLER=1- Number of responsed Message 3Number of transmitted Message 2

for testing

  - During countering the number of responsed Message 1 or 3, the TE should check if the responsed Message 1 or Message 3 is the correct response of Message 0 or Message 2 by checking the device ID

  - Further discuss the message type to be tested

Issue 2-3: Multiplexing

- Agreement

- Single device

Issue 2-4: Antenna configuration

- Agreement

- 1T1R

Issue 2-5: Numerology

- Agreement

- 15kHz SCS

Issue 2-6: R2D message structure for testing

- Agreement

- R-TAS (SIP+CAP) + PRDCH + Preamble

R-TAS	PRDCH	Postamble

Issue 2-7: TBS

- Way forward

- Option 1: 20 bits

- Option 2: 96 bits

Issue 2-8: R2D coding

- Agreement

  - R2D line coding to define the corresponding PRDCH requirement

Issue 2-9: Waveform

- Agreement

  - DFT-s-OFDM with OOK-4

Issue 2-10: M-chips for OOK

- Way forward

  - Option 1: 6

  - Option 2: 24

Issue 2-11: Repetition

- Agreement

  - No repetition configured

Issue 2-12: Channel Bandwidth

- Way forward

  - Option 1: 2 PRBs

  - Option 2: 3 PRBs

Issue 2-13: Channel model

- Agreement

  - TDLA30-10

Issue 2-14: Specification structure

- Way forward

  - Option 1: RAN4 should specify the Ambient IoT device demodulation requirements in “Ambient IoT device radio transmission and reception”

  - Option 2: The demodulation requirements can be captured in clause 10 of TS 38.191

3 BS demodulation requirements

Issue 3-1: Whether to define PDRCH demodulation requirement for Ambient-IoT

- Way forward

  - Option 1: Define PDRCH demodulation requirement.

  - Option 2: Not define PDRCH demodulation requirements

Issue 3-2: D2R message type for testing

- Way forward

  - Option 1: Define PDRCH demodulation requirements with Message 1 reception

  - Option 2: Define PDRCH demodulation requirements with Message 3 reception

Issue 3-3: D2R message structure for testing

- Agreement

  - Option 1: Short D2R transmission without midamble with 31 bits Preamble

  - Option 2: Long D2R transmission with midamble inserted with 31 bits Preamble and 31 bits Midamble

Issue 3-4: Reader detection assumption

- Agreement

  - Coherence detection

Issue 3-5: Antenna configuration

- Agreement

  - Option 1: 1T1R

  - Option 2: 1T2R

Issue 3-6: Numerology

- Agreement

  - 15kHz SCS

Issue 3-7: Modulation

- Way forward

  - Option 1: Both BPSK and OOK modulation scheme

  - Option 2: OOK modulation

Issue 3-8: Waveform

- Way forward

  - Option 1: backscattering a carrier wave (CW)

  - Option 2: Unmodulated single tone (CW)

Issue 3-9: Channel coding and FEC

- Agreement

  - Channel coding with 1/3 Convolutional Coding

Issue 3-10: Repetition

- Agreement

  - Block-level repetition as 2

Issue 3-11: SFO assumption

- Way forward

  - Option 1: 104 ppm~105 ppm

  - Other options

Issue 3-12: Device sampling rate

- Way forward

  - Option 1: 1.92MHz device sampling rate

  - Other options

Issue 3-13: TBS

- Way forward

  - Option 1: 96 bits

  - Other options

Issue 3-14: CBW

- Way forward

  - Option 1: further discuss how to select the proper CBW

  - Option 2: postpone the channel bandwidth and (Tb, Tc, R) discussion until related core part is stable

Issue 3-15: Channel model

- Agreement

  - TDL-A30-10

Issue 3-16: Test methodology

- Agreement

- 10% BLER with BLER=1- Number of responsed Message 0Number of transmitted Message 1  or BLER=1- Number of responsed Message 2Number of transmitted Message 3

for testing

  - During countering the number of responsed Message 0 or 2, the TE should check if the responsed Message 0 or Message 2 is the correct response of Message 1 or Message 3

Issue 3-17: Specification structure

- Way forward

  - Option 1: Specify the Ambient IoT BS demodulation requirements in “Ambient IoT Base Station (BS) and Carrier-Wave (CW) node radio transmission and reception” and “Ambient IoT Base Station (BS) and Carrier-Wave (CW) node conformance testing” if RAN4 agreed to define the Ambient IoT BS demodulation requirements.

  - Option 2: The demodulation requirements can be captured in clause 10 of TS 38.191