3GPP TSG RAN WG4 Meeting #116-bis	R4-2513006

Prague, Czech, 13 - 17 Oct, 2025

3GPP TSG RAN WG1 Meeting #122                                               R1-2506538

Bengaluru, India, 25th – 29th August, 2025

Title:	Reply LS on UL Tx switching for TEI19

Response to:	R4-2505221

Release:	Rel-19

Work Item:	TEI19

Source:	RAN1

To:	RAN4, RAN2

Cc:

Contact Person:	Mohammed Al-Imari

mohammed.al-imari@mediatek.com

Send any reply LS to:	 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# Overall description

RAN1 would like to thank RAN4 for their LS in R4-2505221. RAN1 would like to inform RAN4 and RAN2 that RAN1 reached the following agreement to introduce the functionalities for Scenario#1 in RAN1 specifications:

Agreement:

- A UE capable of Scenario#1 is mandated to support max 2 layers of UL-MIMO on carriers of both bands.

- For a UE capable of Scenario#1, the number of ports of UL-MIMO can be configured as either 1-port or 2-port on any UL carriers as usual. If 1-port is configured on all the carriers of any band, then the UE behaviour follows the normal UL-CA.

- Inform RAN2/RAN4 with the following:

  - The following RRC parameters are used to configure 3Tx Scenario#1;

    - uplinkTxSwitching: existing RRC parameter in R16/R17 UL Tx switching.

    - [uplinkTxSwitching3TxScenario1]: new RRC parameter to configure the UE with 3Tx Scenario#1.

  - The following RRC parameters are not applicable when the UE is configured with [uplinkTxSwitching3TxScenario1];

    - uplinkTxSwitchingOption

    - uplinkTxSwitching-2T-Mode

    - uplinkTxSwitching-DualUL-TxState

  - Note: the following UE capability is assumed to be introduced by RAN4:

    - [uplinkTxSwitchingPeriod3TxScenario1]: UE capability to report the support of 3Tx Scenario#1 and the switching period required for 3Tx Scenario#1

- Adopt the following TP to TS38.214.

< Unchanged parts are omitted > ### 6.1.6 Uplink switching The UE may omit uplink transmission during the uplink switching gap NTx1-Tx2 if the conditions defined in this clause are met and the UE is configured with uplinkTxSwitching or uplinkTxSwitchingMoreBands. The switching gap NTx1-Tx2 is indicated by UE capability uplinkTxSwitchingPeriod2T2T if uplinkTxSwitching-2T-Mode is configured, [uplinkTxSwitchingPeriod3TxScenario1] if [uplinkTxSwitching3TxScenario1] is configured, and uplinkTxSwitchingPeriod otherwise in clauses 6.1.6.1, 6.1.6.2.0, 6.1.6.3, and is determined based on higher layer parameter switchingPeriodConfigForBandPair in clause 6.1.6.2.2 for uplink switching configured with 2, 3 or 4 uplink bands if uplinkTxSwitchingMoreBands is configured: < Unchanged parts are omitted > #### 6.1.6.2 Uplink switching for carrier aggregation ##### 6.1.6.2.0 Uplink switching with two uplink bands For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch, or uplinkTxSwitchingPeriod2T2T, or [uplinkTxSwitchingPeriod3TxScenario1] for a band combination, and if it is for that band combination configured with uplink carrier aggregation: - If the UE is configured with uplink switching with parameter uplinkTxSwitching, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s): - If [uplinkTxSwitching3TxScenario1] is not configured, when the UE is to transmit a 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission is a 1-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. - If [uplinkTxSwitching3TxScenario1] is not configured, when the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission is a 2-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. - For the UE configured with uplinkTxSwitchingOption set to 'switchedUL', when the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission was a 1-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. - For the UE configured with uplinkTxSwitchingOption set to 'dualUL' or with [uplinkTxSwitching3TxScenario1], when the UE is to transmit a 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission was a 1-port transmission on a carrier on the same band and the UE is under the operation state in which 2-port transmission cannot be supported in the same band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. - For the UE configured with uplinkTxSwitchingOption set to 'dualUL', when the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission was a 1-port transmission on another uplink carrier on another band and the UE is under the operation state in which 2-port transmission can be supported in the same band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. - For the UE configured with uplinkTxSwitchingOption set to 'dualUL', if the UE is configured with uplinkTxSwitching-DualUL-TxState set to 'oneT', when the UE is under the operation state in which 2-port transmission can be supported on one carrier on one band followed by no transmission on any carrier on the same band and 1-port transmission on the other carrier on another band the UE shall consider this as if 1-port transmission was transmitted on both uplinks, otherwise the UE shall consider this as if 2-port transmission took place on the transmitting carrier. - If uplinkTxSwitching-2T-Mode or [uplinkTxSwitching3TxScenario1] is configured, when the UE is to transmit a 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission is a 2-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. - If [uplinkTxSwitching3TxScenario1] is configured, when the UE is to transmit a 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission was a 1-port transmission on another uplink carrier on another band and the UE is under the operation state in which 2-port transmission can be supported on the same band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the carriers. - If [uplinkTxSwitching3TxScenario1] is not configured, the UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on one uplink carrier on one band, and any transmission on another uplink carrier on another band. - In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions. < Unchanged parts are omitted > ### 6.2.1 UE sounding procedure < Unchanged parts are omitted > For a UE configured with one or more SRS resource configuration(s), and when the higher layer parameter resourceType in SRS-Resource or SRS-PosResource is set to 'aperiodic': - the UE receives a configuration of SRS resource sets, - the UE receives a downlink DCI, a group common DCI, or an uplink DCI based command where a codepoint of the DCI may trigger one or more SRS resource set(s). For SRS in a resource set with usage set to 'codebook' or 'antennaSwitching', the minimal time interval between the last symbol of the PDCCH triggering the aperiodic SRS transmission and the first symbol of SRS resource is N2 symbols and an additional time duration Tswitch. Otherwise, the minimal time interval between the last symbol of the PDCCH triggering the aperiodic SRS transmission and the first symbol of SRS resource is N2 +14 symbols and an additional time duration Tswitch. The minimal time interval unit of OFDM symbol is counted based on the minimum subcarrier spacing given by min(µPDCCH, µUL) where µUL is given by min(µUL,carrier1, µUL,carrier2, µSRS) when the UE is configured with the higher layer parameter uplinkTxSwitchingOption set to 'dualUL' or configured with [uplinkTxSwitching3TxScenario1] for uplink carrier aggregation, and by µSRS otherwise. µSRS and µPDCCH are the subcarrier spacing configurations for triggered SRS and PDCCH carrying the triggering command respectively. - Tswitch, µUL,carrier1 and µUL,carrier2 are defined in clause 6.4. < Unchanged parts are omitted > ## 6.4 UE PUSCH preparation procedure time If the first uplink symbol in the PUSCH allocation for up to two transport blocks, including the DM-RS, as defined by the slot offset K2 and Koffset, if configured, and the start S and length L of the PUSCH allocation indicated by 'Time domain resource assignment' of the scheduling DCI and including the effect of the timing advance, is no earlier than at symbol L2, where L2 is defined as the next uplink symbol with its CP starting Tproc,2=max(N2+d2,1+d2+d3+d4)(2048+144)⋅κ2-μ⋅TC+Text+Tswitch,d2,2 after the end of the reception of the last symbol of the PDCCH carrying the DCI scheduling the PUSCH, then the UE shall transmit the PUSCH. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the last symbol of the PDCCH carrying the DCI scheduling the PUSCH, the PDCCH candidate that ends later in time is used. < Unchanged parts are omitted > - If uplink switching gap is triggered as defined in clause 6.1.6, equals to the switching gap duration and for the UE configured with higher layer parameter uplinkTxSwitchingOption set to 'dualUL' or configured with [uplinkTxSwitching3TxScenario1] for uplink carrier aggregation µUL=min(µUL,carrier1, µUL,carrier2), otherwise . < Unchanged parts are omitted >

# Actions

RAN1 respectfully asks RAN4 and RAN2 to take the above agreement into consideration for 3Tx UL switching Scenario#1 in TEI-19.

# Dates of next TSG RAN WG1 meetings

TSG RAN WG1 Meeting #122-bis	October 13 – 17, 2025	      Prague, CZ

TSG RAN WG1 Meeting #123	November 17 – 21, 2025	       Dallas, US