3GPP TSG-RAN5 Meeting #109	R5-255638

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

Title:	Discussion on the pass fail requirement of Demodulation Test cases

Source:	MediaTek Inc

Agenda item:	5.4.7.1

Document for:	Discussion and endorsement

1.	Introduction

Pass fail requirement of demodulation TCs are illustrated in each related test specs [1][2][3][4].

Different interpretations of how to judge a DUT pass or fail a demodulation test are observed.

|  | Platform 1 | Platform 2 | Platform 3 |
| --- | --- | --- | --- |
| TE implement of testing tput = 70% TC | 64.13% | 70% | 64.13% |

The criteria should be aligned in conformance testing.

This paper is aiming to address the issue and hopefully consensus can be achieved in the group.

2. 	Discussion

2.1 Judgement of 64.13%

Take 5.2.3.1.1_1 in TS 38.521-4 and 8.2.1.2.2 in TS 38.521-5 as an example.

38.521-4 5.2.3.1.1_1.3.2 Test procedure 1. SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 5.2.3.1.1.0-3 and Table 5.2.3.1.1.0-4. The SS sends downlink MAC padding bits on the DL RMC. 2. Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2.3.1.1_1.4-1 and 5.2.3.1.1_1.4-2 as appropriate. 3. Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5. 4. Repeat steps from 1 to 4 for each subtest in Tables 5.2.3.1.1_1.4-1 and 5.2.3.1.1_1.4-2 as appropriate. G.1.5 Minimum Test time If a pass fail decision in clause G.1.4 can be achieved earlier than the minimum test time, then the test shall not be decided, but continued until the minimum test time is elapsed. G.1.4 Pass Fail limit Testing Throughput = 30 %, then the test limit is Number of successes (ACK) / number of samples ≥ 59 / 233 Testing Throughput = 70 % then the test limit is Number of fails (NACK and statDTX) / number of samples ≤ 66 / 184 Testing BLER = 1% then the test limit is Number of fails (NACK and statDTX) / number of samples ≤ 163 / 13135 There are 3 distinct cases: a) The duration for the number of samples (233, 184 or 13135) is greater than the minimum test time: Then the number of samples (233, 184 or 13135) is predefined and the decision is done according to the number of events (59 successes, 66 fails or 163 fails) b) Since subframe 0 and 5 contain less bits than the remaining subframes, it is allowed to predefine a number of samples contained in an integer number of frames. In this case test-limit-ratio applies. c) The minimum test time is greater than the duration for the number of samples: The minimum test time is predefined and the decision is done comparing the measured ratio at that instant against the test-limit-ratio. NOTE: The test time for most of the tests is governed by the Minimum Test Time.

38.521-5 8.2.1.2.2.1.1_1.3.2 Test procedure 1. UE location according to TS 38.508-1 [12] clause 5.6.1 is provided to the UE through any preconfigured means. 2 Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 38.508[12] table 5.6.2.1-2 for NGSO (LEO-600) if UE supports only NGSO satellites or both GSO and NGSO satellites. Test system shall send same SIB19 information during the duration of the test as defined in clause 8.2.1.2.2.1.1_1.3.3. 3. Deactivate UE prediction of satellite trajectory by any preconfigured means. 4. SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables Table 8.2.1.2.2.1.1-3. The SS sends downlink MAC padding bits on the DL RMC. 5. Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Table 8.2.1.2.2.1.1-2 and Table 8.2.1.2.2.1.1-3 as appropriate. 6. Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Annex G clause G.1.4. 4. Repeat steps from 1 to 3 for each subtest in Table 8.2.1.2.2.1.1-3 as appropriate.

- Example 1: 5.2.3.1.1_1: test procedure -> G.1.5 -> G.1.4.

- Example 2: 8.2.1.2.2: test procedure pointing to G.1.4 directly.

Based on the above highlighted sentences, G.1.5 provides minimum test time while G.1.4 provides the pass fail limit.

According to “Testing Throughput = 70 % then the test limit is”, “Testing tput = 70%” is like a type of test case and the pass limit of this kind of TCs is 1- (66/184) = 0.641 which is implemented in Platform 1 and 3.

Observation 1: Demodulation pass fail requirement 64.1% is derived from G.1.4 Pass Fail limit.

2.2 Judgement of 70%

Another interpretation of the pass fail requirement is,

- Majority of TCs in their test procedure are pointing out to G.1.5 which is min test time and test limit is aligned with RAN4 spec.

38.521-4 Table 5.2.3.1.1_1.4-1: Test Requirement for Rank 1 Test num.; Reference channel; Bandwidth (MHz) / Subcarrier spacing (kHz); Modulation format and code rate; Propagation condition; Correlation matrix and antenna configuration; Reference value. Fraction of maximum throughput (%); SNR (dB). 1-1; R.PDSCH.1-1.1 FDD; 10 / 15; QPSK, 0.30; TDLB100-400; 2x4, ULA Low; 70; -2.6.

- Test limit is not changed in Annex F.

38.521-4 Annex F 55.2.3.1.1_1 4Rx FDD FR1 PDSCH mapping Type A performance - 2x4 MIMO with baseline receiver for both SA and NSA; ± 0.9 dB for > 10Hz doppler ± 1.0 dB for 10Hz doppler; Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. SNR uncertainty due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + SNR uncertainty due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2Tx AWGN flatness and signal flatness ±2.0 dB SNR uncertainty due to finite test time ±0.3 dB for 10Hz Doppler, otherwise ±0.0 dB.

Observation 2: Demodulation pass fail requirement 70% is coming from RAN4 core requirement + RAN5 TT relaxation.

3. 	Conclusion

Based on above discussion, different interpretation and implementation are observed in the test ECO. Aligned and convincible pass fail requirement of demodulation test is required for both LTE and NR.

Observation 1: Demodulation pass fail requirement 64.1% is derived from G.1.4 Pass Fail limit.

Observation 2: Demodulation pass fail requirement 70% is coming from RAN4 core requirement + RAN5 TT relaxation.

Proposal 1: Aligned and convincible pass fail requirement of demodulation test is required for both LTE and NR.

4. 	References

- TS 38.521-4 i80

- TS 38.521-5 i71

- TS 36.521-4 i70

- TS 36.521-1 ia0