3GPP TSG-RAN5 Meeting #109	R5-255643

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

Title:	Discussion on the correctness of ephemeris in RAN5 for NTN

Source:	MediaTek Inc

Agenda item:	5.3.4.8

Document for:	Discussion

1.	Introduction

Based on previous discussion, below ephemeris of both stateVectors format and orbitalParameters format were added in 36.905 for IoT NTN and 38.905 for NR NTN. Those files are generated by GMAT [1][2].

Snapshots are selected from those orbits and captured in 36.508 and 38.508-1 for Rel-17 and Rel-18 TCs [3].

38.508-1

The target of the snapshot is to verify the UE functionality/performance with particular SAT-UE elevation angle, one-way delay as well as doppler. The stateVectors values and orbitalParameters values should provide the same test condition to UE. Unfortunately, orbitalParameters values in current spec may not be correct.

This paper is aiming to address the issue to the group.

2. 	Discussion

According to 38.331,

Ephemeris may be expressed either in format of position and velocity state vector in ECEF or in format of orbital parameters in ECI. Note: The ECI and ECEF coincide at epochTime, i.e., x,y,z axis in ECEF are aligned with x,y,z axis in ECI at epochTime.

In the past, conformance tests are testing with stateVectors ephemeris. Take the ephemeris in section 1 as an example. The related parameters can be deprived based on RAN4 endorsed Keplerian model [4] (The input of this model is ECEF) and shown in below table. Obviously, the calculated angle, delay and doppler can precisely meeting our expectation.

| stateVectors in 38.508-1 (Assumed to be ECEF) | ECEF with physical unit which is the input of the Keplerian model | Calculated parameters based on Keplerian model | Requirement in 38.508-1 |
| --- | --- | --- | --- |
| positionX-r17 -2935552 positionY-r17 4895460 positionZ-r17 1188458 velocityVX-r17 8506 velocityVY-r17 -23314 velocityVZ-r17 118253 | x_ECEF = -3816217.6 m y_ECEF = 6364098 m z_ECEF = 1544995.4 m Vx_ECEF = 510.36 m/s Vy_ECEF = -1398.84 m/s Vz_ECEF = 7095.18 m/s | Angle: 30.0275 degree Delay: 6.6728 ms Doppler: 17.5471 ppm | Angle: 30.03 degree Delay: 6.67 ms Doppler: 17.55 ppm |
| UE location Longitude: 121.56076999, Latitude: 25.08439333 (NGSO satellites), Altitude: 0 |  |  |  |

Observation 1: The angle, delay and doppler derived from stateVectors in TS 38.508-1 and 36.508 can meet the requirement.

However, this is not the case for orbitalParameters.

| orbitalParameters in 38.508-1(Assumed to be ECI) | Transform the orbital from ECI to ECEF which is the input of the Keplerian model | Calculated parameters based on the Keplerian model | Requirement in 38.508-1 |
| --- | --- | --- | --- |
| semiMajorAxis-r17 273626880 eccentricity-r17 769300 periapsis-r17 198830479 longitude-r17 89834718 inclination-r17 65473992 meanAnomaly-r17 7822790 | x_ECEF = -242185.7979 m y_ECEF = 974830.0529 m z_ECEF = -7512807.7439 m Vx_ECEF = -3629.6051 m/s Vy_ECEF = 6233.1498 m/s Vz_ECEF = 911.0448 m/s | Angle: -50.3638 degree Delay: 37.6496 ms Doppler: 13.01 ppm | Angle: 30.03 degree Delay: 6.67 ms Doppler: 17.55 ppm |
| UE location Longitude: 121.56076999, Latitude: 25.08439333 (NGSO satellites), Altitude: 0 |  |  |  |

Observation 2: The angle, delay and doppler derived from orbitalParameters in TS 38.508-1 and 36.508 can NOT meet the requirement.

According to TS 38.133 A.3.36.3, orbitalParameters will be used in some NR NTN RRM tests. It’s required to figure out the correctness of current orbitalParameters.

Above transformation is based on the assumption in 38.331 that all the orbitalParameters are illustrated by ECI. So, we have below conjectures,

- Current orbitalParameters are wrong. -> It’s required to generate correct values.

- Current orbitalParameters are represented in ECEF which are not complied with 38.331. -> Find a formula to transform those parameters from ECEF to ECI.

Proposal 1: Correct orbitalParameters in both TS 38.508-1 and 36.508.

3. 	Conclusion

This paper is aiming to provide verification and hypothesis of current ephemeris. Companies are encouraged to provide your views and proposals.

The following observations and proposals are made:

Observation 1: The angle, delay and doppler derived from stateVectors in TS 38.508-1 and 36.508 can meet the requirement.

Observation 2: The angle, delay and doppler derived from orbitalParameters in TS 38.508-1 and 36.508 can NOT meet the requirement.

Proposal 1: Correct orbitalParameters in both TS 38.508-1 and 36.508.

4. 	References

- R5-237213 GSO satellite Ephemeris file generation methodology for NTN NR UE testing, THALES, RAN5 #101

- R5-237216 NGSO satellite Ephemeris file generation methodology for NTN NR UE testing, THALES, RAN5 #101

- R5-236027 Discussions on IoT NTN test methodology, Mediatek Inc., Keysight Technologies UK Ltd, THALES, RAN5 #101

- R4-2513627 draft big CR for 38.101-5 for NTN NGSO testing, Samsung, Ran4 #116 bis