3GPP TSG RAN WG5 Meeting # 109	R5-255653

# Dallas, Texas, USA, 17 – 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0664 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of applicability for Enhanced receiver type 2 test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom, SGS Wireless |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh3-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The applicability for Enhanced receiver type 2 test cases are FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of TC conditions and applicability for Enhanced receiver type 2 test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The TC for Enahanced receiver type 2 will remain uncompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.0 4.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Guidance to the implementor, new test case conditions Caaa, Cbbb, Fccc, Fddd to be resolved in Table 4.0-1 and Table 4.0-4 by assigning next available number. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

4.0	Test case conditions and selection criteria

For the purposes of the present document, the applicability of conformance test cases conditions given in Table 4.0-1 apply. The tested bands selection criteria given in Table 4.0-2 apply. The tested CA/DC configuration selection criteria given in Table 4.0-3 apply. The subtest selection criteria given in Table 4.0-4 apply. The ICS proformas used in Table 4.0-1, Table 4.0-2, Table 4.0-3 and Table 4.0-4 are defined in TS 38.508-2 [8] unless otherwise stated.

Table 4.0-1: Applicability of conformance test cases conditions

| C001 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 THEN R ELSE N/A |  |
| --- | --- |
| C001a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.1-7/3 THEN R ELSE N/A |  |
| C001b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.5-1/1 THEN R ELSE N/A |  |
| C001c IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.1-2/2e OR A.4.3.1-2/12) THEN R ELSE N/A |  |
| C001d Void |  |
| C001e IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2A.1-1/1 THEN R ELSE N/A |  |
| C001f IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.5-1/1 AND (A.4.3.11-1/1 OR A.4.3.11-1/3) THEN R ELSE N/A |  |
| …… |
| C413 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND ((A.4.1-4A/1 or A.4.1-4A/2) AND A.4.1-4A/5) and A.4.3.2A.1-2/3 and A.4.3.2-1/127a THEN R ELSE N/A |
| C414 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.1-4A/5 and A.4.3.2A.1-2/3 and A.4.3.2-1/127a THEN R ELSE N/A |
| C415 IF A.4.1-1/2 AND A.4.3.2-1/226 AND A.4.3.2-1/41 OR A.4.3.2-1/32 AND A.4.3.2-1/228 OR A.4.3.2-1/229 THEN R ELSE N/A |
| C416 IF A.4.1-1/1 AND A.4.3.1-7a/2 OR A.4.3.1-7a/1 AND A.4.3.2-1/161 THEN R ELSE N/A |
| Caaa IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.1-7a/1 AND NOT A.4.3.1-7a/2 AND A.4.3.2-1/190 THEN R ELSE N/A |
| Cbbb IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND A.4.3.1-7a/2) AND A.4.3.2-1/190 THEN R ELSE N/A |
| NOTE 1: Cxxxx applicability is defined for enhanced type 1 receiver for NR related tests (A.4.3.9-1/1). NOTE 2: Cxxxy applicability is defined for alternative additional DMRS position for co-existence with LTE CRS related tests (A.4.3.2-1/20). NOTE 3: Cxxxz applicability is defined for modified MPR behaviour related test (A.4.3.2-1/25). NOTE 4: Cxxxw applicability is defined for mpr Power Boost related test (A.4.3.2-1/38). |  |

<Unchanged part skipped>

Table 4.0-4: Subtest Selection Criteria for performance and RRM test cases

| Code | Tested Bands Selection Criteria | Comment |
| --- | --- | --- |
| F001 | A.4.3.2-1/9 | UEs supporting CSI-RS based PRACH |
| F002 | A.4.3.5-1/1 | UEs supporting long DRX cycle |
| F003 to F013 | Void |  |
| F014 | A.4.3.6-1/2A AND A.4.3.6-1/92 | UEs supporting independentGapConfig for MRDC and Gap Pattern 13 |
| F015 | A.4.3.5-1/1 AND A.4.3.6-1/2A AND A.4.3.6-1/92 | UEs supporting independentGapConfig for MRDC and long DRX cycle and Gap Pattern 13 |
| F016 | A.4.3.6-1/2 AND A.4.3.6-1/21 | UEs supporting per-FR gap and Gap Pattern 4 |
| F017 | Void |  |
| F018 | Void |  |
| F019 | A.4.3.5-1/1 AND A.4.3.6-1/2 AND A.4.3.6-1/21 | UEs supporting per-FR gap and long DRX cycle and Gap Pattern 4 |
| F020 | Void |  |
| F021 | Void |  |
| F022 | A.4.3.2-1/20 | UE supporting alternative additional DMRS position for co-existence with LTE CRS |
| F023 | A.4.3.2-1/2 | UE supporting 256QAM for PDSCH for FR1 |
| F024 | A.4.3.6-1/70 | UE supporting per-FR NCSG |
| F025 | A.4.3.2-1/213 | UE supporting crs-IM-nonDSS-15kHzSCS-r17 |
| F026 | A.4.3.2-1/214 | UE supporting crs-IM-nonDSS-NWA-15kHzSCS-r17 |
| F027 | A.4.3.2-1/213 OR A.4.3.2-1/215 | UE supporting crs-IM-nonDSS-15kHzSCS-r17 OR crs-IM-nonDSS-30kHzSCS-r17 |
| F028 | A.4.3.2-1/214 OR A.4.3.2-1/216 | UE supporting crs-IM-nonDSS-NWA-15kHzSCS-r17 OR crs-IM-nonDSS-NWA-30kHzSCS-r17 |
| F029 | A.4.3.2-1/203 | UEs supporting simultaneous L1-RSRP measurements for more than one cell when the max RTD among the cells is larger than CP length |
| F030 | Void |  |
| F031 | A.4.3.2-1/179 AND NOT (A.4.3.2-1/178) | UEs supporting MAC-CE activated separate LTM TCI states and do not support MAC-CE activated joint LTM TCI states |
| F032 | A.4.3.2-1/175 AND NOT (A.4.3.2-1/178) | UEs supporting unified TCI with joint DL/UL LTM TCI-state indication for LTM procedure and do not support MAC-CE activated jointstates. |
| F033 | A.4.3.2-1/176 AND NOT (A.4.3.2-1/175 OR A.4.3.2-1/178 OR A.4.3.2-1/179) | UEs supporting unified TCI with separate DL/UL LTM TCI-state indication for LTM procedure and do not support MAC-CE activated joint or separate LTM TCI states and do not support unified TCI with joint DL/UL LTM TCI-state indication for LTM procedure |
| F034-F042 | Void |  |
| F043 | IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/191 AND A.4.3.2-1/179 AND A.4.3.2-1/183 AND A.4.3.2-1/186 AND NOT (A.4.3.2-1/178) AND A.4.3.6-1/102 AND A.4.3.2-1/196 THEN R ELSE N/A | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, MAC-CE activated separate LTM TCI states, RACH-less LTM with dynamic grant, intra-frequency L1-RSRP measurement, reporting based on SSB(s) of candidate cell(s) and do not support MAC-CE activated joint LTM TCI states, interruptions caused by PDCCH order |
| F044 | IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/183 AND A.4.3.2-1/186 AND A.4.3.2-1/191 AND A.4.3.6-1/102 AND A.4.3.2-1/196 THEN R ELSE N/A | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, RACH-less LTM with dynamic grant, intra-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) , interruptions caused by PDCCH order |
| F045 | IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/183 AND A.4.3.2-1/186 AND A.4.3.2-1/191 AND A.4.3.2-1/179 AND NOT (A.4.3.2-1/178) AND A.4.3.6-1/102 THEN R ELSE N/A | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, MAC-CE activated separate LTM TCI states, RACH-less LTM with dynamic grant and do not support MAC-CE activated joint LTM TCI states, interruptions caused by PDCCH order |
| F046 | IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/183 AND A.4.3.2-1/186 AND A.4.3.2-1/191 AND A.4.3.6-1/102 THEN R ELSE N/A | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, RACH-less LTM with dynamic grant, interruptions caused by PDCCH order |
| F047 | IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/183 AND A.4.3.2-1/184 AND A.4.3.2-1/185 AND A.4.3.2-1/186 AND A.4.3.6-1/102 AND A.4.3.2-1/194 THEN R ELSE N/A | UEs supporting 5GS NR SA FR1, intra-frequency LTM for MCG with RACH, RACH-less LTM with dynamic grant and intra-frequency L1-RSRP measurement and reporting based on SSB(s) of candidate cell(s) , interruptions caused by PDCCH order and Support simultaneous transmission to handle the overlap between UL transmission on serving cell(s) and PRACH on candidate cell(s) |
| F048 | IF A.4.3.2-1/224 THEN R ELSE N/A | UE supporting max-HARQ-ProcessNumber-r17 with 32 HARQ processes |
| F049 | IF A.4.3.5-1/18 THEN R ELSE N/A | UE supporting harq-FeedbackDisabled-r17 |
| F050 | A.4.3.2-1/78 AND A.4.3.2-1/56 | UE supporting maxNumberActiveTCI-PerBWP other than n1 and maxSimultaneousResourceSetsPerCC atleast 2 |
| F051 | A.4.3.2-1/178 | UEs supporting MAC-CE activated joint LTM TCI states |
| Fccc | A.4.3.9-1/7 OR A.4.3.9-1/8 | UE supporting Enhanced Type 2 Receiver Type 2-2a OR Enhanced Type 2 Receiver Type 2-2b |
| Fddd | A.4.3.9-1/8 | UE supporting Enhanced Type 2 Receiver Type 2-2b |

<Unchanged part skipped>

4.1.4	Performance conformance test cases

Table 4.1.4-1: Applicability of performance test cases, ref. TS 38.521-4 [4]

| Clause | TC Title | Release | Applicability |  | Tested Bands Selection | Additional Information | Branch | Subtest Selection Criteria |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |  |
| 5 | Demodulation performance requirements (Conducted requirements) |  |  |  |  |  |  |  |
| 5.2 | PDSCH demodulation requirements |  |  |  |  |  |  |  |
| 5.2.1.1.1 | 1Rx FDD FR1 PDSCH performance for RedCap | Rel-17 | C177a | RedCap UEs supporting 5GS FDD FR1 and 1Rx UE capability | D008 |  |  | Subtest 1-4: F023 |
| 5.2.1.1.2 | 1Rx FDD FR1 PDSCH performance for eRedCap | Rel-18 | TBD | eRedCap UEs supporting 5GS FDD FR1 and 1Rx UE capability | TBD | NOTE 1 |  |  |
| 5.2.1.2.1 | 1Rx TDD FR1 PDSCH performance for RedCap | Rel-17 | C177c | RedCap UEs supporting 5GS TDD FR1 and 1Rx UE capability | D009 |  |  | Subtest 1-4: F023 |
| 5.2.1.2.2 | 1Rx TDD FR1 PDSCH performance for eRedCap | Rel-18 | TBD | eRedCap UEs supporting 5GS TDD FR1 and 1Rx UE capability | TBD | NOTE 1 |  |  |
| 5.2.2.1.1_1 | 2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 |  |  | Subtest 1-3: F023 |
| 5.2.2.1.1_2 | 2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with enhanced receiver type 1 for both SA and NSA | Rel-15 | C015x | UEs supporting 5GS FDD FR1 and Enhanced Receiver Type 1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 |  |  |  |
| 5.2.2.1.1_3 | 2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for DL1024QAM for both SA and NSA | Rel-17 | C200 | UEs supporting 5GS FDD FR1 and DL1024QAM and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 |  |  |  |
| 5.2.2.1.1_4 | 2Rx FDD FR1 PDSCH mapping Type A performance – 2x2 MIMO with enhanced DMRS for both SA and NSA | Rel-18 | C341 | UEs supporting 5GS FDD FR1 and Rel-18 enhanced DMRS and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 |  |  |  |
| 5.2.2.1.2_1 | 2Rx FDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 |  |  |  |
| 5.2.2.1.3_1 | 2Rx FDD FR1 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C015b | UEs supporting 5GS FDD FR1 and PDSCH mapping Type B and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 |  |  |  |
| 5.2.2.1.4_1 | 2Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C015y | UEs supporting 5GS FDD FR1 and receiving PDSCH with resource mapping that excludes the REs determined by the higher layer configuration LTE-carrier configuring common RS and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 | Subtest 1-1 execution not necessary if subtest 1-2 is executed. |  | Subtest 1-2: F022 |
| 5.2.2.1.5_1 | 2Rx FDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA | Rel-16 | C074 | UEs supporting 5GS FDD FR1 and alternative 64QAM MCS table for PDSCH and CQI table with target BLER of 10^-5 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 |  |  |  |
| 5.2.2.1.6_1 | 2Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-16 | C120 | UEs supporting 5GS FDD FR1 and aggregationFactorDL > 1 for PDSCH repetition multislots and alternative 64QAM MCS table for PDSCH and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 |  |  |  |
| 5.2.2.1.7_1 | 2Rx FDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-16 | C116 | UEs supporting 5GS FDD FR1 and PDSCH processing capability 2 and PDSCH mapping type B and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 |  |  |  |
| 5.2.2.1.8_1 | 2Rx FDD FR1 PDSCH pre-emption performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-16 | C121 | UEs supporting 5GS FDD FR1 and PDSCH pre-emption indication and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 |  |  |  |
| 5.2.2.1.9_1 | 2Rx FDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C099 | UEs supporting 5GS FDD FR1 and enhanced demodulation processing for HST-SFN joint transmission scheme and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 |  |  |  |
| 5.2.2.1.10_1 | 2Rx FDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 |  |  | Subtest 1-2: F050 |
| 5.2.2.1.11_1 | 2Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA | Rel-16 | C070 | UEs supporting 5GS FDD FR1 and single DCI based spatial division multiplexing scheme and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 |  |  |  |
| 5.2.2.1.12_1 | 2Rx FDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA | Rel-16 | C113 | UEs supporting 5GS FDD FR1 and multi-DCI based multi-TRP and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 |  |  |  |
| 5.2.2.1.13_1 | 2Rx FDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA | Rel-16 | C114 | UEs supporting 5GS FDD FR1 and single DCI based FDMSchemeA and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 |  |  |  |
| 5.2.2.1.14_1 | 2Rx FDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA | Rel-16 | C115 | UEs supporting 5GS FDD FR1 and single-DCI based inter-slot TDM and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 | Test execution not necessary if 5.2.2.1.6_1 is executed. |  |  |
| 5.2.2.1.15_1 | 2Rx FDD FR1 PDSCH with inter-cell interference - 2x2 MIMO for both SA and NSA | Rel-15, Rel-16 | C015d | UEs supporting 5GS FDD FR1 and MMSE-IRC receiver and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 |  |  |  |
|  |  | Rel-17 | C015 | UEs supporting 5GS FDD FR1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 |  |  |  |
| 5.2.2.1.16_1 | 2Rx FDD FR1 for PDSCH with intra cell inter user interference performance for baseline MMSE-IRC receiver – 2x2 MIMO for both NSA and SA | Rel-15, Rel-16 | C015d | UEs supporting 5GS FDD FR1 and MMSE-IRC receiver and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 |  |  |  |
|  |  | Rel-17 | C015 | UEs supporting 5GS FDD FR1 and 2Rx antenna ports, but not supporting FDD bands with 4Rx UE capability | D008 D029 |  |  |  |
| 5.2.2.1.16_2 | 2Rx FDD FR1 for PDSCH with intra cell inter user interference performance for Enhanced Receiver Type 2 – 2x2 MIMO for both NSA and SA | Rel-18 | Caaa | UEs supporting 5GS FDD FR1 and 2Rx antenna ports and advReceiver-MU-MIMO-r18, but not supporting FDD bands with 4Rx UE capability | D008 D029 | Subtest 2-1 execution not necessary if subtest 2-2 is executed. |  | Subtest 2-2: Fccc |
| 5.2.2.1.17 | 2Rx FDD FR1 PDSCH performance for RedCap | Rel-17 | C177b | RedCap UEs supporting 5GS FDD FR1 and 2Rx UE capability | D008 |  |  | Subtest 1-3: F023 |
|  | … | … | … | … |  |  |  |  |
| 5.2.3.1.16_2 | 4Rx FDD FR1 for PDSCH with intra cell inter user interference performance for baseline MMSE-IRC receiver – 4x4 MIMO for both NSA and SA | Rel-15, Rel-16 | C017d | UEs supporting 5GS FDD FR1 and 4Rx antenna ports and MMSE-IRC receiver | D008 |  |  |  |
|  |  | Rel-17 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 |  |  |  |
| 5.2.3.1.16_3 | 4Rx FDD FR1 for PDSCH with intra cell inter user interference performance for Enhanced Receiver Type 2 – 2x4 MIMO for both NSA and SA | Rel-18 | Cbbb | UEs supporting 5GS FDD FR1 and 4Rx antenna ports and advReceiver-MU-MIMO-r18 | D008 | Subtest 3-1 execution not necessary if subtest 3-2 is executed. |  | Subtest 3-2: Fccc |
| 5.2.3.1.16_4 | 4Rx FDD FR1 for PDSCH with intra cell inter user interference performance for Enhanced Receiver Type 2 – 4x4 MIMO for both NSA and SA | Rel-18 | Cbbb | UEs supporting 5GS FDD FR1 and 4Rx antenna ports and advReceiver-MU-MIMO-r18 | D008 | Subtest 4-1 execution not necessary if subtest 4-2 is executed. |  | Subtest 4-2: Fddd |
| 5.2.3.1.17_1 | 4Rx FDD FR1 for PDSCH CRS interference mitigation under NR-LTE coexistence scenario – 2x4 MIMO for both NSA and SA | Rel-17 | C355 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports and supporting CRS-IM-DSS-15kHzSCS-r17 | D008 |  |  |  |
| … | … | … | … | … | … | … |  |  |

<End of change>