3GPP TSG-RAN WG5 Meeting #109	R5-255697

DALLAS, USA, 17th – 21st Nov, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-2 | CR | 0208 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new SLPP position measurement without assistance data test cases for SL-Target UE-assisted mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SLPP positioning procedure test cases are missing now. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adde 9.5.4.10 LMF Initiated Position Measurement without assistance data: SL-Target UE-assisted - Added 9.5.4.11 SL server UE Initiated Position Measurement without assistance data: SL-Target UE-assisted (Target UE is in coverage) - Added 9.5.4.12 SL server UE Initiated Position Measurement without assistance data: SL-Target UE-assisted (Target UE is out of coverage) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The release 18 positioning work item will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5.4.10(new), 9.5.4.11(new), 9.5.4.12(new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.571-3 CR 0178 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

9.5.4.10	LMF Initiated Position Measurement without assistance data: SL-Target UE-assisted

9.5.4.10.1	Test Purpose (TP)

(1)

with { a NAS signalling connection }

ensure that {
  when { UE has no assistance data stored and receives a SLPP location request information message for SL target UE-assisted mode and the UE requires assistance data in order to fulfill the location request }

then { UE sends a SLPP REQUEST ASSISTANCE DATA message followed by a SLPP PROVIDE LOCATION INFORMATION message containing a location measurements }

}

}

9.5.4.10.2	Conformance requirements

As defined in clause 9.5.4.7.2.

9.5.4.10.3	Test description

9.5.4.10.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1.

-	System information combination NR-1 as defined in TS 38.508-1 [30] clause 4.4.3.1.3 is used in NR cell.

NR-SS-UE

-	Sub-test 33: NR-SS-UE1 as specified in clause 8.2.14.

-	Sub-test 34: NR-SS-UE1 as specified in clause 8.2.15.

-	Sub-test 35: NR-SS-UE1 and NR-SS-UE2 as specified in clause 8.2.16.

-	Sub-test 36: NR-SS-UE1 as specified in clause 8.2.17.

-	All NR-SS-UEs use GNSS as the synchronization reference source and operate as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

- None.

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [30], subclause 4.4A on NR Cell 1.

9.5.4.10.3.2	Test procedure sequence

This test case includes sub-test cases dependent on the positioning method(s) supported by the UE. Each sub-test case is identified by a sub-test case number as defined in Table 9.5.4.10.3.2-1 below:

Table 9.5.4.10.3.2-1: Sub-test case numbers

| Sub-Test Case Number | Supported Positioning Methods |
| --- | --- |
| 33 | UE supporting SL-RTT (Rel-18 onwards) |
| 34 | UE supporting SL-AoA (Rel-18 onwards) |
| 35 | UE supporting SL-TDOA (Rel-18 onwards) |
| 36 | UE supporting SL-TOA (Rel-18 onwards) |

Table 9.5.4.10.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS sends a RESET UE POSITIONING STORED INFORMATION message. | <-- | RESET UE POSITIONING STORED INFORMATION | - | - |
| 2 | The SS sends a SLPP message of type Request Capabilities. | <-- | DLInformationTransfer (SLPP REQUEST CAPABILITIES) | - | - |
| 3 | The UE sends a SLPP message of type Provide Capabilities including the UE positioning capabilities. | --> | ULInformationTransfer (SLPP PROVIDE CAPABILITIES) | - | - |
| 4 | IF the UE SLPP message at step 3 includes an acknowledgment request THEN SS sends a SLPP Acknowledgement response. | <-- | DLInformationTransfer (SLPP ACKNOWLEDGEMENT) | - | - |
| 5 | The SS sends a SLPP message of type Request Location Information including a request for location measurements. | <-- | DLInformationTransfer (SLPP REQUEST LOCATION INFORMATION) | - |  |
| 6 | The UE sends a SLPP message of type Request Assistance Data including a request for assistance data. NOTE: This requires a method of triggering a SLPP Request Assistance Data message. | --> | ULInformationTransfer (SLPP REQUEST ASSISTANCE DATA) | 1 | P |
| 7 | The SS sends a SLPP message of type Provide Assistance Data. | <-- | DLInformationTransfer (SLPP PROVIDE ASSISTANCE DATA) | - | - |
| 8 | The UE sends a SLPP message of type Provide Location Information including location measurements. | --> | ULInformationTransfer (SLPP PROVIDE LOCATION INFORMATION) | 1 | P |
| 9 | IF the UE SLPP message at step 8 includes an acknowledgment request THEN SS sends a SLPP Acknowledgement response. | <-- | DLInformationTransfer (SLPP ACKNOWLEDGEMENT) | - | - |

9.5.4.10.3.3	Specific message contents

As defined in clause 9.5.4.7.3.3, with the following exceptions:

Table 9.5.4.10.3.3-1 replaces Table 9.5.4.7.3.3-6, Table 9.5.4.10.3.3-2 replaces Table 9.5.4.7.3.3-9 and Table 9.5.4.10.3.3-3 replaces Table 9.5.4.7.3.3-12.

Table 9.5.4.10.3.3-1: CommonIEsRequestLocationInformation

| Derivation Path: Table 8.5-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 8.5-4 with the following exceptions: |  |  |  |
| CommonIEsRequestLocationInformation ::= SEQUENCE { |  |  |  |
| locationInformationType | locationMeasurementsRequired |  |  |
| } |  |  |  |

Table 9.5.4.10.3.3-2: SLPP REQUEST ASSISTANCE DATA (step 6, Table 9.5.4.7.3.2-2)

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Not present |  |  |
| transactionID | (0..255) | Contains the same value as the corresponding field in SLPP Request Location Information message in step 5, Table 9.5.4.10.3.2-2 |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | (0..255) |  |  |
| sessionID | Not present |  |  |
| acknowledgement SEQUENCE { | Present, or not present. |  |  |
| ackRequested | TRUE |  |  |
| ackIndicator | Not present |  |  |
| } |  |  |  |
| slpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestAssistanceData SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| requestAssistanceData SEQUENCE { |  |  |  |
| commonIEsRequestAssistanceData | Not checked |  |  |
| commonSL-PRS-MethodsIEsRequestAssistanceData | Present. Any value acceptable |  |  |
| sl-AoA-RequestAssistanceData | Not checked |  |  |
| sl-RTT-RequestAssistanceData | Not checked |  |  |
| sl-TDOA-RequestAssistanceData | Present | OCTET STRING containing SL-TDOA-RequestAssistanceData as defined in Table 9.5.4.7.3.3-10 | Sub-test 35 |
| sl-TOA-RequestAssistanceData | Present | OCTET STRING containing SL-TOA-RequestAssistanceData as defined in Table 9.5.4.7.3.3-11 | Sub-test 36 |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |

Table 9.5.4.10.3.3-3: SLPP PROVIDE LOCATION INFORMATION (step 7, Table 9.5.4.10.3.2-2)

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Not present |  |  |
| transactionID | (0..255) | Contains the same value as the corresponding field in SLPP Request Location Information message in step 5, Table 9.5.4.10.3.2-2 |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | (0..255) |  |  |
| sessionID | Not present |  |  |
| acknowledgement SEQUENCE { | Present, or not present. |  |  |
| ackRequested | TRUE |  |  |
| ackIndicator | Not present |  |  |
| } |  |  |  |
| slpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |
| commonIEsProvideLocationInformation | Not present |  |  |
| commonSL-PRS-MethodsIEsProvideLocationInformation | Not checked |  |  |
| sl-AoA-ProvideLocationInformation | Present | OCTET STRING containing SL-AoA-ProvideLocationInformation as defined in Table 9.5.4.4.3.3-3 | Sub-test 34 |
| sl-RTT-ProvideLocationInformation | Present | OCTET STRING containing SL-RTT-ProvideLocationInformation as defined in Table 9.5.4.4.3.3-4 | Sub-test 33 |
| sl-TDOA-ProvideLocationInformation | Present | OCTET STRING containing SL-TDOA-ProvideLocationInformation as defined in Table 9.5.4.4.3.3-5 | Sub-test 35 |
| sl-TOA-ProvideLocationInformation | Present | OCTET STRING containing SL-TOA-ProvideLocationInformation as defined in Table 9.5.4.4.3.3-6 | Sub-test 36 |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |

9.5.4.11	SL server UE Initiated Position Measurement without assistance data: SL-Target UE-assisted (Target UE is in coverage)

9.5.4.11.1	Test Purpose (TP)

(1)

with { Target UE is in coverage and having established PC5 RRC connection with peer UE on unicast sidelink }

ensure that {
  when { UE has no assistance data stored and receives a SLPP location request information message for SL target UE-assisted mode and the UE requires assistance data in order to fulfill the location measurements }

then { UE sends a SLPP REQUEST ASSISTANCE DATA message followed by a SLPP PROVIDE LOCATION INFORMATION message containing location measurements }

}

}

9.5.4.11.2	Conformance requirements

As defined in clause 9.5.4.7.2.

9.5.4.11.3	Test description

9.5.4.11.3.1	Pre-test conditions

System Simulator:

SS-NW

-	NR Cell 1.

-	System information combination NR-36 as defined in TS 38.508-1 [30] clause 4.4.3.1.3 is used in NR cell.

NR-SS-UE

-	Sub-test 33: NR-SS-UE1 as specified in clause 8.2.14.

-	Sub-test 34: NR-SS-UE1 as specified in clause 8.2.15.

-	Sub-test 35: NR-SS-UE1 and NR-SS-UE2 as specified in clause 8.2.16.

-	Sub-test 36: NR-SS-UE1 as specified in clause 8.2.17.

-	All NR-SS-UEs use GNSS as the synchronization reference source and operate as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

-	UE is authorised to perform NR sidelink communication.

-	For V2X capable UE, the UE is equipped with a USIM containing default values as per TS 38.508-1 [30] clause 4.8.3.3.3.

-	For 5G ProSe capable UE, the UE is equipped with a USIM containing default values as per TS 38.508-1 [30] clause 4.8.3.3.4.

-	UE is synchronised on GNSS.

Preamble:

-	The UE is in state 1N-B RRC_IDLE_with_SL and Test Mode (On) with UE test loop mode E as defined in TS 38.508-1 [30], subclause 4.4A using generic procedure parameter Sidelink (On), test mode (On) and GNSS Sync (On) as defined in TS 38.508-1 [30], subclause 4.5.1, using the following procedures:

-	for V2X capable UE, using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22;

-	for 5G ProSe capable UE, using UE initiated unicast mode ProSe Direct communication procedure in subclause 4.9.39.

9.5.4.11.3.2	Test procedure sequence

This test case includes sub-test cases dependent on the positioning method(s) supported by the UE. Each sub-test case is identified by a sub-test case number as defined in Table 9.5.4.11.3.2-1 below:

Table 9.5.4.11.3.2-1: Sub-test case numbers

| Sub-Test Case Number | Supported Positioning Methods |
| --- | --- |
| 33 | UE supporting SL-RTT (Rel-18 onwards) |
| 34 | UE supporting SL-AoA (Rel-18 onwards) |
| 35 | UE supporting SL-TDOA (Rel-18 onwards) |
| 36 | UE supporting SL-TOA (Rel-18 onwards) |

Table 9.5.4.11.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The NR-SS-UE1 sends a RESET UE POSITIONING STORED INFORMATION message. | <-- | RESET UE POSITIONING STORED INFORMATION | - | - |
| 2 | The NR-SS-UE1 sends a SLPP message of type Request Capabilities. | <-- | SLPP REQUEST CAPABILITIES | - | - |
| 3 | The UE sends a SLPP message of type Provide Capabilities including the UE positioning capabilities. | --> | SLPP PROVIDE CAPABILITIES | - | - |
| 4 | IF the UE SLPP message at step 3 includes an acknowledgment request THEN SS sends a SLPP Acknowledgement response. | <-- | SLPP ACKNOWLEDGEMENT | - | - |
| 5 | The NR-SS-UE1 sends a SLPP message of type Request Location Information including a request for location measurements. | <-- | SLPP REQUEST LOCATION INFORMATION | - |  |
| 6 | The UE sends a SLPP message of type Request Assistance Data including a request for assistance data. NOTE: This requires a method of triggering a SLPP Request Assistance Data message. | --> | SLPP REQUEST ASSISTANCE DATA | 1 | P |
| 7 | The NR-SS-UE1 sends a SLPP message of type Provide Assistance Data. | <-- | SLPP PROVIDE ASSISTANCE DATA | - | - |
| 8 | The UE sends a SLPP message of type Provide Location Information including a location measurements. | --> | SLPP PROVIDE LOCATION INFORMATION | 1 | P |
| 9 | IF the UE SLPP message at step 8 includes an acknowledgment request THEN NR-SS-UE1 sends a SLPP Acknowledgement response. | <-- | SLPP ACKNOWLEDGEMENT | - | - |

9.5.4.11.3.3	Specific message contents

As defined in clause 9.5.4.8.3.3, with the following exceptions:

Table 9.5.4.11.3.3-1 replaces Table 9.5.4.8.3.3-7, Table 9.5.4.11.3.2-2 replaces Table 9.5.4.8.3.3-8 and Table 9.5.4.11.3.3-3 replaces Table 9.5.4.8.3.3-12.

Table 9.5.4.11.3.3-1: CommonIEsRequestLocationInformation

| Derivation Path: Table 8.5-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| As defined in Table 8.5-4 with the following exceptions: |  |  |  |
| CommonIEsRequestLocationInformation ::= SEQUENCE { |  |  |  |
| locationInformationType | locationMeasurementsRequired |  |  |
| } |  |  |  |

Table 9.5.4.11.3.3-2: SLPP REQUEST ASSISTANCE DATA (step 6, Table 9.5.4.11.3.2-2)

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Present. Any value acceptable |  |  |
| transactionID | (0..255) | Contains the same value as the corresponding field in SLPP Request Location Information message in step 5, Table 9.5.4.11.3.2-2 |  |
| endTransaction | FALSE |  |  |
| sequenceNumber | (0..255) |  |  |
| sessionID | Present | Contains the same value of the transactionID field in step 2, Table 9.5.4.11.3.2-2. |  |
| acknowledgement SEQUENCE { | Present, or not present. |  |  |
| ackRequested | TRUE |  |  |
| ackIndicator | Not present |  |  |
| } |  |  |  |
| slpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| requestAssistanceData SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| requestAssistanceData SEQUENCE { |  |  |  |
| commonIEsRequestAssistanceData | Not checked |  |  |
| commonSL-PRS-MethodsIEsRequestAssistanceData | Present. Any value acceptable |  |  |
| sl-AoA-RequestAssistanceData | Not checked |  |  |
| sl-RTT-RequestAssistanceData | Not checked |  |  |
| sl-TDOA-RequestAssistanceData | Present | OCTET STRING containing SL-TDOA-RequestAssistanceData as defined in Table 9.5.4.8.3.3-9 | Sub-test 35 |
| sl-TOA-RequestAssistanceData | Present | OCTET STRING containing SL-TOA-RequestAssistanceData as defined in Table 9.5.4.8.3.3-10 | Sub-test 36 |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |

Table 9.5.4.11.3.3-3: SLPP PROVIDE LOCATION INFORMATION (step 8, Table 9.5.4.11.3.2-2)

| Derivation Path: 38.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SLPP-Message ::= SEQUENCE { |  |  |  |
| applicationLayerID | Present. Any value acceptable |  |  |
| transactionID | (0..255) | Contains the same value as the corresponding field in SLPP Request Location Information message in step 5, Table 9.5.4.11.3.2-2 |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | (0..255) |  |  |
| sessionID | Present | Contains the same value of the transactionID field in step 2, Table 9.5.4.11.3.2-2 |  |
| acknowledgement SEQUENCE { | Present, or not present. |  |  |
| ackRequested | TRUE |  |  |
| ackIndicator | Not present |  |  |
| } |  |  |  |
| slpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| provideLocationInformation SEQUENCE { |  |  |  |
| commonIEsProvideLocationInformation | Not present |  |  |
| commonSL-PRS-MethodsIEsProvideLocationInformation | Not checked |  |  |
| sl-AoA-ProvideLocationInformation | Present | OCTET STRING containing SL-AoA-ProvideLocationInformation as defined in Table 9.5.4.4.3.3-3 | Sub-test 34 |
| sl-RTT-ProvideLocationInformation | Present | OCTET STRING containing SL-RTT-ProvideLocationInformation as defined in Table 9.5.4.4.3.3-4 | Sub-test 33 |
| sl-TDOA-ProvideLocationInformation | Present | OCTET STRING containing SL-TDOA-ProvideLocationInformation as defined in Table 9.5.4.4.3.3-5 | Sub-test 35 |
| sl-TOA-ProvideLocationInformation | Present | OCTET STRING containing SL-TOA-ProvideLocationInformation as defined in Table 9.5.4.4.3.3-6 | Sub-test 36 |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |

9.5.4.12	SL server UE Initiated Position Measurement without assistance data: SL-Target UE-assisted (Target UE is out of coverage)

9.5.4.12.1	Test Purpose (TP)

(1)

with { Target UE is out of coverage and having established PC5 RRC connection with peer UE on unicast sidelink }

ensure that {
  when { UE has no assistance data stored and receives a SLPP location request information message for SL target UE-assisted mode and the UE requires assistance data in order to fulfill the location request }

then { UE sends a SLPP REQUEST ASSISTANCE DATA message followed by a SLPP PROVIDE LOCATION INFORMATION message containing a location measurements }

}

}

9.5.4.12.2	Conformance requirements

As defined in clause 9.5.4.7.2.

9.5.4.12.3	Test description

9.5.4.9.12.1	Pre-test conditions

System Simulator:

NR-SS-UE

-	Sub-test 33: NR-SS-UE1 as specified in clause 8.2.14.

-	Sub-test 34: NR-SS-UE1 as specified in clause 8.2.15.

-	Sub-test 35: NR-SS-UE1 and NR-SS-UE2 as specified in clause 8.2.16.

-	Sub-test 36: NR-SS-UE1 as specified in clause 8.2.17.

-	All NR-SS-UEs use GNSS as the synchronization reference source and operate as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

GNSS simulator

-	The GNSS simulator is started and configured for Scenario #1.

UE:

-	UE is authorised to perform NR sidelink communication.

-	For V2X capable UE, the UE is equipped with a USIM containing default values as per TS 38.508-1 [30] clause 4.8.3.3.3.

Table 9.5.4.12.3.1-1: UE/ USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | Service n°119 (V2X) supported |  |
| EFVST |  | As per TS 38.508-1 [30] clause 4.8.3.3.3 |  |
| EFV2XP_PC5 |  | SL-PreconfigurationNR field as defined in TS 38.508-1 [30], table 4.10.1-1 with SLPP condition |  |

-	For 5G ProSe capable UE, the UE is equipped with a USIM containing default values as per TS 38.508-1 [30] clause 4.8.3.3.4 except for SL-PreconfigurationNR with SLPP condition.

Table 9.5.4.12.3.1-2: UE / USIM configuration

| USIM field | Priority | Value | Access Technology Identifier |
| --- | --- | --- | --- |
| EFUST |  | Service n°139 5G ProSe is "available" |  |
| EF5G_PROSE_ST |  | Service n°1, n°2 and n°4 are "available" |  |
| EF5G_PROSE_DD |  | SL-PreconfigurationNR field as defined in TS 38.508-1 [30], table 4.10.1-1 with SLPP condition |  |
| EF5G_PROSE_DC |  | SL-PreconfigurationNR field as defined in TS 38.508-1 [30], table 4.10.1-1 with SLPP condition |  |
| EF5G_PROSE_RU |  | SL-PreconfigurationNR field as defined in TS 38.508-1 [30], table 4.10.1-1 with SLPP condition |  |

-	UE is synchronised on GNSS.

Preamble:

-	The UE is in state 4-A and Test Mode (On), Test Loop Function (Off) as defined in TS 38.508-1 [30], Table 4.5.7.2-1 using generic procedure parameter Sidelink (On),Cast Type (Unicast), UE initiating unicast mode NR sidelink communication, GNSS Sync (On), using the following procedures:

-	for V2X capable UE, using UE initiated unicast mode NR sidelink communication procedure in subclause 4.9.22;

-	for 5G ProSe capable UE, using UE initiated unicast mode ProSe Direct communication procedure in subclause 4.9.39.

9.5.4.12.3.2	Test procedure sequence

As defined in clause 9.5.4.11.3.2.

9.5.4.12.3.3	Specific message contents

As defined in clause 9.5.4.11.3.3.