3GPP TSG-RAN5 Meeting #109	R5-255742

# Dallas, US, 17th – 21st November, 2025

Agenda item:	5.3.16.10

Source:	Keysight Technologies

Title:	Discussion on FR1 MUs for 4Tx test cases

Document for:	Discussion and Endorsement

# Introduction

This contribution aims to continue the MU and TT analysis initiated in [1] for the FR1 4Tx test cases being added under the work plan NR_ENDC_RF_FR1_enh2-UEConTest.

# Discussion

## UE Tx Power Architecture

In proposal 2 endorsed in [1], it was captured that, while not explicitly analysed, MU/TT for 4Tx test cases shall be assumed FFS for UE powers greater than PC1.5 maximum output power.

While doing the analysis of the MU and TT for the pending test cases, we have realized that it is important also to clarify the UE power architecture of the UE Tx branches. While not explicitly analysed, any MU and TT analysis made in [1] and continued in this document will only be valid for UEs implementing PC1.5 using a 4xPC3 power amplifier (PA) architecture.

It is known that such a PA architecture was the first priority considered in the corresponding RAN4 core WID NR_ENDC_ RF_FR1_enh2 and so the known implementations so far.

Proposal 1. While not explicitly analysed, MU/TT for 4Tx test cases shall be assumed FFS for UE implementing 4Tx using a PA architecture other than 4 PC3 Tx branches.

## Test cases analysis

#### Low power tests (in-channel)

- In this subsection the minimum output power tests and the relative power control test will be analyzed given the transmit (ON/)OFF power test cases were already analyzed in [1].

- In the minimum output power test case, after analyzing the performance of our test system, we propose to increase both MU and TT by 0.2dB for f > 3GHz and CBW>40MHz versus the limit defined in corresponding SISO test 6.3.1.

Proposal 2. For the 4Tx minimum output power tests 6.3D.1_2 and 6.3G.1_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT same as defined for 6.3.1 for the sum of power at each of UE antenna connector, except for f > 3GHz and CBW > 40MHz where MU and TT shall be increased by 0.2dB.

- Regarding the relative power control test case, there the minimum output power is also measured. However, the measurement is done as part of a UE power ramp and therefore the test system input path cannot be optimized only for such a low power level. Fortunately, the measurement can be done over chunks of the power ramp in order to allow an optimal configuration of the test system for the power expected, noting the dynamic range of every chuck is smaller than the dynamic range of the entire power ramp.  After analyzing the performance of our test system, we propose to increase both MU and TT by 0.2dB versus the limit defined in corresponding 2Tx UL MIMO relative power control test 6.3D.4.2.

Proposal 3. For the 4Tx relative power control tests 6.3D.4.2_2 and 6.3G.4.2_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT for the relative power measurement 0.2dB higher than defined for 6.3D.4.2 for the sum of power at each of UE antenna connector.

#### Out of band emissions tests

- In this section, the SEM, A-SEM, NR ACLR and UTRA ACLR tests will be analyzed for the 4Tx case.

- In all these test cases, the requirements apply to the sum of the mean power of the UE at each antenna connector. The worst scenario is for the SEM and A-SEM test where differences in the power between the wanted signal and the out of band emission to measure is larger. In that scenario, after analyzing the performance of our test system, we propose to increase MU and TT by 0.3dB for SEM and A-SEM test cases from MU and TT limit already defined in corresponding SISO tests. In case of NR ACLR, we propose an increment of 0.2dB for both MU and TT while for UTRA ACLR we can leverage the MU and TT from the equivalent SISO test.

Proposal 4. For the 4Tx SEM and A-SEM tests 6.5D.2.2_2, 6.5D.2.3_1, 6.5G.2.1_1 and 6.5G.2.2_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT 0.3dB higher than defined for 6.5.2.2 for the sum of power at each of UE antenna connector.

- Proposal 5. For the 4Tx NR ACLR tests 6.5D.2.4.1_2 and 6.5G.2.3.1_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT 0.2dB higher than defined for 6.5.2.4.1 for the sum of power at each of UE antenna connector.

- Proposal 6. For the 4Tx UTRA ACLR tests 6.5D.2.4.2_2 and 6.5G.2.3.2_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT same as defined for 6.5.2.4.2 for the sum of power at each of UE antenna connector.

#### Spurious emissions tests

- Power to measure in these tests is remarkable low, not leaving too much SNR margin for the measurement. The requirements apply to the sum of the mean power of the UE at each antenna connector. That drives into a 6dB SNR degradation due to combination of the signal from 4 measurements, one per antenna connector, what can make the influence of noise increase exponentially if the initial available SNR per connector is not very high. Such an SNR per connector will depend on the maximum output power per connector. For the same spurious requirement to measure, the more the total power in that connector, the harder to measure the spurious because the internal attenuation the measurement device requires to prevent from saturation drives also into an SNR reduction. Therefore, the analysis in this section requires to make assumptions regarding the PA architecture of the UE. The one presented in Proposal 1 will be the one to follow unless otherwise stated.

- It must be observed that currently all spurious test cases, general spurious, additional spurious and spurious co-existence use the same definition of MU, TT is always 0 for these test cases, organized in 4 frequency ranges: 9kHz < f ≤ 3GHz, 3GHz < f ≤ 4GHz, 4GHz < f ≤ 19GHz and 19GHz < f ≤ 26GHz, depending the applicability of the ranges may vary per test.

- After evaluating the 3 mentioned Tx spurious tests, for the 4Tx power architecture assumed in Proposal 1, we propose to increase by 0.5dB the MU defined for the 2 first frequency ranges.

Proposal 7. In 4Tx spurious emission tests 6.5D.3_3.1, 6.5D.3_3.2, 6.5D.3_3.3, 6.5G.3.1_1, 6.5G.3.2_1 and 6.5G.3.3_1, define MU/TT same as defined for 6.5.3.1 for the sum of power at each of UE antenna connector except for f ≤ 4GHz where MU shall be increased by 0.5dB.

# Conclusion

The following observations and proposals were made in this contribution.

Proposal 1. While not explicitly analysed, MU/TT for 4Tx test cases shall be assumed FFS for UE implementing 4Tx using a PA architecture other than 4 PC3 Tx branches.

Proposal 2. For the 4Tx minimum output power tests 6.3D.1_2 and 6.3G.1_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT same as defined for 6.3.1 for the sum of power at each of UE antenna connector, except for f > 3GHz and CBW > 40MHz where MU and TT shall be increased by 0.2dB.

Proposal 3. For the 4Tx relative power control tests 6.3D.4.2_2 and 6.3G.4.2_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT for the relative power measurement 0.2dB higher than defined for 6.3D.4.2 for the sum of power at each of UE antenna connector.

Proposal 4. For the 4Tx SEM and A-SEM tests 6.5D.2.2_2, 6.5D.2.3_1, 6.5G.2.1_1 and 6.5G.2.2_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT 0.3dB higher than defined for 6.5.2.2 for the sum of power at each of UE antenna connector.

Proposal 5. For the 4Tx NR ACLR tests 6.5D.2.4.1_2 and 6.5G.2.3.1_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT 0.2dB higher than defined for 6.5.2.4.1 for the sum of power at each of UE antenna connector.

Proposal 6. For the 4Tx UTRA ACLR tests 6.5D.2.4.2_2 and 6.5G.2.3.2_1, for f ≤ 6GHz and CBW ≤ 100MHz, define MU/TT same as defined for 6.5.2.4.2 for the sum of power at each of UE antenna connector.

Proposal 7. In 4Tx spurious emission tests 6.5D.3_3.1, 6.5D.3_3.2, 6.5D.3_3.3, 6.5G.3.1_1, 6.5G.3.2_1 and 6.5G.3.3_1, define MU/TT same as defined for 6.5.3.1 for the sum of power at each of UE antenna connector except for f ≤ 4GHz where MU shall be increased by 0.5dB.

# References

- R5-254065, “Discussion on FR1 MUs for 4Tx test cases”, Keysight, RAN5#108

- TS 38.521-1, “User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone”, v19.1.0 (2025-09)

- TS 38.101-1, “User Equipment (UE); Radio transmission and reception; Part 1: Range 1 Standalone”, v18.11.0 (2025-09)