3GPP TSG-RAN5 Meeting #109	R5-255775

# Dallas, Texas, USA, 17th - 21st November 2025

Agenda Item:	6.3.27.14

Source:	MCC TF160

Title:	URI to address the UE in a PSAP callback

Document for:	Endorsement

# 1	Introduction

At RAN5#108 clarification has been added to 34.229-1 and 34.229-5 that the PSAP callback in conformance test cases happens using the PDN/PDU session of normal IMS (R5-254971, R5-254978). In the INVITE of the PSAP callback the UE is still addressed by the contact address which has been provided in the INVITE of the eCall. But this contact address belongs to the emergency PDN or PDU session.

# 2	Issue

At eCall establishment the Contact header field of the INVITE typically contains the IP address of the UE as assigned at establishment of the PDN connectivity or PDU session.

In general, this contact address belongs to the emergency PDN or PDU session and it is not a realistic network behaviour to use it to address the UE in the PSAP callback performed on the normal IMS. Instead in a real network the PSAP may use the public user id which it typically gets in the P-Asserted-Id header field.

# 3	Proposal

RAN5 to discuss and decide which URI to use in the INVITE of the PSAP callback to address the UE to achieve a realistic network behaviour. Appropriate candidates seem to be the default public user id or the contact address provided by the UE in the REGISTER for normal IMS.