3GPP TSG-RAN5 Meeting #109	R5-255860

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-2 | CR | 0209 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to A-GNSS Request Location Information for multifrequency |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When running GNSS multifrequency test cases, the A-GNSS Request Location includes a multiFreqMeasReq flag, whose value depends on pics parameters for different signals, QZSS among them. This also materializes in a check for the A-GNSS Provide Location Information, which checks for QZSS measurements if the pics for QZSS are enabled. There is no QZSS scenario defined for 37.571-2 test cases in 3GPP 37.571-5. This means, if a conformant UE supports any QZSS signal, current implementation requires QZSS measurements to be provided from the UE, even if no QZSS satellites are being simulated. This would make a a conformant UE to fail the test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove QZSS pics from MultiFreqAGNSS condition |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.4.2, 7.3.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

Table 7.3.4.2.3.3-11: A-GNSS Request Location Information (step 3, Table 7.3.4.2.3.2-1)

| Derivation Path: Table 5.4-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| As defined in Table 5.4-4 with the following exceptions: |  |  |  |  |
| multiFreqMeasReq |  | TRUE, if UE supports multi frequency GNSS |  | MultiFreqAGNSS |

| Condition | Description |
| --- | --- |
| MultiFreqAGNSS | The UE supports at least one of pc_A_GPS_L2C, pc_A_GPS_L5, pc_GLONASS_G2, pc_GLONASS_G3, pc_GALILEO_E5a, pc_GALILEO_E5b, pc_GALILEO_E6, pc_GALILEO_E5aE5b, pc_BDS_B1C, pc_BDS_B2a or pc_BDS_B3I |

# < Unchanged sections omitted >

# < Unchanged sections omitted >

Table 7.3.4.4.3.3-6: A-GNSS Request Location Information (step 2, Table 7.3.4.4.3.2-1)

| Derivation Path: Table 5.4-4 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| As defined in Table 5.4-4 with the following exceptions: |  |  |  |  |  |  |  |
| assistanceAvailability |  | TRUE |  |  |  |  |  |
| multiFreqMeasReq |  | TRUE, if UE supports multi frequency GNSS |  |  |  | MultiFreqAGNSS |  |

| Condition | Description |
| --- | --- |
| MultiFreqAGNSS | The UE supports at least one of pc_A_GPS_L2C, pc_A_GPS_L5, pc_GLONASS_G2, pc_GLONASS_G3, pc_GALILEO_E5a, pc_GALILEO_E5b, pc_GALILEO_E6, pc_GALILEO_E5aE5b, pc_BDS_B1C, pc_BDS_B2a or pc_BDS_B3I |

# < Unchanged sections omitted >