3GPP TSG-RAN5 Meeting #109	R5-255965

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | 0107 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of band 252 into spurious emission TCs for category M1 and NB-IoT NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_FDD_S_band-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The A-SEM and Spurious Emission requirement for category M1 and NB-IoT NTN band 252 is missing and needs to be added |  |  |  |  |  |  |  |  |
| Summary of change: |  | The A-SEM and Spurious Emission requirement for category M1 and NB-IoT NTN band 252 have been added into clause 6.5A.3.3, 6.5A.4.3, 6.5A.4.4, 6.5B.3.3, 6.5B.4.3 and 6.5B.4.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band 252 A-SEM and Spurious Emission requirement will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5A.3.3, 6.5A.4.3, 6.5A.4.4, 6.5B.3.3, 6.5B.4.3, 6.5B.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | WIC belongs to Rel-19 that will trigger the specific upgrade to Rel-19 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

6.5A.3.3	Additional Spectrum Emission Mask for category M1

6.5A.3.3.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth.

6.5A.3.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category M1 that support satellite access operation.

6.5A.3.3.3	Minimum conformance requirements

When "NS_02N","NS_03N", "NS_06N","NS_07N" or "NS_08N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.3-1.

Table 6.5A.3.3.3-1: Additional requirements for "NS_02N", "NS_03N", "NS_06N", "NS_07N", and "NS_08N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ±0 – 0.7 | -2 for PC3 -5 for PC5 | 4 kHz |
| ±0.7 – 2.8 | -12 for PC3 -15 for PC5 | 4 kHz |
| ±>2.8 | -13 for PC3 and PC5 | 4 kHz |

When "NS_04N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.3-2 for any channel bandwidth configured within 1610-1618.25MHz.

Table 6.5A.3.3.3-2: Additional requirements for "NS_04N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0-160 | -2 | 30kHz |
| ± 160-2300 | -2 to -26 |  |
| ± 2300-18500 | -26 |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |

When "NS_05N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.3.3.3-3 for any channel bandwidth configured within 1618.25-1626.5MHz.

Table 6.5A.3.3.3-3: Additional requirements for "NS_05N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0-160 | -5 | 30kHz |
| ± 160-225 | -5 to -8.5 |  |
| ± 225-650 | -8.5 to -15 |  |
| ± 650-1365 | -15 |  |
| ± 1365-1800 | -23 to -26 |  |
| ± 1800-16500 | -26 |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |

The normative reference for this requirement is TS 36.102 [11] clause 6.5A.3.3.

6.5A.3.3.4	Test description

6.5A.3.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in clause 5.2A. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in Table 6.5A.3.3.4.1-1 to Table 6.5A.3.3.4.1-1a. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.5A.3.3.4.1-1: Test Configuration Table (network signalled value "NS_02N, NS_03N, NS_04N, NS_05N, NS_06N, NS_07N, NS_08N")

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[12] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  |  | 1.4 MHz |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |
|  |  |  |  | FDD and HD-FDD |
| 1 | 1.4 MHz |  | QPSK | 2 |
| 2 | 1.4 MHz |  | QPSK | 5 |
| 3 | 1.4 MHz |  | QPSK | 6 |
| Note 1: The RBstart of partial RB allocation shall be RB#0. |  |  |  |  |

Table 6.5A.3.3.4.1-1a: Test Configuration Table, subPRB allocation (network signalled value “NS_02N, NS_03N, NS_04N, NS_05N, NS_06N, NS_07N, NS_08N”)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 4.3.1 |  |  | Low Range, Mid range, High Range |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  |  | 1.4MHz |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |
|  |  |  |  | FDD and HD-FDD |
| 1 | 1.4 MHz |  | QPSK | ½ |  |
| Note 1: the SCstart shall be 0. |  |  |  |  |  |

1.	Connect SS to the UE antenna connectors as shown in TS 36.508[12] Annex A Figure A.7 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.5A.3.3.4.1-1 to Table 6.5A.3.3.4.1-1a.

5.	Propagation conditions are set according to Annex B.0.

6.	UE location according to TS 36.508 [12] clause 5.6.1 is provided to the UE through any preconfigured means.

7.	Test equipment shall emulate the signal with doppler and delay according to ephemeris defined in TS 36.508 [12] table 5.6.2.1-1 for GSO if UE supports only GSO or both GSO and NGSO satellites and table 5.6.2.1-3 for NGSO (LEO-1200) if UE supports only NGSO satellites. Test system shall send same SIB31 information during the duration of the test as define in TS 36.508[12] clause 5.6.3.1

8.	Deactivate UE prediction of satellite trajectory by any preconfigured means

9.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.5A.3.3.4.3.

6.5A.3.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.5A.3.3.4.1-1 to Table 6.5A.3.3.4.1-1a depending on NS-value. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE to ensure that the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Tables 6.2A.3.5-1 to 6.2A.3.5-1a as appropriate. The period of measurement shall be at least the continuous duration one sub-frame (1ms). For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.5A.3.3.3-1 to 6.5A.3.3.3-3 as appropriate. The centre frequency of the filter shall be stepped in contiguous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active time slots.

6.5A.3.3.4.3	Message contents

Message contents are same as in clause 6.2A.3.4.3.

6.5A.3.3.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Tables 6.2A.3.5-1 to 6.2A.3.5-10a as appropriate, and the power of any UE emission shall fulfil requirements in Table.6.5A.3.3.3-1 to Table 6.5A.3.3.3-3, as applicable.

==============Next change==============

6.5A.4.3	Spurious emission band UE co-existence for category M1

6.5A.4.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions.

6.5A.4.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 17 and forward of UE category M1 that support satellite access operation.

6.5A.4.3.3	Minimum conformance requirements

This clause specifies the requirements for E-UTRA satellite bands for UE coexistence with protected bands.

Table 6.5A.4.3.3-1: Requirements for spurious emissions for UE co-existence

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 252 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 33, 32, 35, 38, 40, 41, 42, 43, 50, 51, 54, 65, 68, 69, 72, 74, 75, 76, 87, 88 NR Band n12, n13, n14, n24, n29, n30, n39, n48, n53, n66, n67, n68, n71, n78, n79, n85, n87, n88, n90, n91, n92, n93, n94, n101, n105, n106, n109, n110 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 253 | E-UTRA Band 5, 26, 31, 41, 48, 72 NR Band n1, n3, n7, n8, n18, n20, n28, n34, n38, n39, n40, n50, n51, n65, n67, n74, n75, n76, n79, n91, n92, n93, n94, n105, n109 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 254 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 31, 41, 48, 54, 66, 70, 71, 72, 85, 87, 88, 103 NR Band n1, n3, n7, n8, n18, n20, n28, n34, n38, n39, n40, n50, n51, n53, n65, n67, n74, n75, n76, n77, n78, n90, n91, n92, n93, n94, n105, n106, n109 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 255 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85, 103 NR Band n1, n3, n7, n8, n18, n20, n28, n34, n38, n39, n40, n50, n51, n53, n65, n67, n74, n75, n76, n90, n91, n92, n93, n94, n105, n106, n109 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 256 | E-UTRA Band 1, 3, 5,7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 33, 32, 35, 38, 40, 41, 42, 43, 50, 51, 54, 65, 68, 69, 72, 74, 75, 76, 87, 88 NR Band n12, n13, n14, n24, n29, n30, n39, n48, n53, n66, n67, n71, n78, n79, n85, n90, n91, n92, n93, n94, n101, n105, n106, n109 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n2, n25, n70 | FDL_low | - | FDL_high | NA | NA | 3 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.4A.2-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5A.4.2-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: The co-existence between 256 and band 2, 25 and 70 is subject to regional/national regulation. |  |  |  |  |  |  |  |

NOTE:	To simplify Table 6.5A.4.3-1, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

The normative reference for this requirement is TS 36.102 [11] subclause 6.5A.4.3.

==============Next change==============

6.5A.4.4	Additional spurious emissions for category M1

6.5A.4.4.0	General

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

NOTE: In addition to the requirements below, additional UE region-specific emissions requirements for European are expected to be added once more information becomes available.

6.5A.4.4.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5A.4.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 13 and forward of UE category M1 that support satellite access operation.

6.5A.4.4.3	Minimum conformance requirements

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

6.5A.4.4.3.1	Minimum requirement (network signalled value "NS_02N")

When "NS_02N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.3.1-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5A.4.2.3-1 from the edge of the channel bandwidth. Network signalling remark NS_02N applies integer-value 2.

Table 6.5A.4.4.3.1-1: Additional requirements for "NS_02N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 1.4MHz |  |  |
| 1559≤ f ≤ 1605 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -50 + 24/5 (f-1605) | 700Hz |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -40 + 24/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

Table 6.5A.4.4.3.1-2: Void

The normative reference for this requirement is TS 36.102[11] subclause 6.5A.4.4.2.

6.5A.4.4.3.2	Minimum requirement (network signalled value "NS_24")

When "NS_24" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.3.2-1.

Table 6.5A.4.4.3.2-1: Additional requirements for "NS_24"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 1.4MHz |  |
| Band 34 | -50 | MHz |
| NOTE 1: This requirement applies at a frequency offset equal or larger than 5 MHz from the upper edge of the channel bandwidth, whenever these frequencies overlap with the specified frequency band. |  |  |

The normative reference for this requirement is TS 36.102[11] subclause 6.5A.4.4.3.

6.5A.4.4.3.3	Minimum requirement (network signalled value "NS_03N")

When "NS_03N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.3.3-1 where BWchannel equals to 1.4MHz. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5A.4.2.3-1 from the edge of the channel bandwidth.

Table 6.5.4.4.3.3-1: Additional out-of-band requirements for "NS_03N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | BWchannel |  |  |
| 1559 ≤ f ≤ 1605 | -50 | 700 Hz | Discreet emissions averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -50 + 60/5 (f-1605) | 700 Hz |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

Table 6.5A.4.4.3.3-2: Void

The normative reference for this requirement is TS 36.102[11] subclause 6.5A.4.4.4.

6.5A.4.4.3.4	Minimum requirement (network signalled value "NS_04N, NS_05N")

When "NS_04N or NS_05N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.3.4-1 where BWchannel equals to 1.4MHz. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5A.4.2.3-1 from the edge of the channel bandwidth.

Table 6.5A.4.4.3.4-1: Additional out-of-band requirements for "NS_04N, NS_05N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | BWchannel |  |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605 ≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| 1628.5 ≤ f ≤ 1631.5 | -30 | 30kHz |  |
| 1631.5 ≤ f ≤ 1636.5 | -30 | 100kHz |  |
| 1636.5 ≤ f ≤ 1646.5 | -30 | 300kHz |  |
| 1646.5 ≤ f ≤ 1666.5 | -30 | 1MHz |  |
| 1666.5 ≤ f ≤ 2200 | -30 | 3MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |  |

Table 6.5A.4.4.3.4-2: Void

6.5A.4.4.3.5	Minimum requirement (network signalled value "NS_06N")

When "NS_06" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.3.5-1.

Table 6.5A.4.4.3.5-1: Additional requirements for "NS_06"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 1.4MHz |  |  |
| 1559 ≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1559 ≤ f < 1610 | -40 | 1MHz |  |

6.5A.4.4.3.6	Minimum requirement (network signalled value "NS_07N")

When "NS_07N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.3.6-1.

Table 6.5A.4.4.3.6-1: Additional requirements for "NS_07N"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 1.4MHz |  |  |
| 1559 ≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1559 ≤ f < 1610 | -40 | 1MHz |  |
| 1930 ≤ f ≤1995 | -40 | 1MHz |  |

6.5A.4.4.3.7	Minimum requirement (network signalled value "NS_08N")

When "NS_08N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.3.7-1.

Table 6.5A.4.4.3.7-1: Additional requirements for "NS_08N"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 1.4MHz |  |  |
| 1559 ≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1559 ≤ f < 1610 | -40 | 1MHz |  |
| 1930 ≤ f ≤1995 | -30 | 1MHz |  |

The normative reference for this requirement is TS 36.102[11] subclause 6.5A.4.4.5

6.5A.4.4.3.5	Void

6.5A.4.4.4	Test description

6.5A.4.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in clause 5.2A. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in Table 6.5A.4.4.4.1-1 to Table 6.5A.4.4.4.1-2a. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.5A.4.4.4.1-1: Test Configuration Table (network signalled value "NS_02N, NS_03N, NS_04N, NS_05N, NS_06N, NS_07N, NS_08N")

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[12] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  |  | 1.4 MHz |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |
|  |  |  |  | FDD and HD-FDD |
| 1 | 1.4 MHz |  | QPSK | 2 |
| 2 | 1.4 MHz |  | QPSK | 5 |
| 3 | 1.4 MHz |  | QPSK | 6 |
| Note 1: The RBstart of partial RB allocation shall be RB#0. |  |  |  |  |

Table 6.5A.4.4.4.1-1a: Test Configuration Table, subPRB allocation (network signalled value “NS_02N, NS_03N, NS_04N, NS_05N, NS_06N, NS_07N, NS_08N”)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 4.3.1 |  |  | Low Range, Mid range, High Range |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  |  | 1.4MHz |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |
|  |  |  |  | FDD and HD-FDD |
| 1 | 1.4 MHz |  | QPSK | ½ |  |
| Note 1: the SCstart shall be 0. |  |  |  |  |  |

6.5B.3.3	Additional Spectrum Emission Mask for category NB1 and NB2

Editor’s note: The following aspects are either missing or not yet determined

-	Tests for network signalling values NS_03N, NS_04N, NS_05N, NS_06N, NS_07N, NS_08N are not complete.

6.5B.3.3.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth.

6.5B.3.3.2	Test applicability

This test case applies to all types of NB-IoT UE release 17 and forward of category NB1 and NB2 that support satellite access operation.

6.5B.3.3.3	Minimum conformance requirements

When "NS_02N", "NS_06N", "NS_07N" or " NS_08N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.3.3.3-1.

Table 6.5B.3.3.3-1: Additional requirements for "NS_02N", "NS_06N", "NS_07N", "NS_08N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0.09–0.28 | -2 for PC3 -5 for PC5 | 4 kHz |
| ± 0.28–0.85 | -12 for PC3 -15 for PC5 | 4 kHz |
| ± >0.85 | -13 for PC3 and PC5 | 4 kHz |

NOTE:	ΔfOOB = 0.09 MHz corresponds to an authorized bandwidth, as defined in C63.26-2015 [22], of 0.38 MHz.

When "NS_03N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.3.3.3-2.

Table 6.5B.3.3.3-2: Additional requirements for "NS_03N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0.09–0.28 | -2 for PC3 -5 for PC5 | 4 kHz |
| ± 0.28–0.85 | -12 for PC3 -15 for PC5 | 4 kHz |
| ± 0.85–1.7 | -13 for PC3 and PC5 | 4 kHz |

NOTE:	ΔfOOB = 0.09 MHz corresponds to an authorized bandwidth, as defined in C63.26-2015 [22], of 0.38 MHz.

When "NS_04N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.3.3.3-3 for any channel bandwidth configured within 1610-1618.25MHz.

Table 6.5B.3.3.3-3: Additional requirements for "NS_04N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0-160 | -2 | 30kHz |
| ± 160-2300 | -2 to -26 |  |
| ± 2300-18500 | -26 |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |

When "NS_05N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.3.3.3-4 for any channel bandwidth configured within 1618.25-1626.5MHz.

Table 6.5B.3.3.3-4: Additional requirements for "NS_05N"

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0-160 | -5 | 30kHz |
| ± 160-225 | -5 to -8.5 |  |
| ± 225-650 | -8.5 to -15 |  |
| ± 650-1365 | -15 |  |
| ± 1365-1800 | -23 to -26 |  |
| ± 1800-16500 | -26 |  |
| NOTE 1: Spectrum emissions are linearly interpolated in dBm versus frequency offset. NOTE 2: The EIRP requirement in regulation is converted to conducted requirement using a 0dBi antenna. |  |  |

6.5B.3.3.4	Test description

6.5B.3.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NB-IoT NTN operating bands specified in sub-clause 5.2B. All these configurations shall be tested with applicable test parameters shown in table 6.5B.3.3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of NPDSCH and NPDCCH before measurement are specified in Annex C.2.

Table 6.5B.3.3.4.1-1: Test Configuration Table (network signalled value “NS_02N”, “NS_03N”, “NS_04N”, “NS_05N”, “NS_06N”, “NS_07N”, “NS_08N”)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | NC |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Low range, Mid range, High range |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A for Maximum Power Reduction (MPR) test case | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 (Note 1) |  | QPSK | 3@0 | 15 |
| 2 (Note 1) |  | QPSK | 3@3 | 15 |
| 3 (Note 1) |  | QPSK | 3@9 | 15 |
| 4 (Note 1) |  | QPSK | 6@0 | 15 |
| 5 (Note 1) |  | QPSK | 6@6 | 15 |
| 6 (Note 1) |  | QPSK | 12@0 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |

==============Next change==============

6.5B.4.3	Spurious emission band UE co-existence for category NB1 and NB2

6.5B.4.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to co-existing systems for the specified bands which has specific requirements in terms of transmitter spurious emissions.

6.5B.4.3.2	Test applicability

This test case applies to all types of NB-IoT UE release 17 and forward of category NB1 and NB2 that support satellite access operation.

6.5B.4.3.3	Minimum conformance requirements

This clause specifies the requirements for E-UTRA satellite bands for UE coexistence with protected bands.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.5B.4.3.3-1: Spurious emission band UE co-existence limits

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) |  | MBW (MHz) | NOTE |
| 252 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 33, 32, 35, 38, 40, 41, 42, 43, 50, 51, 54, 65, 68, 69, 72, 74, 75, 76, 87, 88 NR Band n12, n13, n14, n24, n29, n30, n39, n48, n53, n66, n67, n68, n71, n78, n79, n85, n87, n88, n90, n91, n92, n93, n94, n101, n105, n106, n109, n110 | FDL_low |  | FDL_high |  | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high |  | -50 | 1 | 2 |
| 253 | E-UTRA Band 5, 26, 31, 41, 48, 72 NR Band n1, n3, n7, n8, n18, n20, n28, n34, n38, n39, n40, n50, n51, n65, n67, n74, n75, n76, n79, n91, n92, n93, n94, n105, n109 | FDL_low | - | FDL_high |  | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high |  | -50 | 1 | 2 |
| 254 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 31, 41, 48, 54, 66, 70, 71, 72, 85, 87, 88, 103 NR Band n1, n3, n7, n8, n18, n20, n28, n34, n38, n39, n40, n50, n51, n53, n65, n67, n74, n75, n76, n77, n78, n90, n91, n92, n93, n94, n105, n106, n109 | FDL_low | - | FDL_high |  | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high |  | -50 | 1 | 2 |
| 255 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85, 103 NR Band n1, n3, n7, n8, n18, n20, n28, n34, n38, n39, n40, n50, n51, n53, n65, n67, n74, n75, n76, n90, n91, n92, n93, n94 | FDL_low | - | FDL_high |  | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high |  | -50 | 1 | 2 |
| 256 | E-UTRA Band 1, 3, 5,7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 33, 32, 35, 38, 40, 41, 42, 43, 50, 51, 54, 65, 68, 69, 72, 74, 75, 76, 87, 88 NR Band n12, n13, n14, n24, n29, n30, n39, n48, n53, n66, n67, n71, n78, n79, n85, n90, n91, n92, n93, n94, n101 | FDL_low | - | FDL_high |  | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high |  | -50 | 1 | 2 |
|  | NR Band n2, n25, n70 | FDL_low | - | FDL_high |  | NA | NA | 3 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.4A.2-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5A.4.2-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: The co-existence between 256 and band 2, 25 and 70 is subject to regional/national regulation. |  |  |  |  |  |  |  |  |

NOTE:	To simplify Table 6.5B.4.3.3-1, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation.

The normative reference for this requirement is TS 36.102 [11] subclause 6.5B.4.3.

==============Next change==============

6.5B.4.4	Additional spurious emissions for category NB1 and NB2

6.5B.4.4.0	General

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

NOTE: In addition to the requirements below, additional UE region-specific emissions requirements for European are expected to be added once more information becomes available.

6.5B.4.4.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5B.4.4.2	Test applicability

This test case applies to all types of NB-IoT UE release 17 and forward of category NB1 and NB2 that support satellite access operation.

6.5B.4.4.3	Minimum conformance requirements

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

6.5B.4.4.3.1	Minimum requirement (network signalled value "NS_02N")

When "NS_02N" is indicated in the cell, the power of any UE spurious emission shall not exceed the levels specified in Table 6.5B.4.4.3.1-1 and 6.5B.4.4.3.1-2. This requirement also applies for the frequency ranges that are less than FOOB (MHz) specified in 6.5B.4.2 from the edge of the channel bandwidth. Network signalling remark NS_02N applies integer-value 2.

Table 6.5B.4.4.3.1-1: Additional requirements for "NS_02N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 200kHz |  |  |
| 1559≤ f ≤ 1605 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -50 + 24/5 (f-1605) | 700Hz |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -40 + 24/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

Table 6.5B.4.4.3.1-2: Void

The normative reference for this requirement is TS 36.102 [11] subclauses 6.5B.4.4.2.

6.5B.4.4.3.2	Minimum requirement (network signalled value "NS_24")

When "NS_24" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.4.4.3.2-1.

Table 6.5B.4.4.3.2-1: Additional requirements for "NS_24"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 200kHz |  |
| Band 34 | -50 | 1MHz |
| NOTE 1: This requirement applies at a frequency offset equal or larger than 5 MHz from the upper edge of the channel bandwidth, whenever these frequencies overlap with the specified frequency band. |  |  |

The normative reference for this requirement is TS 36.102 [11] subclauses 6.5B.4.4.3.

6.5B.4.4.3.3	Minimum requirement (network signalled value "NS_03N")

When "NS_03N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5A.4.4.4-1 of TS 36.102 [11] where BWchannel is replaced with 200 kHz, and FOOB (MHz) is replaced with 1.7MHz.

Table 6.5B.4.4.3.3-1: Void

The normative reference for this requirement is TS 36.102 [11] subclauses 6.5B.4.4.4.

6.5B.4.4.3.4	Minimum requirement (network signalled value "NS_04N, NS_05N")

When "NS_04N or NS_05N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in 6.5A.4.4.5 of TS 36.102 [11] where BWchannel is replaced with 200 kHz, and FOOB (MHz) is replaced with 1.7MHz.

The normative reference for this requirement is TS 36.102 [11] subclauses 6.5B.4.4.5.

6.5B.4.4.3.5	Minimum requirement (network signalled value "NS_06N")

When "NS_06N is indicated in the cell, the power of any UE emission shall not exceed the levels specified in 6.5B.4.4.7 of TS 36.102 [11]

The normative reference for this requirement is TS 36.102 [11] subclauses 6.5B.4.4.7.

6.5B.4.4.3.6	Minimum requirement (network signalled value "NS_07N")

When "NS_07N is indicated in the cell, the power of any UE emission shall not exceed the levels specified in 6.5B.4.4.8 of TS 36.102 [11]

The normative reference for this requirement is TS 36.102 [11] subclauses 6.5B.4.4.8.

6.5B.4.4.3.7	Minimum requirement (network signalled value "NS_08N")

When "NS_08N is indicated in the cell, the power of any UE emission shall not exceed the levels specified in 6.5B.4.4.9 of TS 36.102 [11]

The normative reference for this requirement is TS 36.102 [11] subclauses 6.5B.4.4.9.

6.5B.4.4.3.5	Void

6.5B.4.4.4	Test description

6.5B.4.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NB-IOT NTN operating bands specified in sub-clause 5.2B. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 6.5B.4.4.4.1-1 to Table 6.5B.4.4.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of NPDSCH and NPDCCH before measurement are specified in Annex C.2.

Table 6.5B.4.4.4.1-1: Test Configuration Table (network signalled value " NS_02N, NS_03N, NS_04N, NS_05N, NS_06N, NS_07N, NS_08N")

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Low range, Mid range, High range |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A for Maximum Power Reduction (MPR) test case | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 (Note 1) |  | QPSK | 3@0 | 15 |
| 2 (Note 1) |  | QPSK | 3@3 | 15 |
| 3 (Note 1) |  | QPSK | 3@9 | 15 |
| 4 (Note 1) |  | QPSK | 6@0 | 15 |
| 5 (Note 1) |  | QPSK | 6@6 | 15 |
| 6 (Note 1) |  | QPSK | 12@0 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |

==============Next change==============

6.5B.4.4.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Tables 6.2B.3.5-1 to 6.2B.3.5-2 as appropriate.

6.5B.4.4.5.1	Test requirement (network signalled value "NS_02N ")

When "NS_02N" is indicated in the cell, the power of any UE spurious emission shall not exceed the levels specified in Table 6.5B.4.4.5.1-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) specified in 6.5B.4.2 from the edge of the channel bandwidth. Network signalling remark NS_02N applies integer-value 2.

Table 6.5B.4.4.5.1-1: Additional requirements for "NS_02N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 200kHz |  |  |
| 1559≤ f ≤ 1605 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -50 + 24/5 (f-1605) | 700Hz |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -40 + 24/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

Table 6.5B.4.4.5.1-2: Void

6.5B.4.4.5.2	Test requirement (network signalled value "NS_24")

When "NS_24" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.4.4.5.2-1.

Table 6.5B.4.4.5.2-1: Additional requirements for "NS_24"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 200kHz |  |
| Band 34 | -50 | MHz |
| NOTE 1: This requirement applies at a frequency offset equal or larger than 5 MHz from the upper edge of the channel bandwidth, whenever these frequencies overlap with the specified frequency band. |  |  |

6.5B.4.4.5.3	Test requirement (network signalled value "NS_03N ")

When "NS_03N" is indicated in the cell, the power of any UE spurious emission shall not exceed the levels specified in Table 6.5B.4.4.5.3-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) specified in 6.5B.4.2 from the edge of the channel bandwidth.

Table 6.5B.4.4.5.3-1: Additional requirements for "NS_03N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 200kHz |  |  |
| 1559≤ f ≤ 1605 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -50 + 24/5 (f-1605) | 700Hz |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -40 + 24/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

Table 6.5B.4.4.5.3-2: Void

6.5B.4.4.5.4	Test requirement (network signalled value "NS_04N ")

When "NS_04N" is indicated in the cell, the power of any UE spurious emission shall not exceed the levels specified in Table 6.5B.4.4.5.4-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) specified in 6.5B.4.2 from the edge of the channel bandwidth.

Table 6.5B.4.4.5.4-1: Additional requirements for "NS_04N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 200kHz |  |  |
| 1559≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| 1628.5 ≤ f ≤ 1631.5 | -30 | 30kHz |  |
| 1631.5 ≤ f ≤ 1636.5 | -30 | 100kHz |  |
| 1636.5 ≤ f ≤ 1646.5 | -30 | 300kHz |  |
| 1646.5 ≤ f ≤ 1666.5 | -30 | 1MHz |  |
| 1666.5 ≤ f ≤ 2200 | -30 | 3MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

Table 6.5B.4.4.5.4-2: Void

6.5B.4.4.5.5	Test requirement (network signalled value "NS_05N ")

When "NS_05N" is indicated in the cell, the power of any UE spurious emission shall not exceed the levels specified in Table 6.5B.4.4.5.5-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) specified in 6.5B.4.2 from the edge of the channel bandwidth.

Table 6.5B.4.4.5.5-1: Additional requirements for "NS_05N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 200kHz |  |  |
| 1559≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -40 + 60/5 (f-1605) | 1MHz |  |
| 1628.5 ≤ f ≤ 1631.5 | -30 | 30kHz |  |
| 1631.5 ≤ f ≤ 1636.5 | -30 | 100kHz |  |
| 1636.5 ≤ f ≤ 1646.5 | -30 | 300kHz |  |
| 1646.5 ≤ f ≤ 1666.5 | -30 | 1MHz |  |
| 1666.5 ≤ f ≤ 2200 | -30 | 3MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

Table 6.5B.4.4.5.5-2: Void

6.5B.4.4.5.6	Test requirement (network signalled value "NS_06N")

When "NS_06N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.4.4.5.6-1.

Table 6.5B.4.4.5.6-1: Additional requirements for "NS_06N"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 200kHz |  |  |
| 1559 ≤ f < 1610 | -50 | 700 Hz | Averaged over any 2-millisecond active transmission interval |
| 1559 ≤ f < 1610 | -40 | 1MHz |  |

6.5B.4.4.5.7	Test requirement (network signalled value "NS_07N")

When "NS_07N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.4.4.5.7-1.

Table 6.5B.4.4.5.7-1: Additional requirements for "NS_07N"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 200kHz |  |  |
| 1559 ≤ f < 1610 | -50 | 700 Hz | Averaged over any 2-millisecond active transmission interval |
| 1559 ≤ f < 1610 | -40 | 1MHz |  |
| 1930 ≤ f ≤1995 | -40 | 1MHz | Frequency range covers DL of B2/n2 and B25/n25 |

6.5B.4.4.5.8	Test requirement (network signalled value "NS_08N")

When "NS_08N" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5B.4.4.5.8-1.

Table 6.5B.4.4.5.8-1: Additional requirements for "NS_08N"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 200kHz |  |  |
| 1559 ≤ f < 1610 | -50 | 700 Hz | Averaged over any 2-millisecond active transmission interval |
| 1559 ≤ f < 1610 | -40 | 1MHz |  |
| 1930 ≤ f ≤1995 | -40 | 1MHz | Frequency range covers DL of B2/n2 and B25/n25 |

==============End of change==============