3GPP TSG-RAN5 Meeting #109	R5-256140

# Dallas, US, Nov.17th – 21st 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4219 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Annex F for ATG test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To update Annex F to capture MU and TT values for follows ATG RRM test cases: - L1-RSRP reporting TCs 19.5.3.1, 19.5.3.2 - L1-SINR reporting TCs 19.5.4.1, 19.5.4.2, 19.5.4.3 - CGI reporting TC 19.5.5.1 - intra-frequency CSI-RSRP accuracy TCs 19.6.6.1.1, 19.6.6.1.2 - intra-frequency CSI-RSRQ accuracy TC 19.6.7.1 - intra-frequency CSI-SINR accuracy TC 19.6.8.1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - MU and TT values for ATG RRM test cases mentioned above are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.1.2, F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 CR 4213, 4214, 4215, 4216, 4217, 4218 TR 38.903 CR 1106, 1107, 1108, 1109, 1110, 1111 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RRM TT Associated TC CRs: R5-256134, R5-256135, R5-256136, R5-256137, R5-256138, R5-256139 Associated 38.903 CRs: R5-256141, R5-256142, R5-256143, R5-256144, R5-256145, R5-256146 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

F.1.1.2	Measurement of RRM requirements

This clause defines the maximum test system uncertainty for the RRM requirements. The maximum uncertainty values allowed for the typical RRM measurement uncertainty contributors is defined in Table F.1.1.2-1 and Table F.1.1.2-2. Unless explicitly stated for a particular test case, these maximum uncertainty values should be used as starting point to perform the test tolerance analysis in TR 38.903 [22] for each of the test cases. Specific test cases might require a tighter measurement uncertainty value for some of the contributors. Exceptions to the general values in Table F.1.1.2-1 and Table F.1.1.2-2 shall be handled case by case.

<Unchanged contents are omitted>

Table F.1.1.2-17: Maximum test system uncertainty for RRM requirements for SA FR1 Air-to-ground test cases

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 19.2.1.1 Intra-frequency handover from FR1 to FR1; known target cell | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 19.2.1.2 Inter-frequency handover from FR1 to FR1; unknown target cell | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 19.2.2.1 Intra-frequency distance-based conditional Handover from FR1 to FR1 | Same as 6.3.3.1 | Same as 6.3.3.1 |
| 19.2.2.2 Inter-frequency distance-based conditional Handover from FR1 to FR1 | Same as 6.3.3.2 | Same as 6.3.3.2 |
| 19.2.3.1.1 Intra-frequency RRC Re-establishment in FR1 for ATG | Same as 6.3.2.1.1 | Same as 6.3.2.1.1 |
| 19.2.3.1.2 Inter-frequency RRC Re-establishment in FR1 with unknown target cell without serving cell timing for ATG | Same as 6.3.2.1.2 | Same as 6.3.2.1.2 |
| 19.2.3.3.1 NR SA FR1-FR1 RRC connection release with Redirection for ATG | Same as 6.3.2.3.1 | Same as 6.3.2.3.1 |
| 19.5.3.1 NR SA FR1 SSB based L1-RSRP measurement in non-DRX for ATG | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 19.5.3.2 NR SA FR1 CSI-RS based L1-RSRP measurement in non-DRX for ATG | Same as 6.6.4.3 | Same as 6.6.4.3 |
| 19.5.4.1 NR SA FR1 L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured in non-DRX for ATG | Same as 6.6.8.1 | Same as 6.6.8.1 |
| 19.5.4.2 NR SA FR1 L1-SINR measurement with SSB based CMR and dedicated IMR in non-DRX for ATG | Same as 6.6.8.2 | Same as 6.6.8.2 |
| 19.5.4.3 NR SA FR1 L1-SINR measurement with CSI-RS based CMR and dedicated IMR in non-DRX for ATG | Same as 6.6.8.3 | Same as 6.6.8.3 |
| 19.5.5.1 NR SA FR1 CGI identification of NR neighbor cell for ATG | Same as 6.6.7.1 | Same as 6.6.7.1 |
| 19.6.6.1.1 NR SA FR1 CSI-RSRP absolute measurement accuracy for ATG | Average Noc ±1.5 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB SSB PRB Ês1 / Noc ±0.3 dB CSI-RS PRB Ês1 / Noc ±0.3 dB SSB PRB Ês2 / Noc ±0.3 dB CSI-RS PRB Ês2 / Noc ±0.3 dB | Noc is the AWGN on NR freq1 Ês1 / Noc is the cell 1 SNR Ês2 / Noc is the cell 2 SNR SSB PRB is the measurement PRB for associated SSB #RBJ to RBJ+19 CSI-RS PRB is the measurement PRB for CSI-RS #RB0 to RB105 |
| 19.6.6.1.2 NR SA FR1 CSI-RSRP relative measurement accuracy for ATG | Same as 19.6.6.1.1 | Same as 19.6.6.1.1 |
| 19.6.7.1 NR SA FR1 CSI-RSRQ absolute measurement accuracy for ATG | Same as 19.6.6.1.1 | Same as 19.6.6.1.1 |
| 19.6.8.1 NR SA FR1 CSI-SINR absolute measurement accuracy for ATG | Same as 19.6.6.1.1 | Same as 19.6.6.1.1 |
| 19.6.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.1.2.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.1.2.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.2.1 NR SA FR1 SS-RSRQ absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.2.2.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.2.2.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.3.1 NR SA FR1 SS-SINR absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.3.2.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.3.2.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.4.1.1 NR SA FR1 SSB based L1-RSRP absolute measurement accuracy for ATG | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 19.6.4.1.2 NR SA FR1 SSB based L1-RSRP relative measurement accuracy for ATG | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 19.6.4.2.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy for ATG | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 19.6.4.2.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy for ATG | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 19.6.5.1.1 NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG | Same as 6.7.9.1.1 | Same as 6.7.9.1.1 |
| 19.6.5.1.2 NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG | Same as 6.7.9.1.1 | Same as 6.7.9.1.1 |
| 19.6.5.2 NR SA FR1 L1-SINR absolute measurement accuracy with SSB based CMR and dedicated IMR for ATG | Same as 6.7.9.2 | Same as 6.7.9.2 |
| 19.6.5.3.1 NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG | Same as 6.7.9.1.1 | Same as 6.7.9.1.1 |
| 19.6.5.3.2 NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG | Same as 6.7.9.1.1 | Same as 6.7.9.1.1 |

<Unchanged contents are omitted>

F.1.3.2	Measurement of RRM requirements

Because the relationships between the test system uncertainties and the test tolerances are often complex, it is not always possible to give a simple derivation of the test requirement in this document. The analysis is recorded in 3GPP TR 38 903 [22].

<Unchanged contents are omitted>

Table F.1.3.2-15: Derivation of test requirements for RRM requirements for SA FR1 Air-to-ground test cases

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 19.2.1.1 Intra-frequency handover from FR1 to FR1; known target cell | Same as 6.3.1.1 | Same as 6.3.1.1 | Same as 6.3.1.1 |
| 19.2.1.2 Inter-frequency handover from FR1 to FR1; unknown target cell | Same as 6.3.1.3 | Same as 6.3.1.3 | Same as 6.3.1.3 |
| 19.2.2.1 Intra-frequency distance-based conditional Handover from FR1 to FR1 | Same as 6.3.3.1 | Same as 6.3.3.1 | Same as 6.3.3.1 |
| 19.2.2.2 Inter-frequency distance-based conditional Handover from FR1 to FR1 | Same as 6.3.3.2 | Same as 6.3.3.2 | Same as 6.3.3.2 |
| 19.2.3.1.1 Intra-frequency RRC Re-establishment in FR1 for ATG | Same as 6.3.2.1.1 | Same as 6.3.2.1.1 | Same as 6.3.2.1.1 |
| 19.2.3.1.2 Inter-frequency RRC Re-establishment in FR1 with unknown target cell without serving cell timing for ATG | Same as 6.3.2.1.2 | Same as 6.3.2.1.2 | Same as 6.3.2.1.2 |
| 19.2.3.3.1 NR SA FR1-FR1 RRC connection release with Redirection for ATG | Same as 6.3.2.3.1 | Same as 6.3.2.3.1 | Same as 6.3.2.3.1 |
| 19.5.3.1 NR SA FR1 SSB based L1-RSRP measurement in non-DRX for ATG | Same as 6.6.4.1 | Same as 6.6.4.1 | Same as 6.6.4.1 |
| 19.5.3.2 NR SA FR1 CSI-RS based L1-RSRP measurement in non-DRX for ATG | Same as 6.6.4.3 | Same as 6.6.4.3 | Same as 6.6.4.3 |
| 19.5.4.1 NR SA FR1 L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured in non-DRX for ATG | Same as 6.6.8.1 | Same as 6.6.8.1 | Same as 6.6.8.1 |
| 19.5.4.2 NR SA FR1 L1-SINR measurement with SSB based CMR and dedicated IMR in non-DRX for ATG | Same as 6.6.8.2 | Same as 6.6.8.2 | Same as 6.6.8.2 |
| 19.5.4.3 NR SA FR1 L1-SINR measurement with CSI-RS based CMR and dedicated IMR in non-DRX for ATG | Same as 6.6.8.3 | Same as 6.6.8.3 | Same as 6.6.8.3 |
| 19.5.5.1 NR SA FR1 CGI identification of NR neighbor cell for ATG | Same as 6.6.7.1 | Same as 6.6.7.1 | Same as 6.6.7.1 |
| 19.6.6.1.1 NR SA FR1 CSI-RSRP absolute measurement accuracy for ATG | Test configuration 1 & 2 |  |  |
|  | Test 1: Noc: -106dBm/15kHz Cell 1 Es/Noc: 6dB Cell 2 Es/Noc: 1dB Reported Cell 2 CSI-RSRP values: ±6.03dB (+4.5dB for extreme conditions) Test 2: Noc: -88dBm/15kHz Cell 1 Es/Noc: 6dB Cell 2 Es/Noc: 1dB Reported Cell 2 CSI-RSRP values: ±9.53dB (+3dB for extreme conditions) Test 3: Noc: -114 + ΔBG offset dBm/15kHz Cell 1 Es/Noc: 3dB Cell 2 Es/Noc: 0dB Reported Cell 2 CSI-RSRP values: ±6.03dB (+4.5dB for extreme conditions) | Test 1: -1.55dB 0dB 0.40dB via mapping Test 2: 0dB 0dB 0.40dB via mapping Test 3: 0dB 0dB 0dB via mapping | Test 1: Noc: -107.55dBm/15kHz Cell 1 Es/Noc: 6dB Cell 2 Es/Noc: 1.4dB RSRP_44 to RSRP_56 RSRP_40 to RSRP_61 for extreme conditions Test 2: Noc: -88dBm/15kHz Cell 1 Es/Noc: 6dB Cell 2 Es/Noc: 1.4dB RSRP_60 to RSRP_79 RSRP_57 to RSRP_82 for extreme conditions Test 3: Noc: -114+ΔBG offset dBm/15kHz Cell 1 Es/Noc: 3dB Cell 2 Es/Noc: 0dB RSRP_36 to RSRP_49 for BG A RSRP_37 to RSRP_49 for BG B RSRP_37 to RSRP_50 for BG C RSRP_38 to RSRP_50 for BG D RSRP_38 to RSRP_51 for BG E RSRP_39 to RSRP_51 for BG F RSRP_39 to RSRP_52 for BG G RSRP_40 to RSRP_52 for BG H RSRP_43 to RSRP_55 for BG N For extreme conditions: RSRP_32 to RSRP_53 for BG A RSRP_32 to RSRP_54 for BG B RSRP_33 to RSRP_54 for BG C RSRP_33 to RSRP_55 for BG D RSRP_34 to RSRP_55 for BG E RSRP_34 to RSRP_56 for BG F RSRP_35 to RSRP_56 for BG G RSRP_35 to RSRP_57 for BG H RSRP_38 to RSRP_60 for BG N |
|  | Test configuration 3 |  |  |
|  | Test 2: Noc: -94dBm/15kHz Cell 1 Es/Noc: 6dB Cell 2 Es/Noc: 1dB Reported Cell 2 CSI-RSRP values: ±9.53dB (+3dB for extreme conditions) Test 3: Noc: -114 + ΔBG offset dBm/15kHz Cell 1 Es/Noc: 3dB Cell 2 Es/Noc: 0dB Reported Cell 2 CSI-RSRP values: ±6.03dB (+4.5dB for extreme conditions) | Test 2: 0dB 0dB 0.40dB via mapping Test 3: -4.10dB 0dB 0dB via mapping | Test 2: Noc: -94dBm/15kHz Cell 1 Es/Noc: 6dB Cell 2 Es/Noc: 1.4dB RSRP_57 to RSRP_76 RSRP_54 to RSRP_79 for extreme conditions Test 3: Noc: -118.1 +ΔBG offset dBm/15kHz Cell 1 Es/Noc: 3dB Cell 2 Es/Noc: 0dB RSRP_35 to RSRP_47 for BG A RSRP_36 to RSRP_48 for BG C RSRP_37 to RSRP_49 for BG D RSRP_37 to RSRP_49 for BG E For extreme conditions: RSRP_31 to RSRP_52 for BG A RSRP_32 to RSRP_53 for BG C RSRP_32 to RSRP_53 for BG D RSRP_33 to RSRP_54 for BG E |
| 19.6.6.1.2 NR SA FR1 CSI-RSRP relative measurement accuracy for ATG | Test configuration 1 & 2 |  |  |
|  | Test 1: Noc: -106dBm/15kHz Cell 1 Es/Noc: 6dB Cell 2 Es/Noc: 1dB Reported Cell 2 CSI-RSRP – Cell 1 RSRP values: ±3.42dB Test 2: Noc: -88dBm/15kHz Cell 1 Es/Noc: 6dB Cell 2 Es/Noc: 1dB Reported Cell 2 CSI-RSRP – Cell 1 RSRP values: ±3.42dB Test 3: Noc: -114 + ΔBG offset dBm/15kHz Cell 1 Es/Noc: 3dB Cell 2 Es/Noc: 0dB Reported Cell 2 CSI-RSRP – Cell 1 RSRP values: ±3.42dB | Test 1: 0dB 0dB 0.40dB via mapping Test 2: 0dB 0dB 0.40dB via mapping Test 3: 0dB 0dB 0dB via mapping | Test 1: Noc: -106dBm/15kHz Cell 1 Es/Noc: 6dB Cell 2 Es/Noc: 1.4dB RSRP_x - 9 to RSRP_x -2 Test 2: Noc: -88dBm/15kHz Cell 1 Es/Noc: 6dB Cell 2 Es/Noc: 1.4dB RSRP_x - 9 to RSRP_x -2 Test 3: Noc: -114+ΔBG offset dBm/15kHz Cell 1 Es/Noc: 3dB Cell 2 Es/Noc: 0dB RSRP_x - 7 to RSRP_x +1 RSRP_x is the reported Cell 1 CSI-RSRP. |
|  | Test configuration 3 |  |  |
|  | Test 2: Noc: -94dBm/15kHz Cell 1 Es/Noc: 6dB Cell 2 Es/Noc: 1dB Reported Cell 2 CSI-RSRP – Cell 1 RSRP values: ±3.42dB Test 3: Noc: -114 + ΔBG offset dBm/15kHz Cell 1 Es/Noc: 3dB Cell 2 Es/Noc: 0dB Reported Cell 2 CSI-RSRP – Cell 1 RSRP values: ±3.42dB | Test 2: 0dB 0dB 0.40dB via mapping Test 3: 0dB 0dB 0dB via mapping | Test 2: Noc: -94dBm/15kHz Cell 1 Es/Noc: 6dB Cell 2 Es/Noc: 1.4dB RSRP_x - 9 to RSRP_x -2 Test 3: Noc: -114+ΔBG offset dBm/15kHz Cell 1 Es/Noc: 3dB Cell 2 Es/Noc: 0dB RSRP_x - 7 to RSRP_x +1 RSRP_x is the reported Cell 1 CSI-RSRP. |
| 19.6.7.1 NR SA FR1 CSI-RSRQ absolute measurement accuracy for ATG | Test configuration 1 & 2 |  |  |
|  | Test 1: Noc: -85dBm/15kHz Cell 1 Es/Noc: 3dB Cell 2 Es/Noc: 3dB Reported Cell 2 CSI-RSRQ values: ±2.72dB (+1.5dB for extreme conditions) Test 2: Noc: -101dBm/15kHz Cell 1 Es/Noc: -2.9dB Cell 2 Es/Noc: -2.9dB Reported Cell 2 CSI-RSRQ values: ±3.74dB (+1.5dB for extreme conditions) Test 3: Noc: -114 + ΔBG offset dBm/15kHz Cell 1 Es/Noc: -4dB Cell 2 Es/Noc: -4dB Reported Cell 2 CSI-RSRQ values: ±3.74dB (+1.5dB for extreme conditions) | Test 1: -1.55dB 0dB 0dB via mapping Test 2: 0dB 0dB 0dB via mapping Test 3: 0dB 0dB 0dB via mapping | Test 1: Noc: -86.55dBm/15kHz Cell 1 Es/Noc: 3dB Cell 2 Es/Noc: 3dB RSRQ_52 to RSRP_62 RSRQ_49 to RSRP_65 for extreme conditions Test 2: Noc: -101dBm/15kHz Cell 1 Es/Noc: -2.9dB Cell 2 Es/Noc: -2.9dB RSRQ_46 to RSRP_60 RSRQ_45 to RSRP_61 for extreme conditions Test 3: Noc: -114+ΔBG offset dBm/15kHz Cell 1 Es/Noc: -4dB Cell 2 Es/Noc: -4dB RSRQ_44 to RSRP_59 RSRQ_43 to RSRP_60 for extreme conditions |
|  | Test configuration 3 |  |  |
|  | Test 1: Noc: -91dBm/15kHz Cell 1 Es/Noc: 3dB Cell 2 Es/Noc: 3dB Reported Cell 2 CSI-RSRQ values: ±2.72dB (+1.5dB for extreme conditions) Test 3: Noc: -114 + ΔBG offset dBm/15kHz Cell 1 Es/Noc: -4dB Cell 2 Es/Noc: -4dB Reported Cell 2 CSI-RSRQ values: ±3.74dB (+1.5dB for extreme conditions) | Test 1: -1.55dB 0dB 0dB via mapping Test 3: 0dB 0dB 0dB via mapping | Test 1: Noc: -92.55dBm/15kHz Cell 1 Es/Noc: 3dB Cell 2 Es/Noc: 3dB RSRQ_52 to RSRP_62 RSRQ_49 to RSRP_65 for extreme conditions Test 3: Noc: -114+ΔBG offset dBm/15kHz Cell 1 Es/Noc: -4dB Cell 2 Es/Noc: -4dB RSRQ_44 to RSRP_59 RSRQ_43 to RSRP_60 for extreme conditions |
| 19.6.8.1 NR SA FR1 CSI-SINR absolute measurement accuracy for ATG | Test 1: Noc: -93dBm/15kHz Cell 1 Es/Noc: 4.54dB Cell 2 Es/Noc: 2.66dB Reported Cell 2 CSI-RSRQ values: ±4.40dB (+0.5dB for extreme conditions) Test 2: Noc: -116 + ΔBG offset dBm/15kHz Cell 1 Es/Noc: -4dB Cell 2 Es/Noc: -4dB Reported Cell 2 CSI-RSRQ values: ±4.12dB (+0.5dB for extreme conditions) | Test 1: -0.15dB 0dB 0dB via mapping Test 2: 0dB 0.45dB 0.45dB via mapping | Test 1: Noc: -93.15dBm/15kHz Cell 1 Es/Noc: 4.54dB Cell 2 Es/Noc: 2.66dB SINR_31 to SINR_49 SINR_30 to SINR_50 for extreme conditions Test 3: Noc: -116+ΔBG offset dBm/15kHz Cell 1 Es/Noc: -3.55dB Cell 2 Es/Noc: -3.55dB SINR_28 to SINR_44 SINR_27 to SINR_45 for extreme conditions |
| 19.6.1.1.1 NR SA FR1 SS-RSRP absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.1.1.2 NR SA FR1 SS-RSRP relative measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.1.2.1 NR SA FR1-FR1 SS-RSRP absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.1.2.2 NR SA FR1-FR1 SS-RSRP relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.2.1 NR SA FR1 SS-RSRQ absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.2.2.1 NR SA FR1-FR1 SS-RSRQ absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.2.2.2 NR SA FR1-FR1 SS-RSRQ relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.3.1 NR SA FR1 SS-SINR absolute measurement accuracy for ATG | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 | Same as 6.7.1.1.1 |
| 19.6.3.2.1 NR SA FR1-FR1 SS-SINR absolute measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.3.2.2 NR SA FR1-FR1 SS-SINR relative measurement accuracy for ATG | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 | Same as 6.7.1.2.1 |
| 19.6.4.1.1 NR SA FR1 SSB based L1-RSRP absolute measurement accuracy for ATG | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 19.6.4.1.2 NR SA FR1 SSB based L1-RSRP relative measurement accuracy for ATG | Same as 6.7.4.1.2 | Same as 6.7.4.1.2 | Same as 6.7.4.1.2 |
| 19.6.4.2.1 NR SA FR1 CSI-RS based L1-RSRP absolute measurement accuracy for ATG | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 | Same as 6.7.4.1.1 |
| 19.6.4.2.2 NR SA FR1 CSI-RS based L1-RSRP relative measurement accuracy for ATG | Same as 6.7.4.1.2 | Same as 6.7.4.1.2 | Same as 6.7.4.1.2 |
| 19.6.5.1.1 NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG | Same as 4.7.7.1.1 | Same as 4.7.7.1.1 | Same as 4.7.7.1.1 |
| 19.6.5.1.2 NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and no dedicated IMR for ATG | Same as 4.7.7.1.2 | Same as 4.7.7.1.2 | Same as 4.7.7.1.2 |
| 19.6.5.2 NR SA FR1 L1-SINR absolute measurement accuracy with SSB based CMR and dedicated IMR for ATG | Same as 4.7.7.2 | Same as 4.7.7.2 | Same as 4.7.7.2 |
| 19.6.5.3.1 NR SA FR1 L1-SINR absolute measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG | Same as 4.7.7.3.1 | Same as 4.7.7.3.1 | Same as 4.7.7.3.1 |
| 19.6.5.3.2 NR SA FR1 L1-SINR relative measurement accuracy with CSI-RS based CMR and dedicated IMR for ATG | Same as 4.7.7.1.2 | Same as 4.7.7.1.2 | Same as 4.7.7.1.2 |

<End of Change 1>