3GPP TSG-RAN5 Meeting #109	R5-256209

DALLAS, USA, Nov 17th – 21st 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-5 | CR | 0115 | rev | - | Current version: | 18.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to fixed reference channels for SCS 30kHz FR1-NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The description of slot allocation for frames in fixed reference channels for SCS 30kHz FR1-NTN was wrong. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The slot allocation for frames in fixed reference channel for receiver requirements in Table A.3.4.1.1A-1, Table A.3.4.1.1A-2, Table A.3.4.1.1A-3 Table A.3.4.1.1A-4 are updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The fixed reference channels for SCS 30kHz FR1-NTN cannot be correctly referred to. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.4.1.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

A.3.4.1.1A	Fixed reference channels for SCS 30kHz FR1-NTN

In addition to general description, no user data is scheduled on slot #1 within 20 ms in order to avoid SIB and PDSCH transmissions in one slot and simplify test configuration.

Table A.3.4.1.1A-1: Fixed reference channel for receiver requirements (SCS 30kHz, FDD, QPSK 1/3, NGSO)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 15 | 20 |  |  |  |  |
| Subcarrier spacing configuration |  | 1 | 1 | 1 |  |  |  |  |
| Allocated resource blocks |  | 24 | 38 | 51 |  |  |  |  |
| Subcarriers per resource block |  | 12 | 12 | 12 |  |  |  |  |
| Allocated slots per 2 Frames |  | 16 | 16 | 16 |  |  |  |  |
| MCS Index |  | 4 | 4 | 4 |  |  |  |  |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK |  |  |  |  |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 |  |  |  |  |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3, 20,…,39 | Bits | N/A | N/A | N/A |  |  |  |  |
| For Slots 4,…,19 | Bits | 1608 | 2472 | 3368 |  |  |  |  |
| Transport block CRC | Bits | 16 | 16 | 16 |  |  |  |  |
| LDPC base graph |  | 2 | 2 | 2 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3, 20,…,39 | CBs | N/A | N/A | N/A |  |  |  |  |
| For Slots 4,…,19 | CBs | 1 | 1 | 1 |  |  |  |  |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3, 20,…,39 | Bits | N/A | N/A | N/A |  |  |  |  |
| For Slots 4,…,19 | Bits | 5184 | 8208 | 11016 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 1.286 | 1.978 | 2.694 |  |  |  |  |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1 from TS 38.101-1 [5]. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per 2 frames |  |  |  |  |  |  |  |  |

Table A.3.4.1.1A-2: Fixed reference channel for receiver requirements (SCS 30kHz, FDD, 64QAM, NGSO)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 15 | 20 |  |  |  |  |
| Subcarrier spacing configuration |  | 1 | 1 | 1 |  |  |  |  |
| Allocated resource blocks |  | 24 | 38 | 51 |  |  |  |  |
| Subcarriers per resource block |  | 12 | 12 | 12 |  |  |  |  |
| Allocated slots per 2 Frames |  | 16 | 16 | 16 |  |  |  |  |
| MCS Index |  | 24 | 24 | 24 |  |  |  |  |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |
| Modulation |  | 64 QAM | 64 QAM | 64 QAM |  |  |  |  |
| Target Coding Rate |  | 3/4 | 3/4 | 3/4 |  |  |  |  |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3, 20,…,39 | Bits | N/A | N/A | N/A |  |  |  |  |
| For Slots 4,…,19 | Bits | 11784 | 18432 | 25104 |  |  |  |  |
| Transport block CRC | Bits | 24 | 24 | 24 |  |  |  |  |
| LDPC base graph |  | 1 | 1 | 1 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3, 20,…,39 | CBs | N/A | N/A | N/A |  |  |  |  |
| For Slots 4,…,19 | CBs | 2 | 3 | 3 |  |  |  |  |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3, 20,…,39 | Bits | N/A | N/A | N/A |  |  |  |  |
| For Slots 4,…,19 | Bits | 15552 | 24624 | 33048 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 9.427 | 14.746 | 20.083 |  |  |  |  |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1 from TS 38.101-1 [5]. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per 2 frames |  |  |  |  |  |  |  |  |

Table A.3.4.1.1A-3: Fixed reference channel for receiver requirements (SCS 30kHz, FDD, QPSK 1/3, GSO)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 15 | 20 |  |  |  |  |
| Subcarrier spacing configuration |  | 1 | 1 | 1 |  |  |  |  |
| Allocated resource blocks |  | 24 | 38 | 51 |  |  |  |  |
| Subcarriers per resource block |  | 12 | 12 | 12 |  |  |  |  |
| Allocated slots per 32 Frames |  | 16 | 16 | 16 |  |  |  |  |
| MCS Index |  | 4 | 4 | 4 |  |  |  |  |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK |  |  |  |  |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 |  |  |  |  |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3, 20,…,639 | Bits | N/A | N/A | N/A |  |  |  |  |
| For Slots 4,…,19 | Bits | 1608 | 2472 | 3368 |  |  |  |  |
| Transport block CRC | Bits | 16 | 16 | 16 |  |  |  |  |
| LDPC base graph |  | 2 | 2 | 2 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3, 20,…,639 | CBs | N/A | N/A | N/A |  |  |  |  |
| For Slots 4,…,19 | CBs | 1 | 1 | 1 |  |  |  |  |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3, 20,…,639 | Bits | N/A | N/A | N/A |  |  |  |  |
| For Slots 4,…,19 | Bits | 5184 | 8208 | 11016 |  |  |  |  |
| Max. Throughput averaged over 32 frames | Mbps | 0.080 | 0.124 | 0.168 |  |  |  |  |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1 from TS 38.101-1 [5]. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per 32 frames |  |  |  |  |  |  |  |  |

Table A.3.4.1.1A-4: Fixed reference channel for receiver requirements (SCS 30kHz, FDD, 64QAM, GSO)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 15 | 20 |  |  |  |  |
| Subcarrier spacing configuration |  | 1 | 1 | 1 |  |  |  |  |
| Allocated resource blocks |  | 24 | 38 | 51 |  |  |  |  |
| Subcarriers per resource block |  | 12 | 12 | 12 |  |  |  |  |
| Allocated slots per 32 Frames |  | 16 | 16 | 16 |  |  |  |  |
| MCS Index |  | 24 | 24 | 24 |  |  |  |  |
| MCS Table for TBS determination |  | 64QAM |  |  |  |  |  |  |
| Modulation |  | 64 QAM | 64 QAM | 64 QAM |  |  |  |  |
| Target Coding Rate |  | 3/4 | 3/4 | 3/4 |  |  |  |  |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3, 20,…,639 | Bits | N/A | N/A | N/A |  |  |  |  |
| For Slots 4,…,19 | Bits | 11784 | 18432 | 25104 |  |  |  |  |
| Transport block CRC | Bits | 24 | 24 | 24 |  |  |  |  |
| LDPC base graph |  | 1 | 1 | 1 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3, 20,…,639 | CBs | N/A | N/A | N/A |  |  |  |  |
| For Slots 4,…,19 | CBs | 2 | 3 | 3 |  |  |  |  |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |
| For Slots 0,1,2,3, 20,…,639 | Bits | N/A | N/A | N/A |  |  |  |  |
| For Slots 4,…,19 | Bits | 15552 | 24624 | 33048 |  |  |  |  |
| Max. Throughput averaged over 32 frames | Mbps | 0.589 | 0.922 | 1.255 |  |  |  |  |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1 from TS 38.101-1 [5]. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per 32 frames |  |  |  |  |  |  |  |  |

==============End of change==============