3GPP TSG-RAN5 Meeting #109	R5-256226

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1209 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to amend missing info of 6.2D.2 for R17 HST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The MPR test case defined for UL MIMO case under PC6. But the test requirements is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To add test requirements for 6.2D.2 for HST PC6. To amend the previous agreements to 6.2D.2 for HST PC6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The text remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<Start of change>>>

6.2D.2	UE maximum output power reduction for UL MIMO

Editor's note:	The following aspects are either missing or not yet determined:

-	Measurement Uncertainties and Test Tolerances are FFS for PC2 and PC4.

-	The test requirement for PCs other than PC3, PC5 and PC6 is FFS.

<<<Skipped unchanged parts>>>

6.2D.2.5	Test requirements

The maximum output power, derived in step 5 shall be within the range prescribed by the nominal maximum output power and tolerance in following tables.

Table 6.2D.2.5-1: UE Power Class test requirements for Power Class 1 (for Bands n257, n258, n261)

FFS

Table 6.2D.2.5-2: UE Power Class test requirements for Power Class 1 (for Bands n260)

FFS

Table 6.2D.2.5-3: UE Power Class test requirements for Power Class 2 (n257, 258, 261)

FFS

Table 6.2.2D.5-4: UE Power Class test requirements for Power Class 3 (n257, 258, 261)

| Test Configuration Table | Test ID | BW (MHz) | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Table 6.2D.2.4.1-4 | 1 | <=200MHz | 22.4 | 4.0 | 3 | 15.4-TT | 43 |
|  |  | 400MHz | 22.4 | 5.0 | 4 | 13.4-TT | 43 |
|  | 2 | <=200MHz | 22.4 | 4.0 | 3 | 15.4-TT | 43 |
|  |  | 400MHz | 22.4 | 5.0 | 4 | 13.4-TT | 43 |
| Table 6.2D.2.4.1-5 | 1 | <=200MHz | 22.4 | 3.5 | 3 | 15.9-TT | 43 |
|  | 2 | <=200MHz | 22.4 | 4.0 | 3 | 15.4-TT | 43 |
|  | 3 | <=200MHz | 22.4 | 4.0 | 3 | 15.4-TT | 43 |
|  | 4 | <=200MHz | 22.4 | 4.0 | 3 | 15.4-TT | 43 |
|  | 5 | <=200MHz | 22.4 | 5.0 | 4 | 13.4-TT | 43 |
|  | 6 | <=200MHz | 22.4 | 5.0 | 4 | 13.4-TT | 43 |
|  | 7 | <=200MHz | 22.4 | 5.0 | 4 | 13.4-TT | 43 |
|  | 8 | <=200MHz | 22.4 | 5.0 | 4 | 13.4-TT | 43 |
|  | 9 | <=200MHz | 22.4 | 7.5 | 5 | 11.9-TT | 43 |
|  | 10 | <=200MHz | 22.4 | 7.5 | 5 | 11.9-TT | 43 |
|  | 11 | <=200MHz | 22.4 | 7.5 | 5 | 11.9-TT | 43 |
|  | 12 | <=200MHz | 22.4 | 7.5 | 5 | 11.9-TT | 43 |
| Table 6.2D.2.4.1-6 | 1 | 400MHz | 22.4 | 5 | 4 | 13.4-TT | 43 |
|  | 2 | 400MHz | 22.4 | 5 | 4 | 13.4-TT | 43 |
|  | 3 | 400MHz | 22.4 | 5 | 4 | 13.4-TT | 43 |
|  | 4 | 400MHz | 22.4 | 6.5 | 5 | 10.9-TT | 43 |
|  | 5 | 400MHz | 22.4 | 6.5 | 5 | 10.9-TT | 43 |
|  | 6 | 400MHz | 22.4 | 6.5 | 5 | 10.9-TT | 43 |
|  | 7 | 400MHz | 22.4 | 9 | 5 | 8.4-TT | 43 |
|  | 8 | 400MHz | 22.4 | 9 | 5 | 8.4-TT | 43 |
|  | 9 | 400MHz | 22.4 | 9 | 5 | 8.4-TT | 43 |
| Note 1: Void. Note 2: Void. Note 3: Void. Note 4: Void. |  |  |  |  |  |  |  |

Table 6.2D.2.5-5: UE Power Class test requirements for Power Class 3 (n260)

| Test Configuration Table | Test ID | BW (MHz) | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Table 6.2D.2.4.1-4 | 1 | <=200MHz | 20.6 | 4.0 | 3 | 13.6-TT | 43 |
|  |  | 400MHz | 20.6 | 5.0 | 4 | 11.6-TT | 43 |
|  | 2 | <=200MHz | 20.6 | 4.0 | 3 | 13.6-TT | 43 |
|  |  | 400MHz | 20.6 | 5.0 | 4 | 11.6-TT | 43 |
| Table 6.2D.2.4.1-5 | 1 | <=200MHz | 20.6 | 3.5 | 3 | 14.1-TT | 43 |
|  | 2 | <=200MHz | 20.6 | 4.0 | 3 | 13.6-TT | 43 |
|  | 3 | <=200MHz | 20.6 | 4.0 | 3 | 13.6-TT | 43 |
|  | 4 | <=200MHz | 20.6 | 4.0 | 3 | 13.6-TT | 43 |
|  | 5 | <=200MHz | 20.6 | 5.0 | 4 | 11.6-TT | 43 |
|  | 6 | <=200MHz | 20.6 | 5.0 | 4 | 11.6-TT | 43 |
|  | 7 | <=200MHz | 20.6 | 5.0 | 4 | 11.6-TT | 43 |
|  | 8 | <=200MHz | 20.6 | 5.0 | 4 | 11.6-TT | 43 |
|  | 9 | <=200MHz | 20.6 | 7.5 | 5 | 8.1-TT | 43 |
|  | 10 | <=200MHz | 20.6 | 7.5 | 5 | 8.1-TT | 43 |
|  | 11 | <=200MHz | 20.6 | 7.5 | 5 | 8.1-TT | 43 |
|  | 12 | <=200MHz | 20.6 | 7.5 | 5 | 8.1-TT | 43 |
| Table 6.2D.2.4.1-6 | 1 | 400MHz | 20.6 | 5 | 4 | 11.6-TT | 43 |
|  | 2 | 400MHz | 20.6 | 5 | 4 | 11.6-TT | 43 |
|  | 3 | 400MHz | 20.6 | 5 | 4 | 11.6-TT | 43 |
|  | 4 | 400MHz | 20.6 | 6.5 | 5 | 9.1-TT | 43 |
|  | 5 | 400MHz | 20.6 | 6.5 | 5 | 9.1-TT | 43 |
|  | 6 | 400MHz | 20.6 | 6.5 | 5 | 9.1-TT | 43 |
|  | 7 | 400MHz | 20.6 | 9 | 5 | 6.6-TT | 43 |
|  | 8 | 400MHz | 20.6 | 9 | 5 | 6.6-TT | 43 |
|  | 9 | 400MHz | 20.6 | 9 | 5 | 6.6-TT | 43 |
| Note 1: Void. Note 2: Void. Note 3: Void. Note 4: Void. |  |  |  |  |  |  |  |

Table 6.2D.2.5-5a: Test Tolerance (Power class 3)

| Test Metric | FR2a | FR2b | FR2c |
| --- | --- | --- | --- |
| Max device size ≤ 30 cm | 3.24 dB, NTC 3.41 dB, ETC | 3.24 dB, NTC 3.41 dB, ETC | TBD, NTC TBD, ETC |

Table 6.2D.2.5-6: UE Power Class test requirements for Power Class 4 (for Bands n257, n258, n261)

FFS

Table 6.2D.2.5-7: UE Power Class test requirements for Power Class 4 (for Bands n260)

FFS

Table 6.2D.2.5-8: UE Power Class test requirements for Power Class 5 (n257)

| Test Configuration Table | Test ID | BW (MHz) | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Table 6.2D.2.4.1-4 | 1 | <=200MHz | 30 | 4.0 | 3 | 23-TT | 43 |
|  |  | 400MHz | 30 | 5.0 | 4 | 21-TT | 43 |
|  | 2 | <=200MHz | 30 | 4.0 | 3 | 23-TT | 43 |
|  |  | 400MHz | 30 | 5.0 | 4 | 21-TT | 43 |
| Table 6.2D.2.4.1-5 | 1 | <=200MHz | 30 | 3.5 | 3 | 23.5-TT | 43 |
|  | 2 | <=200MHz | 30 | 4.0 | 3 | 23-TT | 43 |
|  | 3 | <=200MHz | 30 | 4.0 | 3 | 23-TT | 43 |
|  | 4 | <=200MHz | 30 | 4.0 | 3 | 23-TT | 43 |
|  | 5 | <=200MHz | 30 | 5.0 | 4 | 21-TT | 43 |
|  | 6 | <=200MHz | 30 | 5.0 | 4 | 21-TT | 43 |
|  | 7 | <=200MHz | 30 | 5.0 | 4 | 21-TT | 43 |
|  | 8 | <=200MHz | 30 | 5.0 | 4 | 21-TT | 43 |
|  | 9 | <=200MHz | 30 | 7.5 | 5 | 17.5-TT | 43 |
|  | 10 | <=200MHz | 30 | 7.5 | 5 | 17.5-TT | 43 |
|  | 11 | <=200MHz | 30 | 7.5 | 5 | 17.5-TT | 43 |
|  | 12 | <=200MHz | 30 | 7.5 | 5 | 17.5-TT | 43 |
| Table 6.2D.2.4.1-6 | 1 | 400MHz | 30 | 5 | 4 | 21-TT | 43 |
|  | 2 | 400MHz | 30 | 5 | 4 | 21-TT | 43 |
|  | 3 | 400MHz | 30 | 5 | 4 | 21-TT | 43 |
|  | 4 | 400MHz | 30 | 6.5 | 5 | 18.5-TT | 43 |
|  | 5 | 400MHz | 30 | 6.5 | 5 | 18.5-TT | 43 |
|  | 6 | 400MHz | 30 | 6.5 | 5 | 18.5-TT | 43 |
|  | 7 | 400MHz | 30 | 9 | 5 | 16-TT | 43 |
|  | 8 | 400MHz | 30 | 9 | 5 | 16-TT | 43 |
|  | 9 | 400MHz | 30 | 9 | 5 | 16-TT | 43 |

Table 6.2D.2.5-8a: UE Power Class test requirements for Power Class 5 (n258)

| Test Configuration Table | Test ID | BW (MHz) | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Table 6.2D.2.4.1-4 | 1 | <=200MHz | 30.4 | 4.0 | 3 | 23.4-TT | 43 |
|  |  | 400MHz | 30.4 | 5.0 | 4 | 21.4-TT | 43 |
|  | 2 | <=200MHz | 30.4 | 4.0 | 3 | 23.4-TT | 43 |
|  |  | 400MHz | 30.4 | 5.0 | 4 | 21.4-TT | 43 |
| Table 6.2D.2.4.1-5 | 1 | <=200MHz | 30.4 | 3.5 | 3 | 23.9-TT | 43 |
|  | 2 | <=200MHz | 30.4 | 4.0 | 3 | 23.4-TT | 43 |
|  | 3 | <=200MHz | 30.4 | 4.0 | 3 | 23.4-TT | 43 |
|  | 4 | <=200MHz | 30.4 | 4.0 | 3 | 23.4-TT | 43 |
|  | 5 | <=200MHz | 30.4 | 5.0 | 4 | 21.4-TT | 43 |
|  | 6 | <=200MHz | 30.4 | 5.0 | 4 | 21.4-TT | 43 |
|  | 7 | <=200MHz | 30.4 | 5.0 | 4 | 21.4-TT | 43 |
|  | 8 | <=200MHz | 30.4 | 5.0 | 4 | 21.4-TT | 43 |
|  | 9 | <=200MHz | 30.4 | 7.5 | 5 | 17.9-TT | 43 |
|  | 10 | <=200MHz | 30.4 | 7.5 | 5 | 17.9-TT | 43 |
|  | 11 | <=200MHz | 30.4 | 7.5 | 5 | 17.9-TT | 43 |
|  | 12 | <=200MHz | 30.4 | 7.5 | 5 | 17.9-TT | 43 |
| Table 6.2D.2.4.1-6 | 1 | 400MHz | 30.4 | 5 | 4 | 21.4-TT | 43 |
|  | 2 | 400MHz | 30.4 | 5 | 4 | 21.4-TT | 43 |
|  | 3 | 400MHz | 30.4 | 5 | 4 | 21.4-TT | 43 |
|  | 4 | 400MHz | 30.4 | 6.5 | 5 | 18.9-TT | 43 |
|  | 5 | 400MHz | 30.4 | 6.5 | 5 | 18.9-TT | 43 |
|  | 6 | 400MHz | 30.4 | 6.5 | 5 | 18.9-TT | 43 |
|  | 7 | 400MHz | 30.4 | 9 | 5 | 16.4-TT | 43 |
|  | 8 | 400MHz | 30.4 | 9 | 5 | 16.4-TT | 43 |
|  | 9 | 400MHz | 30.4 | 9 | 5 | 16.4-TT | 43 |

Table 6.2D.2.5-8b: Test Tolerance (Power class 5)

| Test Metric | FR2a |
| --- | --- |
| Max device size ≤ 30 cm | 3.38 dB, NTC 3.56 dB, ETC |

Table 6.2D.2.5-9: UE Power Class test requirements for Power Class 6 (for Bands n257, n261)

| Test Configuration Table | Test ID | BW (MHz) | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Table 6.2D.2.4.1-4 | 1 | <=200MHz | 30 | 4.0 | 3 | 23-TT | 43 |
|  |  | 400MHz | 30 | 5.0 | 4 | 21-TT | 43 |
|  | 2 | <=200MHz | 30 | 4.0 | 3 | 23-TT | 43 |
|  |  | 400MHz | 30 | 5.0 | 4 | 21-TT | 43 |
| Table 6.2D.2.4.1-5 | 1 | <=200MHz | 30 | 3.5 | 3 | 23.5-TT | 43 |
|  | 2 | <=200MHz | 30 | 4.0 | 3 | 23-TT | 43 |
|  | 3 | <=200MHz | 30 | 4.0 | 3 | 23-TT | 43 |
|  | 4 | <=200MHz | 30 | 4.0 | 3 | 23-TT | 43 |
|  | 5 | <=200MHz | 30 | 5.0 | 4 | 21-TT | 43 |
|  | 6 | <=200MHz | 30 | 5.0 | 4 | 21-TT | 43 |
|  | 7 | <=200MHz | 30 | 5.0 | 4 | 21-TT | 43 |
|  | 8 | <=200MHz | 30 | 5.0 | 4 | 21-TT | 43 |
|  | 9 | <=200MHz | 30 | 7.5 | 5 | 17.5-TT | 43 |
|  | 10 | <=200MHz | 30 | 7.5 | 5 | 17.5-TT | 43 |
|  | 11 | <=200MHz | 30 | 7.5 | 5 | 17.5-TT | 43 |
|  | 12 | <=200MHz | 30 | 7.5 | 5 | 17.5-TT | 43 |
| Table 6.2D.2.4.1-6 | 1 | 400MHz | 30 | 5 | 4 | 21-TT | 43 |
|  | 2 | 400MHz | 30 | 5 | 4 | 21-TT | 43 |
|  | 3 | 400MHz | 30 | 5 | 4 | 21-TT | 43 |
|  | 4 | 400MHz | 30 | 6.5 | 5 | 18.5-TT | 43 |
|  | 5 | 400MHz | 30 | 6.5 | 5 | 18.5-TT | 43 |
|  | 6 | 400MHz | 30 | 6.5 | 5 | 18.5-TT | 43 |
|  | 7 | 400MHz | 30 | 9 | 5 | 16-TT | 43 |
|  | 8 | 400MHz | 30 | 9 | 5 | 16-TT | 43 |
|  | 9 | 400MHz | 30 | 9 | 5 | 16-TT | 43 |

Table 6.2D.2.5-9a: UE Power Class test requirements for Power Class 6 (n258)

| Test Configuration Table | Test ID | BW (MHz) | PPowerclass | MPRf,c | T(MPRf,c) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Table 6.2D.2.4.1-4 | 1 | <=200MHz | 30.4 | 4.0 | 3 | 23.4-TT | 43 |
|  |  | 400MHz | 30.4 | 5.0 | 4 | 21.4-TT | 43 |
|  | 2 | <=200MHz | 30.4 | 4.0 | 3 | 23.4-TT | 43 |
|  |  | 400MHz | 30.4 | 5.0 | 4 | 21.4-TT | 43 |
| Table 6.2D.2.4.1-5 | 1 | <=200MHz | 30.4 | 3.5 | 3 | 23.9-TT | 43 |
|  | 2 | <=200MHz | 30.4 | 4.0 | 3 | 23.4-TT | 43 |
|  | 3 | <=200MHz | 30.4 | 4.0 | 3 | 23.4-TT | 43 |
|  | 4 | <=200MHz | 30.4 | 4.0 | 3 | 23.4-TT | 43 |
|  | 5 | <=200MHz | 30.4 | 5.0 | 4 | 21.4-TT | 43 |
|  | 6 | <=200MHz | 30.4 | 5.0 | 4 | 21.4-TT | 43 |
|  | 7 | <=200MHz | 30.4 | 5.0 | 4 | 21.4-TT | 43 |
|  | 8 | <=200MHz | 30.4 | 5.0 | 4 | 21.4-TT | 43 |
|  | 9 | <=200MHz | 30.4 | 7.5 | 5 | 17.9-TT | 43 |
|  | 10 | <=200MHz | 30.4 | 7.5 | 5 | 17.9-TT | 43 |
|  | 11 | <=200MHz | 30.4 | 7.5 | 5 | 17.9-TT | 43 |
|  | 12 | <=200MHz | 30.4 | 7.5 | 5 | 17.9-TT | 43 |
| Table 6.2D.2.4.1-6 | 1 | 400MHz | 30.4 | 5 | 4 | 21.4-TT | 43 |
|  | 2 | 400MHz | 30.4 | 5 | 4 | 21.4-TT | 43 |
|  | 3 | 400MHz | 30.4 | 5 | 4 | 21.4-TT | 43 |
|  | 4 | 400MHz | 30.4 | 6.5 | 5 | 18.9-TT | 43 |
|  | 5 | 400MHz | 30.4 | 6.5 | 5 | 18.9-TT | 43 |
|  | 6 | 400MHz | 30.4 | 6.5 | 5 | 18.9-TT | 43 |
|  | 7 | 400MHz | 30.4 | 9 | 5 | 16.4-TT | 43 |
|  | 8 | 400MHz | 30.4 | 9 | 5 | 16.4-TT | 43 |
|  | 9 | 400MHz | 30.4 | 9 | 5 | 16.4-TT | 43 |

Table 6.2D.2.5-9b: Test Tolerance (Power class 6)

| Test Metric | FR2a |
| --- | --- |
| Max device size ≤ 30 cm | 3.37 dB |

<<<End of change>>>